| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G333 |
| --- | --- |

| Title: | Accuracy improvement of HM's transform bases |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yasuko Sugito Atsuro Ichigaya Shinichi Sakaida NHK Science & Technology Research Laboratories 1-10-11, Kinuta, Setagaya-ku, Tokyo, JAPAN | Tel: Email: | +81-3-5494-3353 sugitou.y-gy@nhk.or.jp |
| Source: | NHK (Japan Broadcasting Corporation) |  |  |

_____________________________

# Abstract

This contribution proposes high accuracy 16x16 and 32x32 transform bases based on HM's transform proposed by Cisco and TI[1]. The word "accuracy" means orthogonality and isometry (normality of transform) of basis vectors. To derive transform bases, two options are used. Option 1 is the same as JCTVC-F193[2]. Option 2 has feature that value range of elements is wider than Option 1 and waveform of basis is made consideration. Proposed bases are verified according to CE10's coding efficiency[3] of normal QP range and low QP range. The results for normal QP range (QP=22, 27, 32, 37) show around 0% of average gains for luma. Almost all of the results of sequences for low QP range (QP=1, 5, 9, 13) show gains for luma, peak gain is about 1.3% without any process time increasing.

# Introduction

The core transform of current HM[1] is improved the balance between orthogonality and normality than HM 2.0. The transform is designed based on DCT and its features are similar to DCT's.

1. Basis vectors are almost orthogonal.

2. Norms of basis vectors are almost same.

Redesigning bases, we focused attention on orthogonality of basis vectors as well as their norms. To improve above features 1 and 2, we introduced the cost function for calculating elements of bases.

# Description

We redesigned 4x4 to 32x32 transform bases. As a result, 16x16 and 32x32 transform bases are different from HM's, while 4x4 and 8x8 transform bases are the same as HM's.

## Option 1

Option 1 is the same as JCTVC-F193[2]. Hereinafter, we take the derivation of 16x16 transform basis for example.

Figure 1 shows the matrix product of HM's 16x16 transform basis and its transpose. The element of the matrix in the i th row and j th column represents the inner product of the i th basis vector and the j th basis vector. If there is the ideal 16x16 transform basis based on HM's transform, the matrix product of it and its transpose will be the result of multiplying identity matrix by 65536 (= 642x16), as indicated by Figure 2.

Figure 1: Matrix product of HM's 16x16 transform basis and its transpose

Figure 2: Matrix product of the ideal 16x16 transform basis and its transpose

Figure 3: Matrix product of the proposed basis and its transpose (Option 1 and 2)

In view of the difference between Figure 1's matrix and Figure 2's matrix, we defined the cost function as below.

C=ai|(xi,xi)−N|+i,j|(xi,xj)|.    (1)

(a>0, xi represents the i th basis vector, i,j=1,2,3,...,16, i≠j, Nis the ideal size of norm: 65536)

In formula (1), the variable Crepresents cost and would be 0 if xis are the ideal 16x16 transform basis vectors. We would like to obtain xis which minimize C at any given range.

Here, the 16x16 transform basis is obtained by the following conditions.

- Odd rows of 16x16 transform basis vectors are already determined by 8x8 transform basis vectors.

- Basis vectors have symmetry as DCT's basis vectors.

- Correspondence relations of elements at even rows of basis vectors are determined in common with DCT.

Therefore, we can rewrite formula (1) as below.

C'=16|(x2,x2)−65536|+j=even,414|(x2,xj)|.    (2)

In formula (2), 16 indicates size of basis. In consideration of quantization, we focused on isometry of basis vectors than orthogonality of them.

As we obtained minimum ofC', we first calculated 256 times the 2nd 16x16 DCT basis vector and rounded it to integer. The vector obtained is r2=(90,87,80,70,57,43,26,9,−9,−26,−43,−57,−70,−80,−87,−90).

Secondly, in formula (2), by setting elements of x2 in the range of r2±1 which is same as HM's transform, we calculated basis vectors x4,x6,x8,...,x14 corresponding to x2and minimum of C'. The basis vector x2 which minimize C' is x2=(91,87,79,70,56,43,27,8,−8,−27,−43,−56,−70,−79,−87,−91). To maintain the shape of a cosine wave, we selected elements of x2in descending order.

The proposed 16x16 transform basis is shown in Figure 4. Colored elements show difference from current HM's.

Figure 3 shows the matrix product of the proposed 16x16 transform basis and its transpose, colored hatching means improved elements. Figure 3's matrix is closer to Figure 2's matrix than Figure 1's matrix. This means accuracy of the proposed basis is improved than HM's.

The proposed 32x32 transform basis was obtained by the same method as the proposed 16x16 transform basis. The proposed 32x32 transform basis is shown in Figure 5. Colored elements show difference from current HM's.

Figure 4: Proposed 16x16 transform basis (Option 1 and 2)

Figure 5: Proposed 32x32 transform basis (Option 1)

## Option 2

Option 2 has feature that the value range ofx2's elements is wider than Option 1 and waveform of basis is made consideration.

In Option 2, the cost function described in formula(1) and (2) is also used, but the value range of x2's elements is set as r2±2. And the basis vector x2 which minimize possible C' and whose waveform is as close to DCT (a  cosine wave) is derived. By calculating Option 2, the proposed 16x16 transform basis is the same as Option1 (Figure 4), and the proposed 32x32 transform basis is shown in Figure 6. Colored elements show difference from current HM's.

Figure 7 and 8 are waveforms of the 2nd basis vector of the proposed 32x32 transform basis for Option 1 and 2, respectively. The waveform in Figure 8 (Option 2) is closer to scaled DCT than Figure 7 (Option 1).

Figure 6: Proposed 32x32 transform basis (Option 2)

Figure 7: Graph of the proposed 32x32 transform basis (Option 1) and scaled 32x32 DCT basis

Figure 8: Graph of the proposed 32x32 transform basis (Option 2) and scaled 32x32 DCT basis

# Coding results

We verified proposed bases according to conditions of CE10's coding efficiency[3] of normal QP range and low QP range.

## Option 1

Table 1 shows BD-rate results for the proposed bases using HM 4.0 as the anchor for common conditions (QP=22, 27, 32, 37)[4]. Averages of BD-rate show some losses, but these are less than 0.1%.

Table 2 shows BD-rate results for low QP range (QP=1, 5, 9, 13) and 2 sec of video.

Table 3 shows BD-rate Y results by sequences for low QP range, colored hatching means gain. It shows accuracy improvement provides gains to almost all of sequences.

All of the results show approximately-same complexity as HM 4.0.

Table 1: BD-rate results for normal QP range (QP=22, 27, 32, 37), Option 1

Table 2: BD-rate results for low QP range (QP=1, 5, 9, 13), Option 1

Table 3: BD-rate Y results by sequences, low QP range (QP=1, 5, 9, 13), Option 1

## Option 2

Table 4 shows BD-rate results for the proposed bases using HM 4.0 as the anchor for common conditions (QP=22, 27, 32, 37)[4]. Averages of BD-rate show some losses, but these are less than 0.1%.

Table 5 shows BD-rate results for low QP range (QP=1, 5, 9, 13) and 2 sec of video.

Table 6 shows BD-rate Y results by sequences for low QP range, colored hatching means gain. It shows accuracy improvement provides gains to almost all of sequences.

All of the results show approximately-same complexity as HM 4.0.

Table 4: BD-rate results for normal QP range (QP=22, 27, 32, 37), Option 2

Table 5: BD-rate results for low QP range (QP=1, 5, 9, 13), Option 2

Table 6: BD-rate Y results by sequences, low QP range (QP=1, 5, 9, 13), Option 2

# Conclusions

We proposed high accuracy 16x16 and 32x32 transform bases based on current HM's transform[1]. We used two options to derive these bases and verified them according to CE10's coding efficiency[3] of normal QP range and low QP range.

All of the results show approximately-same complexity as HM 4.0. The results of low QP range showed accuracy improvement provides gains to almost all of sequences for luma. And the results of Option 2 show higher gains than Option 1's.

# References

[1] A. Fuldseth, G. Bjontegaard, M. Sadafale, M. Budagavi, "Transform design for HEVC with 16 bit intermediate data representation," JCTVC-E243, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Geneva, CH, March 2011

[2] Yasuko Sugito, Atsuro Ichigaya, Shinichi Sakaida, "Accuracy improvement of Cisco and TI's transform (JCTVC-E243)," JCTVC-F193, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Torino, IT, July 2011

[3]Pankaj Topiwala, Madhukar Budagavi, Arild Fuldseth, Rajan Joshi, Elena Alshina, "CE10: Core transform design," JCTVC-F910, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Torino, IT, July 2011

[4]Frank Bossen, "Common test conditions and software reference configurations," JCTVC-F900, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Torino, IT, July 2011

# Patent rights declaration(s)

NHK (Japan Broadcasting Corporation) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).