| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G335_r1 |
| --- | --- |

| Title: | Cross-check report on 7 bit transform matrices (JCTVC-G496) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Yasuko Sugito Atsuro Ichigaya NHK Science & Technology Research Laboratories 1-10-11, Kinuta, Setagaya-ku, Tokyo, JAPAN | Tel: Email: | +81-3-5494-3353 sugitou.y-gy@nhk.or.jp |
| Source: | NHK (Japan Broadcasting Corporation) |  |  |

_____________________________

# Abstract

This contribution reports cross-check results on 7 bit transform matrices proposed by Cisco and TI[1]. The verification task has been done successfully.

# Introduction

Since HM 3.0, the core transform using 8 bit transform matrices proposed by Cisco and TI[2] has been implemented in the HM software. This time Cisco and TI proposed the core transform using 7 bit transform matrices[1] and the transform structure is exactly the same properties as HM's transform. They integrated these matrices into HM 4.0 code.

In proposed matrices, 7 bit coefficients are not half of HM's 8bit coefficients, nor are they simply rounded from the DCT coefficients to integer. Possible to equal norms of basis vectors and improve normality, several coefficients are varied as with the 8 bit. For example, 4x4 transform matrices are given as follows.

HM's 8 bit 4x4 transform:

{64, 64, 64, 64}

{83, 36,-36,-83}

{64,-64,-64, 64}

{36,-83, 83,-36}

Proposed 7 bit 4x4 transform:

{32, 32, 32, 32}
		{41, 19,-19,-41}
		{32,-32,-32, 32}
		{19,-41, 41,-19}

7 bit 4x4 transform simply rounded from the DCT coefficients to integer:

{32, 32, 32, 32}
		{42, 17,-17,-42}
		{32,-32,-32, 32}
		{17,-42, 42,-17}

# Verification

According to CE 10's evaluation method[3], verification task was done for normal QP range (QP=22, 27, 32, 37), low QP range (QP=1, 5, 9, 13), and high QP range (QP=36, 42, 47, 51).

# Results

Table 1, 2, and 3 show BD-rate results for normal QP range (QP=22, 27, 32, 37), low QP range (QP=1, 5, 9, 13), and high QP range (QP=36, 42, 47, 51) respectively.

At the condition, Class C, PartyScene, RA_LC, QP=1, decoder crash occurs in both HM 4.0 and proponents' software. Therefore decoding time was calculated except the results of the condition. All of the results show approximately-same complexity as HM 4.0.

All results match exactly provided by proponents.

Table 1: BD-rate results for normal QP range (QP=22, 27, 32, 37)

Table 2: BD-rate results for low QP range (QP=1, 5, 9, 13)

Table 3: BD-rate results for high QP range (QP=36, 42, 47, 51)

# Conclusions

We have cross-checked 7 bit transform matrices proposed by Cisco and TI. Results match exactly provided by proponents. The verification task has been done successfully.

# References

[1] A. Fuldseth, G. Bjontegaard, M. Budagavi, V. Sze, "Core transform design for HEVC with 7 bit coefficients," JCTVC-G496, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Geneva, CH, November, 2011

[2] A. Fuldseth, G. Bjontegaard, M. Sadafale, M. Budagavi, "Transform design for HEVC with 16 bit intermediate data representation," JCTVC-E243, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Geneva, CH, March, 2011

[3] Pankaj Topiwala, Madhukar Budagavi, Arild Fuldseth, Rajan Joshi, Elena Alshina, "CE10: Core transform design," JCTVC-F910, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Torino, IT, July, 2011