| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G337 |
| --- | --- |

| Title: | Non-CE8.c.7: Crosscheck of Single-source SAO and ALF virtual boundary processing (JCTVC-G212) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Tomohiro Ikai 1-9-2 Nakase, Mihama-ku, Chiba, JAPAN | Tel: Email: | +81 43 299 8526 ikai.tomohiro@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

# Abstract

This contribution provides cross-check results of the proposals on SAO and ALF virtual boundary processing (JCTVC-G212). It is confirmed that the bitrate and PNSR values completely match with those provided by the proponents. The cross checker didn’t observe any subjective artifacts related to the proposal such as virtual boundary line. Because this technique eliminates line buffers/data transfers and benefits the DF and SAO and ALF’s one pass processing, it is suggested to discuss this technique’s adoption.

# On methods

This proposal consists of the following methods.

- 9x9 filter shape

- Reference samples restrictions in alf filtering near Virtual Boundary (VB_ALF)

- Alf filter coefficients adaptation near Virtual Boundary to avoid subjective artifacts

- BA computation reference sample restriction near Virtual Boundary

- SAO class information reduction(VB_SAO_CLASS_STORE)

The proposed methods are implemented properly under the cross-checkers investigation. The cross-checker thinks this proposal is very valuable for eliminating line buffers. The following figure illustrate overall concept.

virtual boundary

referring pixels upper than VB is prohibited and coeffs may be changed to avoid subjective artifacts // 4 lines

no change for filtering // 56 lines (=64 - 8)

referring pixels lower than VB is prohibited and coeffs may be changed to avoid subjective artifacts // 4 lines

# Experimental result

Table 1: Performance of proposal compared to anchor (HM-4.0)

|  | All Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.2% | 0.3% | 0.2% |
| Class B | 0.0% | 0.5% | 0.3% |
| Class C | 0.1% | 0.1% | 0.3% |
| Class D | -0.1% | 0.2% | 0.3% |
| Class E | 0.1% | 1.0% | 0.8% |
| Class F |  |  |  |
| Overall | 0.0% | 0.4% | 0.3% |
|  | 0.0% | 0.4% | 0.3% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 101% |  |  |
|  | Random Access HE |  |  |
|  | Y | U | V |
| Class A | 0.1% | 0.6% | 0.6% |
| Class B | 0.0% | 0.9% | 0.6% |
| Class C | -0.1% | 0.0% | 0.2% |
| Class D | -0.6% | 0.1% | 0.2% |
| Class E |  |  |  |
| Class F |  |  |  |
| Overall | -0.1% | 0.4% | 0.4% |
|  | -0.1% | 0.4% | 0.4% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 101% |  |  |
|  | Low delay B HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | -0.2% | 0.6% | 0.3% |
| Class C | -0.3% | -0.1% | -0.1% |
| Class D | -0.8% | -0.1% | 0.1% |
| Class E | 0.0% | 0.6% | 0.7% |
| Class F |  |  |  |
| Overall | -0.3% | 0.2% | 0.2% |
|  | -0.3% | 0.2% | 0.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |

# Conclusion

This contribution provides cross-check results of the proposals on SAO and ALF virtual boundary processing (JCTVC-G212). It is confirmed that the bitrate and PNSR values completely match with those provided by the proponents. The cross checker didn’t observe any subjective artifacts related to the proposal such as virtual boundary line. Because this technique eliminates line buffers/data transfers and benefits the DF and SAO and ALF’s one pass processing, it is suggested to discuss this technique’s adoption.

# References

[1]  C.-M. Fu, C.-Y. Chen, C.-Y. Tsai, Y.-W. Huang, S. Lei, “CE8.c.2: Single-source SAO and ALF virtual boundary processing”, JCTVC-G212, JCT-VC 7th Meeting, Geneva, 2011