| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G0338 |
| --- | --- |

| Title: | Non-CE8.a: Crosscheck of Non-CE8 Subtest a: Harmonization of CE8a Tool 2(SDBA) and Tool 3(HV8) (JCTVC-G0656) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Tomohiro Ikai 1-9-2 Nakase, Mihama-ku, Chiba, JAPAN | Tel: Email: | +81 43 299 8526 ikai.tomohiro@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

# Abstract

This contribution provides cross-check results of Non-CE8 Subtest a: Harmonization of CE8a Tool 2 Shape-dependent BA (SDBA) and Tool 3 Block-based filter adaptation with up to 8 filters (HV8) (JCTVC-G656). It is confirmed that the bitrate and PNSR values completely match with those provided by the proponents. The evaluation technique’s purpose is to balance complexity and coding performance.

With smaller maximum filters, the technique’s bdrate reduction was reported 0.1% to 0.2% gain. The enc time and dec time was reported about the same as HM4.0.

# Methods

This proposal to be checked (JCTVC-G656) [1] consists of the following two methods.

- HV8: 8 filters(JCTVC-G647)[2]

- SDBA: Shape dependent BA(JCTVC-G0609[3])

The HM4.0 uses RA(Reagion base mode) and BA(block base mode) for classify picture into several classes.

HV8 only uses up to 8 filters by reducing the number of RA and BA classes from 16 to 8. This can reduce overall ALF’s encoding complexity which is useful in practical usage.

SDBA enhance BA mode’s performance by introducing shape dependent feature computation technique, whose computation is similar to HM4.0.

# Experimental result

Table 1: Performance of proposal compared to anchor (HM-4.0)

|  | All Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | -0.04% | -0.02% | -0.02% |
| Class B | -0.07% | -0.03% | -0.03% |
| Class C | -0.19% | -0.02% | -0.02% |
| Class D | -0.13% | 0.05% | 0.05% |
| Class E | -0.23% | -0.02% | -0.02% |
| Class F |  |  |  |
| Overall | -0.13% | -0.01% | -0.01% |
|  | -0.13% | -0.01% | -0.01% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 101% |  |  |
|  | Random Access HE |  |  |
|  | Y | U | V |
| Class A | -0.07% | -0.18% | -0.33% |
| Class B | -0.03% | -0.08% | -0.07% |
| Class C | -0.13% | -0.05% | -0.05% |
| Class D | -0.18% | 0.09% | -0.18% |
| Class E |  |  |  |
| Class F |  |  |  |
| Overall | -0.10% | -0.06% | -0.15% |
|  | -0.10% | -0.06% | -0.14% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay B HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.01% | -0.08% | -0.18% |
| Class C | -0.19% | -0.04% | -0.15% |
| Class D | -0.20% | 0.36% | 0.20% |
| Class E | -0.26% | 0.05% | 0.35% |
| Class F |  |  |  |
| Overall | -0.14% | 0.06% | 0.02% |
|  | -0.14% | 0.04% | -0.02% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay P HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.07% | -0.07% | -0.01% |
| Class C | -0.21% | -0.07% | -0.11% |
| Class D | -0.35% | -0.40% | -0.36% |
| Class E | -0.27% | -0.29% | 0.03% |
| Class F |  |  |  |
| Overall | -0.17% | -0.19% | -0.12% |
|  | -0.17% | -0.20% | -0.15% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |

# Conclusion

This contribution provides cross-check results of Non-CE8 Subtest a: Harmonization of CE8a Tool 2 Shape-dependent BA (SDBA) and Tool 3 Block-based filter adaptation with up to 8 filters (HV8) (JCTVC-G656). This proposal was confirmed that with smaller maximum filters, the technique’s bdrate reduction was reported 0.1% to 0.2% gain. The enc time and dec time was reported about the same as HM4.0.

# References

[1] P. Lai, F. C. A. Fernandes, I.-K. Kim, “Non-CE8 Subtest a: Harmonization of CE8a Tool 2 Shape-dependent BA (SDBA) and Tool 3 Block-based filter adaptation with up to 8 filters (HV8)”, JCTVC-G0656, JCT-VC 7th Meeting, Geneva, 2011

[2] P. Lai, F. C. A. Fernandes, I.-K. Kim, “CE8 Subtest a, Tool 3: Block-based filter adaptation with up to 8 filters (HV8)”, JCTVC-G0647, JCT-VC 7th Meeting, Geneva, 2011

[3] T. Yamakage, T. Watanabe, T. Chujoh, C.-Y. Chen, C.-M. Fu, C.-Y. Tsai, Y.-W. Huang, S. Lei, M. Karczewicz, I. S. Chong, T. Ikai, A. Segall, T. Yamamoto, “CE8.a.2: Directional feature calculation on subset of pixels”, JCTVC-G0609, JCT-VC 7th Meeting, Geneva, 2011