| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G342 |
| --- | --- |

| Title: | CE3: Cross-verification of Qualcomm’s adaptive motion vector resolution (JCTVC-G727) by Intel |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Yi-Jen Chiu 2200 Mission College Blvd. Santa Clara CA USA 95054 | Tel: Email: | 408-765-9494 yi-jen.chiu@intel.com |
|  | Wenhao Zhang 8F Raycom Infotech Park A, No. 2 KeXueYuan South Rd., Haidian District, Beijing, China, 100190 | Tel: Email: | 86-10-82171657 wenhao.zhang@intel.com |
|  | Lidong Xu 8F Raycom Infotech Park A, No. 2 KeXueYuan South Rd., Haidian District, Beijing, China, 100190 | Tel: Email: | 86-10-82171425 lidong.xu@intel.com |
|  | Yu Han 8F Raycom Infotech Park A, No. 2 KeXueYuan South Rd., Haidian District, Beijing, China, 100190 | Tel: Email: | 86-10-82171388 yu.han@intel.com |
| Source: | Intel Corp. |  |  |

_____________________________

# Abstract

This contribution provides the cross-check results of adaptive motion vector resolution proposed by Qualcomm in JCTVC-G727. The reference results data were provided by the proponent. The BD rate, operation counts and 1D&2D memory reduction perfectly match the reference results. The encoding and decoding time also follow the reference data.

# Introduction

Two HM 4.0 based software packages were obtained from proponent, one is “Quarter” version and the other is “Eighth” version. They were successfully compiled and tested under the common test conditions defined in JCTVC-F900. The “random access”, “low delay B” and “low delay P” cases were run for the test with the provided code.

# Experimental Results

The encoder was compiled in 64bit Linux OS and tested in a parallel computing system. The decoder was complied with Visual Studio 2008 and tested on a single server with following configurations: Windows Server 2003 Enterprise x64 Edition, Intel Xeon CPU E5335 @ 2.00GHz, 8GB RAM. Table 1-2 shows the summary results for JCTVC-G727 proposal as cross-checked by Intel, and detailed results are available in the attached excel sheets.

| Quarter | HE configuration |  |  | LC configuration |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | RA | LD | LD_P | RA | LD | LD_P |
| Y BD rate | -0.2% | -0.1% | 0.1% | -0.3% | -0.2% | 0.1% |
| U BD rate | -0.6% | -0.3% | 0.0% | -0.5% | 0.0% | 0.1% |
| V BD rate | -0.5% | -0.2% | 0.0% | -0.4% | -0.2% | 0.0% |
| Enc. time | 95% | 93% | 97% | 94% | 92% | 95% |
| Dec. time | 94% | 93% | 100% | 95% | 94% | 97% |
| Mults | -5% | -8% | -8% | -5% | -8% | -8% |
| Adds | -5% | -8% | -8% | -6% | -9% | -8% |
| Access | -3% | -5% | -5% | -3% | -5% | -5% |
| MemBand 2D Ave. | -1% | -2% | -2% | -1% | -2% | -2% |
| MemBand 2D Max. | 5% | 5% | 2% | 10% | 10% | 1% |
| MemBand 1D Ave. | -1% | -2% | -2% | -1% | -2% | -2% |
| MemBand 1D Max. | 5% | 5% | 2% | 9% | 9% | 1% |

Table 1: Cross-check Results of the “Quarter” version

| Eighth | HE configuration |  |  | LC configuration |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | RA | LD | LD_P | RA | LD | LD_P |
| Y BD rate | -0.3% | 0.1% | -2.9% | -0.4% | -0.2% | -2.5% |
| U BD rate | -0.5% | -0.2% | -2.5% | -1.1% | -1.4% | -2.9% |
| V BD rate | -0.6% | -0.6% | -2.7% | -1.1% | -1.8% | -3.2% |
| Enc. time | 116% | 118% | 115% | 119% | 119% | 117% |
| Dec. time | 97% | 98% | 102% | 97% | 95% | 101% |
| Mults | 18% | 12% | 17% | 18% | 13% | 17% |
| Adds | 18% | 12% | 17% | 18% | 13% | 17% |
| Access | 10% | 5% | 7% | 10% | 5% | 7% |
| MemBand 2D Ave. | 3% | 1% | 2% | 3% | 1% | 3% |
| MemBand 2D Max. | 10% | 12% | 9% | 11% | 6% | 12% |
| MemBand 1D Ave. | 3% | 1% | 2% | 3% | 1% | 3% |
| MemBand 1D Max. | 8% | 12% | 9% | 11% | 7% | 12% |

Table 2: Cross-check Results of the “Eighth” version

# Conclusions

We have cross-checked the results of adaptive motion vector resolution presented by Qualcomm with HM 4.0 anchor. The simulation results are confirmed.