| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G343 |
| --- | --- |

| Title: | Non-CE9: Improvement in temporal candidate of merge mode and AMVP |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Na Zhang Xiaopeng Fan Siwei Ma Debin Zhao Harbin Institute of Technology Harbin, China | Email: | hitnzhang@gmail.com fxp@hit.edu.cn swma@pku.edu.cn dbzhao@jdl.ac.cn |
| Source: | Harbin Institute of Technology |  |  |

_____________________________

# Abstract

This contribution proposes an improved scheme in temporal candidate of merge mode and AMVP. In order to utilize motion information from the below and right sides more effectively, the scheme considers three additional temporal candidate checking positions relative to HM4.0 in some cases. Simulation results show that the proposed scheme in average provides 0.1% bit rate saving for random access, and 0.2% bit rate saving for low delay with almost the same time cost. This proposal have been crosschecked by MSRA and the document number is JCTVC-G406.

# Introduction

Currently in both merge mode and AMVP, neighboring motion information can be used for coding a current block. They check the same candidate locations, although the process of deriving candidates is slightly different. The locations of neighboring blocks are shown in Error: Reference source not found.

Figure 1. Candidate checking positions for merge mode (a) and AMVP (b) in HM4.0

In the WD4[1], merge mode first checks four spatial candidate positions A1, B1, B0, A0. Only when the number of valid candidates is less than four, can B2 position be checked. Therefore, up to four spatial candidates are to be derived. The temporal candidate, TRb, is located right bottom to the temporal collocated PU as shown in Figure 1. If this candidate is not available, the centered collocated candidate is used. However, AMVP first checks A0 and A1 positions, and the first valid candidate will be the motion vector predictor on the left side. Then AMVP checks B0, B1 and B2 positions, and similarly the first valid candidate will be the motion vector predictor on the above side. The way of obtaining the temporal candidate in AMVP is similar to merge mode.

In HM 4.0, in order to introduce MVP candidates from the below and right sides, the right bottom neighboring corner block (TRb) of a collocated picture is considered when deriving the temporal candidate. However, if only the right bottom neighboring corner block is considered, it may be not enough to exploit the motion information on the below and right sides especially when the temporal prediction is more effective.

# Proposed Method

In order to utilize the motion information from the below and right sides more effectively, the proposed method considers three additional temporal candidate checking positions in some cases which are from the below, right neighboring blocks and right bottom corner block (Tb, Tr and TRb' ) of a collocated PU as shown in Figure 2.

Figure 2. Proposed candidate checking positions for merge mode and AMVP

To provide more effective solution, we implemented and tested two solutions as follows.

Solution A:

The procedure of Solution A is as follows:

Step 1: In both merge mode and AMVP, check abs(POC(currPic)-POC(colPic)). If it is larger than 1 as shown in Figure 3(a), goto step 2; If it is equal to 1 as shown in Figure 3(b), Figure 4(a), Figure 4(b) (indicating that the collocated picture is close to the current picture), goto step 3.

Step 2: When deriving the temporal candidate, first check TRb. If it is not valid, then check median(Tb, Tr ,TRb' ). Otherwise, the median checking is skipped. After these operations, if the temporal candidate is still not available, the TCtr is used.

Step 3: If the number of valid candidates after checking all the candidates of the original candidate list(before checking the three kinds of addtional bi-predictive merge candidates) is less than the maximum number of candidates(5 for merge mode, 3 for AMVP), check median(Tb, Tr, TRb' ). If it is valid, add the candidate to the end of original candidate list.

Figure 3: Absolute POC difference of a GOP size in random access configurations

Figure 4: Absolute POC difference in low delay configurations

Solution B:

If the number of valid candidates after checking all the candidates of the original candidate list is less than the maximum number of candidates(5 for merge mode, 3 for AMVP), check median(Tb, Tr ,TRb' ) . If it is valid, add the candidate to the end of the original candidate list.

The maximum number of candidates under each of the two modes in both of the solutions keeps unchanged.

# Experimental Results:

Both of the solutions are implemented in HM4.0 and simulated under the common test conditions Error: Reference source not found. Error: Reference source not found shows results of the solution A. Based on the results we can see that the solution A in average provides 0.1% bit rate saving for random access, and 0.2% bit rate saving for low delay with no obvious complexity increasing at both encoder and decoder.

Table 1 Test result of solution A

|  | Random Access HE |  |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y |  | U | V |
| Class A | -0.1% | -0.1% | -0.3% | -0.1% |  | -0.1% | -0.1% |
| Class B | -0.1% | 0.0% | 0.0% | -0.1% |  | -0.1% | 0.0% |
| Class C | -0.1% | 0.0% | 0.0% | -0.1% |  | -0.1% | -0.2% |
| Class D | -0.1% | -0.1% | -0.1% | -0.1% |  | -0.2% | -0.1% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.1% | 0.0% | -0.1% | -0.1% |  | -0.1% | -0.1% |
|  | -0.1% | 0.0% | -0.1% | -0.1% |  | -0.1% | -0.1% |
| Enc Time[%] | 100% |  |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  |  | 101% |  |  |
|  | Low delay B HE |  |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y |  | U | V |
| Class A |  |  |  |  |  |  |  |
| Class B | -0.2% | -0.3% | -0.1% | -0.2% |  | -0.1% | 0.1% |
| Class C | -0.2% | -0.2% | -0.2% | -0.2% |  | -0.1% | -0.3% |
| Class D | -0.2% | -0.2% | 0.0% | -0.2% |  | -0.1% | -0.3% |
| Class E | -0.2% | 0.1% | 0.7% | -0.2% |  | 0.0% | -0.1% |
| Overall | -0.2% | -0.2% | 0.1% | -0.2% |  | -0.1% | -0.1% |
|  | -0.2% | -0.2% | 0.0% | -0.2% |  | -0.1% | -0.1% |
| Enc Time[%] | 101% |  |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  |  | 101% |  |  |

|  | Low delay P HE |  |  |  | Low delay P LC |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y |  | U | V |
| Class A |  |  |  |  |  |  |  |
| Class B | -0.2% | 0.1% | 0.2% | -0.3% |  | 0.0% | -0.2% |
| Class C | -0.2% | -0.2% | 0.0% | -0.2% |  | -0.3% | -0.2% |
| Class D | -0.1% | -0.2% | -0.3% | -0.2% |  | 0.1% | -0.5% |
| Class E | -0.2% | -0.2% | -0.4% | -0.3% |  | -0.5% | -0.7% |
| Overall | -0.2% | -0.1% | -0.1% | -0.3% |  | -0.1% | -0.4% |
|  | -0.2% | -0.2% | -0.2% | -0.3% |  | -0.2% | -0.4% |
| Enc Time[%] | 100% |  |  |  | 101% |  |  |
| Dec Time[%] | 101% |  |  |  | 102% |  |  |

Error: Reference source not found shows the performance of the solution B. Simulation results show that the solution B has better prediction performance than the solution A for almost one third of the test sequences, and has almost the same prediction performance as the solution A for the other test sequences in random access configurations. Taking Table 1 and Table 2 for comparison, we can see that in RA-LC configuration, solution A in average provides 0.1% bit rate saving for Class D, while solution B in average provides 0.2% bit rate saving for Class D. Moreover, due to that in low delay configurations, the picture order count (POC) difference between the current picture and the collocated picture is always 1, the solution A and the solution B have the same prediction performance in these cases.

Table 1 Test result of solution B

|  | Random Access HE |  |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y |  | U | V |
| Class A | -0.1% | -0.2% | -0.3% | -0.1% |  | 0.0% | 0.0% |
| Class B | -0.1% | 0.0% | 0.0% | -0.1% |  | -0.1% | 0.0% |
| Class C | -0.1% | -0.1% | 0.0% | -0.1% |  | -0.1% | -0.1% |
| Class D | -0.1% | -0.1% | -0.1% | -0.2% |  | -0.2% | -0.1% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.1% | -0.1% | -0.1% | -0.1% |  | -0.1% | -0.1% |
|  | -0.1% | -0.1% | -0.1% | -0.1% |  | -0.1% | -0.1% |
| Enc Time[%] | 100% |  |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  |  | 101% |  |  |
|  | Low delay B HE |  |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y |  | U | V |
| Class A |  |  |  |  |  |  |  |
| Class B | -0.2% | -0.3% | -0.1% | -0.2% |  | -0.1% | 0.1% |
| Class C | -0.2% | -0.2% | -0.2% | -0.2% |  | -0.1% | -0.3% |
| Class D | -0.2% | -0.2% | 0.0% | -0.2% |  | -0.1% | -0.3% |
| Class E | -0.2% | 0.1% | 0.7% | -0.2% |  | 0.0% | -0.1% |
| Overall | -0.2% | -0.2% | 0.1% | -0.2% |  | -0.1% | -0.1% |
|  | -0.2% | -0.2% | 0.0% | -0.2% |  | -0.1% | -0.1% |
| Enc Time[%] | 100% |  |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  |  | 102% |  |  |
|  | Low delay P HE |  |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y |  | U | V |
| Class A |  |  |  |  |  |  |  |
| Class B | -0.2% | 0.1% | 0.2% | -0.3% |  | 0.0% | -0.2% |
| Class C | -0.2% | -0.2% | 0.0% | -0.2% |  | -0.3% | -0.2% |
| Class D | -0.1% | -0.2% | -0.3% | -0.2% |  | 0.1% | -0.5% |
| Class E | -0.2% | -0.2% | -0.4% | -0.3% |  | -0.5% | -0.7% |
| Overall | -0.2% | -0.1% | -0.1% | -0.3% |  | -0.1% | -0.4% |
|  | -0.2% | -0.2% | -0.2% | -0.3% |  | -0.2% | -0.4% |
| Enc Time[%] | 101% |  |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  |  | 101% |  |  |

# Conclusion

This proposal presents two solutions to utilize the temporal motion information more effectively for both merge mode and AMVP. Both solution A and solution B in average provide 0.1% bit rate saving for random access, and 0.2% bit rate saving for low delay with almost the same time cost as HM 4.0. However, the performance of solution B is slightly better than that of solution A for random access. We recommend solution B to be adopted into HM.

# Acknowledge

We would like to thank MSRA for doing the crosscheck for us.

# Reference

- “WD4: Working Draft 4 of High-Efficiency Video Coding”, JCTVC-F803, JCT-VC 6th Meeting: Torino, July, 2011

- J.-L.  Lin, Y.-P.  Tsai, Y.-W.  Huang, Shawmin  Lei, “Improved Advanced Motion Vector Prediction”, JCTVC-D125, JCT-VC 4th Meeting: Daegu, January, 2011

- Frank Bossen, “Common test conditions and software reference configurations”, JCTVC-F900, JCT-VC 6th Meeting: Torino, July, 2011

# Patent rights declaration(s)

Harbin Institute of Technology may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Proposed Draft Text

## 7.1 Soltuion A

##### Derivation process for luma motion vectors for merge mode

This process is only invoked when PredMode is equal to MODE_ INTER and merge_flag [ xP ][ yP ] is equal to 1, where ( xP, yP ) specify the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture.

Inputs of this process are

- a luma location ( xP, yP ) of the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture,

- variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH,

- a variable PartIdx specifying the index of the current prediction unit within the current coding unit.

Outputs of this process are

–	the luma motion vectors mvL0 and mvL1,

–	the reference indices refIdxL0 and refIdxL1,

–	the prediction list utilization flags predFlagL0 and predFlagL1.

The motion vectors mvL0 and mvL1, the reference indices refIdxL0 and refIdxL1, and the prediction utilization flags predFlagL0 and predFlagL1 are derived as specified by the following ordered steps:

- The derivation process for merging candidates from neighboring prediction unit partitions in subclause Error: Reference source not found is invoked with luma location ( xP, yP ), the width and the height of the prediction unit nPSW and nPSH and the partition index PartIdx as inputs and the output is assigned to the availability flags availableFlagN, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N and the motion vectors mvL0N and mvL1N with N being replaced by A0, A1, B0, B1 or B2.

- The derivation process of reference indices for temporal merging candidate in subclause Error: Reference source not found is invoked with luma location ( xP, yP ), nPSW, nPSH as the inputs and the output is directly assigned to refIdxLX.

- The derivation process for temporal luma motion vector prediction in subclause  is invoked with luma location ( xP, yP ), refIdxLX as the inputs and with the output being the availability flag availableFlagLXN and the temporal motion vector mvLXN. The variables availableFlagN and predFlagLXN(with X being 0 or 1, N being replaced by Col1 or Col2, respectively) are derived as specified below.

availableFlagN = availableFlagL0N || availableFlagL1N

predFlagLXN= availableFlagLXN

- The merging candidate list, mergeCandList, is constructed of which elements are given as specified order:

  - A1, if availableFlagA1 is equal to 1

  - B1, if availableFlagB1 is equal to 1

  - B0, if availableFlagB0 is equal to 1

  - A0, if availableFlagA0 is equal to 1

  - B2, if availableFlagB2 is equal to 1

  - Col1, if availableFlagCol1 is equal to 1

  - Col2, if availableFlagCol2 is equal to 1

- When merging candidates have the same motion vectors and the same reference indices, the merging candidates are removed from the list except the merging candidate which has the smallest order in the mergeCandList.

- The variable numMergeCand and numOrigMergeCand are set to the number of merging candidates in the mergeCandList and the variable maxNumMergeCand is set to 5.

- When slice_type is equal to B, the following applies.

  - The derivation process for combined bi-predictive merging candidates specified in subclause Error: Reference source not found is invoked with mergeCandList, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N, the motion vectors mvL0N and mvL1N of every candidate N being in mergeCandList, numMergeCand, numOrigMergeCand and maxNumMergeCand given as input and the output is assigned to mergeCandList, numMergeCand, the reference indices refIdxL0combCandk and refIdxL1combCandk, the prediction list utilization flags predFlagL0combCandk and predFlagL1combCandk and the motion vectors mvL0combCandk and mvL1combCandk of every new candidate combCandk being added in mergeCandList. The number of candidates being added numCombMergeCand is set equal to ( numMergeCand – numOrigMergeCand ). When numCombMergeCand is greater than 0, k ranges from 0 to numCombMergeCand − 1, inclusive.

  - The derivation process for non-scaled bi-predictive merging candidates specified in subclause Error: Reference source not found is invoked with mergeCandList, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N, the motion vectors mvL0N and mvL1N of every candidate N being in mergeCandList, numMergeCand, numOrigMergeCand and maxNumMergeCand given as input and the output is assigned to mergeCandList, numMergeCand, the reference indices refIdxL0nscaleCandl and refIdxL1nscaleCandl, the prediction list utilization flags predFlagL0nscaleCandl and predFlagL1nscaleCandl, the motion vectors mvL0nscaleCandl and mvL1nscaleCandl of every new candidate nscaleCandl being added in mergeCandList. The number of candidates being added numNscaleMergeCand is set equal to ( numMergeCand – numOrigMergeCand – numCombMergeCand ). When numNscaleMergeCand is greater than 0, l ranges from 0 to numNscaleMergeCand – 1, inclusive.

- The derivation process for zero motion vector merging candidates specified in subclause Error: Reference source not found is invoked with the mergeCandList, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N, the motion vectors mvL0N and mvL1N of every candidate N being in mergeCandList, the NumMergeCand and the MaxNumMergeCand as the inputs and the output is assigned to mergeCandList, numMergeCand, the reference indices refIdxL0zeroCandm and refIdxL1zeroCandm, the prediction list utilization flags predFlagL0zeroCandm and predFlagL1zeroCandm, the motion vectors mvL0zeroCandm and mvL1zeroCandm of every new candidate zeroCandm being added in mergeCandList. The number of candidates being added numZeroMergeCand is set equal to ( numMergeCand – numOrigMergeCand – numCombMergeCand – numNscaleMergeCand ). When numZeroMergeCand is greater than 0, m ranges from 0 to numZeroMergeCand – 1, inclusive.

- The following assignments are made with N being the candidate at position merge_idx[ xP][ yP ] in the merging candidate list mergeCandList ( N = mergeCandList[ merge_idx[ xP][ yP ] ] ) and X being replaced by 0 or 1:

  - mvLX[ 0 ] = mvLXN[ 0 ]

  - mvLX[ 1 ] = mvLXN[ 1 ]

  - refIdxLX = refIdxLXN

  - predFlagLX = predFlagLXN

##### Derivation process for luma motion vector prediction

Inputs to this process are

- a luma location ( xP, yP ) specifying the top-left luma sample of the current prediction unit relative to the top-left sample of the current picture,

- variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH.

- the reference index of the current prediction unit partition refIdxLX (with X being 0 or 1).

Output of this process is

- the prediction mvpLX of the motion vector mvLX (with X being 0 or 1).

The motion vector predictor mvpLX is derived in the following ordered steps.

- The derivation process for motion vector predictor candidates from neighboring prediction unit partitions in subclause Error: Reference source not found is invoked with luma location ( xP, yP ), the width and the height of the prediction unit nPSW and nPSH, and refIdxLX (with X being 0 or 1, respectively) as inputs and the availability flags availableFlagLXN and the motion vectors mvLXN with N being replaced by A, B as the output.

- The derivation process for temporal luma motion vector prediction in subclause Error: Reference source not found is invoked with luma location ( xP, yP ) , the width and the height of the prediction unit nPSW and nPSH, and refIdxLX (with X being 0 or 1, respectively) as the inputs and with the output being the availability flag availableFlagLXN and the temporal motion vector predictor mvLXN with N being replaced by Col1 or Col2.

- The, is constructed of which elements are given as specified order:

- mvLXA, if availableFlagLXA is equal to 1

- mvLXB, if availableFlagLXB is equal to 1

- mvLXCol1, if availableFlagLXCol1 is equal to 1

- mvLXCol2, if availableFlagLXCol2 is equal to 1

- When motion vectors have the same value, the motion vectors are removed from the list except the motion vector which has the smallest order in the mvpListLX.

- When mvpListLX is empty, a zero motion vector is added as follows.

  - mvpListLC[ 0 ][0] = 0

  - mvpListLC[ 0 ][1] = 0

- The variable numMVPCandLX is set to the number of elements within the mvpListLX and maxNumMVPCand is set to 2.

- The motion vector predictor list is modifed to contain exactly maxNumMVPCand motion vector predictor candidates as follows.

    - If numMVPCandLX is less than maxNumMVPCand, the derivation process for zero motion vector predictor candidates specified in subclause Error: Reference source not found is invoked with mvpListLX and numMVPCandLX given as input and the output is assigned to mvpListLX and numMVPCandLX.

    - Otherwise (numMVPCandLX is equal to or greater than maxNumMVPCand), all motion vector predictor candidates mvpListLX[ idx ] with idx greater than maxNumMVPCand − 1 are removed from the list.

- The motion vector of mvpListLX[ mvp_idx_lX[ xP, yP ] ] is assigned to mvpLX.

##### Derivation process for temporal luma motion vector prediction

Inputs to this process are

- a luma location ( xP, yP ) specifying the top-left luma sample of the current prediction unit relative to the top-left sample of the current picture,

- variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH,

- the reference index of the current prediction unit partition refIdxLX (with X being 0 or 1).

Outputs of this process are (with N being replaced by Col1 or Col2 and with X being replaced by 0 or 1)

- the motion vector prediction mvLXN,

- the availability flag availableFlagLXN.

The function RefPicOrderCnt( pic, refidx, LX ) is specified by the value of PicOrderCnt of the picture that is the reference picture RefPicListX[ refidx ] of pic with X being 0 or 1. PicOrderCnt of the reference picture shall be maintained until the picture is marked as “non-existing”.

[Ed. (BB): PicOrderCnt(.) needs to be defined. Currently the Decoding process for picture order count is not described]

Depending on the values of slice_type and collocated_from_l0_flag, the variable colPic, specifying the picture that contains the co-located partition, is derived as follows.

- If slice_type is equal to B and collocated_from_l0_flag is equal to 0, the variable colPic specifies the picture that contains the co-located partition as specified by RefPicList1[ 0 ].

- Otherwise (slice_type is equal to B and collocated_from_l0_flag is equal to 1 or slice_type is equal to P) , the variable colPic specifies the picture that contains the co-located partition as specified by RefPicList0[ 0 ].

Variable colnPu and its position ( xPColn, yPColn)(with n being replaced by Rb, r, b, Rb' , Ctr)are derived as follows:

- Right-bottom neighboring luma position of the current prediction unit is defined by

- xPRb = xP + nPSW

- yPRb = yP + nPSH

The variable colRbPu  is set as the prediction unit covering the modified position given by

( ( xPRb >> 4 ) << 4, ( yPRb >> 4 ) << 4 ) inside the colPic.

- Bottom neighboring luma position of the current prediction unit is defined by

- xPb = xP + nPSW - 1

- yPb = yP + nPSH

The variable colbPu is set as the prediction unit covering the modified position given by

( ( xPb >> 4 ) << 4, ( yPb >> 4 ) << 4 ) inside the colPic.

- Right neighboring luma position of the current prediction unit is defined by

- xPr = xP + nPSW

- yPr = yP + nPSH-1

The variable colrPu is set as the prediction unit covering the modified position given by

( ( xPr >> 4 ) << 4, ( yPr >> 4 ) << 4 ) inside the colPic.

- Right-bottom luma position of the current prediction unit is defined by

- xPRb' = xP + nPSW - 1

- yPRb' = yP + nPSH - 1

The variable colRb'Pu is set as the prediction unit covering the modified position given by

( ( xPRb ' >> 4 ) << 4, ( xPRb ' >> 4 ) << 4 ) inside the colPic.

- Central luma position of the current prediction unit is defined by

- xPCtr = xP + ( nPSW >> 1 ) – 1

- yPCtr =yP + ( nPSH >> 1 ) – 1

The variable colCtrPu is set as the prediction unit covering the modified position given by

( ( xPCtr >> 4 ) << 4, ( yPCtr >> 4 ) << 4 ) inside the colPic.

- ( xPColn, yPColn ) is set equal to the top-left luma sample of the colnPu relative to the top-left luma sample of the colPic with n being replaced by Rb, r, b, Rb' or Ctr.

The variables mvLXColn and availableFlagLXColn (with n being replaced by Rb, r, b, Rb' or Ctr) are derived as follows.

- If colnPu is coded in an intra prediction mode or colnPu is unavailable, both components of mvLXColn are set equal to 0 and availableFlagLXColn is set equal to 0.

- Otherwise (colnPu is not coded in an intra prediction mode and colnPu is available), the variables mvColn and refIdxColn are derived as follows,

  - If PredFlagL0[ xPColn ][ yPColn] is equal to 0, the motion vector mvColn and the reference index refIdxColn are set equal to MvL1[ xPColn ][ yPColn] and RefIdxL1[ xPColn ][ yPColn], respectively.

  - Otherwise (PredFlagL0[ xPColn ][ yPColn ] is equal to 1), the following applies.

  - If PredFlagL1[ xPColn ][ yPColn ] is equal to 0, the motion vector mvColn and the reference index refIdxColn are set equal to MvL0[ xPColn ][ yPColn ] and RefIdxL0[  xPColn ][ yPColn ], respectively.

  - Otherwise (PredFlagL1[  xPColn][ yPColn] is equal to 1), the following assignments are made.

      - If all PicOrderCnts of pictures in all reference picture lists are smaller than PicOrderCnt( currPic ), the motion vector mvColn, the reference index refIdxColn and ListColn are set equal to MvLN[ xPColn ][ yPColn ], RefIdxColLN and LN, respectively, with N being value of RefIdxLX.

      - Otherwise (one of PicOrderCnt of pictures in all reference picture lists are larger than PicOrderCnt( currPic ) ), the motion vector mvColn, the reference index refIdxColn and ListColn are set equal to MvLN[ xPColn ][ yPColn ], RefIdxColLN and LN, respectively, with N being value of collocated_from_l0_flag.

and the variable availableFlagLXColn is set equal to 1 and the following applies.

  - If PicOrderCnt( colPic ) – RefPicOrderCnt( colPic, refIdxColn, ListColn ) is equal to PicOrderCnt( currPic ) - RefPicOrderCnt( currPic, refIdxLX, LX ),

  - mvLXColn = mvColn

  - Otherwise, mvLXColn is derived as scaled version of the motion vector mvColn as specified below

  - tx = ( 16384 + Abs( td / 2 ) ) / td

  - DistScaleFactor = Clip3( -1024, 1023, ( tb * tx + 32 ) >> 6 )

  - mvLXColn = ClipMv( Sign( DistScaleFactor * mvColn ) * 			
				( (Abs( DistScaleFactor * mvColn ) + 127 ) >> 8 ) )

[Ed. (WJ): software has a clip function for scaled motion vector. Do we need this? AVC does not have it]

where td and tb are derived as

  - td = Clip3( -128, 127, PicOrderCnt( colPic ) – RefPicOrderCnt( colPic, refIdxColn, ListColn ) )

  - tb = Clip3( -128, 127, PicOrderCnt( currPic ) – RefPicOrderCnt( currPic, refIdxLX, LX ) )

The motion vector prediction mvLXCol1 and the availability flag availableFlagLXCol1 are derived in the following ordered steps:

bExistMVCol = 0

- If colRbPu is not coded in an intra prediction mode and colRbPu is available

- mvLXCol1 = mvLXColRb

- availableFlagLXCol1 = availableFlagLXColRb

- bExistMVCol = 1

- If bExistMVCol is equal to 0 and abs(PicOrderCnt( colPic ) - PicOrderCnt( currPic )) is larger than 1

- If colbPu is not coded in an intra prediction mode and colbPu is available or colrPu is not       coded in an intra prediction mode and colrPu is available or colRb'Pu is not coded in an intra prediction mode and colRb'Pu is available

  - mvLXCol1 = median(mvLXColb , mvLXColr , mvLXColRb')

  - availableFlagLXCol1 =availableFlagLXColb || availableFlagLXColr || availableFlagLXColRb'

  - bExistMVCol = 1

- If bExistMVCol is equal to 0

- If colCtrPu is not coded in an intra prediction mode and colCtrPu is available

  - mvLXCol1 = mvLXColCtr

  - availableFlagLXCol1 = availableFlagLXColCtr

  - bExistMVCol = 1

- If bExistMVCol is equal to 0

Both components of mvLXCol1 are set equal to 0 and availableFlagLXCol1 is set equal to 0.

The motion vector prediction mvLXCol2 and the availability flag availableFlagLXCol2 are derived in the following ordered steps:

- If abs(PicOrderCnt( colPic ) - PicOrderCnt( currPic )) is only 1 and the number of valid candidates after checking all the candidates of the original candidate list is less than the maximum number of candidates(5 for merge mode, 3 for AMVP)

- If colbPu is not coded in an intra prediction mode and colbPu is available or colrPu is not coded in an intra prediction mode and colrPu is available or colRb'Pu is not coded in an intra prediction mode and colRb'Pu is available

  - mvLXCol2 = median(mvLXColb , mvLXColr , mvLXColRb')

  - availableFlagLXCol2 =availableFlagLXColb || availableFlagLXColr || availableFlagLXColRb'

- Otherwise, both components of mvLXCol2 are set equal to 0 and availableFlagLXCol2 is set equal to 0.

- Otherwise, both components of mvLXCol2 are set equal to 0 and availableFlagLXCol2 is set equal to 0.

## 7.2 Soltuion B

##### Derivation process for luma motion vectors for merge mode

This process is only invoked when PredMode is equal to MODE_ INTER and merge_flag [ xP ][ yP ] is equal to 1, where ( xP, yP ) specify the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture.

Inputs of this process are

- a luma location ( xP, yP ) of the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture,

- variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH,

- a variable PartIdx specifying the index of the current prediction unit within the current coding unit.

Outputs of this process are

–	the luma motion vectors mvL0 and mvL1,

–	the reference indices refIdxL0 and refIdxL1,

–	the prediction list utilization flags predFlagL0 and predFlagL1.

The motion vectors mvL0 and mvL1, the reference indices refIdxL0 and refIdxL1, and the prediction utilization flags predFlagL0 and predFlagL1 are derived as specified by the following ordered steps:

- The derivation process for merging candidates from neighboring prediction unit partitions in subclause Error: Reference source not found is invoked with luma location ( xP, yP ), the width and the height of the prediction unit nPSW and nPSH and the partition index PartIdx as inputs and the output is assigned to the availability flags availableFlagN, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N and the motion vectors mvL0N and mvL1N with N being replaced by A0, A1, B0, B1 or B2.

- The derivation process of reference indices for temporal merging candidate in subclause Error: Reference source not found is invoked with luma location ( xP, yP ), nPSW, nPSH as the inputs and the output is directly assigned to refIdxLX.

- The derivation process for temporal luma motion vector prediction in subclause  is invoked with luma location ( xP, yP ), refIdxLX as the inputs and with the output being the availability flag availableFlagLXN and the temporal motion vector mvLXN. The variables availableFlagN and predFlagLXN(with X being 0 or 1, N being replaced by Col1 or Col2, respectively) are derived as specified below.

availableFlagN = availableFlagL0N || availableFlagL1N

predFlagLXN= availableFlagLXN

- The merging candidate list, mergeCandList, is constructed of which elements are given as specified order:

  - A1, if availableFlagA1 is equal to 1

  - B1, if availableFlagB1 is equal to 1

  - B0, if availableFlagB0 is equal to 1

  - A0, if availableFlagA0 is equal to 1

  - B2, if availableFlagB2 is equal to 1

  - Col1, if availableFlagCol1 is equal to 1

  - Col2, if availableFlagCol2 is equal to 1

- When merging candidates have the same motion vectors and the same reference indices, the merging candidates are removed from the list except the merging candidate which has the smallest order in the mergeCandList.

- The variable numMergeCand and numOrigMergeCand are set to the number of merging candidates in the mergeCandList and the variable maxNumMergeCand is set to 5.

- When slice_type is equal to B, the following applies.

  - The derivation process for combined bi-predictive merging candidates specified in subclause Error: Reference source not found is invoked with mergeCandList, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N, the motion vectors mvL0N and mvL1N of every candidate N being in mergeCandList, numMergeCand, numOrigMergeCand and maxNumMergeCand given as input and the output is assigned to mergeCandList, numMergeCand, the reference indices refIdxL0combCandk and refIdxL1combCandk, the prediction list utilization flags predFlagL0combCandk and predFlagL1combCandk and the motion vectors mvL0combCandk and mvL1combCandk of every new candidate combCandk being added in mergeCandList. The number of candidates being added numCombMergeCand is set equal to ( numMergeCand – numOrigMergeCand ). When numCombMergeCand is greater than 0, k ranges from 0 to numCombMergeCand − 1, inclusive.

  - The derivation process for non-scaled bi-predictive merging candidates specified in subclause Error: Reference source not found is invoked with mergeCandList, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N, the motion vectors mvL0N and mvL1N of every candidate N being in mergeCandList, numMergeCand, numOrigMergeCand and maxNumMergeCand given as input and the output is assigned to mergeCandList, numMergeCand, the reference indices refIdxL0nscaleCandl and refIdxL1nscaleCandl, the prediction list utilization flags predFlagL0nscaleCandl and predFlagL1nscaleCandl, the motion vectors mvL0nscaleCandl and mvL1nscaleCandl of every new candidate nscaleCandl being added in mergeCandList. The number of candidates being added numNscaleMergeCand is set equal to ( numMergeCand – numOrigMergeCand – numCombMergeCand ). When numNscaleMergeCand is greater than 0, l ranges from 0 to numNscaleMergeCand – 1, inclusive.

- The derivation process for zero motion vector merging candidates specified in subclause Error: Reference source not found is invoked with the mergeCandList, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N, the motion vectors mvL0N and mvL1N of every candidate N being in mergeCandList, the NumMergeCand and the MaxNumMergeCand as the inputs and the output is assigned to mergeCandList, numMergeCand, the reference indices refIdxL0zeroCandm and refIdxL1zeroCandm, the prediction list utilization flags predFlagL0zeroCandm and predFlagL1zeroCandm, the motion vectors mvL0zeroCandm and mvL1zeroCandm of every new candidate zeroCandm being added in mergeCandList. The number of candidates being added numZeroMergeCand is set equal to ( numMergeCand – numOrigMergeCand – numCombMergeCand – numNscaleMergeCand ). When numZeroMergeCand is greater than 0, m ranges from 0 to numZeroMergeCand – 1, inclusive.

- The following assignments are made with N being the candidate at position merge_idx[ xP][ yP ] in the merging candidate list mergeCandList ( N = mergeCandList[ merge_idx[ xP][ yP ] ] ) and X being replaced by 0 or 1:

  - mvLX[ 0 ] = mvLXN[ 0 ]

  - mvLX[ 1 ] = mvLXN[ 1 ]

  - refIdxLX = refIdxLXN

  - predFlagLX = predFlagLXN

##### Derivation process for luma motion vector prediction

Inputs to this process are

- a luma location ( xP, yP ) specifying the top-left luma sample of the current prediction unit relative to the top-left sample of the current picture,

- variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH.

- the reference index of the current prediction unit partition refIdxLX (with X being 0 or 1).

Output of this process is

- the prediction mvpLX of the motion vector mvLX (with X being 0 or 1).

The motion vector predictor mvpLX is derived in the following ordered steps.

- The derivation process for motion vector predictor candidates from neighboring prediction unit partitions in subclause Error: Reference source not found is invoked with luma location ( xP, yP ), the width and the height of the prediction unit nPSW and nPSH, and refIdxLX (with X being 0 or 1, respectively) as inputs and the availability flags availableFlagLXN and the motion vectors mvLXN with N being replaced by A, B as the output.

- The derivation process for temporal luma motion vector prediction in subclause Error: Reference source not found is invoked with luma location ( xP, yP ) , the width and the height of the prediction unit nPSW and nPSH, and refIdxLX (with X being 0 or 1, respectively) as the inputs and with the output being the availability flag availableFlagLXN and the temporal motion vector predictor mvLXN with N being replaced by Col1 or Col2.

- The, is constructed of which elements are given as specified order:

- mvLXA, if availableFlagLXA is equal to 1

- mvLXB, if availableFlagLXB is equal to 1

- mvLXCol1, if availableFlagLXCol1 is equal to 1

- mvLXCol2, if availableFlagLXCol2 is equal to 1

- When motion vectors have the same value, the motion vectors are removed from the list except the motion vector which has the smallest order in the mvpListLX.

- When mvpListLX is empty, a zero motion vector is added as follows.

  - mvpListLC[ 0 ][0] = 0

  - mvpListLC[ 0 ][1] = 0

- The variable numMVPCandLX is set to the number of elements within the mvpListLX and maxNumMVPCand is set to 2.

- The motion vector predictor list is modifed to contain exactly maxNumMVPCand motion vector predictor candidates as follows.

    - If numMVPCandLX is less than maxNumMVPCand, the derivation process for zero motion vector predictor candidates specified in subclause Error: Reference source not found is invoked with mvpListLX and numMVPCandLX given as input and the output is assigned to mvpListLX and numMVPCandLX.

    - Otherwise (numMVPCandLX is equal to or greater than maxNumMVPCand), all motion vector predictor candidates mvpListLX[ idx ] with idx greater than maxNumMVPCand − 1 are removed from the list.

- The motion vector of mvpListLX[ mvp_idx_lX[ xP, yP ] ] is assigned to mvpLX.

##### Derivation process for temporal luma motion vector prediction

Inputs to this process are

- a luma location ( xP, yP ) specifying the top-left luma sample of the current prediction unit relative to the top-left sample of the current picture,

- variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH,

- the reference index of the current prediction unit partition refIdxLX (with X being 0 or 1).

Outputs of this process are (with N being replaced by Col1 or Col2 and with X being replaced by 0 or 1)

- the motion vector prediction mvLXN,

- the availability flag availableFlagLXN.

The function RefPicOrderCnt( pic, refidx, LX ) is specified by the value of PicOrderCnt of the picture that is the reference picture RefPicListX[ refidx ] of pic with X being 0 or 1. PicOrderCnt of the reference picture shall be maintained until the picture is marked as “non-existing”.

[Ed. (BB): PicOrderCnt(.) needs to be defined. Currently the Decoding process for picture order count is not described]

Depending on the values of slice_type and collocated_from_l0_flag, the variable colPic, specifying the picture that contains the co-located partition, is derived as follows.

- If slice_type is equal to B and collocated_from_l0_flag is equal to 0, the variable colPic specifies the picture that contains the co-located partition as specified by RefPicList1[ 0 ].

- Otherwise (slice_type is equal to B and collocated_from_l0_flag is equal to 1 or slice_type is equal to P) , the variable colPic specifies the picture that contains the co-located partition as specified by RefPicList0[ 0 ].

Variable colnPu and its position ( xPColn, yPColn)(with n being replaced by Rb, r, b, Rb' , Ctr)are derived as follows:

- Right-bottom neighboring luma position of the current prediction unit is defined by

- xPRb = xP + nPSW

- yPRb = yP + nPSH

The variable colRbPu  is set as the prediction unit covering the modified position given by

( ( xPRb >> 4 ) << 4, ( yPRb >> 4 ) << 4 ) inside the colPic.

- Bottom neighboring luma position of the current prediction unit is defined by

- xPb = xP + nPSW - 1

- yPb = yP + nPSH

The variable colbPu is set as the prediction unit covering the modified position given by

( ( xPb >> 4 ) << 4, ( yPb >> 4 ) << 4 ) inside the colPic.

- Right neighboring luma position of the current prediction unit is defined by

- xPr = xP + nPSW

- yPr = yP + nPSH-1

The variable colrPu is set as the prediction unit covering the modified position given by

( ( xPr >> 4 ) << 4, ( yPr >> 4 ) << 4 ) inside the colPic.

- Right-bottom luma position of the current prediction unit is defined by

- xPRb' = xP + nPSW - 1

- yPRb' = yP + nPSH - 1

The variable colRb'Pu is set as the prediction unit covering the modified position given by

( ( xPRb ' >> 4 ) << 4, ( xPRb ' >> 4 ) << 4 ) inside the colPic.

- Central luma position of the current prediction unit is defined by

- xPCtr = xP + ( nPSW >> 1 ) – 1

- yPCtr =yP + ( nPSH >> 1 ) – 1

The variable colCtrPu is set as the prediction unit covering the modified position given by

( ( xPCtr >> 4 ) << 4, ( yPCtr >> 4 ) << 4 ) inside the colPic.

- ( xPColn, yPColn ) is set equal to the top-left luma sample of the colnPu relative to the top-left luma sample of the colPic with n being replaced by Rb, r, b, Rb' or Ctr.

The variables mvLXColn and availableFlagLXColn (with n being replaced by Rb, r, b, Rb' or Ctr) are derived as follows.

- If colnPu is coded in an intra prediction mode or colnPu is unavailable, both components of mvLXColn are set equal to 0 and availableFlagLXColn is set equal to 0.

- Otherwise (colnPu is not coded in an intra prediction mode and colnPu is available), the variables mvColn and refIdxColn are derived as follows,

  - If PredFlagL0[ xPColn ][ yPColn] is equal to 0, the motion vector mvColn and the reference index refIdxColn are set equal to MvL1[ xPColn ][ yPColn] and RefIdxL1[ xPColn ][ yPColn], respectively.

  - Otherwise (PredFlagL0[ xPColn ][ yPColn ] is equal to 1), the following applies.

  - If PredFlagL1[ xPColn ][ yPColn ] is equal to 0, the motion vector mvColn and the reference index refIdxColn are set equal to MvL0[ xPColn ][ yPColn ] and RefIdxL0[  xPColn ][ yPColn ], respectively.

  - Otherwise (PredFlagL1[  xPColn][ yPColn] is equal to 1), the following assignments are made.

      - If all PicOrderCnts of pictures in all reference picture lists are smaller than PicOrderCnt( currPic ), the motion vector mvColn, the reference index refIdxColn and ListColn are set equal to MvLN[ xPColn ][ yPColn ], RefIdxColLN and LN, respectively, with N being value of RefIdxLX.

      - Otherwise (one of PicOrderCnt of pictures in all reference picture lists are larger than PicOrderCnt( currPic ) ), the motion vector mvColn, the reference index refIdxColn and ListColn are set equal to MvLN[ xPColn ][ yPColn ], RefIdxColLN and LN, respectively, with N being value of collocated_from_l0_flag.

and the variable availableFlagLXColn is set equal to 1 and the following applies.

  - If PicOrderCnt( colPic ) – RefPicOrderCnt( colPic, refIdxColn, ListColn ) is equal to PicOrderCnt( currPic ) - RefPicOrderCnt( currPic, refIdxLX, LX ),

  - mvLXColn = mvColn

  - Otherwise, mvLXColn is derived as scaled version of the motion vector mvColn as specified below

  - tx = ( 16384 + Abs( td / 2 ) ) / td

  - DistScaleFactor = Clip3( -1024, 1023, ( tb * tx + 32 ) >> 6 )

  - mvLXColn = ClipMv( Sign( DistScaleFactor * mvColn ) * 			
				( (Abs( DistScaleFactor * mvColn ) + 127 ) >> 8 ) )

[Ed. (WJ): software has a clip function for scaled motion vector. Do we need this? AVC does not have it]

where td and tb are derived as

  - td = Clip3( -128, 127, PicOrderCnt( colPic ) – RefPicOrderCnt( colPic, refIdxColn, ListColn ) )

  - tb = Clip3( -128, 127, PicOrderCnt( currPic ) – RefPicOrderCnt( currPic, refIdxLX, LX ) )

The motion vector prediction mvLXCol1 and the availability flag availableFlagLXCol1 are derived in the following ordered steps:

bExistMVCol = 0

- If colRbPu is not coded in an intra prediction mode and colRbPu is available

- mvLXCol1 = mvLXColRb

- availableFlagLXCol1 = availableFlagLXColRb

- bExistMVCol = 1

- If bExistMVCol is equal to 0

- If colCtrPu is not coded in an intra prediction mode and colCtrPu is available

  - mvLXCol1 = mvLXColCtr

  - availableFlagLXCol1 = availableFlagLXColCtr

  - bExistMVCol = 1

- If bExistMVCol is equal to 0

Both components of mvLXCol1 are set equal to 0 and availableFlagLXCol1 is set equal to 0.

The motion vector prediction mvLXCol2 and the availability flag availableFlagLXCol2 are derived in the following ordered steps:

- If the number of valid candidates after checking all the candidates of the original candidate list is less than the maximum number of candidates(5 for merge mode, 3 for AMVP)

- If colbPu is not coded in an intra prediction mode and colbPu is available or colrPu is not coded in an intra prediction mode and colrPu is available or colRb'Pu is not coded in an intra prediction mode and colRb'Pu is available

  - mvLXCol2 = median(mvLXColb , mvLXColr , mvLXColRb')

  - availableFlagLXCol2 =availableFlagLXColb || availableFlagLXColr || availableFlagLXColRb'

- Otherwise, both components of mvLXCol2 are set equal to 0 and availableFlagLXCol2 is set equal to 0.

Otherwise, both components of mvLXCol2 are set equal to 0 and availableFlagLXCol2 is set equal to 0.