| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G344 |
| --- | --- |

| Title: | Cross-verification report for motion vector predictor candidate clipping removal (JCTVC-G134) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Hirofumi Aoki Keiichi Chono 1753, Shimonumabe, Nakahara-Ku Kawasaki, Kanagawa, 211-8666 JAPAN | Tel: Email: | +81 50 3757 5438 hao@bu.jp.nec.com chono@ct.jp.nec.com |
| Source: | NEC Corporation |  |  |

_____________________________

# Abstract

This contribution presents a study and cross-verification results of removal of clipping operation to motion vector predictor candidates proposed in JCTVC-G134. The cross-verification tasks were successfully completed and it was confirmed that the implementation conforms to the description in JCTVC-G134 and results exactly match those presented in JCTVC-G134. The necessity of the clipping is also raised as one of open issues to be discussed in the WD4 document. It is recommended that this issue is discussed and fixed at the 7th JCT-VC meeting.

# Software analysis

Software was provided by Qualcomm and it was successfully compiled by Microsoft Visual C++ 2008. It was confirmed that the software implementation conforms to the description in JCTVC-G134 [1]. The implementation is simple and straightforward, and there have been no problems found in the software.

# Remarks

In the HEVC WD4 [2], clipping operation is applied for motion vector predictor candidates. Meanwhile the necessity of the clipping is raised as one of open issues to be discussed in [2]. As described in [1], the clipping operation increases the complexity of motion vector predictor derivation process and valid motion vector can be derived even from unclipped motion vector predictor. It is recommended that this issue is discussed and fixed at the 7th JCT-VC meeting.

# Simulation results

Experiments to verify simulation results shown in [1] were conducted under the common test conditions described in JCTVC-F900 [3] with the software provided by the proponent. Our computing platform used for the verification is shown below.

| Computing platform | Usage |
| --- | --- |
| OS: Windows 7 64-bit Professional CPU: Xeon X5680 3.46GHz (12 cores) Memory: 12GiB | Up to 12 processes for encoding and decoding ran at the same time. |

Table 1 shows the summary of results. An attached spreadsheet, JCT-G344.xls, contains the detailed results. It was confirmed that BD-PSNR/Rate values are exactly same as those provided by the proponent.

Table 1: Summary results against CE4 anchor.

|  |  | Random Access HE |  |  |  |  |  | Random Access LC |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y |  | U |  | V |  | Y |  | U |  | V |
| Class A |  | 0.01% |  | -0.08% |  | -0.26% |  | 0.00% |  | 0.02% |  | 0.04% |
| Class B |  | 0.01% |  | 0.06% |  | 0.01% |  | 0.00% |  | -0.02% |  | 0.08% |
| Class C |  | 0.02% |  | 0.01% |  | -0.01% |  | -0.01% |  | 0.02% |  | 0.05% |
| Class D |  | 0.02% |  | 0.03% |  | -0.02% |  | -0.02% |  | 0.03% |  | 0.02% |
| Class E |  |  |  |  |  |  |  |  |  |  |  |  |
| Overall |  | 0.01% |  | 0.01% |  | -0.06% |  | -0.01% |  | 0.01% |  | 0.05% |
|  |  | 0.01% |  | 0.01% |  | -0.06% |  | -0.01% |  | 0.01% |  | 0.04% |
| Enc Time[%] |  | 100% |  |  |  |  |  | 100% |  |  |  |  |
| Dec Time[%] |  | 99% |  |  |  |  |  | 99% |  |  |  |  |
|  |  | Low delay B HE |  |  |  |  |  | Low delay B LC |  |  |  |  |
|  |  | Y |  | U |  | V |  | Y |  | U |  | V |
| Class A |  |  |  |  |  |  |  |  |  |  |  |  |
| Class B |  | 0.02% |  | -0.05% |  | -0.17% |  | 0.02% |  | 0.00% |  | -0.01% |
| Class C |  | 0.01% |  | 0.00% |  | -0.10% |  | 0.02% |  | 0.06% |  | -0.20% |
| Class D |  | 0.03% |  | 0.10% |  | -0.15% |  | 0.00% |  | -0.13% |  | -0.11% |
| Class E |  | -0.01% |  | -0.32% |  | 0.39% |  | -0.03% |  | -0.19% |  | -0.19% |
| Overall |  | 0.01% |  | -0.05% |  | -0.04% |  | 0.01% |  | -0.05% |  | -0.11% |
|  |  | 0.01% |  | -0.05% |  | -0.04% |  | 0.01% |  | -0.10% |  | -0.16% |
| Enc Time[%] |  | 100% |  |  |  |  |  | 100% |  |  |  |  |
| Dec Time[%] |  | 100% |  |  |  |  |  | 100% |  |  |  |  |
|  | Low delay P HE |  |  |  |  |  | Low delay P LC |  |  |  |  |  |
|  | Y |  | U |  | V |  | Y |  | U |  | V |  |
| Class A |  |  |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.03% |  | 0.00% |  | -0.05% |  | 0.01% |  | 0.02% |  | -0.03% |  |
| Class C | 0.00% |  | 0.00% |  | -0.10% |  | 0.02% |  | -0.07% |  | 0.09% |  |
| Class D | 0.04% |  | -0.08% |  | -0.10% |  | 0.00% |  | 0.03% |  | -0.02% |  |
| Class E | -0.03% |  | -0.20% |  | -0.09% |  | 0.00% |  | -0.07% |  | 0.15% |  |
| Overall | 0.01% |  | -0.06% |  | -0.08% |  | 0.01% |  | -0.02% |  | 0.04% |  |
|  | 0.01% |  | -0.08% |  | -0.07% |  | 0.01% |  | -0.01% |  | 0.01% |  |
| Enc Time[%] | 100% |  |  |  |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 99% |  |  |  |  |  | 99% |  |  |  |  |  |

# Conclusions

This contribution has presented a study and cross-verification results of removal of clipping operation to motion vector predictor candidates proposed in JCTVC-G134. The cross-verification tasks were successfully completed and it was confirmed that the implementation conforms to the description in JCTVC-G134 and results exactly match those presented in JCTVC-G134. The necessity of the clipping is also raised as one of open issues to be discussed in the WD4 document. It is recommended that this issue is discussed and fixed at the 7th JCT-VC meeting.

# References

[1] M. Coban and M. Karczewicz, “Motion Vector Predictor Candidate Clipping Removal,” JCT-VC document JCTVC-G134, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 7th meeting, Geneva, November 2011.

[2] B. Bross, et al., “WD4: Working Draft 4 of High-Efficiency Video Coding,” JCT-VC document JCTVC-F803, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 6th meeting, Torino, July 2011.

[3] F. Bossen, “Common test conditions and software reference configurations,” JCT-VC document JCTVC-F900, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 6th meeting, Torino, July 2011.

# Patent rights declaration(s)

NEC Corporation does not have any current or pending patent rights relating to the technology described in this contribution.