| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G345 WG11 Number: m 21907 |
| --- | --- |

| Title: | Non-CE7: Restricted mode-dependent 8x8 DST for Intra prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposed Draft |  |  |
| Author(s) or Contact(s): | Atsuro Ichigaya Yasuko Sugito Shin-ichi Sakaida NHK Science & Technology Research Laboratories 1-10-11, Kinuta, Setagaya-ku, Tokyo, Japan | Tel: Email: | +81-3-5494-3354 ichigaya.a-go@nhk.or.jp |
| Source: | NHK (Japan Broadcasting Corporation) |  |  |

_____________________________

# Abstract

Current HM is implemented the intra prediction mode-dependent DCT/DST at 4x4 TUs. In the previous meeting, F282 proposed the transform switching for 8x8 DST which applied DST to all 8x8 TUs. Though it achieved 0.2% gain at AI-HE, the encoding and decoding time were increased 2-3%.

This contribution proposed restricted DCT/DST switching for 8x8 DST depending on TU and PU sizes. The proposed restriction of this proposal is: when CU, PU and TU sizes are 8x8, mode-dependent DCT/DST is applied for 8x8 TU coding. This method does not require any additional signaling or any modifications from F282. The experimental results showed improvement with the BD-Rate in AIHE of 0.1 to 0.6 % and in AILC of 0.2% to 0.5%. The results showed higher gain at high resolution sequences in classes A, B, and E in particular.

# Introduction

Unified intra prediction removes correlation between contiguous pixels. Such residues have small energy alongside of reference pixels. On the other hand, the residues on the far side from reference pixels have larger energy for not having strong correlation with the reference pixels. Hence, DST type 7 is suitable for coding residue of unified intra prediction mode [1].

Current HM is implemented only in the intra prediction mode-dependent DCT/DST for 4x4 TUs. However, the 4x4 TU is not highly selected when coding higher resolution sequences. The mode-dependent DCT/DST for 8x8 TUs is available to improve the coding performance against such sequences. JCTVC-F282 [2] proposed mode dependent DCT/DST for 8x8 intra TUs. This method switches DCT and DST type 7 depending on the intra prediction directions. The results of the coding experiment showed good gain. However, the process time was also increased. In the results, the proposal was not adopted in HM4.0.

In this contribution, we propose two methods for the mode-dependent DCT/DST for 8x8 TUs and present results of each coding experiment.

# Restricted mode dependent 8x8 DCT/DST

In the case of the smallest 4x4 TU, most pixels of a block have a sufficiently strong correlation with reference pixels for the short distance between reference pixels and current pixels. With the structure, DST works well for the coding residue. In the case of 8x8 TU, the correlation is not necessarily strong. Therefore, mode-dependent DCT/DST should be restricted when applying to 8x8 TU.

In the case of intra coding, TU size is not larger than PU. We only consider the two cases: Case 1: PU equal to TU, Case 2: PU is larger than TU. When PU and TU are the same sizes, it can be considered that the prediction works well for most pixels of the TU. In the case2, it can be considered that the prediction works better in average, however, the prediction is not necessarily performing its best in each TU. Reflecting the situation, we propose the restricted mode dependent 8x8 DCT/DST by according to its PU and TU size.

Case 1. PU=TU                      Case 2. PU=4TU

There are two restrictions for the mode dependent 8x8 DCT/DST. Method 1 is rather a weaker restriction: the DCT/DST switching is applied when PU and TU are the same sizes. Method 2 is stronger restriction: it is applied when CU, PU and TU are the same sizes. Method 2 can be considered as a RQT depth dependent mode. Any other setting is the same as JCTVC-F282. We confirmed the coding performances under common condition F900 [3]. The experiment results were cross checked by MERL [4] and Samsung [5].

## Method 1

This method proposes weaker restriction for the DCT/DST switching for 8x8 and 4x4 TUs. The mode-dependent DCT/DST is applied when PU and TU are the same sizes. PU partitioning of intra coding can choose 2Nx2N and NxN mode. There are three cases which PU and TU are the same sizes for 8x8 and 4x4 TU. The combinations are shown as bellow in green boxes.

|  | TU depth |  |  |
| --- | --- | --- | --- |
| CU size | 0 | 1 | 2 |
| 64 | N/A | 32 | 16 |
| 32 | 32 | 16 | 8 |
| 16 | 16 | 8 | 4 |
| 8 | 8 | 4 | N/A |

Marked cases are to apply DCT/DST switching

|  | PU partitioning |  |
| --- | --- | --- |
| CU size | 2Nx2N | NxN |
| 64 | N/A | 32 |
| 32 | 32 | 16 |
| 16 | 16 | 8 |
| 8 | 8 | 4 |

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.6% | 0.2% | 0.2% | -0.3% | -0.4% | -0.6% |
| Class B | -0.3% | 0.3% | 0.3% | -0.1% | -0.4% | -0.4% |
| Class C | -0.2% | 0.0% | 0.1% | -0.3% | -0.3% | -0.3% |
| Class D | -0.1% | -0.1% | -0.1% | -0.3% | -0.2% | -0.2% |
| Class E | -0.4% | 1.1% | 0.8% | -0.3% | -0.2% | -0.3% |
| Overall | -0.3% | 0.3% | 0.2% | -0.2% | -0.3% | -0.4% |
|  | -0.3% | 0.2% | 0.2% | -0.3% | -0.3% | -0.4% |
| Enc Time[%] | 102% |  |  | 102% |  |  |
| Dec Time[%] | 102% |  |  | 101% |  |  |

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | 0.3% | 0.4% | -0.1% | -0.5% | -0.6% |
| Class B | -0.1% | 0.3% | 0.2% | -0.1% | -0.2% | -0.3% |
| Class C | -0.1% | 0.0% | 0.1% | -0.2% | -0.5% | -0.1% |
| Class D | -0.1% | 0.1% | -0.2% | -0.1% | -0.2% | -0.1% |
| Class E |  |  |  |  |  |  |
| Overall | -0.1% | 0.2% | 0.1% | -0.1% | -0.3% | -0.3% |
|  | -0.1% | 0.1% | 0.1% | -0.1% | -0.3% | -0.3% |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 99% |  |  | 102% |  |  |

## Method 2

This method proposed stronger restriction for 8x8 TUs. The mode-dependent DCT/DST is applied when CU, PU and TU are the same sizes. In this case, 4x4 TUs are same as current HM.

|  | TU depth |  |  |
| --- | --- | --- | --- |
| CU size | 0 | 1 | 2 |
| 64 | N/A | 32 | 16 |
| 32 | 32 | 16 | 8 |
| 16 | 16 | 8 | 4 |
| 8 | 8 | 4 | N/A |

Marked cases are to apply DCT/DST switching

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.6% | 0.1% | 0.0% | -0.4% | -0.4% | -0.7% |
| Class B | -0.2% | 0.0% | 0.0% | -0.2% | -0.5% | -0.5% |
| Class C | -0.1% | -0.2% | -0.2% | -0.3% | -0.5% | -0.5% |
| Class D | -0.1% | -0.3% | -0.3% | -0.2% | -0.3% | -0.3% |
| Class E | -0.4% | 0.7% | 0.4% | -0.5% | -0.3% | -0.4% |
| Overall | -0.3% | 0.0% | 0.0% | -0.3% | -0.4% | -0.5% |
|  | -0.3% | 0.0% | -0.1% | -0.3% | -0.4% | -0.5% |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.2% | -0.1% | -0.5% | -0.1% | -0.2% | -0.6% |
| Class B | -0.2% | 0.2% | -0.1% | -0.1% | -0.2% | -0.1% |
| Class C | -0.1% | -0.3% | -0.1% | -0.2% | -0.4% | -0.3% |
| Class D | -0.1% | 0.1% | -0.4% | -0.1% | -0.3% | -0.1% |
| Class E |  |  |  |  |  |  |
| Overall | -0.1% | 0.0% | -0.2% | -0.1% | -0.3% | -0.3% |
|  | -0.1% | 0.0% | -0.3% | -0.1% | -0.3% | -0.3% |
| Enc Time[%] | 101% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 102% |  |  |

# Complexity

Quantization & Scaling Matrices: We retain the same quantization and inverse quantization matrices as used in HM 4.0.

Transform Matrices: We only require the storage of one additional matrix corresponding to DST at block size 8x8. The 8x8 transform is implemented via full matrix multiplication. Here the norms of the basis vectors of 8x8 DST are matched to be similar to those for 8x8 DCT in HM 4.0.

const short g_as_DST_MAT_8 [8][8]=

{

{    16,    32,    46,    59,    70,    79,    84,    87,},

{    46,    79,    87,    70,    32,   -16,   -59,   -84,},

{    70,    84,    32,   -46,   -87,   -59,    16,    79,},

{    84,    46,   -59,   -79,    16,    87,    32,   -70,},

{    87,   -16,   -84,    32,    79,   -46,   -70,    59,},

{    79,   -70,   -16,    84,   -59,   -32,    87,   -46,},

{    59,   -87,    70,   -16,   -46,    84,   -79,    32,},

{    32,   -59,    79,   -87,    84,   -70,    46,   -16,},

};

Operation Counts: The matrix multiplication for 8x8 DST in our proposal requires 1024 multiplications, 1024 additions and 128 shifts. The worst cast and average cost of this proposal show below.

- Worst cost

| CU | PU | TU |  | E243 | HM4.0 | Method1 | Method2 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 16x16 | 16x16 | 16x16 | Mults | 2752 | 0 | 0 | 0 |
|  |  |  | Adds | 3712 | 0 | 0 | 0 |
|  |  |  | Shifts | 512 | 0 | 0 | 0 |
|  |  | 8x8 | Mults | 352 | 0 | 0 | 0 |
|  |  |  | Adds | 576 | 0 | 0 | 0 |
|  |  |  | Shifts | 128 | 0 | 0 | 0 |
|  |  | 4x4 | Mults | 48 | +80 | 0 | +80 |
|  |  |  | Adds | 96 | +32 | 0 | +32 |
|  |  |  | Shifts | 32 | 0 | 0 | 0 |
|  | 8x8 | 8x8 | Mults | 352 | 0 | +672 | 0 |
|  |  |  | Adds | 576 | 0 | +448 | 0 |
|  |  |  | Shifts | 128 | 0 | 0 | 0 |
|  |  | 4x4 | Mults | 48 | +80 | 0 | +80 |
|  |  |  | Adds | 96 | +32 | 0 | +32 |
|  |  |  | Shifts | 32 | 0 | 0 | 0 |
| 8x8 | 8x8 | 8x8 | Mults | 352 | 0 | +672 | +672 |
|  |  |  | Adds | 576 | 0 | +448 | +448 |
|  |  |  | Shifts | 128 | 0 | 0 | 0 |
|  |  | 4x4 | Mults | 48 | +80 | 0 | +80 |
|  |  |  | Adds | 96 | +32 | 0 | +32 |
|  |  |  | Shifts | 32 | 0 | 0 | 0 |
|  | 4x4 | 4x4 | Mults | 48 | +80 | +80 | +80 |
|  |  |  | Adds | 96 | +32 | +32 | +32 |
|  |  |  | Shifts | 32 | 0 | 0 | 0 |

- Average cost

| CU | PU | TU |  | E243 | HM4.0 | Method1 | Method2 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 16x16 | 16x16 | 16x16 | Mults | 2752.0 | 0.0 | 0.0 | 0.0 |
|  |  |  | Adds | 3712.0 | 0.0 | 0.0 | 0.0 |
|  |  |  | Shifts | 512.0 | 0.0 | 0.0 | 0.0 |
|  |  | 8x8 | Mults | 352.0 | 0.0 | 0.0 | 0.0 |
|  |  |  | Adds | 576.0 | 0.0 | 0.0 | 0.0 |
|  |  |  | Shifts | 128.0 | 0.0 | 0.0 | 0.0 |
|  |  | 4x4 | Mults | 48.0 | +11.4 | 0.0 | +11.4 |
|  |  |  | Adds | 96.0 | +17.1 | 0.0 | +17.1 |
|  |  |  | Shifts | 32.0 | 0.0 | 0.0 | 0.0 |
|  | 8x8 | 8x8 | Mults | 352.0 | 0.0 | +480.0 | 0.0 |
|  |  |  | Adds | 576.0 | 0.0 | +320.0 | 0.0 |
|  |  |  | Shifts | 128.0 | 0.0 | 0.0 | 0.0 |
|  |  | 4x4 | Mults | 48.0 | +11.4 | 0.0 | +11.4 |
|  |  |  | Adds | 96.0 | +17.1 | 0.0 | +17.1 |
|  |  |  | Shifts | 32.0 | 0.0 | 0.0 | 0.0 |
| 8x8 | 8x8 | 8x8 | Mults | 352.0 | 0.0 | +480.0 | +672.0 |
|  |  |  | Adds | 576.0 | 0.0 | +320.0 | +448.0 |
|  |  |  | Shifts | 128.0 | 0.0 | 0.0 | 0.0 |
|  |  | 4x4 | Mults | 48.0 | +11.4 | 0.0 | +11.4 |
|  |  |  | Adds | 96.0 | +17.1 | 0.0 | +17.1 |
|  |  |  | Shifts | 32.0 | 0.0 | 0.0 | 0.0 |
|  | 4x4 | 4x4 | Mults | 48.0 | +11.4 | +11.4 | +80.0 |
|  |  |  | Adds | 96.0 | +17.1 | +17.1 | +32.0 |
|  |  |  | Shifts | 32.0 | 0.0 | 0.0 | 0.0 |

Bit Shift at Intermediate Levels: In our implementation, we shift the output of 8x8 DST (used either as vertical or horizontal transform) by 7-bits similar to the 7-bit shift for DCT in HM 4.0.

Scanning: In this proposal we have retained the conventional HM 4.0 scans

Mapping table of DCT/DST applying:

|  | Mode | Vertical Transform | Horizontal Transform |
| --- | --- | --- | --- |
| 0 | PLANAR | DST | DST |
| 1 | VER | DST | DCT |
| 2 | HOR | DCT | DST |
| 3 | DC | DCT | DCT |
| 4 | VER-8 | DST | DST |
| 5 | VER-4 | DST | DST |
| 6 | VER+4 | DST | DCT |
| 7 | VER+8 | DST | DCT |
| 8 | HOR-4 | DST | DST |
| 9 | HOR+4 | DCT | DST |
| 10 | HOR+8 | DCT | DST |
| 11 | VER-6 | DST | DST |
| 12 | VER-2 | DST | DST |
| 13 | VER+2 | DST | DCT |
| 14 | VER+6 | DST | DCT |
| 15 | HOR-6 | DST | DST |
| 16 | HOR-2 | DST | DST |
| 17 | HOR+2 | DCT | DST |
| 18 | HOR+6 | DCT | DST |
| 19 | VER-7 | DST | DST |
| 20 | VER-5 | DST | DST |
| 21 | VER-3 | DST | DST |
| 22 | VER-1 | DST | DST |
| 23 | VER+1 | DST | DCT |
| 24 | VER+3 | DST | DCT |
| 25 | VER+5 | DST | DCT |
| 26 | VER+7 | DST | DCT |
| 27 | HOR-7 | DST | DST |
| 28 | HOR-5 | DST | DST |
| 29 | HOR-3 | DST | DST |
| 30 | HOR-1 | DST | DST |
| 31 | HOR+1 | DCT | DST |
| 32 | HOR+3 | DCT | DST |
| 33 | HOR+5 | DCT | DST |
| 34 | HOR+7 | DCT | DST |

# Conclusion

In this contribution, we have proposed two restrictions for the mode-dependent DCT/DST. Method 1 achieves 0.3% gain for AI-HE and 0.2% for AI-LC. Method 2 achieves 0.3% gain for both AI-HE and LC. Both methods showed almost same amount of gains. However, encoding/decoding process time of Method 1 increased 2% and the time of Method 2 was almost same as the anchor. Furthermore, this method is effective in coding high resolution sequences in Class A, B and E. It is recommended to adopt Method 2 for HM5 and WD5.

# References

- J. Han, A. Saxena and K. Rose, “Towards jointly optimal spatial prediction and adaptive transform in video/image coding,” IEEE International Conference on Acoustics, Speech and Signal Processing (ICASSP), March 2010, pp. 726-729.

- A. Saxena and F. Fernandes, “CE7: Mode-dependent 8x8 DCT/DST for Intra Prediction,” ITU-T Doc. JCTVC-F282, Torino, IT, July 2011.

- “Common test conditions and software reference configurations.” ITU-T Doc. JCTVC-F900, Torino, IT, July 2011

- R. Cohen, “Non-CE7: Cross verification of JCTVC-G345, Restricted mode-dependent 8x8 DST for Intra prediction,” ITU-T Doc. JCTVC-G817, Geneva, CH, Nov. 2011.

- A. Saxena, “Cross Check of JCTVC-G345: Restricted Mode-Dependent 8x8 DST for Intra prediction” ITU-T Doc. JCTVC-G630, Geneva, CH, Nov. 2011.

# Patent rights declaration(s)

NHK (Japan Broadcasting Corporation) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).