| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G348 |
| --- | --- |

| Title: | Cross-check report of NTT's intra prediction approach (JCTVC-G350) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Tomonobu Yoshino Sei Naito 2-1-15, Ohara, Fujimino-shi, Saitama, JAPAN | Tel: Email: | +81 49 278 7587 to-yoshino@kddi.com |
| Source: | KDDI Corp. (KDDI R&D Laboratories Inc.) |  |  |

_____________________________

# Abstract

This contribution reports cross-check results of JCTVC-G 350.

# Introduction

Intra prediction approach by NTT is presented in document JCTVC-G350. This contribution reports the cross-check result of this scheme. The software version was HM4.0. The reference was generated by using the common test conditions [1]. We compiled provided source code on 64-bit WindowsXP environment, and the experiments were performed on the same PCs.

Table 1 Experimental results for All-Intra conditions

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | -0.2% | -0.1% | -0.1% | -0.2% | -0.2% |
| Class B | -0.4% | -0.4% | -0.5% | -0.5% | -0.7% | -0.7% |
| Class C | -0.6% | -0.7% | -0.7% | -0.8% | -0.9% | -1.0% |
| Class D | -0.5% | -0.5% | -0.6% | -0.6% | -0.7% | -0.7% |
| Class E | -0.6% | -0.6% | -0.4% | -0.7% | -0.6% | -0.6% |
| Overall | -0.4% | -0.5% | -0.5% | -0.5% | -0.6% | -0.6% |
|  | -0.4% | -0.5% | -0.5% | -0.5% | -0.6% | -0.6% |
| Enc Time[%] | 103% |  |  | 103% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

- F. Bossen, “Common test conditions and software reference configurations,” JCTVC-F900, 6th. JCT-VC Meeting, Torino, IT, July 2011.

# Conclusions

We have verified results reported in JCTVC-G350. The results of the cross-check matched with NTT's report. The encoding/decoding time also matched closely to those reported by the proponent.