| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G349 WG11 Number: m21911 |
| --- | --- |

| Title: | CE1: Subset A: Parallel context processing for coefficient coding using block-based context updates |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hisao Sasai Takahiro Nishi 1006 Kadoma, Kadoma, Osaka, Japan | Tel: Email: | +81 6 6900 9689 sasai.hisao@jp.panasonic.com |
| Source: | Panasonic Corporation |  |  |

_____________________________

# Abstract

This contribution is a test report for JCTVC-E226 listed in CE1 Subset A. Proposed technique is aimed to improve the throughput of the entropy coder for CABAC. The context updates make difficult to increase throughput due to their serial dependencies. The proposed modifications have been implemented in HMv4 and their coding efficiencies were evaluated for coefficient cording parameters "last_significant_coeff_x","last_significant_coeff_y","significant_coeff_flag","coeff_abs_level_greater1_flag"and"coeff_abs_level_greater2_flag" respectively. The parallelization capability by the proposal comes at a cost of less than 0.1% performance loss.

- Introduction

The CABAC has high serial dependencies from multiple feedback loop, which poses practical problems for real-time processing of high resolution video sequences. It is difficult to increase throughput due to the dependencies. At both of encoder and decoder, "range update" and "context update" are performed on feedback loop. In the current WD[1], same context index is called continuously (bin by bin), e.g. "significant_coeff_flag" in high frequency. It is the bottleneck limiting the bin throughput of the whole process.

- Proposed Solution

## BBCU (Block based context updates) for coefficient coding parameters

For coefficient coding operation, "last_significant_coeff_x", "last_significant_coeff_y", "significant_coeff_flag", "coeff_abs_level_greater1_flag" and "coeff_abs_level_greater2_flag" are encoded and decoded in same block loop. Figure 1 shows the operation flow of CABAC. Within the block loop, first of all context selection is conducted. Then, the probability ( state of context ) corresponding the selected context is loaded. At the encoder, a target bin for significance map is encoded by using the probability. At the decoder, a target bin for significance map is parsed by using the probability. In this stage, the range in arithmetic coder is updated bin by bin. Therefore, the operation  "Bin Code" has serial dependency on following bins. Finally, the probability is updated by encoded or parsed bin. The updated probability is used for following bin operation. Accordingly, the operation "Context Update on Current States" has also serial dependency on following probability derivation.

In proposed solution, temporal storing process and set process are added as shown in Figure 2. At the beginning of the operation of coefficient parameter coding per block, current state of context are stored to temporal memory. Then the same operation as CABAC is conducted within the block loop except for the operation "Context Update on Stored States". The difference is which of stored or current probability is updated at this stage. In our solution, stored probability is updated, but current probability which is used for encoding or decoding is no changed. Therefore, there is no dependency on the probability within the block loop. Finally, stored probability (state of context ) which is updated within the block loop is set to current probability (state of context) as following block operation.

Since the additional operation is out of block loop, the additional operation is limited.

Figure 1. Operation flow for CABAC.   	Figure 2. Operation flow for proposal method.

## The benefit of proposed solution

As an example for simple architecture, a single operation flow can be assumed. The following conditions are used for the worst case cycle calculation.

- 1 cycle for Context Selection, Context Load, Bin Code and Context Update.

- Context selection can be done simultaneously when there is no dependency.

- Context load can be done simultaneously when there is no dependency.

- Context can be loaded after Context Update.

- No pre-calculation processing is used.

Figure 3. shows the difference in worst case between proposed solution and current HMv4 for "coeff_abs_level_greater2_flag".

Figure 3. the difference between HMv4 and proposed method in worst case for coeff_abs_level_greater2_flag.

Note: worst case is all sub-block (4x4) coefficients are greater than 2.

The context selection for coeff_abs_level_greater2_flag depends on the coefficient position. After 5 coefficient position in scan order, same context (Ctx E) is used for left of coefficients.

The worst case cycle number 41 (in HMv4) can be reduced to 19 for sub-block (4x4) (54% reduction).

The number of cycles in the worst case can be calculated in the same manner ( Table 1 ).

Table 1. The number of cycles in the worst case for coefficient coding parameters

As another example for hardware architecture, the multiple bins per cycle operation can be assumed [2].  For those architectures, we can modify the method of multiple bin processing for bypass mode to apply not equal probabilities. For "coeff_abs_level_greater2_flag" in HMv4, same contexts are used for 12 sequential bins. By using BBCU, we can use fixed probabilities for those successive bin coding. For those operation, additional range-update pre-calculators would be needed to be realized more than 2 bins per cycle.

- Results

The proposal method is implemented in HM4.0 and checked under the common test condition described in JCTVC-F900 [3]. The computing platform is Linux 64bits and the executables were compiled on 64-bit Linux with gcc 4.4.3.

The following tool set is tested on CE1 Subset A[4];

TEST1. 	All coefficient coding parameters

TEST2.		All coefficient coding parameters except for "significant_coeff_flag"

TEST3.		"last_significant_coeff_x" and "last_significant_coeff_y"

TEST4.		"coeff_abs_level_greater1_flag"and"coeff_abs_level_greater2_flag"

TEST5.		"significant_coeff_flag"

The summary of those test are listed in Table 2. The detail results are available in excel sheet.

Table 2. BD BR for summary results

|  | All Intra HE |  |  | Random Access HE |  |  | Low delay B HE |  |  | Low delay P HE |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V | Y | U | V |
| TEST1 | 0.2% | 0.3% | 0.3% | 0.1% | 0.1% | 0.0% | 0.1% | -0.1% | 0.0% | 0.1% | -0.2% | -0.2% |
| TEST2 | 0.1% | 0.1% | 0.1% | 0.0% | 0.0% | -0.1% | 0.0% | -0.2% | -0.1% | 0.0% | -0.1% | -0.3% |
| TEST3 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| TEST4 | 0.1% | 0.1% | 0.1% | 0.0% | 0.1% | -0.1% | 0.0% | -0.2% | 0.0% | 0.0% | 0.0% | -0.1% |
| TEST5 | 0.1% | 0.2% | 0.2% | 0.1% | 0.2% | 0.1% | 0.1% | -0.1% | 0.1% | 0.1% | 0.2% | -0.1% |

The recommended combination is TEST 2. The BD BR and execution time is shown in Table 3.

Table 3. BD BR for Qualcomm's proposal on Removal of the parsing dependency of residual coding on intra mode (high efficiency)

- Conclusion

In this contribution, a coefficient coding using block-based context updates for CABAC is proposed. The proposed method can improve CABAC worst case cycles. An experimental result shows less than 0.1% performance drop in average BD-rate with improving CABAC throughput. It is suggested to consider the inclusion of this proposed method in the HM/WD of HEVC.

- References

- JCT-VC, “WD4: Working Draft 4 of High-Efficiency Video Coding”, JCTVC-F803, JCT-VC 6th Meeting, Torino, July, 2011.

- B. Shi, W. Zheng, H.-S. Lee, D.-X. Li, and M. Zhang, “Pipelined Architecture Design of H.264/AVC CABAC Real-Time Decoding,” in IEEE International Conference on Circuits and Systems for Communications, May 2008, pp. 492 –496.

- Frank Bossen, “Common test conditions and software reference configurations”, JCTVC-F900, Torino, July 2011.

- Rajan Joshi et al., “Description of Core Experiment 1: Entropy Coding”, JCTVC-F901, Torino, July 2011.

# Patent rights declaration(s)

Panasonic Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).