| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G350_r1 |
| --- | --- |

| Title: | Modification of angular intra prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shohei Matsuo Seishi Takamura Hirohisa Jozawa 1-1, Hikari-no-oka, Yokosuka-shi, Kanagawa 239-0847, JAPAN | Tel: Email: | +81-46-859-2070 matsuo.shohei@lab.ntt.co.jp |
| Source: | NTT (Nippon Telegraph and Telephone) Corporation |  |  |

_____________________________

# Abstract

This document introduces an angular intra prediction method exploiting interpolation technique whose tap length is more than 2. In the proposed method, reference samples for angular intra prediction are generated by either 4-tap DCT-IF or conventional 2-tap filter. The proposed intra prediction is applied to chrominance components as well as luminance component. Compared to the HM4.0 anchor, in the HE configuration, the overall average coding gains of Y, Cb and Cr were about 0.4%, 0.5% and 0.5%, respectively. Regarding LC configuration, those of Y, Cb and Cr were about 0.5%, 0.6% and 0.6%, respectively. The maximum coding gains of Y, Cb and Cr were about 1.9%, 1.8% and 2.2%, respectively, for the sequence “BasketballDrill” in LC case. In the HE configuration, the encoding and decoding run-times of the proposal were about 100.75% and 100.34% against the anchor on average, respectively. In the LC configuration, those of the proposal were about 101.67% and 100.66%, respectively. Additional experimental results showed that the proposal provided about 0.1 to 0.4% coding gain on average for inter-frame coding.

# Introduction

In HM4.0, intra prediction modes are defined as follows: planar (mode 0), vertical (mode 1), horizontal (mode 2), DC (mode 3), angular (mode 4 to 34) and fromLuma (mode 35: this mode is used only for chroma). In the angular mode, 31 prediction directions are prepared as shown in Sec 8.3.1 of Working Draft [1]. The number of the prediction directions of HM is much larger than that of H.264/AVC standard. The angular prediction of HM can generate more flexible prediction signals and reduce effectively the prediction error energy. When the angular prediction mode is selected, in some prediction modes, the reference samples are generated by linear interpolation. Specifically, the following equation is used for generating the prediction samples.

predSamples[ x, y ] = ( ( 32 – iFact )*refMain[ x+iIdx+1 ] + iFact*refMain[ x+iIdx+2] + 16 ) >> 5

The array predSamples[ x, y ] and refMain[ x, y ] denote the predicted samples and the reference pixels, respectively. The parameter iIdx and iFact denote the index and the multiplication factor determined by the intra prediction mode. From the equation above, the predicted samples are generated by adding the weighted values of neighboring reference pixels. In other words, the reference pixels are interpolated by 2-tap simple interpolation filter. In this contribution, an interpolation filter whose tap length is more than 2 is applied to the generation of reference pixels for angular intra prediction.

# Angular intra prediction exploiting longer-tap interpolation

## Basic idea

This section explains the motivation of this proposal. Assume that Ai,j and Pi,j are the locally-decoded reference pixels and the pixels to be predicted, respectively, as shown in Figure 1. The index i and j denote spatial coordinates for horizontal and vertical position, respectively. When the vertical mode is selected, for example, the value of A0,0 is copied to P0,0, P0,1, …, P0,b-1. The parameter b denotes the prediction block size. When the angular modes are selected, the reference pixels such as A,0 (0<<1) are generated to make the prediction signals. In the current implementation of HM4.0, the reference pixels are generated by 2-tap linear interpolation filter. The filter tap length is fixed regardless of the characteristics of the input image and coding information.

When the prediction block size is larger, the texture tends to be flat and simple. Each reference pixel may have the similar characteristics. Therefore, the 2-tap interpolation of HM4.0 may be enough. In addition, when the prediction block size is larger, the pixels at the bottom-right corner in the prediction block are far from the reference pixels. There is a possibility that the effect of modification of interpolation filter is small. However, when the prediction block size is smaller, the texture would be complicated. In addition, the pixels at the bottom-right corner in the prediction block are close to the reference pixels. Therefore, the longer-tap interpolation may be able to generate the prediction signals that reduce the prediction error energy.

DCT-based Interpolation Filter (DCT-IF) was proposed in the contribution [2, 3] and it is improved and currently employed in HM4.0 because of its effectiveness and unified design. In this contribution, when the angular intra predictions are selected, n-tap (n>2) DCT-IF is applied to smaller block sizes. DCT-IF approach is currently applied to the interpolation of the proposal. However, of course, other interpolation approaches can be also easily applied to the proposal.

Figure 1: Angular intra prediction.

## Encoding algorithm

The detailed encoding algorithm is summarized as follows.

S1: Set m that indicates the threshold value of the prediction block size.

S2: Input the block size of intra prediction. If the block size is larger than mxm, the tap length of the interpolation filter for angular prediction is set to 2. Otherwise, the tap length of interpolation filter for angular prediction is set to n (n>2).

S3: Input the value of the intra prediction mode. If the input value is equal to the angular prediction, go to S4. Otherwise, the corresponding conventional intra prediction process is performed as defined in Working Draft [1]. After the process is done, go to S5.

S4: Angular prediction is done by using the interpolation filter defined in S2. Specifically, if the tap length is equal to 2, the conventional angular prediction is performed. Otherwise, n-tap interpolation filter is used for generating the reference pixels. After the angular prediction is done, go to S5.

S5: Move on to the next prediction block. Repeat the loop (S1 to S5) until all the intra prediction process is completed.

## Intra prediction parameter syntax

In the current implementation, the proposed encoder and decoder share the tap length n and the threshold value of the block size m. Therefore, there is no additional semantics related to the proposed intra prediction.

## Decoder description

In Sec. 8.3.3.1.6 of Working Draft [1], the specification of angular prediction is described. In the section, the equation (8-44) is defined as follows.

predSamples[ x, y ] = ( ( 32 – iFact )*refMain[ x+iIdx+1 ] + iFact*refMain[ x+iIdx+2] + 16 ) >> 5

Assume that the 4-tap interpolation filter is employed. The corresponding equation is replaced as follows.

predSamples[ x, y ] = ( a0*refMain[ x+iIdx] + a1*refMain[x+iIdx+1]

+ a2*refMain[x+iIdx+2] + a3*refMain[x+iIdx+3] + r ) >> s

In the equation above, ai denotes the interpolation filter coefficients. The coefficients ai, the offset parameter r and the shift parameter s are determined by the employed interpolation filter.

# Experimental results

The proposal was implemented in HM 4.0. It is applied to chrominance components as well as luminance component. The configurations of the proposal are subject to the following conditions:

- Threshold value of prediction block size: m=16

- Tap length: 4-tap (n=4)

- Filter type: DCT-IF [2, 3] with the smoothing parameter =0.07

The other configurations are subject to the common test condition [4].

The objective performance is shown in Table 1. The gains (BD-rate [5]) against the anchor of HM 4.0 were calculated. In the HE configuration, the average gains were about 0.4 %, 0.5% and 0.5% for Y, Cb and Cr, respectively. In the LC configuration, those were 0.5 %, 0.6% and 0.6% for Y, Cb and Cr, respectively. The maximum coding gains of Y, Cb and Cr were about 1.9%, 1.8% and 2.2% for the sequence “BasketballDrill” in LC case, respectively. It is confirmed that the proposal is effective for Class C.

Regarding the computational complexity, in the HE configuration, the encoding and decoding run-times against the anchor were about 100.75% and 100.34% on average, respectively. In the LC configuration, those were about 101.67% and 100.66% on average, respectively.

To confirm the influence for the inter-frame coding, the results of other GOP structures were also obtained. The results are shown in Table 2 in the end of this document (7 Appendix). It is confirmed that the coding performance is improved and that the increment of run-time is around 1% or less.

Table 1: Simulation results of intra-frame coding.

# Conclusion

In this contribution, an angular intra prediction using longer-tap interpolation was proposed. Average gains of coding efficiency against the anchor HM4.0 of HE case were about 0.4%, 0.5% and 0.5% for Y, Cb and Cr, respectively. Those of LC case were about 0.5 %, 0.6% and 0.6% for Y, Cb and Cr, respectively. In the HE configuration, the average encoding and decoding run-times of the proposal were about 100.75% and 100.34% against the anchor, respectively. In the LC configuration, those of the proposal were about 101.67% and 100.66%, respectively. Other interpolation approaches as well as DCT-IF can be easily applied to the proposal.

It is recommended to incorporate the proposal into intra prediction method of HEVC.

Thanks to KDDI for the cross-check of the coding results and the run-times [6].

# References

[1] B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan and T. Wiegand, “WD4: Working Draft 4 of High-Efficiency Video Coding,” JCTVC-F803_d5, 6th. JCT-VC Meeting, Torino, IT, 14-22 July, 2011.

[2] K. McCann, W.-J. Han, I.-K. Kim, J.-H Min, E. Alshina, A. Alshin, T. Lee, J. Chen, V. Seregin, S. Lee, Y.-M. Hong, M.-S. Cheon and N. Shlyakhov, “Samsung’s Response to the Call for Proposals on Video Compression Technology,” JCTVC-A124_r2, 1st. JCT-VC Meeting, Dresden, DE, 15-23 April, 2010.

[3] E. Alshina, J. Chen, A. Alshin, N. Shlyakhov, and W.-J. Han, “CE3: Experimental results of DCTIF by Samsung,” JCTVC-D344_r1, 4th. JCT-VC Meeting, Daegu, KR, 20-28 January, 2011.

[4] F. Bossen, “Common test conditions and software reference configurations,” JCTVC-F900, 6th. JCT-VC Meeting, Torino, IT, 14-22 July, 2011.

[5] G. Bjontegaard, “Calculation of average PSNR differences between RD-curves,” ITU-T SG16 Q.6 Document, VCEG-M33, 13th VCEG Meeting, Austin, Texas, USA, April 2001.

[6] T. Yoshino and S. Naito, “Cross-check report of NTT's intra prediction approach (JCTVC-G350),” JCTVC-G348, 7th. JCT-VC Meeting, Geneva, CH, 21-30 November, 2011.

# Patent rights declaration(s)

NTT Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Appendix

The inter-frame coding performance with the proposal is shown in the following table.

Table 2: Simulation results of inter-frame coding.