| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G352 WG11 Number: m21914 |
| --- | --- |

| Title: | Parameterization of Default Quantization Matrices |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ehsan Maani, Munsi Haque, Ali Tabatabai 1730 N 1st Street San Jose, CA 95112 | Tel: Email: | +1-408-352-4099 ehsan.maani@am.sony.com munsi.haque@am.sony.com ali.tabatabai@am.sony.com |
| Source: | Sony Electronics Inc. ./ Sony Corp. |  |  |

_____________________________

# Abstract

Default quantization matrices have to be stored in the decoder for the case where no explicit visual scaling matrices are transmitted in the bit stream. These matrices, in AVC case where explicitly defined and stored in decoder since the transform sizes included only 8x8 and 4x4 transform. However, in HEVC, with additional 16x16, 32x32, and non-square transforms the memory size to store these matrices may be excessive. This document presents a method to parameterize the default matrices such that they can be efficiently stored at the decoder memory. When these matrices are needed they can be derived using simple operations. Using this technique can save up to 8 KB of ROM compared to explicit representation of the matrices.

- Parameterization of visual matrices

Many different parameterization technique were used in the literature to derive the best visual quantization matrices as part of an optimization. Such parameterization was necessary since to limit the degree of freedom and thus be able to solve the optimization problem efficiently. Among the best of these techniques is the symmetric quadratic model (with 4 parameters)[1]. Many of the existing general purpose quantization matrices were either derived using such parameterization as part of an optimization or can be efficiently represented using this model (with small residual error). In this document, we use the  AVC's default matrices to derive a set of parameters that can efficiently represent these matrices. Nevertheless, this technique may also be applied to other quantization matrices if they are shown to result in better visual quality for HEVC.

Given a set of 4 integer parameters, , the value of the quantization matrix for the location  is obtain by

,

where represents the bit depth precision and . Using this model, the 4 parameters can be derived for each quantization matrix using a least square fit. Then, only these parameters are stored in the decoder (and the encoder) as default matrices. During the run time, if needed, parameters can be used to derive the exact quantization matrices. In this document, we show the concept of QM parameterization using AVC's 4x4 and 8x8 quantization matrices. A bit precision of is also assumed to derive the following quantization matrices and their associated parameters:

Table 1. AVC Intra 4x4 matrix parameterization.

| AVC Intra | Parametric | Error |
| --- | --- | --- |
| 6 13 20 28 13 20 28 32 20 28 32 37 28 32 37 42 | 6 13 21 27 13 20 27 33 21 27 32 37 27 33 37 42 | 0 0 -1 1 0 0 1 -1 -1 1 0 0 1 -1 0 0 |

Table 2. AVC Inter 4x4 matrix parameterization.

| AVC Inter | Parametric | Error |
| --- | --- | --- |
| 10 14 20 24 14 20 24 27 20 24 27 30 24 27 30 34 | 10 15 20 24 15 19 24 27 20 24 27 31 24 27 31 33 | 0 -1 0 0 -1 1 0 0 0 0 0 -1 0 0 -1 1 |

Table 3. AVC Intra 8x8 quantization matrix parameterization.

| AVC Intra | Parametric | Error |
| --- | --- | --- |
| 6 10 13 16 18 23 25 27 10 13 16 18 23 25 27 29 13 16 18 23 25 27 29 31 16 18 23 25 27 29 31 33 18 23 25 27 29 31 33 36 23 25 27 29 31 33 36 38 25 27 29 31 33 36 38 40 27 29 31 33 36 38 40 42 | 7 10 13 16 19 22 24 27 10 13 16 19 22 24 27 29 13 16 19 22 24 27 29 32 16 19 22 24 27 29 32 34 19 22 24 27 29 32 34 36 22 24 27 29 32 34 36 38 24 27 29 32 34 36 38 40 27 29 32 34 36 38 40 41 | -1 0 0 0 -1 1 1 0 0 0 0 -1 1 1 0 0 0 0 -1 1 1 0 0 -1 0 -1 1 1 0 0 -1 -1 -1 1 1 0 0 -1 -1 0 1 1 0 0 -1 -1 0 0 1 0 0 -1 -1 0 0 0 0 0 -1 -1 0 0 0 1 |

Table 4. AVC Inter 8x8 quantization matrix parameterization.

| AVC Inter | Parametric | Error |
| --- | --- | --- |
| 9 13 15 17 19 21 22 24 13 15 17 19 21 22 24 25 15 17 19 21 22 24 25 27 17 19 21 22 24 25 27 28 19 21 22 24 25 27 28 30 21 22 24 25 27 28 30 32 22 24 25 27 28 30 32 33 24 25 27 28 30 32 33 35 | 11 13 15 17 19 20 22 24 13 15 17 19 20 22 24 25 15 17 19 20 22 24 25 27 17 19 20 22 24 26 27 29 19 20 22 24 26 27 29 30 20 22 24 26 27 29 30 32 22 24 25 27 29 30 32 33 24 25 27 29 30 32 33 34 | -2 0 0 0 0 1 0 0 0 0 0 0 1 0 0 0 0 0 0 1 0 0 0 0 0 0 1 0 0 -1 0 -1 0 1 0 0 -1 0 -1 0 1 0 0 -1 0 -1 0 0 0 0 0 0 -1 0 0 0 0 0 0 -1 0 0 0 1 |

This method can also be combined with scaling to results in new quantization matrices based on previously known matrices of different size. For example, by employing this mode, quantization matrices for large transform sizes in HEVC (i.e., 16x16 and 32x32) are derived below using AVC's 8x8 quantization matrices. The parameters for all cases and transform sizes are shown in the table below:

Table 5. Parameters to be stored for each transform size.

| Transform Size | Slice Type | Parameters (a, b, c, d) |
| --- | --- | --- |
| 4x4 | Intra | -256, -901, 8264, 5652 |
| 8x8 | Intra | -56, -127, 3364, 6898 |
| 16x16 | Intra | -14, -32, 1682, 6898 |
| 32x32 | Intra | -4, -8, 841, 6898 |
| 4x4 | Inter | -192, -563, 5440, 9907 |
| 8x8 | Inter | -28, -50, 2070, 11260 |
| 16x16 | Inter | -7, -13, 1035, 11260 |
| 32x32 | Inter | -2, -3, 517, 11260 |

Some of matrices for large size transforms are shown as examples in the Appendix. Furthermore, the default matrices proposed in [2] are also parameterized in the Appendix to show the effectiveness of this technique in representing other quantization matrices.

# Conclusion

This proposal shows how a 4-parameter model can efficiently represent a set of large quantization matrices. The error in these quantization matrices due to parameterization is negligible and will not visibly impact the visual quality. However, it can save significant amount of memory (more than 8 KB of ROM) at both the encoder and the decoder.  Another advantage of this technique compared to other methods that derive large transform's quantization matrices from smaller matrices is the additional degree of freedom it provides for larger matrices to be radically different from 4x4 or 8x8. Furthermore, updating the matrices through their parameters is relatively easy (compared to modifying the derivation logic of large matrices from 4x4 or 8x8) if later a better visual quantization matrix is found.

# Patent rights declaration(s)

Sony Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- Huipin Zhang and Guy Cote, "Modeling Quantization Matrices for Perceptual Image / Video Encoding", Visual Communications and Image Processing 2008, Proc. of SPIE-IS&T Electronic Imaging, SPIE Vol. 6822, 682210

- J. Tanaka, Y. Morigami, and T. Suzuki, " Quantization Matrix for HEVC," JCTVC-E073, March 2011, Geneva, CH.

# Appendix

## Large transform matrices

Using the parameters of , we demonstrate some of the larger size quantization matrices that result from these values as examples:

Table 6. Intra 16x16 parameterized quantization matrix.

7 8 10 12 13 15 16 18 19 20 22 23 24 26 27 28 8 10 12 13 15 16 18 19 20 22 23 24 26 27 28 29 10 12 13 15 16 18 19 20 22 23 24 26 27 28 29 31 12 13 15 16 18 19 20 22 23 24 26 27 28 29 31 32 13 15 16 18 19 20 22 23 24 26 27 28 29 31 32 33 15 16 18 19 20 22 23 24 26 27 28 29 30 32 33 34 16 18 19 20 22 23 24 26 27 28 29 30 32 33 34 35 18 19 20 22 23 24 26 27 28 29 30 32 33 34 35 36 19 20 22 23 24 26 27 28 29 30 32 33 34 35 36 37 20 22 23 24 26 27 28 29 30 32 33 34 35 36 37 38 22 23 24 26 27 28 29 30 32 33 34 35 36 37 38 39 23 24 26 27 28 29 30 32 33 34 35 36 37 38 39 40 24 26 27 28 29 30 32 33 34 35 36 37 38 39 40 40 26 27 28 29 31 32 33 34 35 36 37 38 39 40 40 41 27 28 29 31 32 33 34 35 36 37 38 39 40 40 41 42 28 29 31 32 33 34 35 36 37 38 39 40 40 41 42 43

Table 7. Inter 16x16 parameterized quantization matrix.

11 12 13 14 15 16 17 18 19 20 20 21 22 23 24 25 12 13 14 15 16 17 18 19 20 20 21 22 23 24 25 25 13 14 15 16 17 18 19 20 20 21 22 23 24 25 25 26 14 15 16 17 18 19 20 20 21 22 23 24 25 25 26 27 15 16 17 18 19 20 20 21 22 23 24 25 25 26 27 28 16 17 18 19 20 20 21 22 23 24 25 25 26 27 28 29 17 18 19 20 20 21 22 23 24 25 25 26 27 28 29 29 18 19 20 20 21 22 23 24 25 25 26 27 28 29 29 30 19 20 20 21 22 23 24 25 25 26 27 28 29 29 30 31 20 20 21 22 23 24 25 25 26 27 28 29 29 30 31 31 20 21 22 23 24 25 25 26 27 28 29 29 30 31 31 32 21 22 23 24 25 25 26 27 28 29 29 30 31 31 32 33 22 23 24 25 25 26 27 28 29 29 30 31 31 32 33 33 23 24 25 25 26 27 28 29 29 30 31 31 32 33 33 34 24 25 25 26 27 28 29 29 30 31 31 32 33 33 34 35 25 25 26 27 28 29 29 30 31 31 32 33 33 34 35 35

Table 8. Intra 32x32 parametrized quantization matrix.

7 8 8 9 10 11 12 12 13 14 15 15 16 17 17 18 19 20 20 21 22 22 23 24 24 25 25 26 27 27 28 28 8 8 9 10 11 12 12 13 14 15 15 16 17 17 18 19 20 20 21 22 22 23 24 24 25 25 26 27 27 28 28 29 8 9 10 11 12 12 13 14 15 15 16 17 17 18 19 20 20 21 22 22 23 24 24 25 25 26 27 27 28 28 29 30 9 10 11 12 12 13 14 15 15 16 17 17 18 19 20 20 21 22 22 23 24 24 25 25 26 27 27 28 28 29 30 30 10 11 12 12 13 14 15 15 16 17 17 18 19 20 20 21 22 22 23 24 24 25 25 26 27 27 28 28 29 30 30 31 11 12 12 13 14 15 15 16 17 17 18 19 20 20 21 22 22 23 24 24 25 25 26 27 27 28 28 29 30 30 31 31 12 12 13 14 15 15 16 17 17 18 19 20 20 21 22 22 23 24 24 25 25 26 27 27 28 28 29 30 30 31 31 32 12 13 14 15 15 16 17 17 18 19 20 20 21 22 22 23 24 24 25 25 26 27 27 28 28 29 30 30 31 31 32 32 13 14 15 15 16 17 17 18 19 20 20 21 22 22 23 24 24 25 25 26 27 27 28 28 29 30 30 31 31 32 32 33 14 15 15 16 17 17 18 19 20 20 21 22 22 23 24 24 25 25 26 27 27 28 28 29 30 30 31 31 32 32 33 33 15 15 16 17 17 18 19 20 20 21 22 22 23 24 24 25 25 26 27 27 28 28 29 30 30 31 31 32 32 33 33 34 15 16 17 17 18 19 20 20 21 22 22 23 24 24 25 25 26 27 27 28 28 29 30 30 31 31 32 32 33 33 34 34 16 17 17 18 19 20 20 21 22 22 23 24 24 25 25 26 27 27 28 28 29 30 30 31 31 32 32 33 33 34 34 35 17 17 18 19 20 20 21 22 22 23 24 24 25 25 26 27 27 28 28 29 30 30 31 31 32 32 33 33 34 34 35 35 17 18 19 20 20 21 22 22 23 24 24 25 25 26 27 27 28 28 29 30 30 31 31 32 32 33 33 34 34 35 35 36 18 19 20 20 21 22 22 23 24 24 25 25 26 27 27 28 28 29 30 30 31 31 32 32 33 33 34 34 35 35 36 36 19 20 20 21 22 22 23 24 24 25 25 26 27 27 28 28 29 30 30 31 31 32 32 33 33 34 34 35 35 36 36 37 20 20 21 22 22 23 24 24 25 25 26 27 27 28 28 29 30 30 31 31 32 32 33 33 34 34 35 35 36 36 37 37 20 21 22 22 23 24 24 25 25 26 27 27 28 28 29 30 30 31 31 32 32 33 33 34 34 35 35 36 36 37 37 38 21 22 22 23 24 24 25 25 26 27 27 28 28 29 30 30 31 31 32 32 33 33 34 34 35 35 36 36 37 37 38 38 22 22 23 24 24 25 25 26 27 27 28 28 29 30 30 31 31 32 32 33 33 34 34 35 35 36 36 37 37 38 38 38 22 23 24 24 25 25 26 27 27 28 28 29 30 30 31 31 32 32 33 33 34 34 35 35 36 36 37 37 38 38 38 39 23 24 24 25 25 26 27 27 28 28 29 30 30 31 31 32 32 33 33 34 34 35 35 36 36 37 37 38 38 38 39 39 24 24 25 25 26 27 27 28 28 29 30 30 31 31 32 32 33 33 34 34 35 35 36 36 37 37 38 38 38 39 39 40 24 25 25 26 27 27 28 28 29 30 30 31 31 32 32 33 33 34 34 35 35 36 36 37 37 38 38 38 39 39 40 40 25 25 26 27 27 28 28 29 30 30 31 31 32 32 33 33 34 34 35 35 36 36 37 37 38 38 38 39 39 40 40 40 25 26 27 27 28 28 29 30 30 31 31 32 32 33 33 34 34 35 35 36 36 37 37 38 38 38 39 39 40 40 40 41 26 27 27 28 28 29 30 30 31 31 32 32 33 33 34 34 35 35 36 36 37 37 38 38 38 39 39 40 40 40 41 41 27 27 28 28 29 30 30 31 31 32 32 33 33 34 34 35 35 36 36 37 37 38 38 38 39 39 40 40 40 41 41 42 27 28 28 29 30 30 31 31 32 32 33 33 34 34 35 35 36 36 37 37 38 38 38 39 39 40 40 40 41 41 42 42 28 28 29 30 30 31 31 32 32 33 33 34 34 35 35 36 36 37 37 38 38 38 39 39 40 40 40 41 41 42 42 42 28 29 30 30 31 31 32 32 33 33 34 34 35 35 36 36 37 37 38 38 38 39 39 40 40 40 41 41 42 42 42 43

## Modeling of other default matrices

Default matrices shown in [2] can also be used for parameterization using this technique. 4x4 and 8x8 matrices in [2] are the same as those of Section 1. 16x16 and 32x32 matrices are also shown here for comparison:

| Transform Size | Slice Type | Parameters (a, b, c, d) | MSE |
| --- | --- | --- | --- |
| 16x16 | Intra | -20, -30, 1759, 6426 | 0.5273 |
| 32x32 | Intra | -7, -7, 921, 6122 | 0.7520 |
| 16x16 | Inter | -12, -11, 1093, 10933 | 0.3359 |
| 32x32 | Inter | -4, -2, 559, 10830 | 0.5820 |

Assuming one matrix per transform size per component per slice type (Inter and Intra). One byte per matrix element. This estimation excludes any non-square transform sizes.