| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G353 |
| --- | --- |

| Title: | Non-CE4 : QP prediction using spatial QP |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hiroya Nakamura Masayoshi Nishitani Shigeru Fukushima Toru Kumakura JVC KENWOOD Corporation 58-7, Shinmei-cho, Yokosuka, Kanagawa 239-8550, Japan | Tel: Email: | +81-46-836-6459 nakamura.hiroya@ jvckenwood.com nishitani.masayoshi@ jvckenwood.com fukushima.shigeru@ jvckenwood.com kumakura.toru@ jvckenwood.com |
| Source: | JVC KENWOOD Corporation |  |  |

_____________________________

# Abstract

In the HM4.0, the predictive QP is always the QP of left neighbor quantization group for delta QP signaling. In this contribution, it is proposed to calculate the predictive QP based on the QP of previously coded neighboring quantization groups. It is reported that 0.3%, 0.2%, 0.3%, 0.3%, 0.4%, and 0.5% luminance BD-rate reductions were obtained by the proposed method for AIHE, AILC, RAHE, RALC, LDHE, and LDLC configurations, respectively.

# Introduction

In the HM 4.0 [1], delta QP is calculated as the difference between the QP of current quantization group and the predictive QP. The predictive QP is always used the QP of previously coded left neighboring quantization group of current coding units in the current slice if available. If the predictive QP is more different from the current QP, the larger magnitude of delta QP is, the more bits will be required. Coded bit size of delta QP is not negligible especially when QP is controlled elaborately for adaptive quantization.

Therefore, the technique to calculate the predictive QP is significant to reduce the amount of coded bits.

# Algorithm Description

## Current algorithm (HM4.0)

Delta QP is calculated as the difference between the QP of current quantization group and the predictive QP which is the QP of previously coded neighboring quantization group. In the HM4.0, the predictive QP is the QP of left neighboring quantization group of coding units in the current slice. If the left neighboring quantization group in the current slice is not available, the predictive QP is the QP of the previous quantization group in decoding order in the current slice. For the first quantization group of coding units in the slice, the predictive QP is initially set equal to slice QP at the start of each slice.

## Proposed algorithm

In this contribution, the predictive QP is calculated as the weighted average between the QPl(QP of left quantization group) and the QPa (QP of above quantization group). The weighted coefficients of the QP of left and above quantization group are given based on the detection whether or not the QPl or the QPa is equal to the QPal (QP of above-left quantization group).

The predictive QP is derived as the following steps.

- If the QPl is equal to the QPal,
the predictive QP is calculated as the weighted average between the QPl and the QPa if both left and above quantization group are available.
The predictive QP is calculated by the following formula:

predictiveQP=QPl+3∗QPa+2>>2

- Otherwise, if the QPa is equal to the QPal,
the predictive QP is calculated as the weighted average between the QPl and the QPa if both left and above quantization group are available.
The predictive QP is calculated by the following formula:

predictiveQP=3∗QPl+QPa+2>>2

- Otherwise,
the predictive QP is calculated as the average between the QPl and the QPa if both left and above quantization group are available. 
The predictive QP is calculated by the following formula:

predictiveQP=QPl+QPa+1>>1

After the predictive QP is derived, the following is applied.

In the encoding side, delta QP is calculated as the difference between the QP of current quantization group and the predictive QP.

In the decoding side, QP of current quantization group is calculated to add delta QP to the predictive QP.

## Implementation

### Software

This proposed technique is implemented into HM4.0 CE4 anchor software.

# Experiments

## Test conditions

The common test conditions [2] are used.

## Experimental environment

The experimental environment used is presented in Table1.

Table 1 Experimental environment

| CPU | Intel Core i7-2600K 3.40GHz |
| --- | --- |
| Memory | 16GB (DDR3) |
| OS | Windows 7 Professional 64bit |
| Compiler | Microsoft Visual C++ 2008 Express edition SP1 |
| Executable | x64 |

Notes: The decoding time is measured with the writing of yuv file.

## Simulation results

Table 2, 3, and 4 present the simulation results for intra only, random access, and low delay, respectively.

Table 2 Simulation results for intra only

|  | All Intra HE |  |  |  | All Intra LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | dQP incr | Y | U | V | dQP incr |
| ClassA | -0.1% | -0.1% | -0.1% | -3.4% | 0.0% | 0.0% | 0.0% | -0.9% |
| ClassB | -0.2% | -0.2% | -0.2% | -5.8% | -0.2% | -0.2% | -0.2% | -4.3% |
| ClassC | -0.4% | -0.4% | -0.4% | -8.2% | -0.3% | -0.3% | -0.3% | -7.3% |
| ClassD | -0.4% | -0.3% | -0.3% | -8.9% | -0.4% | -0.3% | -0.4% | -8.2% |
| ClassE | -0.5% | -0.5% | -0.4% | -7.9% | -0.3% | -0.3% | -0.3% | -6.1% |
| Overall | -0.3% | -0.3% | -0.3% | -6.7% | -0.2% | -0.2% | -0.2% | -5.3% |
| Enc Time[%] | 100% |  |  |  | 98% |  |  |  |
| Dec Time[%] | 100% |  |  |  | 100% |  |  |  |

Table 3 Simulation results for random access

|  | Random Access HE |  |  |  | Random Access LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | dQP incr | Y | U | V | dQP incr |
| ClassA | -0.2% | -0.1% | -0.4% | -4.9% | -0.1% | -0.1% | -0.2% | -3.5% |
| ClassB | -0.3% | -0.2% | -0.2% | -6.8% | -0.3% | -0.3% | -0.3% | -6.1% |
| ClassC | -0.4% | -0.3% | -0.5% | -8.9% | -0.5% | -0.4% | -0.4% | -9.0% |
| ClassD | -0.4% | -0.4% | -0.3% | -8.6% | -0.4% | -0.6% | -0.5% | -8.7% |
| ClassE | - | - | - | - | - | - | - | - |
| Overall | -0.3% | -0.3% | -0.3% | -7.3% | -0.3% | -0.4% | -0.3% | -6.8% |
| Enc Time[%] | 100% |  |  |  | 100% |  |  |  |
| Dec Time[%] | 99% |  |  |  | 100% |  |  |  |

Table 4 Simulation results for low delay B

|  | Low Delay HE |  |  |  | Low Delay LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | dQP incr | Y | U | V | dQP incr |
| ClassA | - | - | - | - | - | - | - | - |
| ClassB | -0.3% | -0.5% | -0.5% | -7.4% | -0.4% | -0.4% | -0.4% | -7.0% |
| ClassC | -0.4% | -0.2% | -0.3% | -9.0% | -0.6% | -0.5% | -0.4% | -9.6% |
| ClassD | -0.4% | -1.0% | -0.2% | -8.6% | -0.5% | -0.4% | -0.3% | -9.1% |
| ClassE | -0.4% | 0.1% | -0.6% | -9.8% | -0.5% | -1.5% | -0.1% | -9.7% |
| Overall | -0.4% | -0.4% | -0.4% | -8.6% | -0.5% | -0.6% | -0.3% | -8.6% |
| Enc Time[%] | 100% |  |  |  | 100% |  |  |  |
| Dec Time[%] | 100% |  |  |  | 99% |  |  |  |

The simulation results report that the proposed technique provides 0.2-0.3% BD-rate [3] gain for intra only settings, 0.3% gain for random access settings, and 0.4-0.5% gain for low delay B settings. (See the attached excel sheet)

# Conclusions

This contribution provides the technique to calculate the predictive QP for delta QP signaling. The proposed technique is composed simple algorithm, and is available tool for HEVC.

We recommend that the proposed technique be adopted in next WD and HM.

# References

[1] “WD4: Working Draft 4 of High-Efficiency Video Coding”,

JCTVC-F803, Torino, July 2011

[2] “Common test conditions and software reference configurations”,

JCTVC-F900, Torino, July 2011

[3] “Improvements of the BD-PSNR model,” ITU-T SG16 Q.6 Document,

VCEG-AI11, Berlin, July 2008.

# Patent rights declaration(s)

JVC KENWOOD Holdings, Inc. or JVC KENWOOD Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# WD changes

### 7.4.9	Transform coefficient semantics

The transform coefficient levels are parsed into the arrays transCoeffLevel[ x0 ][ y0 ][ trafoDepth ][ cIdx ][ n ]. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered transform block relative to the top-left luma sample of the picture. The array index trafoDepth specifies the current subdivision level of a coding unit into blocks for the purpose of transform coding. trafoDepth is equal to 0 for blocks that correspond to coding units. The array index cIdx specifies an indicator for the colour component; it is equal to 0 for luma, equal to 1 for Cb, and equal to 2 for Cr. The array index n specifies the scanning position of the transform coefficient levels.

When PredMode is equal to MODE_INTRA, different scanning orders are used. The array ScanType[ log2TrafoSize − 2 ][ IntraPredMode ], specifying the scanning order for various luma transform block sizes and intra prediction modes, is derived as specified in Table 7-2Error: Reference source not found.

Table 7-2 – Specification of ScanType[ log2TrafoSize − 2 ][ IntraPredMode ]

| IntraPredMode | log2TrafoSize − 2 |  |  |  |
| --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 |
| 0 | 1 | 1 | 0 | 0 |
| 1 | 2 | 2 | 0 | 0 |
| 2-3 | 0 | 0 | 0 | 0 |
| 4-5 | 1 | 1 | 0 | 0 |
| 6 | 0 | 0 | 0 | 0 |
| 7-8 | 2 | 2 | 0 | 0 |
| 9-10 | 0 | 0 | 0 | 0 |
| 11-12 | 1 | 1 | 0 | 0 |
| 13-14 | 0 | 0 | 0 | 0 |
| 15-16 | 2 | 2 | 0 | 0 |
| 17-19 | 0 | 0 | 0 | 0 |
| 20-23 | 1 | 1 | 0 | 0 |
| 24-27 | 0 | 0 | 0 | 0 |
| 28-31 | 2 | 2 | 0 | 0 |
| 32-33 | 0 | 0 | 0 | 0 |
| 34 | 0 | 0 | 0 | 0 |
| 35 | 0 | 0 | 0 | 0 |

cu_qp_delta can change the value of QPY for a quantization group of coding units where the quantization group of coding units is specified as follows:

- If a coding unit with the split_coding_unit_flag[ x0 ][ y0 ] equal to 0 and the log2CUSize is greater than or equal to log2MinCUDQPSize, the quantization group includes this coding unit only.

- Otherwise, if a coding unit with the split_coding_unit_flag[ x0 ][ y0 ] equal to 1 and the log2CUSize is equal to log2MinCUDQPSize, the quantization group includes all coding units split from this coding unit.

The decoded value of cu_qp_delta shall be in the range of –( 26+ QpBdOffsetY / 2 ) to +( 25+ QpBdOffsetY / 2 ), inclusive. cu_qp_delta shall be inferred to be equal to 0 when it is not present for any quantization group of coding units.

The value of QPY is derived as

QPY = ( ( ( QPY,PREV + cu_qp_delta +52+ 2*QpBdOffsetY )%( 52 + QpBdOffsetY ) ) - QpBdOffsetY	(7-1)

where QPY,PREV is the luma quantization parameter, QPY, of the left neighbor quantization group of coding units in the current slice. If the left neighbor quantization group in the current slice is not available, QPY,PREV is the luma quantization parameter, of the previous quantization group in decoding order in the current slice. For the first quantization group of coding units in the slice QPY,PREV is initially set equal to SliceQPY at the start of each slice.

where QPY,PREV is predictor of the luma quantization parameter, QPY for delta QP coding, and is calculated QP of previously coded neighbor quantization groups around the current coding unit. QPY,PREV is calculated as the weighted average between the QP of left quantization group and the QP of  above quantization group. The weighted coefficients of left and above quantization group are given based on the detection which is equal to the QP of above-left quantization group.

The predictive QP is derived as following steps.

- If the QP of left quantization group is equal to the QP of above-left quantization group,

the predictive QP is calculated as the weighted average between the QP of left quantization group and the QP of above quantization group if both left and above quantization group are available.The value of predictive QP is derived as:

predictiveQP = (QPL+3* QPA+3)>>2		(7-xx)

(2)  Otherwise, if the QP of above quantization group is equal to the QP of above-left quantization group,

the predictive QP is calculated as the weighted average between the QP of left quantization group and the QP of above quantization group if both left and above quantization group are available. The value of predictive QP is derived as:

predictiveQP = (3*QPL+ QPA+3)>>2		(7-xx)

(3)  Otherwise,

the predictive QP is calculated as the average between the QP of left quantization group and the QP of above quantization group if both left and above quantization group are available. The value of predictive QP is derived as:

predictiveQP = (QPL+ QPA+1)>>1		(7-xx)

where QPL and QPA are quantization parameter of left and above quantization group, respectively.

If QPY,PREV is not available, QPY,PREV is the luma quantization parameter, of the previous quantization group in decoding order in the current slice. For the first quantization group of coding units in the slice QPY,PREV is initially set equal to SliceQPY at the start of each slice.

QP of current quantization group is calculated to add delta QP to the predictive QP.

The value of QP’Y is derived as

QP’Y = QPY + QpBdOffsetY		(7-2)