| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G357 |
| --- | --- |

| Title: | Non-CE4: Improvement of Intra prediction based QP prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jun Xu 1730 N. First Street San Jose, CA 95112, USA | Tel: Email: | +1-408-352-4086 jun.xu@am.sony.com |
|  | Ali Tabatabai 1730 N. First Street San Jose, CA 95112, USA | Tel: Email: | +1-408-352-4715 ali.tabatabai@am.sony.com |
|  | Kazushi Sato Sony City Osaki 9F 2-10-1 Osaki, Shinagawa, 141-8610 Tokyo, Japan | Tel: Email: | +81 50 3750 2741 Kazushi.Sato@jp.sony.com |
| Source: | Sony Electronics Inc. |  |  |

_____________________________

# Abstract

This proposal introduced QP prediction algorithm for Intra CU based on QPs from neighing CUs. Experimental results show that under the common test conditions defined in CE4, the BD-rate gains for All Intra LoCo conditions are 0.5% for Y, 0.5% for U and 0.4% for V and for All Intra HE conditions are 0.5% for Y, 0.5% for U and 0.5% for V. Especially, proposed solution can save delta-QP bits for 10.8% in AI_LC and 11.3% in AI_HE.

# Introduction

The cu_delta_QP signalling at sub-LCU level was adopted at the 5th JCT-VC meeting. The cu_delat_QP is obtained as the difference between current QP and predicted QP (refQP). There were several contributions to propose various QP prediction algorithms, which were studied in Core Experiment 4 (CE4 subset 1.3.c)[2].

In CE4 1.3.c[1][3], intra prediction based QP prediction was proposed. If the intra prediction is approximately upwards, refQP is the QP from above neighboring CU. If the intra prediction is approximately leftwards, refQP is the QP from left neighboring CU. Otherwise, refQP is derived based on availabilities. If both neighbors are available, refQP is the average of QPs from left and above neighbors. If either left or above neighbor is available, refQP is the QP from available neighbor. If neither of left and above neighbors is available, refQP is the last encoded QP in encoding order.

# Solution

To achieve more accurate prediction, we propose to loosen the condition of intra prediction based QP prediction.  The proposed solution defines three sets for the intra prediction modes where the way to derive refQP is different. One set contains intra prediction direction is roughly upwards as shown in red circle in Figure 1, and refQP is predicted from QP of the above neighbor. Another set contains intra prediction direction is roughly leftwards as shown in yellow circle in Figure 1, and refQP is predicted from QP of the left neighbor. The last set contains Planar, DC and PCM modes and some intra prediction direction close to leftwards and upwards as shown in blue square in Figure 1, and refQP is predicted from the average QP of the above and left neighbors.

In this contribution, the new algorithm is defined as:

If intraPredOrder(in Table 8-1 of JCTVC-F803[2]) is in the range of [5, 17], refQP is the QP from above neighboring CU if it is available.

If intraPredOrder is in the range of [21, 33], refQP is the QP from left neighboring CU if it is available.

Otherwise, i.e. the neighboring CU is not available or intra prediction mode is DC, Planar or PCM, refQP is derived as CE4 1.3.c.[3]

Table 8-1 – Specification of intraPredAngle (JCTVC-F803)

| intraPredOrder | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| intraPredAngle | - | -32 | -26 | -21 | -17 | -13 | -9 | -5 | -2 | - | 2 | 5 | 9 | 13 | 17 | 21 | 26 |
| intraPredOrder | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 |
| intraPredAngle | 32 | -26 | -21 | -17 | -13 | -9 | -5 | -2 | - | 2 | 5 | 9 | 13 | 17 | 21 | 26 | 32 |

refQP = above QP

refQP = left QP

Horizontal angles

Vertical angles

DC+Planar+PCM

Figure 1 Demonstration of proposed algorithm

# Experiment setting and results

## Coding conditions

The algorithm is implemented on top of CE4 1.3.c software. The conditions are the same as specified in CE4 subset 1 (JCTVC-F904[1]).

## Results

A summary of results is listed in Table 2, where the anchor is CE4 anchor. Complete results are available in “JCTVC-G357.xls”.

Table 2 Simulation results using proposed solution

|  | All Intra HE |  |  |  | All Intra LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A | -0.2% | -0.2% | -0.2% | -6.9% | -0.2% | -0.2% | -0.3% | -6.7% |
| Class B | -0.4% | -0.4% | -0.4% | -10.2% | -0.4% | -0.4% | -0.4% | -10.3% |
| Class C | -0.5% | -0.5% | -0.5% | -12.0% | -0.6% | -0.5% | -0.6% | -12.8% |
| Class D | -0.5% | -0.4% | -0.4% | -11.4% | -0.6% | -0.5% | -0.5% | -12.4% |
| Class E | -0.8% | -0.8% | -0.7% | -15.0% | -0.8% | -0.8% | -0.8% | -15.4% |
| Overall | -0.5% | -0.5% | -0.4% | -10.8% | -0.5% | -0.5% | -0.5% | -11.3% |
|  | -0.5% | -0.5% | -0.4% |  | -0.5% | -0.5% | -0.5% |  |
| Enc Time[%] | 99% |  |  |  | 98% |  |  |  |
| Dec Time[%] | 100% |  |  |  | 99% |  |  |  |

# Conclusion

This proposal improved the QP prediction performance using intra prediction modes. Significant savings for cu_delta_QP bits were observed while no complexity was introduced. We proposed to adopt this solution to HEVC.

# Reference

- K. Sato, etc, “Description of Core Experiment 4: Quantization”, JCTVC-F904r3, Torino, July 2011.

- B. Bross, etc, “WD4: Working Draft 4 of High-Efficiency Video Coding”, JCTVC-F803, Torino, July 2011.

- H. Aoki, etc, “CE4 Subtest 1: Spatial QP prediction based on intra prediction and prediction mode (test 1.3.c)”, JCTVC-G067, Geneva, November, 2011

# Patent rights declaration(s)

Sony Electronics Inc.  may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Appendix A: Proposed modification of HEVC Working Draft

This work makes modification with respect to JCTVC-G067[3] Appendix A.2 shown as follows. The modified or added portions are highlighted with yellow color.

### 8.5.x.2		Intra quantisation parameter prediction process

Inputs to this process are

–	a syntax element pcm_flag specifying if the top-left coding unit of the current quantisation group of coding units is coded in PCM mode

–	a variable intraPredMode specifying the intra prediction mode of the top-left prediction unit of the current quantisation group of coding units relative to the top-left sample of the current picture.

Output of this process is

–	the predicted quantisation parameter QPY, PRED.

The rearranged intra prediction order intraPredOrder is derived as follows.

–	If one of the following conditions is true, intraPredOrder is set to 0.

- pcm_flag is equal to 1.

- intraPredMode is equal to 0 (Intra_Planar) or 3 (Intra_DC).

–	Otherwise, if intraPredMode is equal to 1 (Intra_Vertical), intraPredOrder is set to 9.

–	Otherwise, if intraPredMode is equal to 2 (Intra_Horizontal), intraPredOrder is set to 25.

–	Otherwise, intraPredOrder is derived as specified in Table 8-7.

The predicted quantisation parameter QPY, PRED is derived as follows.

–	If intraPredOrder is greater than 04 and less than 18, and the above neighbouring quantisation group of coding units in the current slice is available, then QPY, PRED is set equal to the luma quantisation parameter of the above neighbouring quantisation group of coding units in the current slice.

–	Otherwise, if intraPredOrder is greater than or equal to 18 more than 21 and less than 34, and the left neighbouring quantisation group of coding units in the current slice is available, then QPY, PRED is set equal to the luma quantisation parameter of the left neighbouring quantisation group of coding units in the current slice.

–	Otherwise, default quantisation parameter prediction process as specified in subclause 8.5.x.1 is invoked with the output being the reference quantisation parameter QPY, PRED.