| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G359_r2 WG11 Number: m21921 |
| --- | --- |

| Title: | Non-CE6: Coding of luma intra prediction modes that are not in the MPM set |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Robert Cohen Xiaozhong Xu Anthony Vetro Huifang Sun Mitsubishi Electric Research Laboratories 201 Broadway Cambridge, MA 02139 U.S.A | Tel: Email: | +1-617-621-7503 cohen@merl.com xiaozhongxu@gmail.com avetro@merl.com hsun@merl.com |
| Source: | Mitsubishi Electric Research Laboratories |  |  |

_____________________________

# Abstract

This tool proposes a method for coding the intra prediction mode when it is not contained in the Most Probable Mode (MPM) set. In the current Working Draft, when the current mode is not in the MPM set, it is explicitly signaled, and when CAVLC is used, a ranking table is maintained. This proposed tool instead signals the difference between the current mode and an estimate derived from the MPM set. No ranking table is needed because smaller differences are assigned smaller codewords. The estimate for the current mode is the median or mean of the two angular modes in the MPM set, or the mode adjacent to one angular mode in the MPM set. This tool requires no change to the existing Prediction Unit syntax. Average BD-Rate improvements compared to HM 4.0 are 0.3% for AI-HE configurations, and 0.2% for AI-LC, with no change in encoder or decoder run times. An additional experiment performed yields similar results when bypass mode is used to code the difference value.

# Description of proposed method

The purpose of the tool proposed in this document is to improve upon the coding of the intra prediction mode when it is not in the MPM set. Currently in Working Draft 4 (WD4) [1], a flag is signaled indicating whether the intra prediction mode for the current block is contained in a set of previously-decoded intra prediction modes, known as the Most Probable Mode (MPM) set. If the current mode is in the MPM set, another flag is sent indicating which of those modes to use. If the current mode is not in the MPM set, then it is explicitly signaled in the bit-stream. In addition, when the CAVLC entropy coder is used, an adaptive VLC table is updated via a ranking table based upon the value of the current mode.

In this proposal, instead of signaling the mode value directly, we signal the difference between the current mode and an estimate of the current mode based upon the contents of the MPM set. The estimate is computed using a few add/subtract and shift operations on the MPM set. This is in contrast to earlier methods of differential coding [2] in which the estimate was calculated based upon neighboring pixel values, which incurred an increased complexity over HM.

The Prediction Unit syntax of this proposed tool is identical to that of WD4. The only change is in how rem_intra_luma_pred_mode is processed and coded, because it represents the difference between the current intra prediction mode and its estimate. Because differences are typically lower in magnitude than the mode value itself, no ranking table is needed because small differences are assigned shorter codewords. Hence, there is no need for the CAVLC entropy coding table to adapt. Because there is no PU syntax change from WD4, parallelism in parsing of the bit-stream is still supported, as it does not require knowledge of neighboring intra prediction modes to parse bits for the current mode.

## Computing estimate from two angular modes in the MPM set

There are two kinds of luma intra prediction modes: Angular modes in which intra prediction is performed along a certain direction, and non-angular modes such as DC and Planar prediction. When both modes in the MPM set are angular, we use them to compute an estimate for the current mode. Conceptually, we would like the estimate to have an angle similar to the two MPM modes. Hence, if the vectors representing the directions of the two MPM modes form an acute angle, the estimate will be the mean or median of that angle. An example of this case is shown in Figure 1.

Figure 1. Computing estimate from modes that form an acute angle

Here, the two modes in the MPM set are 11 and 13. The modes are mapped to a sequential order, shown in circles, in order to compute the estimate. Modes 11 and 13 are mapped to the values 6 and 2. Their mean is (6 + 2)/2 = 4, so the mapped estimate is 4. Suppose the actual current mode is 5, which has a mapped value of 3. The difference between the mapped estimate and the mapped current mode is therefore 4 – 3 = 1. This is the value that is signaled as rem_intra_luma_pred_mode.

When the angle between the modes in the MPM set is obtuse, we use a circular representation of the modes to compute the mean. For example, if the unmapped modes in the MPM set were 13 (up-right) and 17 (down-left), then the above method would produce an unmapped estimate of 10 (up-left), which is almost at a right angle to the modes in the MPM set. To obtain an estimate that is more aligned with the existing modes, we would like the estimate to be the median between one mode and a mirrored version of the other mode in the MPM set, as shown in Figure 3. This adjustment is accomplished by rotating the initial estimate by 90°.

Figure 2. Computing estimate from modes that form an obtuse angle

Figure 3. Computing estimate from modes that form an obtuse angle

Here, the mapped value of the estimate before adjustment is 8. To rotate by 90°, we add (NumModes  1)/2 = 8. The result is 16. The mapped estimate therefore becomes 16, which corresponds to unmapped intra prediction mode 17. This mode is equivalent to the median between unmapped mode 16 and a mirrored version of unmapped mode 13 indicated by the dotted line. Thus, intra prediction mode 17 is the final estimate, which is more aligned with modes 13 and 16 as compared to the estimate before adjustment. If after rotation by 90° the estimate is outside the range of possible modes, it is rotated by 180° by adding or subtracting (NumModes  1) to the mapped value.

For this example, the difference between the mapped estimate 16 and the mapped current mode is then signaled as rem_intra_luma_pred_mode. In the current implementation, depending upon the number of angular modes in the MPM set, 1 or 2 is added to this difference before being signaled as rem_intra_luma_pred_mode, to allow representation of the non-angular modes by 0 or 1, if needed.

## Computing estimate from one angular modes in the MPM set

When only one mode in the MPM set is angular, we compute the estimate as being the mode adjacent to that angular mode. Generally, this is the mode with a mapped sequential mode order that is one less than the mapped angular mode from the MPM set. We then signal the difference between this estimate and the mapped current mode. The difference is incremented by one before signaling as rem_intra_luma_pred_mode, because a value of 0 has a special meaning as described in the next subsection.

## Processing when none of the modes in the MPM set are angular or if the current mode is non-angular

If the current mode is angular and the MPM set comprises the DC and Planar modes, then we can’t use the MPM set to compute an angular estimate. In this case, the current mode is explicitly signaled. The current mode can be mapped to a smaller value before coding, because in this case we know that neither DC nor Planar modes will be signaled.

If the current mode is non-angular and the MPM set contains one angular and one non-angular mode, then we signal a rem_intra_luma_pred_mode of zero. When the decoder sees that rem_intra_luma_pred_mode is zero while the number of angular modes in the MPM set is one, then it knows that the current mode must be the non-angular mode not contained in the MPM set. For example, if the MPM set contains DC mode and mode 12, then a value of zero for rem_intra_luma_pred_mode indicates that the current mode is Planar.

## Processing when the current mode is in the MPM set

When the current mode is already in the MPM set, everything is done as already described in WD 4. The proposed method operates only when the current mode is not contained in the MPM set, i.e., when rem_intra_luma_pred_mode is signaled.

# Performance of proposed method

The proposed method was implemented on top of HM 4.0, and compared to a reference coded with HM 4.0 for the common conditions described in [3]. The performance is summarized in Error: Reference source not found. Full results are included in the accompanying spreadsheet. BD-Rate metrics from -0.3% to -0.4% are obtained for AI-HE configurations and -0.1% to -0.3% for AI-LC configurations.

Table 1. Performance of proposed method

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.3% | 0.1% | 0.0% | -0.1% | -0.1% | -0.1% |
| Class B | -0.3% | 0.0% | 0.1% | -0.2% | -0.1% | -0.1% |
| Class C | -0.3% | 0.0% | -0.1% | -0.3% | -0.2% | -0.2% |
| Class D | -0.3% | -0.1% | -0.2% | -0.3% | -0.2% | -0.2% |
| Class E | -0.4% | 0.1% | 0.1% | -0.3% | -0.2% | -0.2% |
| Overall | -0.3% | 0.0% | 0.0% | -0.2% | -0.1% | -0.1% |
|  | -0.3% | 0.0% | 0.0% | -0.2% | -0.1% | -0.2% |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 98% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | 0.3% | 0.1% | 0.0% | 0.3% | 0.0% |
| Class B | -0.2% | 0.1% | 0.0% | -0.1% | -0.2% | 0.0% |
| Class C | -0.2% | -0.2% | 0.0% | -0.2% | -0.2% | -0.1% |
| Class D | -0.1% | 0.1% | -0.4% | -0.2% | -0.3% | 0.1% |
| Class E |  |  |  |  |  |  |
| Overall | -0.1% | 0.1% | -0.1% | -0.1% | -0.1% | 0.0% |
|  | -0.1% | 0.0% | -0.1% | -0.1% | -0.1% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

# Using bypass mode to code rem_intra_luma_pred_mode

During the current JCT-VC meeting (November 2011), several contributions presented results using bypass mode when coding intra prediction mode data with CABAC. In order to see if the method proposed here is compatible with bypass mode coding, we performed another experiment during the meeting in which the codewords for rem_intra_luma_pred_mode were coded using bypass (equiprobable) mode in CABAC. The results show that bypass mode is compatible with coding rem_intra_luma_pred_mode.

Table 2. Proposed method using bypass mode to code rem_intra_luma_pred_mode

| Proposed method |  |  |  |  | Proposed method with bypass mode |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | All Intra HE |  |  |  |  | All Intra HE |  |  |
|  | Y | U | V |  |  | Y | U | V |
| Class A | -0.29% | 0.1% | 0.0% |  | Class A | -0.30% | 0.0% | 0.0% |
| Class B | -0.25% | 0.0% | 0.1% |  | Class B | -0.24% | 0.0% | 0.0% |
| Class C | -0.32% | 0.0% | -0.1% |  | Class C | -0.33% | 0.0% | -0.1% |
| Class D | -0.31% | -0.1% | -0.2% |  | Class D | -0.38% | -0.1% | -0.2% |
| Class E | -0.44% | 0.1% | 0.1% |  | Class E | -0.44% | -0.1% | -0.1% |
| Overall | -0.31% | 0.0% | 0.0% |  | Overall | -0.33% | -0.1% | -0.1% |
|  | -0.31% | 0.0% | 0.0% |  |  | -0.31% | 0.0% | 0.0% |

# Conclusions

In the current Working Draft (WD4), when the intra prediction mode is not in the MPM set, it is explicitly signaled as rem_intra_luma_pred_mode. For the method proposed in this document, rem_intra_luma_pred_mode instead represents the difference between the current intra prediction mode and an estimate based upon the modes in the MPM set. The estimate is calculated using only a few add, subtract, and shift operations via a circular representation of prediction mode directions. Because smaller differences are more common than larger differences, compression efficiency is improved by assigning shorter codewords to smaller differences. Complexity is also reduced because this fixed mapping eliminates the need for an adaptive ranking table. The Prediction Unit syntax for this proposed method is identical to that of WD4. Average BD-Rate improvements compared to HM 4.0 are 0.3% for AI-HE configurations, and 0.2% for AI-LC, with no change in encoder or decoder run times. An additional experiment confirms that the bypass mode of CABAC can be used to code rem_intra_luma_pred_mode with negligible changes in performance.

# Appendix: Changes to syntax and semantics

Changes from WD4  JCTVC-F803_d5.doc are highlighted in yellow.

7.3.7 Prediction unit syntax

There are no changes to the existing Prediction Unit syntax.

8.3.1 Derivation process for luma intra prediction mode

Only portions of step 5 are changed:

…

- IntraPredMode[ xB ][ yB ] is derived by applying the following procedure:

- If prev_intra_pred_flag[ xB ][ yB ] is true, the IntraPredMode[ xB ][ yB ] is set equal to  candModeList[mpm_idx][ xB ][ yB ]]

- Otherwise IntraPredMode[ xB ][ yB ] is derived by applying the following ordered steps:

  - IntraPredMode[ xB ][ yB ] = rem_intra_pred_mode

  - If candModeList[ 0 ] is greater than candModeList[ 1 ], then two values are swapped. [Ed. (WJ): may be better expression needed]

  - When IntraPredMode[ xB ][ yB ] is equal or greater than candModeList[ 0 ], the value of IntraPredMode[ xB ][ yB ] is increased by one

  - When IntraPredMode[ xB ][ yB ] is equal or greater than candModeList[ 1 ], the value of IntraPredMode[ xB ][ yB ] is increased by one

  - NumAngularPredMode = 2

  - If candModeList[ 0 ] is equal to 0 or 3, NumAngularPredMode is decreased by one.

  - If candModeList[ 1 ] is equal to 0 or 3, NumAngularPredMode is decreased by one.

  - If NumAngularPredMode is equal to 0, then the following steps are applied:

    - IntraPredMode[ xB ][ yB ] is increased by one.

    - If IntraPredMode[ xB ][ yB ] is greater than or equal to 3, then IntraPredMode[ xB ][ yB ] is increased by one.

  - If intraPredModeNum is equal to 18, then k is set to the value 0; otherwise k is set to the value 1

  - If NumAngularPredMode is equal to 1, then one the following steps are applied:

    - If IntraPredMode[ xB ][ yB ] is equal to 0 and candModeList[ 1 ] is not equal to 3, then IntraPredMode[ xB ][ yB ] = 3

    - If IntraPredMode[ xB ][ yB ] is not equal to 0, then all the following steps are applied in order:

      - If candModeList[ 0 ] is equal to 0 or 3, then candModeList[ 0 ] = mapIntraModeDec[k][ candModeList[ 1 ] ]; otherwise candModeList[ 0 ] = mapIntraModeDec[k][ candModeList[ 0 ] ]

      - IntraPredMode[ xB ][ yB ] is decreased by one

      - If IntraPredMode[ xB ][ yB ] is even, IntraPredMode[ xB ][ yB ] is incremented by 2

      - IntraPredMode[ xB ][ yB ] = mapResToIntraMode( IntraPredMode[ xB ][ yB ], candModeList[ 0 ], intraPredModeNum )

      - IntraPredMode[ xB ][ yB ] is set equal to mapIntraModeDec[k][ IntraPredMode[ xB ][ yB ] ]

  - If NumAngularPredMode is equal to 2, then the following steps are applied:

    - If IntraPredMode[ xB ][ yB ] is equal to 0, then IntraPredMode[ xB ][ yB ] = 3 and this process terminates

    - If IntraPredMode[ xB ][ yB ] is equal to 1, then IntraPredMode[ xB ][ yB ] = 0 and this process terminates

    - IntraPredMode[ xB ][ yB ] is decreased by 2

    - candModeList[ 0 ] = mapIntraModeDec[k][ candModeList[ 0 ] ], and candModeList[ 1 ] = mapIntraModeDec[k][ candModeList[ 1 ] ]

    - predModeMean = ( candModeList[ 0 ] + candModeList[ 1 ] ) / 2

    - If candModeList[ 1 ] -  candModeList[ 0 ] is less than intraPredModeNum/2, then the following steps are performed

      - if predModeMean is greater than or equal to 2 * (candModeList[ 1 ] – predModeMean) – 1, or if predModeMean is greater than or equal to (2 * (predModeMean - candModeList[ 0 ]), then increase IntraPredMode[ xB ][ yB ] by 2

      - IntraPredMode[ xB ][ yB ] = mapResToIntraMode( IntraPredMode[ xB ][ yB ], predModeMean, intraPredModeNum )

    - If candModeList[ 1 ] -  candModeList[ 0 ] is not less than intraPredModeNum/2, then the following steps are performed

      - predModeMean = predModeMean + intraPredModeNum / 2

      - If predModeMean is greater than or equal to intraPredModeNum, then predModeMean = predModeMean - (intraPredModeNum – 1)

      - If predModeMean is less than or equal to zero, then predModeMean = predModeMean + (intraPredModeNum – 1)

      - If IntraPredMode[ xB ][ yB ]is equal to (intraPredModeNum – 2), then IntraPredMode[ xB ][ yB ] is increased by 1.

      - IntraPredMode[ xB ][ yB ] = mapResToIntraMode( IntraPredMode[ xB ][ yB ], predModeMean, intraPredModeNum )

      - If IntraPredMode[ xB ][ yB ] is greater than or equal to candModeList[ 0 ] and IntraPredMode[ xB ][ yB ] is less than ( candModeList[ 0 ] + candModeList[ 1 ] ) / 2, then IntraPredMode[ xB ][ yB ] is increased by 1

      - If IntraPredMode[ xB ][ yB ] is less than or equal to candModeList[ 1 ] and IntraPredMode[ xB ][ yB ] is greater than ( candModeList[ 0 ] + candModeList[ 1 ] ) / 2, then IntraPredMode[ xB ][ yB ] is decreased by 1

      - IntraPredMode[ xB ][ yB ] is set equal to mapIntraModeDec[k][ IntraPredMode[ xB ][ yB ] ]

Table 8-A specifies the mapping table between intraPredModeCoding and the rearranged intra prediction order intraPredOrderCoding.

Table 8-A – Specification of mapIntraModeDec[k][value] [Ed.: For k=1, this is almost same as intraPredOrder table in WD4_d5]

| value | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| mapIntraModeDec[0][value] | 17 | 5 | 13 | 0 | 9 | 7 | 3 | 1 | 11 | 15 | 16 | 8 | 6 | 4 | 2 | 10 | 12 | 14 |
| mapIntraModeDec[1][value] | 34 | 9 | 25 | 0 | 1 | 5 | 13 | 17 | 21 | 29 | 33 | 3 | 7 | 11 | 15 | 19 | 23 | 27 |
| value | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 |  |
| mapIntraModeDec[1][value] | 31 | 2 | 4 | 6 | 8 | 10 | 12 | 14 | 16 | 18 | 20 | 22 | 24 | 26 | 28 | 30 | 32 |  |

8.3.1.1 Description of the mapResToIntraMode( Residual, Estimate, NumModes) function:

- If Residual is odd, then Residual = (Residual + 1) / 2; otherwise Residual = -1 * Residual / 2

- Residual = Estimate + Residual;

- If Residual is greater than or equal to NumModes, then Residual = Residual – (NumModes – 1); otherwise if Residual is less than or equal to 0, then Residual = Residual + (NumModes – 1)

- The function returns the value: Residual

Table 9-3 – codeNum and codeword with vlcNum equal to 20

| - codeNum | - Codeword |
| --- | --- |
| - 0 | - 000 |
| - 1 | - 001 |
| - 2 | - 010 |
| - 3 | - 011 |
| - 4 | - 1000 |
| - 5 | - 1001 |
| - 6 | - 1010 |
| - 7 | - 1011 |
| - 8 | - 11000 |
| - 9 | - 11001 |
| - 10 | - 11010 |
| - 11 | - 11011 |
| - 12 | - 11100 |
| - 13 | - 11101 |
| - 14 | - 11110 |
| - 15 | - 11111 |

Table 9-4 – codeNum and codeword with vlcNum equal to 21

| - codeNum | - Codeword |
| --- | --- |
| - 0 | - 000 |
| - 1 | - 0010 |
| - 2 | - 0011 |
| - 3 | - 0100 |
| - 4 | - 0101 |
| - 5 | - 01100 |
| - 6 | - 01101 |
| - 7 | - 01110 |
| - 8 | - 01111 |
| - 9 | - 10000 |
| - 10 | - 10001 |
| - 11 | - 10010 |
| - 12 | - 10011 |
| - 13 | - 10100 |
| - 14 | - 10101 |
| - 15 | - 10110 |
| - 16 | - 10111 |
| - 17 | - 11000 |
| - 18 | - 110010 |
| - 19 | - 110011 |
| - 20 | - 110100 |
| - 21 | - 110101 |
| - 22 | - 110110 |
| - 23 | - 110111 |
| - 24 | - 111000 |
| - 25 | - 111001 |
| - 26 | - 111010 |
| - 27 | - 111011 |
| - 28 | - 111100 |
| - 29 | - 111101 |
| - 30 | - 111110 |
| - 31 | - 1111110 |
| - 32 | - 1111111 |

- 9.2.2.5 Initialisation process for intraModeTable

Outputs of this process are initial values of the variable array intraModeTable.

The variable array intraModeTable is initialized as specified in Table 9-5.

Table 9-5 – Specification of intraModeTable[k][codeNum]

|  | - codeNum |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - k | - 0 | - 1 | - 2 | - 3 | - 4 | - 5 | - 6 | - 7 | - 8 | - 9 | - 10 | - 11 | - 12 | - 13 | - 14 | - 15 | - 16 |
| - 0 | - 0 | - 3 | - 2 | - 15 | - 8 | - 11 | - 1 | - 10 | - 7 | - 4 | - 14 | - 9 | - 6 | - 5 | - 13 | - 12 |  |
| - 1 | - 0 | - 2 | - 3 | - 29 | - 1 | - 8 | - 30 | - 21 | - 28 | - 16 | - 7 | - 15 | - 20 | - 31 | - 9 | - 11 | - 6 |

|  | - codeNum |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - k | - 17 | - 18 | - 19 | - 20 | - 21 | - 22 | - 23 | - 24 | - 25 | - 26 | - 27 | - 28 | - 29 | - 30 | - 31 | - 32 | - 33 |
| - 1 | - 4 | - 5 | - 12 | - 10 | - 14 | - 22 | - 19 | - 17 | - 27 | - 13 | - 18 | - 23 | - 26 | - 32 | - 24 | - 25 |  |

#### 9.2.4.6 Parsing process for rem_intra_luma_pred_mode

This process is invoked when entropy_coding_mode_flag is equal to 0 for parsing syntax element rem_intra_luma_pred_mode in subclause Error: Reference source not found.

Inputs to this process are bits from slice data,and a variable log2CUSize specifying the size of the current coding unit. and a variable array intraModeTable.

Outputs of this process are the syntax element rem_intra_luma_pred_mode and value-updated intraModeTable.

The variable log2PUSize is derived as follows.

–	If PartMode is equal to PART_NxN, log2PUSize is set equal to log2CUSize – 1,

–	Otherwise, log2PUSize is set equal to log2CUSize.

The value of rem_intra_luma_pred_mode is derived as follows.

–	Based on log2PUSize, the value of a variable intraPredModeNum indicating the number of intra prediction modes for the given size of prediction unit is obtained according to Error: Reference source not found.

–	If intraPredModeNum is equal to 4, read one bit and assign its value to rem_intra_luma_pred_mode. No further step is carried out. Othewise,

–	The values of variable k and vlcNum are obtained as follows

  - If intraPredModeNum is equal to 18, set vlcNum and k equal to 20 and 0, respectively.

  - If intraPredModeNum is equal to 35, set vlcNum and k equal to 21 and 1, respectively.

  - The parsing process described in subclause Error: Reference source not found is invoked with vlcNum as input and the variable codeNum as output.

  - The value of rem_intra_luma_pred_mode is set equal to intraModeTable[k][codeNum] codeNum.

  - The value of a variable counterNum is set to 0. The variable array intraModeTable[k] is updated by invoking process in subclause Error: Reference source not found with intraModeTable[k], codeNum and counterNum as inputs.

#### Initialisation process for context variables

Table 9-6 – Values of variable m and n for rem_intra_luma_pred_mode ctxIdx

| Initialisation variables | rem_intra_luma_pred_mode ctxIdx |  |  |  |
| --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 |
| m | -3 | -2 | 1 | 2 |
| n | 65 | 61 | 55 | 54 |

- 9.3.2.8 Binarization process for rem_intra_luma_pred_mode

Input to this process is a request for the syntax element rem_intra_luma_pred_mode, cNumBins, binIdx, ctxIdx, and intraPredModeNum.

Output of this process is the binarization of the syntax element.

The binarization for rem_intra_luma_pred_mode is as follows:given by .

Table 9-7 – Binarization for rem_intra_luma_pred_mode

| Value of rem_intra_luma_pred_mode | Bin string |
| --- | --- |
| less than 32 | FL, cMax = cNumBins |
| 32 | 111110 |
| 33 | 111111 |

The value rem_intra_luma_pred_mode is mapped to the codeword using the mapping from Table 9-49 according to the value of intraPredModeNum.

While parsing each bin, the variable binIdx is incremented by 1 starting with binIdx being set equal to 0 for the first bin.

When binIdx equals zero, ctxIdx is set equal 3. For binIdx values greater than zero, ctxIdx is set equal to zero.

Table 9-8 – Binarization for rem_intra_luma_pred_mode

| Value of rem_intra_luma_pred_mode | Bin string |  |  |
| --- | --- | --- | --- |
|  | intraPredModeNum = 18 | intraPredModeNum = 35 | intraPredModeNum = 4 |
| 0 | 000 | 000 | 0 |
| 1 | 001 | 0010 | 1 |
| 2 | 010 | 0011 |  |
| 3 | 011 | 0100 |  |
| 4 | 1000 | 0101 |  |
| 5 | 1001 | 01100 |  |
| 6 | 1010 | 01101 |  |
| 7 | 1011 | 01110 |  |
| 8 | 11000 | 01111 |  |
| 9 | 11001 | 10000 |  |
| 10 | 11010 | 10001 |  |
| 11 | 11011 | 10010 |  |
| 12 | 11100 | 10011 |  |
| 13 | 11101 | 10100 |  |
| 14 | 11110 | 10101 |  |
| 15 | 11111 | 10110 |  |
| 16 |  | 10111 |  |
| 17 |  | 11000 |  |
| 18 |  | 110010 |  |
| 19 |  | 110011 |  |
| 20 |  | 110100 |  |
| 21 |  | 110101 |  |
| 22 |  | 110110 |  |
| 23 |  | 110111 |  |
| 24 |  | 111000 |  |
| 25 |  | 111001 |  |
| 26 |  | 111010 |  |
| 27 |  | 111011 |  |
| 28 |  | 111100 |  |
| 29 |  | 111101 |  |
| 30 |  | 111110 |  |
| 31 |  | 1111110 |  |
| 32 |  | 1111111 |  |

# References

- B. Bross, et al., “WD4: Working Draft 4 of High-Efficiency Video Coding,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-F803_d5, Torino, IT, July 2011.

- E. Maani, et al., “CE6.c: Differential Coding of Intra Modes,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-F8566, Torino, IT, July 2011.

- F. Bossen., “Common conditions and software reference configurations”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-F900, Torino, IT, July 2011

# Patent rights declaration(s)

Mitsubishi Electric may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).