| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G360 |
| --- | --- |

| Title: | CE5: Redundancy removal for Run-mode in CAVLC (F286) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Jun Xu 1730 N. First Street San Jose, CA 95112, USA | Tel: Email: | +1-408-352-4086 jun.xu@am.sony.com |
|  | Ali Tabatabai 1730 N. First Street San Jose, CA 95112, USA | Tel: Email: | +1-408-352-4715 ali.tabatabai@am.sony.com |
| Source: | Sony Electronics Inc. |  |  |

_____________________________

# Abstract

This report presents the results of JCTVC-F286[1] studied in CE5[2]. Simulation results show that BD-rate difference is -0.1% for Y, -0.1% for U, -0.1% for V in AI_LC, -0.1% for Y, -0.1% for U, 0.1% for V in RA_LC, and -0.1% for Y, -0.0% for U, -0.0% for V in LB_LC.

# Introduction

To remove the redundancy, JCTVC-F286 proposed to use truncated unary code for all cases where VLC table index = 0 is selected. In HM4.0, the VLC table indexes are stored in the variables shown as follows.

const UInt g_auiVlcTable8x8Inter[29]   ={8,0,0,5,5,5,5,5,5,5,6,6,6,6,6,6,6,6,6,6,6,6,6,6,6,6,6,6,3};

#if MOD_INTRA_TABLE

const UInt g_auiVlcTable8x8Intra[29]   = {8,0,0,0,0,1,5,5,5,5,5,5,5,5,5,5,5,5,1,1,1,1,1,1,1,1,1,1,1};

#else

const UInt g_auiVlcTable8x8Intra[29]   = {8,0,0,5,5,5,5,5,5,5,6,6,6,6,6,6,6,6,6,6,6,6,6,6,6,6,6,6,3};

#endif

#if CAVLC_COEF_LRG_BLK

#if MOD_INTRA_TABLE == 0

const UInt g_auiVlcTable16x16Intra[29] = {8,0,0,0,0,5,5,5,5,5,5,5,5,5,5,5,5,5,1,1,1,1,1,1,1,1,1,1,2};

#endif

const UInt g_auiVlcTable16x16Inter[29] = {8,0,1,1,1,1,2,2,2,2,2,2,2,6,6,6,6,6,6,3,3,3,3,3,3,3,3,3,3};

#endif

In implementation we virtually extend the number of VLC tables from 14 to 18. The extra four VLC tables are VLC table index 14 encodes max number of symbols is 5, VLC table index 15 encodes max number of symbols is 7, VLC table index 16 encodes max number of symbols is 9 and VLC table index 17 encodes max number of symbols is 11. The new VLC table indexes are stored in the variables shown as follows.

const UInt g_auiVlcTable8x8Inter[29]   = {8,14,15,5,5,5,5,5,5,5,6,6,6,6,6,6,6,6,6,6,6,6,6,6,6,6,6,6,3};

#if MOD_INTRA_TABLE

const UInt g_auiVlcTable8x8Intra[29]   = {8,14,15,16,17,1,5,5,5,5,5,5,5,5,5,5,5,5,1,1,1,1,1,1,1,1,1,1,1};

#else

const UInt g_auiVlcTable8x8Intra[29]   = {8,14,15,5,5,5,5,5,5,5,6,6,6,6,6,6,6,6,6,6,6,6,6,6,6,6,6,6,3};

#endif

#if CAVLC_COEF_LRG_BLK

#if MOD_INTRA_TABLE == 0

const UInt g_auiVlcTable16x16Intra[29] = {8,14,15,16,17,5,5,5,5,5,5,5,5,5,5,5,5,5,1,1,1,1,1,1,1,1,1,1,2};

#endif

const UInt g_auiVlcTable16x16Inter[29] = {8,14,1,1,1,1,2,2,2,2,2,2,2,6,6,6,6,6,6,3,3,3,3,3,3,3,3,3,3};

#endif

Accordingly, we extend the VLC length look-up table from static const int VLClength[14][128] to static const int VLClength[18][128] for RDOQ purpose in encoder. In the real VLC encoding/decoding function  Void TEncCavlc::xWriteVlc(UInt uiTableNumber, UInt uiCodeNumber)/ Int TDecCavlc::xReadVlc( Int n ), we actually called the existing function Void TEncCavlc::xWriteUnaryMaxSymbol( UInt uiSymbol, UInt uiMaxSymbol )/ Void TDecCavlc::xReadUnaryMaxSymbol( UInt& ruiSymbol, UInt uiMaxSymbol ) for VLC table index 14~17.

So there are no new tables introduced in decoder!

# Experiment setting and results

## Coding conditions

The conditions are the same as specified in JCTVC-E905[2].

## Results

A summary of results is listed in Table 1. Complete results are available in “JCTVC-G360.xls”.

Table 1 Simulation results using proposed solution

|  | All Intra LC |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | -0.1% | 0.0% | 0.0% |
| Class B | -0.1% | -0.1% | -0.1% |
| Class C | -0.1% | -0.1% | -0.1% |
| Class D | -0.1% | -0.1% | -0.1% |
| Class E | -0.1% | -0.1% | -0.1% |
| Overall | -0.1% | -0.1% | -0.1% |
|  | -0.1% | -0.1% | -0.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Random Access LC |  |  |
|  | Y | U | V |
| Class A | -0.1% | 0.0% | 0.2% |
| Class B | -0.1% | -0.1% | 0.0% |
| Class C | -0.1% | -0.2% | -0.1% |
| Class D | -0.1% | -0.2% | 0.3% |
| Class E |  |  |  |
| Overall | -0.1% | -0.1% | 0.1% |
|  | -0.1% | -0.1% | 0.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay B LC |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | -0.1% | 0.1% | 0.3% |
| Class C | -0.1% | 0.0% | -0.3% |
| Class D | -0.1% | 0.1% | -0.2% |
| Class E | -0.1% | 0.0% | 0.0% |
| Overall | -0.1% | 0.0% | 0.0% |
|  | -0.1% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 99% |  |  |

# Conclusion

This contribution reported results of JCTVC-F286 in CE5. BD-rate savings were observed while no complexity was introduced. We proposed to adopt this solution to HEVC.

# Reference

- J. Xu, A. Tabatabai, “Redundancy removal for Run-mode in CAVLC”, JCTVC-F286, Torino, Italy, Jul. 2011.

- “CAVLC entropy coding improvements”, JCTVC-F905, Torino, July 2011.

# Patent rights declaration(s)

Sony Electronics Inc.  may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).