| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G362 |
| --- | --- |

| Title: | Non-CE4: Efficient binarization of delta_QP in CABAC with signaling of Max and Min QP |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jun Xu 1730 N. First Street San Jose, CA 95112, USA | Tel: Email: | +1-408-352-4086 jun.xu@am.sony.com |
|  | Kenji Kondo 2-10-1, Osaki, Shinagawa-ku Tokyo, 141-8610, JAPAN | Tel: Email: | +81-3-5435-3308 Kenji.Kondo@jp.sony.com |
|  | Kazushi Sato Sony City Osaki 9F 2-10-1 Osaki, Shinagawa, 141-8610 Tokyo, Japan | Tel: Email: | +81 50 3750 2741 Kazushi.Sato@jp.sony.com |
|  | Ali Tabatabai 1730 N. First Street San Jose, CA 95112, USA | Tel: Email: | +1-408-352-4715 ali.tabatabai@am.sony.com |
| Source: | Sony Electronics Inc. |  |  |

_____________________________

# Abstract

In this proposal, the binarization of cu_delat_QP in CABAC is changed to remove redundancy and improve coding efficiency in the scenario of signaling Max and Min QP. Experimental results show that further savings on delta_QP bits can be achieved in CABAC. BD-rate savings are -0.1% with -2.0% savings in delta_QP bits for AI_HE, -0.1% with -2.7% savings in delta_QP bits for RA_HE, -0.1% with -2.9% savings in delta_QP bits for LB_HE over JCTVC-G462.Compared to CE4 anchor, BD-rate is -0.1% with -3.1% in delta_QP bits for AI_HE, 0.0% with -3.9% in delta_QP bits for RA_HE and -0.1% with -4.6% in delta_QP bits for LB_HE.

# Introduction

JCTVC-F174[1] proposed to signal Max and Min QP of a slice in the slice header, which was further studied in CE4 1.2.a[2]. Results are reported in JCTVC-G462[3].The idea is based on observation that QP values vary in a small range rather than the whole valid range of [0, 51]. Based on the range of QP values as well as predicted QP (refQP) from prediction, the possible values of cu_delta_QP can be determined. Then adaptive code index mapping table for cu_delta_QP can be designed accordingly to remove redundancy since some of the symbols will not occur. In CABAC, truncated unary code with maximum value of delta_QP index is used for binarization of delta_QP index.

# Solution

In fact, given maxQP, minQP and refQP, more redundancy can be removed. If we send the absolute value of delta_QP, we can derive the sign of delta_QP based on maxQP, minQP and refQP. Then some sign bits can be saved.

The proposed algorithm is as follows:

If delta_QP==0,

send flag 0.

Else

Send flag 1.

AbsDelta_QPMinus1 = |delta_QP|-1.

MaxAbsDelta_QPMinus1 = max(maxQP-refQP-1, refQP-minQP-1)

Binarize AbsDelta_QPMinus1 with truncated unary code with max= MaxAbsDelta_QPMinus1

If  refQP + AbsDelta_QPMinus1+1<= maxQP && refQP - AbsDelta_QPMinus1-1>=minQP

Send sign of delta_QP

To compare with JCTVC-F174, we study an example to show the advantage of proposed algorithm.

For example, if maxQP=32, minQP=24 and refQP=30, the indexing is done as follows:

| Index | dQP value | QP | JCTVC-F174/JCTVC-G462 | Proposed |
| --- | --- | --- | --- | --- |
| 0 | 0 | 30 | 0 | 0 |
| 1 | 1 | 31 | 10 | 100 |
| 2 | -1 | 29 | 110 | 101 |
| 3 | 2 | 32 | 1110 | 1100 |
| 4 | -2 | 28 | 11110 | 1101 |
| 5 | -3 | 27 | 11110 | 1110 |
| 6 | -4 | 26 | 1111110 | 11110 |
| 7 | -5 | 25 | 11111110 | 111110 |
| 8 | -6 | 24 | 11111111 | 111111 |

From the example, we can tell that proposed algorithm saves significant number of bits compared to JCTVC-F174/JCTVC-G462, while only proposed algorithm spends one more bit.

# Experiment setting and results

## Coding conditions

The algorithm is implemented on top of JCTVC-F174/JCTVC-G462 software. The conditions are the same as specified in CE4 subset 1 (JCTVC-F904[2]).

## Results

A summary of results of proposed algorithm is listed in Table 1, where the anchor is JCTVC-F174/JCTVC-G462.  Complete results are available in “JCTVC-G362.xls”.

Table 1 proposed solution vs JCTVC-G462

|  | All Intra HE |  |  |  |
| --- | --- | --- | --- | --- |
|  | Y | U | V | dQP incr. |
| Class A | -0.1% | -0.1% | -0.1% | -2.2% |
| Class B | -0.1% | -0.1% | -0.1% | -1.9% |
| Class C | -0.1% | -0.1% | -0.1% | -1.8% |
| Class D | -0.1% | -0.1% | -0.1% | -2.2% |
| Class E | -0.1% | -0.1% | -0.1% | -1.8% |
| Overall | -0.1% | -0.1% | -0.1% | -2.0% |
|  | -0.1% | -0.1% | -0.1% |  |
| Enc Time[%] | #NUM! |  |  |  |
| Dec Time[%] | #NUM! |  |  |  |
|  | Random Access HE |  |  |  |
|  | Y | U | V | dQP incr. |
| Class A | -0.1% | -0.1% | -0.1% | -2.6% |
| Class B | -0.1% | -0.1% | -0.1% | -2.3% |
| Class C | -0.1% | -0.1% | -0.1% | -2.6% |
| Class D | -0.1% | -0.1% | -0.1% | -3.2% |
| Class E |  |  |  |  |
| Overall | -0.1% | -0.1% | -0.1% | -2.7% |
|  | -0.1% | -0.1% | -0.1% |  |
| Enc Time[%] | #NUM! |  |  |  |
| Dec Time[%] | #NUM! |  |  |  |
|  | Low delay B HE |  |  |  |
|  | Y | U | V | dQP incr. |
| Class A |  |  |  |  |
| Class B | -0.1% | -0.1% | -0.1% | -2.6% |
| Class C | -0.1% | -0.1% | -0.1% | -2.8% |
| Class D | -0.1% | -0.1% | -0.1% | -3.6% |
| Class E | -0.1% | -0.1% | -0.1% | -2.5% |
| Overall | -0.1% | -0.1% | -0.1% | -2.9% |
|  | -0.1% | -0.1% | -0.1% |  |
| Enc Time[%] | #NUM! |  |  |  |
| Dec Time[%] | #NUM! |  |  |  |

Compared with CE4 anchor, the results are summarized in Table 2. Complete results are available in “JCTVC-G362_vs_CE4_anchor.xls”.

Table 2 Proposed vs HM4.0 CE4 anchor

|  | All Intra HE |  |  |  |
| --- | --- | --- | --- | --- |
|  | Y | U | V | dQP incr. |
| Class A | -0.1% | -0.1% | -0.1% | -3.1% |
| Class B | -0.1% | -0.1% | -0.1% | -2.4% |
| Class C | -0.1% | -0.1% | -0.1% | -2.7% |
| Class D | -0.1% | -0.1% | -0.1% | -4.1% |
| Class E | -0.2% | -0.2% | -0.1% | -3.2% |
| Overall | -0.1% | -0.1% | -0.1% | -3.1% |
|  | -0.1% | -0.1% | -0.1% |  |
| Enc Time[%] | 99% |  |  |  |
| Dec Time[%] | 100% |  |  |  |
|  | Random Access HE |  |  |  |
|  | Y | U | V | dQP incr. |
| Class A | -0.1% | 0.0% | -0.4% | -3.7% |
| Class B | -0.1% | -0.1% | 0.0% | -3.0% |
| Class C | -0.1% | -0.1% | -0.2% | -3.8% |
| Class D | 0.1% | 0.2% | 0.0% | -5.2% |
| Class E |  |  |  |  |
| Overall | 0.0% | 0.0% | -0.1% | -3.9% |
|  | 0.0% | 0.0% | -0.1% |  |
| Enc Time[%] | 100% |  |  |  |
| Dec Time[%] | 100% |  |  |  |
|  | Low delay B HE |  |  |  |
|  | Y | U | V | dQP incr. |
| Class A |  |  |  |  |
| Class B | -0.1% | 0.1% | -0.4% | -3.4% |
| Class C | -0.2% | -0.3% | 0.0% | -4.2% |
| Class D | 0.0% | -0.3% | 0.3% | -5.6% |
| Class E | 0.0% | 1.1% | -0.5% | -5.5% |
| Overall | -0.1% | 0.1% | -0.1% | -4.6% |
|  | -0.1% | 0.1% | 0.0% |  |
| Enc Time[%] | 100% |  |  |  |
| Dec Time[%] | 101% |  |  |  |

# Conclusion

This proposal improved the QP prediction performance using efficient binarization of cu_delta_QP when maxQP and minQP are available. Significant savings for cu_delta_QP bits were observed while no complexity was introduced. We proposed to adopt this solution to HEVC.

# Reference

- K. Sugimoto, S. Sekiguchi, “Signaling of Max and Min QP in slice”, JCTVC-F174, Torino, July 2011.

- K. Sato, etc, “Description of Core Experiment 4: Quantization”, JCTVC-F904r3, Torino, July 2011.

- K. Sugimoto, A. Minezawa, S. Sekiguchi, “CE4 subset 1.2.a: results of Max/Min QP signaling”, JCTVC-G462, Geneva, November 2011.

# Patent rights declaration(s)

Sony Electronics Inc.  may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).