| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G363 |
| --- | --- |

| Title: | CE4: Improvement of delta-QP Coding (1.3.a) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Jun Xu 1730 N. First Street San Jose, CA 95112, USA | Tel: Email: | +1-408-352-4086 jun.xu@am.sony.com |
|  | Kenji Kondo 2-10-1, Osaki, Shinagawa-ku Tokyo, 141-8610, JAPAN | Tel: Email: | +81 50 3750 2741 Kenji.Kondo@jp.sony.com |
|  | Kazushi Sato Sony City Osaki 9F 2-10-1 Osaki, Shinagawa, 141-8610 Tokyo, Japan | Tel: Email: | +81 50 3750 2741 Kazushi.Sato@jp.sony.com |
|  | Ali Tabatabai 1730 N. First Street San Jose, CA 95112, USA | Tel: Email: | +1-408-352-4715 ali.tabatabai@am.sony.com |
| Source: | Sony Electronics Inc. |  |  |

_____________________________

# Abstract

This report presents the results of CE4 1.3.a. Two tests are defined for CE4 1.3.a. One is CE4 anchor + 1.3.a, which provides BD-rate 0.0% in AI_HE, 0.0% in RA_HE, -0.1% in LB_HE and 0.0% in AI_LC, 0.0% in RA_LC, -0.1% in LB_LC, and dQP increasing is 0.1% in AI_HE, -0.8% in RA_HE, -1.6% in LB_HE and 0.1% in AI_LC, -1.0% in RA_LC, -2.0% in LB_LC. The other is CE4 1.3.c + 1.3.a, which provides BD-rate 0.1% in AI_HE, 0.1% in RA_HE, 0.0% in LB_HE and 0.1% in AI_LC, 0.0% in RA_LC, 0.0% in LB_LC, and dQP increasing is 1.3% in AI_HE, 0.3% in RA_HE, 0.1% in LB_HE and 1.6% in AI_LC, 0.1% in RA_LC, -0.3% in LB_LC, compared to CE4 1.3.c.

# Introduction

In CE4 1.3.a [1], JCTVC-F422-subset 2.2[2] is tested alone and along with several proposals on QP prediction in CE4.

JCTVC-F422-subset 2.2 proposed that QP of the CUs with CBP=0 is not reliable for QP prediction. Therefore it is proposed to regard QP of such CU as “unavailable”. This proposal adds CBP!=0 into the if-else conditions of each CU.

To test the performance of JCTVC-F422-subset 2.2, in CE4 1.3.a, two tests are conducted.

- CE4 anchor + CE4 1.3.a

- CE4 1.3.c + 1.3.a

# Experiment setting and results

## Coding conditions

The conditions are the same as specified in CE4 subset 1 (JCTVC-F904[1]).

## Results

### CE4 anchor + 1.3.a

A summary of results is listed in Table 1, where the anchor is CE4 anchor. Complete results are available in “JCTVC-G363_a.xls”.

Table 1 Simulation results of CE4 anchor + 1.3.a vs CE4 anchor

|  | All Intra HE |  |  |  | All Intra LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A | 0.0% | 0.0% | 0.0% | 0.3% | 0.0% | 0.0% | 0.0% | 0.3% |
| Class B | 0.0% | 0.0% | 0.0% | 0.2% | 0.0% | 0.0% | 0.0% | 0.2% |
| Class C | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.2% | 0.0% | 0.0% | 0.0% | 0.2% |
| Class E | 0.0% | -0.1% | 0.0% | -0.3% | 0.0% | 0.0% | 0.0% | -0.1% |
| Overall | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.1% |
|  | 0.0% | 0.0% | 0.0% |  | 0.0% | 0.0% | 0.0% |  |
| Enc Time[%] | #NUM! |  |  |  | #NUM! |  |  |  |
| Dec Time[%] | #NUM! |  |  |  | #NUM! |  |  |  |
|  | Random Access HE |  |  |  | Random Access LC |  |  |  |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A | 0.0% | -0.1% | -0.1% | -0.5% | -0.1% | -0.1% | -0.1% | -0.8% |
| Class B | 0.0% | 0.0% | -0.1% | -1.1% | 0.0% | -0.1% | -0.2% | -1.3% |
| Class C | 0.0% | 0.0% | -0.1% | -0.8% | -0.1% | -0.1% | 0.0% | -1.1% |
| Class D | 0.0% | 0.0% | 0.0% | -0.7% | 0.0% | -0.1% | 0.0% | -0.8% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | -0.1% | -0.8% | 0.0% | -0.1% | -0.1% | -1.0% |
|  | 0.0% | 0.0% | -0.1% |  | 0.0% | -0.1% | -0.1% |  |
| Enc Time[%] | #NUM! |  |  |  | #NUM! |  |  |  |
| Dec Time[%] | #NUM! |  |  |  | #NUM! |  |  |  |
|  | Low delay B HE |  |  |  | Low delay B LC |  |  |  |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A |  |  |  |  |  |  |  |  |
| Class B | -0.1% | -0.1% | -0.1% | -1.8% | -0.1% | -0.1% | -0.2% | -2.1% |
| Class C | -0.1% | -0.2% | -0.2% | -1.3% | -0.1% | -0.3% | 0.0% | -1.6% |
| Class D | -0.1% | -0.6% | 0.5% | -1.0% | 0.0% | -0.1% | -0.4% | -1.2% |
| Class E | -0.1% | -0.6% | -0.4% | -2.6% | -0.1% | -0.8% | -0.6% | -3.2% |
| Overall | -0.1% | -0.4% | 0.0% | -1.6% | -0.1% | -0.3% | -0.3% | -2.0% |
|  | -0.1% | -0.3% | 0.0% |  | -0.1% | -0.3% | -0.3% |  |
| Enc Time[%] | #NUM! |  |  |  | #NUM! |  |  |  |
| Dec Time[%] | #NUM! |  |  |  | #NUM! |  |  |  |

### CE4 1.3.c + 1.3.a

A summary of results is listed in Table 2, where the anchor is CE4 1.3.c. Complete results are available in “JCTVC-G363_b.xls”.

Table 2 Simulation results of CE4 1.3.c + 1.3.a vs CE4 1.3.c

|  | All Intra HE |  |  |  | All Intra LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A | 0.0% | 0.1% | 0.1% | 1.0% | 0.0% | 0.0% | 0.1% | 1.3% |
| Class B | 0.1% | 0.0% | 0.0% | 1.3% | 0.1% | 0.1% | 0.1% | 1.7% |
| Class C | 0.0% | 0.0% | 0.0% | 0.9% | 0.1% | 0.0% | 0.0% | 1.1% |
| Class D | 0.0% | 0.0% | 0.0% | 0.7% | 0.0% | 0.0% | 0.0% | 1.0% |
| Class E | 0.2% | 0.2% | 0.2% | 2.9% | 0.2% | 0.1% | 0.1% | 3.1% |
| Overall | 0.1% | 0.1% | 0.1% | 1.3% | 0.1% | 0.1% | 0.1% | 1.6% |
|  | 0.1% | 0.1% | 0.1% |  | 0.1% | 0.1% | 0.1% |  |
| Enc Time[%] | #NUM! |  |  |  | #NUM! |  |  |  |
| Dec Time[%] | #NUM! |  |  |  | #NUM! |  |  |  |
|  | Random Access HE |  |  |  | Random Access LC |  |  |  |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A | 0.1% | 0.3% | 0.4% | 0.4% | 0.0% | 0.1% | 0.1% | -0.1% |
| Class B | 0.0% | 0.0% | -0.1% | -0.5% | 0.0% | 0.0% | 0.0% | -0.5% |
| Class C | 0.1% | 0.0% | -0.1% | 0.7% | 0.1% | 0.0% | 0.1% | 0.6% |
| Class D | 0.0% | 0.0% | 0.1% | 0.7% | 0.0% | -0.1% | 0.0% | 0.6% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | 0.1% | 0.1% | 0.1% | 0.3% | 0.0% | 0.0% | 0.0% | 0.1% |
|  | 0.1% | 0.1% | 0.1% |  | 0.0% | 0.0% | 0.0% |  |
| Enc Time[%] | #NUM! |  |  |  | #NUM! |  |  |  |
| Dec Time[%] | #NUM! |  |  |  | #NUM! |  |  |  |
|  | Low delay B HE |  |  |  | Low delay B LC |  |  |  |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A |  |  |  |  |  |  |  |  |
| Class B | -0.1% | -0.1% | -0.2% | -1.0% | -0.1% | -0.1% | 0.0% | -1.6% |
| Class C | 0.0% | -0.1% | 0.2% | 0.5% | 0.0% | 0.2% | 0.2% | 0.3% |
| Class D | -0.1% | -0.5% | 0.6% | 0.7% | 0.1% | 0.7% | 0.1% | 0.5% |
| Class E | 0.0% | 0.0% | -0.7% | 0.6% | 0.0% | 0.0% | 0.0% | -0.2% |
| Overall | 0.0% | -0.2% | 0.0% | 0.1% | 0.0% | 0.2% | 0.1% | -0.3% |
|  | 0.0% | -0.1% | 0.0% |  | 0.0% | 0.2% | 0.1% |  |
| Enc Time[%] | #NUM! |  |  |  | #NUM! |  |  |  |
| Dec Time[%] | #NUM! |  |  |  | #NUM! |  |  |  |

# Conclusion

This report presented CE4 1.3.a test results. We proposed to adopt this solution to HEVC.

# Reference

- “Description of Core Experiment 4: Quantization”, JCTVC-F904r3, Torino, July 2011.

- K. Kondo, K. Sato, J. Xu, “Improvement of delat-QP coding”, JCTVC-F422, Torino, July 2011.

# Patent rights declaration(s)

Sony Electronics Inc.  may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).