| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G367 |
| --- | --- |

| Title: | CE5: cross-check for Samsung’s CAVLC (JCTVC-G532) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Jun Xu, Munsi Haque 1730 N. First Street San Jose, CA 95112, USA | Tel: Email: | +1-408-352-4086 jun.xu@am.sony.com munsi.haque@am.sony.com |
| Source: | Sony Electronics Inc. |  |  |

_____________________________

# Abstract

This report presents the cross verification results of JCTVC-G532. Studying of software shows that implementation matches with descriptions in JCTVC-G532. Simulation results match with results from proponents of JCTVC-G532. Simulation time from cross-checking results is close to that from proponents’ results.

# Introduction

The algorithm of JCTVC-G532 was first proposed as JCTVC-F458 and further studied in CE5 [1]. Here is the quote from CE5 about JCTVC-F458.

“In JCTVC-F458, it is proposed that intra chroma coefficients are coded using intra-block run-level index mapping instead of inter-block run-level index mapping. Currently in HM3.0 all chroma coefficients are coded using inter-block run-level index mapping. It is reported by doing so coding efficiency can be improved.”

Software of JCTVC-F458 was released on 9/13/2011. The cross-check was conducted using the following settings.

#define  RUN_PRED_MODE                    1          // F458

#define  CAVLC_EXCEPTION                  0          // F466  using encoder_intra_loco_exception.cfg (using PP_Mov_0041_1280x720_30.265 sequence)

# Results

A summary of results is listed in Table 1. Detailed results can be found in “JCTVC-G367.xls”.

Table 1 Results of cross-checking JCTVC-G532

|  | All Intra LC |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | -0.02% | 0.16% | -0.03% |
| Class B | -0.07% | 0.50% | 0.43% |
| Class C | -0.11% | 0.62% | 0.49% |
| Class D | -0.13% | 0.58% | 0.31% |
| Class E | -0.02% | 0.37% | 0.51% |
| Overall | -0.07% | 0.45% | 0.34% |
|  | -0.07% | 0.42% | 0.27% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 98% |  |  |
|  | Random Access LC |  |  |
|  | Y | U | V |
| Class A | 0.03% | 0.98% | 0.80% |
| Class B | -0.02% | 0.63% | 0.69% |
| Class C | -0.04% | 0.58% | 0.66% |
| Class D | -0.04% | 0.81% | 0.59% |
| Class E |  |  |  |
| Overall | -0.02% | 0.74% | 0.68% |
|  | -0.02% | 0.72% | 0.68% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 102% |  |  |
|  | Low delay B LC |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.01% | 0.32% | 0.43% |
| Class C | -0.04% | 0.34% | 0.05% |
| Class D | -0.02% | 0.25% | -0.16% |
| Class E | -0.12% | -0.10% | 0.18% |
| Overall | -0.03% | 0.23% | 0.14% |
|  | -0.03% | 0.15% | 0.13% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |

# Reference

- X. Wang, et al., “CAVLC entropy coding improvements,” JCT-VC document JCTVC-F905, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 6th meeting, Torino, July 2011.