| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G369 |
| --- | --- |

| Title: | CE6.c: cross-check for SDIP (JCTVC-F532) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Jun Xu 1730 N. First Street San Jose, CA 95112, USA | Tel: Email: | +1-408-352-4086 jun.xu@am.sony.com |
|  | Ali Tabatabai 1730 N. First Street San Jose, CA 95112, USA | Tel: Email: | +1-408-352-4715 ali.tabatabai@am.sony.com |
| Source: | Sony Electronics Inc. |  |  |

_____________________________

# Abstract

This report presents the cross verification results of CE6 4.3.1 JCTVC-F532. Studying of software shows that software implementation matches with descriptions in CE6. BD-rate and PSNR match with results from proponents. Simulation time information is close to that from proponents.

# Introduction

SDIP [1] was further studied in CE6 subset c [2]. In this cross verification, three cases were tested as follows.

Case 1: SDIP (with all harmonization listed in JCTVC-F532 [1]) without 1x16/16x1 TU;

Case 2: Case 1 with fast SDIP optimization algorithm;

Case 3: Case 1 simplification with non-square TU only (removing non-square PU).

# Results

## Case 1

A summary of results is listed in Error: Reference source not found. Detailed results can be found in “JCTVC-G369_case1.xls”.

Table 1 Cross-checking results of SDIP case 1

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.51% | -1.55% | -1.94% | -1.25% | -1.91% | -2.54% |
| Class B | -1.25% | -2.36% | -2.58% | -2.33% | -2.43% | -2.97% |
| Class C | -1.53% | -1.43% | -1.73% | -2.30% | -2.44% | -2.89% |
| Class D | -1.51% | -1.18% | -1.13% | -2.12% | -2.05% | -2.14% |
| Class E | -2.18% | -5.34% | -4.88% | -3.76% | -7.06% | -5.61% |
| Overall | -1.35% | -2.22% | -2.34% | -2.28% | -2.95% | -3.10% |
|  | -1.35% | -2.21% | -2.32% | -2.28% | -2.93% | -3.07% |
| Enc Time[%] | 128% |  |  | 134% |  |  |
| Dec Time[%] | 102% |  |  | 98% |  |  |

## Case 2

A summary of results is listed in. Detailed results can be found in “JCTVC-G369_case2.xls”.

Table 2 Cross-checking results of SDIP case 2

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.28% | -1.11% | -1.35% | -0.76% | -1.29% | -1.84% |
| Class B | -0.90% | -1.97% | -2.11% | -1.61% | -1.63% | -2.14% |
| Class C | -1.04% | -1.02% | -1.26% | -1.58% | -1.80% | -2.18% |
| Class D | -1.13% | -0.93% | -0.92% | -1.59% | -1.62% | -1.74% |
| Class E | -1.62% | -4.27% | -3.87% | -2.72% | -5.69% | -4.44% |
| Overall | -0.96% | -1.74% | -1.82% | -1.60% | -2.20% | -2.35% |
|  | -0.96% | -1.74% | -1.80% | -1.60% | -2.20% | -2.33% |
| Enc Time[%] | 114% |  |  | 120% |  |  |
| Dec Time[%] | 101% |  |  | 98% |  |  |

## Case 3

A summary of results is listed in. Detailed results can be found in “JCTVC-G369_case3.xls”.

Table 3 Cross-checking results of SDIP case 3

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.24% | -0.80% | -0.95% | -0.50% | -1.12% | -1.35% |
| Class B | -0.82% | -1.65% | -1.66% | -1.38% | -1.66% | -1.93% |
| Class C | -0.70% | -0.69% | -0.80% | -0.78% | -0.96% | -1.22% |
| Class D | -0.60% | -0.50% | -0.50% | -0.65% | -0.81% | -0.79% |
| Class E | -1.63% | -3.12% | -2.82% | -2.37% | -4.89% | -3.69% |
| Overall | -0.76% | -1.28% | -1.29% | -1.09% | -1.72% | -1.71% |
|  | -0.76% | -1.27% | -1.27% | -1.09% | -1.74% | -1.70% |
| Enc Time[%] | 115% |  |  | 114% |  |  |
| Dec Time[%] | 103% |  |  | 98% |  |  |

# Reference

- “CE6.b Test Summary and spec text of SDIP”, JCTVC-F532, Torino, July 2011

- “CE6: Intra Coding Improvements”, JCTVC-F906, Torino, July 2011.