| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G370 |
| --- | --- |

| Title: | Binarisation of last position for higher throughput |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Cheung Auyeung 1730 N. First Street, MD:3NW San Jose, CA 95112 USA | Tel: Email: | 1-408-352-4725 cheung.auyeung@am.sony.com |
| Source: | Sony Electronics Inc. |  |  |

_____________________________

# Abstract

For encoding with CABAC, HM4.0 binarised the x and y coordinates of the last position with unary codes interleaved with fixed binary codes. To improve throughput, this contribution propose to reorder the binarisation with the unary code of x and y followed by fixed binary codes of x and y. Experimental results reported 0.0% BD-rate gain.

# Introduction

As shown in Figure 1, HM4.0 adopted JCTVC-F357 [1] to binarise the last position coordinates (x,y) of transform coefficients with unary codes and fixed binary codes. The unary binarisation and the fixed binarisation are interleaved. The fixed bins are encoded in bypass mode with CABAC and they do not require contexts.

Figure 1: To encode the last position coordinates (x,y) of transform coefficients by HM4 with CABAC, the unary binarisation is interleaved with fixed binarisation.

# Proposal

As shown in Figure 2, this contribution propose to group the x and y fixed bin together to improve throughput since multiple bypass bins can be coded in a single cycle for increased throughput [2].

Figure 2: To improve throughput, the unary bins of both x and y are encoded first, then the fixed bins of x and y are encoded later in bypass mode with CABAC.

# Simulation Results

The algorithms were integrated into HM4.0. The simulations were performed in three Microsoft HPC clusters where the common test conditions and reference configurations specified by the Software AhG are followed:

- All intra simulations are performed on AMD Opteron Processor 6136 cluster @ 2.4GHz.

- All RA simulations are performed on Intel Xeon X5690 cluster @ 3.47GHz.

- All LD simulations are performed on Intel Xeon X5680 cluster @ 3.33GHz.

|  | All Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 101% |  |  |
|  | Random Access HE |  |  |
|  | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% |
| Class E |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Low delay B HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |

Table 1: No change in coding efficiency is observed for grouping the bypass bins together.

# Summary

This contribution proposes grouping the bypass bins of the last position coordinates together to improve the throughput of the CABAC. Specifically, it proposes encoding the unary bins of the last position coordinates together followed by encoding the fixed bins of the last position coordinates. There is no impact on coding efficiency.

# References

- Vadim Seregin, Il-Koo Kim, “Binarisation modification for last position coding”, JCTVC-F130, 6th Meeting: Torino, IT, 14-22 July, 2011.

- Vivienne Sze, Madhukar Budagavi, “Parallel Context Processing of Coefficient Level”, JCTVC-F130, 6th Meeting: Torino, IT, 14-22 July, 2011.

# Patent rights declaration(s)

Sony Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Appendix: Text for Working Draft based on JCTVC-F803-v7

The changes are highlighted in yellow.

7.3.10		Residual coding CABAC syntax

| residual_coding_cabac( x0, y0, log2TrafoSize, trafoDepth, scanIdx, cIdx ) { | Descriptor |
| --- | --- |
| last_significant_coeff_x_prefix | ae(v) |
| last_significant_coeff_y_prefix | ae(v) |
| halfTraFoSize = 1 << ( log2TrafoSize – 1 ) |  |
| If (log2TrafoSize ≥ 3 && last_sginficant_coeff_x_prefix == halfTraFoSize) { |  |
| last_significant_coeff_x_suffix | ae(v) |
| last_significant_coeff_x = last_significant_coeff_x_prefix+ last_significant_coeff_x_suffix |  |
| } else |  |
| last_significant_coeff_x = last_significant_coeff_x_prefix |  |
| If (log2TrafoSize ≥ 3 && last_sginficant_coeff_y_prefix == halfTraFoSize) { |  |
| last_significant_coeff_y_suffix | ae(v) |
| last_significant_coeff _y= last_significant_coeff_y_prefix+ last_significant_coeff_y_suffix |  |
| } else |  |
| last_significant_coeff_y= last_significant_coeff_y_prefix |  |
| numCoeff = 0 |  |
| xC = ScanOrder[ log2TrafoSize − 2 ][ log2TrafoSize − 2 ][ scanIdx ][ numCoeff ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoSize − 2 ][ log2TrafoSize − 2 ][ scanIdx ][ numCoeff ][ 1 ] |  |
| while( ( xC != last_significant_coeff_x ) ¦¦ ( yC != last_significant_coeff_y ) ) { |  |
| numCoeff++ |  |
| xC = ScanOrder[ log2TrafoSize − 2 ][ log2TrafoSize − 2 ][ scanIdx ][ numCoeff ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoSize − 2 ][ log2TrafoSize − 2 ][ scanIdx ][ numCoeff ][ 1 ] |  |
| } |  |
| for( n = numCoeff − 1; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoSize − 2 ][ log2TrafoSize − 2 ][ scanIdx ][ n ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoSize − 2 ][ log2TrafoSize − 2 ][ scanIdx ][ n ][ 1 ] |  |
| significant_coeff_flag[ xC ][ yC ] | ae(v) |
| } |  |
| numLastSubset = numCoeff >> 4 |  |
| for( i = numLastSubset − 1; i >= 0; i− − ) { |  |
| offset = i << 4 |  |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoSize − 2 ][ log2TrafoSize − 2 ][ 0 ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoSize − 2 ][ log2TrafoSize − 2 ][ 0 ][ n + offset` ][ 1 ] |  |
| if( significant_coeff_flag[ xC ][ yC ] ) |  |
| coeff_abs_level_greater1_flag[ n ] | ae(v) |
| } |  |
| for( n = 15; n >= 0; n− − ) { |  |
| if( coeff_abs_level_greater1_flag[ n ] ) |  |
| coeff_abs_level_greater2_flag[ n ] | ae(v) |
| } |  |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoSize − 2 ][ log2TrafoSize − 2 ][ 0 ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoSize − 2 ][ log2TrafoSize − 2 ][ 0 ][ n + offset` ][ 1 ] |  |
| if( significant_coeff_flag[ xC ][ yC ] ) { |  |
| coeff_sign_flag[ n ] | ae(v) |
| } |  |
| for( n = 15; n >= 0; n− − ) { |  |
| if( coeff_abs_level_greater2_flag[ n ] ) |  |
| coeff_abs_level_minus3[ n ] | ae(v) |
| xC = ScanOrder[ log2TrafoSize − 2 ][ log2TrafoSize − 2 ][ 0 ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoSize − 2 ][ log2TrafoSize − 2 ][ 0 ][ n + offset` ][ 1 ] |  |
| if( significant_coeff_flag[ xC ][ yC ] ) { |  |
| transCoeffLevel[ x0 ][ y0 ][ trafoDepth ][ cIdx ][ n + offset ] = ( coeff_abs_level_minus3[ n ] + 3 ) * ( 1 − 2 * coeff_sign_flag[ n ] ) |  |
| } else |  |
| transCoeffLevel[ x0 ][ y0 ][ trafoDepth ][ cIdx ][ n + offset ] = 0 |  |
| } |  |
| } |  |
| } |  |

9.3.2.9		Binarization process for last_significant_coefficient_x and last_significant_coefficient_y

Input to this process is a request for a binarization for the syntax element last_significant_coefficient_pos with pos being x or y and log2TrafoSize.

Output of this process is the binarization of the syntax element.

The variable halfTrafoSize is set equal to 1 << ( log2TrafoSize – 1 ).

- If log2TrafoSize is smaller than 3, last_significant_coefficient_pos_prefix is equal to last_significant_coefficient_pos. The TU binarization process as specified in subclause Error: Reference source not found is invoked with the value of the syntax element last_significant_coeff_pos_prefix and cMax = halfTrafoSize + 1 as input and the bin string as output.

- Otherwise if last_significant_coefficient_pos is smaller than halfTrafoSize, last_significant_coefficient_pos_prefix is equal to last_significant_coefficient_pos. The TU binarization process as specified in subclause Error: Reference source not found is invoked with the value of the syntax element last_significant_coefficient_pos_prefix and cMax = halfTrafoSize as input and the bin string as output.

- Otherwise (log2TrafoSize is greater than 2 and last_significant_coefficient_pos is greater than halfTrafoSize – 1), last_significant_coefficient_pos_prefix is equal to halfTrafoSize and last_significant_coefficient_pos_suffix is equal to last_significant_coefficient_pos - halfTrafoSize. The bin string of the syntax element last_significant_coefficient_pos_prefix  and last_significant_coefficient_pos_suffix are derived as follows.

  - The bin string of last_significant_coefficient_pos_prefix is derived by the TU binarization process as specified in subclause Error: Reference source not found with the symbol value of halfTrafoSize and cMax = halfTrafoSize as input and the bin string as output.

  - The bin string of last_significant_coefficient_pos_suffix is derived by the FL binarization process as specified in subclause Error: Reference source not found with the symbol value of the syntax element last_significant_coefficient_suffix and cMax = halfTrafoSize − 1 as input and the bin string as output.