| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G371 |
| --- | --- |

| Title: | CE8 Subset b: cross-check for Cisco’s ALF (JCTVC-G498) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Jun Xu 1730 N. First Street San Jose, CA 95112, USA | Tel: Email: | +1-408-352-4086 jun.xu@am.sony.com |
|  | Ali Tabatabai 1730 N. First Street San Jose, CA 95112, USA | Tel: Email: | +1-408-352-4715 ali.tabatabai@am.sony.com |
| Source: | Sony Electronics Inc. |  |  |

_____________________________

# Abstract

This report presents the cross verification results of CE8 subset b/5.4.1 JCTVC-G498. Studying of software shows that implementation matches with description in JCTVC-G498. Simulation results match with results from proponents. Timing information is not available from proponents. Cross-check timing information is collected under the same conditions both for the anchor and cross-checked software.

# Introduction

In CE8 subset b [1], JCTVC-F542 [2] was further studied and tested. The description was quoted from CE8 as follows.

“This experiment evaluates ALF where estimation and transmission of the ALF coefficients is performed on a block-by-block basis, typically 64x64 or 128x128 (or rectangular size).  The ALF process is single-pass within the picture and without any dependency between the current block and future blocks in processing order.  The ALF targets a complexity operation point that is significantly lower than that of HM 4.0-ALF.”

JCTVC-G498 [3] provides the algorithm and results from proponents of JCTVC-F542.

# Results

A summary of results is listed in Error: Reference source not found. Detailed results can be found in “JCTVC-G371.xls”.

Table 1 Cross-checking results for JCTVC-G498.

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.70% | 1.90% | 1.39% | -2.35% | 0.40% | 0.40% |
| Class B | 0.40% | 2.73% | 2.22% | -1.35% | 0.50% | 0.51% |
| Class C | 0.46% | 3.80% | 3.97% | -0.89% | 0.55% | 0.56% |
| Class D | 0.20% | 3.13% | 3.49% | -0.40% | 0.31% | 0.32% |
| Class E | 0.65% | 4.46% | 6.12% | -1.91% | 0.90% | 0.90% |
| Overall | 0.47% | 3.12% | 3.24% | -1.35% | 0.51% | 0.52% |
|  | 0.47% | 3.08% | 3.22% | -1.34% | 0.52% | 0.52% |
| Enc Time[%] | 79% |  |  | 105% |  |  |
| Dec Time[%] | 91% |  |  | 107% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 1.51% | 7.47% | 6.45% | -3.62% | 1.43% | 0.71% |
| Class B | 0.97% | 4.09% | 2.56% | -2.43% | 0.55% | 0.54% |
| Class C | 0.88% | 2.60% | 3.02% | -1.51% | 0.92% | 0.85% |
| Class D | 0.87% | 2.15% | 2.45% | -1.64% | 0.48% | 0.38% |
| Class E |  |  |  |  |  |  |
| Overall | 1.05% | 4.08% | 3.56% | -2.31% | 0.83% | 0.62% |
|  | 1.06% | 4.03% | 3.55% | -2.32% | 0.84% | 0.61% |
| Enc Time[%] | 95% |  |  | 101% |  |  |
| Dec Time[%] | 90% |  |  | 109% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 1.10% | 3.61% | 2.52% | -2.18% | 0.57% | 0.72% |
| Class C | 1.00% | 3.22% | 3.50% | -1.99% | 0.95% | 0.80% |
| Class D | 0.74% | 3.46% | 3.74% | -1.06% | 0.94% | 0.97% |
| Class E | 1.30% | 3.15% | 3.39% | -3.13% | 1.34% | 0.86% |
| Overall | 1.02% | 3.39% | 3.23% | -2.03% | 0.90% | 0.83% |
|  | 1.02% | 3.37% | 3.25% | -2.02% | 0.90% | 0.84% |
| Enc Time[%] | 97% |  |  | 101% |  |  |
| Dec Time[%] | 92% |  |  | 107% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 1.15% | 3.87% | 2.55% | -5.08% | 0.22% | -0.28% |
| Class C | 0.44% | 2.40% | 2.71% | -2.47% | 0.62% | 0.71% |
| Class D | 0.07% | 2.14% | 2.10% | -0.90% | 0.30% | 0.22% |
| Class E | 1.32% | 4.33% | 3.83% | -7.02% | -0.16% | 0.37% |
| Overall | 0.74% | 3.16% | 2.72% | -3.75% | 0.27% | 0.21% |
|  | 0.73% | 3.18% | 2.67% | -3.74% | 0.34% | 0.20% |
| Enc Time[%] | 95% |  |  | 101% |  |  |
| Dec Time[%] | 91% |  |  | 108% |  |  |

# Reference

- “Description of Core Experiment 8 (CE8): Non-deblocking loop filtering”, JCTVC-F908, Torino, July 2011.

- “Low complexity adaptive loop filter with sub-frame delay”, JCTVC-F542, Torino, July 2011.

- “CE8: ALF with low latency and reduced complexity for HEVC”, JCTVC-G498, Geneva, November 2011.