| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G374 |
| --- | --- |

| Title: | Improving the Intra Prediction Based on a Uniform Probability Model |  |  |
| --- | --- | --- | --- |
| Status: | [Input Document to JCT-VC] |  |  |
| Purpose: | [Proposal] |  |  |
| Author(s) or Contact(s): | Lingzhi Liu 2330 Central Expy. Santa Clara, CA 95050 USA | Tel: Email: | +1 4083304996 lingzhi.liu@huawei.com |
| Source: | HiSilicon Technologies |  |  |

_____________________________

# Abstract

The prediction values of intra prediction are calculated through the predefined angTable table in HM. Performance decreasing is observed when there are varied diagonal textures. This contribution presents a new table based on a uniform probability model. The new and original angTable are chosen alternatively depending on the left and above intra prediction modes. Simulation results show that 0.9% (intra_HE) and 0.8% (intra_loco) coding gain are achieved for BasketballDill, and there is no change for other sequences. This technique can improve the performance of UIP for variational textures.

# Problem Statement

Current Unified Intra Prediction (UIP) has up to 33 directional modes [1][2]. According to the symmetric feature, a predefined 9-entry angTable table is defined to performance the calculation as in equation (1). Each entry i in the table is corresponding to a dedicated angle in the image space. Only adding, shifting and simple multiplication operations are required to get the prediction values which makes UIP well balanced for performance and complexity.

angTable[9]    = {0,    2,    5,   9,  13,  17,  21,  26,  32}      (1)

There are two remaining problems in the angTable design in UIP. First, it is not clear how the entries in the table are obtained. Second, a performance decreasing is observed for the sequence of BasketballDrill in which there are varied diagonal textures.

This contribution proposes a new angTable with the value of (2). In intra prediction, when both the intra prediction mode of left and above are not available or inter coded, or one of the modes is close to the diagonal direction, new table is select to do the prediction, otherwise the original table is selected.

angTable2[9]    = {0,    2,    5,   9,  13,  18,  23,  28,  32}     (2)

Section 2 describes the deriving method for the new table based on a uniform possibility assumption of the incoming angles. Section 3 shows the testing results and Section 4 concludes the contribution.

# Uniform Possibility Model and the Derivation of the new angTable

## Weighted intra prediction of UIP and its angles

If a line with angle θ reaches the predicting pixel P through two neighboring reference pixels Pl and Pr, the predicted value of P can be calculated by a weighted prediction method. The weighing factor wl and wr are according to the distance between Pl and Pr with the corresponding pixel P’ which locates on the same line. wl and wr can be obtained from Figure 1 as:

wl=cot(θs)−cot(θ)cot(θs)−cot(θe)wr=1−wl                            (3)

Where θs and θe are the angles of right/starting and left/ending reference pixels, θ is the angle of the line.

Figure 1. UIP intra prediction from the reference pixels

Current implementation of UIP is based on (3) and the angles defined in angTable are

θi =arccot(angTable[i]/32)                                 (4)

## Uniform probability model and equivalent averaging prediction angle

If θ is an angel in the angular interval [θ0, θ1], and it corresponds to the uniform distribution between θ0 and θ1, the averaging weighting factor Wl can be computed through definite integral.

Set interval [θ0, θ1] is partitioned into N sub-intervals, and

Δθ=θ1−θ0N                       (5)

Then, according to (3), the averaging predicting factor

Wl=n=1N1N∗cot(θs)−cot(θ0+n∗Δθ)cot(θs)−cot(θe)n=1NΔθθ1−θ0∗cot(θs)−cot(θ0+n∗Δθ)cot(θs)−cot(θe)                       (6)

When N→∞, (6) is turned to a definite integral:

Wl=limN→∞n=1N1N∗cot(θs)−cot(θ0+n∗Δθ)cot(θs)−cot(θe)limN→∞n=0NΔθθ1−θ0∗cot(θs)−cot(θ0+n∗Δθ)cot(θs)−cot(θe)1θ1−θ0∗cot(θs)−cot(θ0+θ)cot(θs)−cot(θe)dθ1θ1−θ0cot(θs)−cot(θ)cot(θs)−cot(θe)dθcot(θs)cot(θs)−cot(θe)−1θ1−θ0cot(θs)−cot(θe)∗cot(θ)dθ=1cot(θs)−cot(θe)∗cot(θs)−ln(sin(θ1))−ln(sin(θ0))θ1−θ0         (7)

Set

θequ=arccot(ln(sin(θ1))−ln(sin(θ0))θ1−θ0)       (8)

(7) can be written as

Wl=cot(θs)−cot(θequ)cot(θs)−cot(θe)             (9)

(9) has similar expression as (3), and θequ can be treated as the equivalent averaging prediction angle (EAPA).

## derivation of the new angTable entries

Because current intra prediction mode distribution in HEVC shows the highest priority of angle π/2 and a certain priority of π/4, to derive the EAPA of the new angTable, the fan-shape of π/4 in Figure 2 is divided into 7 equal intervals. Here Δθ=(π/2-π/4)/7, and the ith angular intervals is [π/4+i* Δθ , π/4+(i+1)* Δθ) (i=0..6).

Based on (8), the EAPA θequ,i of the ith angular interval can be calculated. The corresponding angles for the new angTable entries are then obtained as

θ[9]= (π/2, θequ,6, θequ,5, …, θequ,0, π/4)   (10)

According to (4) and (10), the entries in the angTable table are obtained as:

angTable2[i]=round(cot(θ[i])*32)       (i=0…8)   (11)

Figure 1. Partitions of the fan-shape of π/4 with 7 equal angle intervals

The calculated angTable2 is

θ[9]= {0,    2,    5,   9,  13,  18,  23,  29,  32}  (12)

Because the priority of angle π/4 is weaker than that of π/2, the angular interval Δθ is adjusted to (π/2-(π/4+ Δstep))/7, and the corresponding angTables are generated to verify the performances of each Δstep. It has been found that change 29 to 28 can get better results for videos with various textures. Then the final new angTable for UIP intra prediction is:

angTable2[9]    = {0,    2,    5,   9,  13,  18,  23,  28,  32}

# Experimental results

Table 1 is the testing results of Case1 by using the new angTable2 only, which means the original angTable is replaced by angTable2.

Table 1. Case1: Replace the original angTable with new angTable2

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
| Class C | -0.2% | -0.2% | -0.2% | -0.2% | -0.2% | -0.3% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.1% | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 98% |  |  |

The overall performances are the same. But noticeable coding gain is obtained for the sequences of BasketballDrill832x480

Table 2. Comparison of coding performance of BasketballDrill for Case1

| BasketballDrill832x480 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| AI_HE |  |  | AI_LC |  |  |
| Y | U | V | Y | U | V |
| -1.02% | -0.80% | -0.90% | -0.90% | -0.77% | -0.98% |

The maximum 0.16% and 0.18% coding lose is from PeopleOnStreet for HE and Loco cases. Other coding losses are less than 0.12%.

Case1 implies that the original angTable has performance decreasing when there are varying diagonal textures.

Test Case2 is alternatively using the original or the new angTable. The condition for using the new angTable is: when both the intra prediction mode of left and above are not available or they are inter blocks, or one of the modes is 4, 7, 10, 11, 14, 15, 18, 19, 26, 27 or 34, which means one of the neighboring modes is close to the diagonal direction. For intra chroma prediction, only the new table is used.

Table 3 lists the testing results of Case2 by using the above method.

Table 3. Case2: alternatively using original and new angTable

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | -0.2% | -0.1% | -0.2% | -0.2% | -0.2% | -0.2% |
| Class D | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |

There are marginal overall coding gains. The coding gain for BasketballDrill is mostly kept as in Table 4.

Table 4. Comparison of coding performance of BasketballDrill for Case2

| BasketballDrill832x480 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| AI_HE |  |  | AI_LC |  |  |
| Y | U | V | Y | U | V |
| -0.92% | -0.55% | -0.73% | -0.82% | -0.57% | -0.79% |

By applying the method, most of the coding gain is kept for BasketballDrill. Others are kept the same performance as the anchor.

Test Case3 is the same as Case2 except that for intra chroma prediction, only the original table is used.

Table 5. Case3: Case2 except chroma using original table

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% |
| Class B | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | -0.2% | -0.1% | -0.1% | -0.2% | -0.1% | -0.1% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  | 102% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |

Table 6. Comparison of coding performance of BasketballDrill for Case3

| BasketballDrill832x480 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| AI_HE |  |  | AI_LC |  |  |
| Y | U | V | Y | U | V |
| -0.82% | -0.52% | -0.47% | -0.73% | -0.46% | -0.40% |

Compared with Case2 and Case3, we can see that using new table for chroma is generally better.

# WD Changes

## Case1

Figure 8-1 – Intra prediction angle definition (informative)

Table 8-1 – Specification of intraPredAngle

| intraPredOrder | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| intraPredAngle | - | -32 | -28 | -23 | -18 | -13 | -9 | -5 | -2 | - | 2 | 5 | 9 | 13 | 18 | 23 | 28 |
| intraPredOrder | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 |
| intraPredAngle | 32 | -28 | -23 | -18 | -13 | -9 | -5 | -2 | - | 2 | 5 | 9 | 13 | 18 | 23 | 28 | 32 |

## Case2

8.3.1.1.6 Specification of Intra_Angular prediction mode

Inputs to this process are:

–	neighbouring samples p[ x, y ], with x, y = -1..2*nS-1,

–	a variable nS specifying the prediction size.

–	variable arrays IntraPredMode (If available) that are previously (in decoding order) derived for adjacent coding units.

Output of this process is:

–	predicted samples predSamples[ x, y ], with x, y =0..nS-1.

This intra prediction mode is invoked when intraPredMode is in the range of 3..33.

Table 8-2 specifies the mapping table between intraPredMode and the rearranged intra prediction order intraPredOrder.

Table 8-2 – Specification of intraPredOrder

| intraPredMode | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| intraPredOrder | - | - | - | - | 1 | 5 | 13 | 17 | 21 | 29 | 33 | 3 | 7 | 11 | 15 | 19 | 23 | 27 |
| intraPredMode | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 |  |
| intraPredOrder | 31 | 2 | 4 | 6 | 8 | 10 | 12 | 14 | 16 | 18 | 20 | 22 | 24 | 26 | 28 | 30 | 32 |  |

The variable iTable is derived as follows:

–	Two variables intraPredModeA and intraPredModeB are specified in subclause 8.3.1.

–	If intraPredModeA is equal to -1 and intraPredModeB is equal to -1, or either intraPredModeA or intraPredModeB is equal to 4, 7, 10, 11, 14, 15, 18, 19, 26, 27 or 34, iTable is set equal to 0.

–	Otherwise, iTable is set equal to 1.

Figure 8-1 illustrates the total 34 intra angles and Table 8-1 specifies the mapping table between intraPredOrder and the angle parameter intraPredAngle when iTable is equal to 0.

Figure 8-2 – Intra prediction angle definition when iTable is equal to 0(informative)

Table 8-3 – Specification of intraPredAngle when iTable is equal to 0

| intraPredOrder | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| intraPredAngle | - | -32 | -28 | -23 | -18 | -13 | -9 | -5 | -2 | - | 2 | 5 | 9 | 13 | 18 | 23 | 28 |
| intraPredOrder | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 |
| intraPredAngle | 32 | -28 | -23 | -18 | -13 | -9 | -5 | -2 | - | 2 | 5 | 9 | 13 | 18 | 23 | 28 | 32 |

Table 8-4 further specifies the mapping table between intraPredOrder and the inverse angle parameter invAngle when iTable is equal to 0.

Table 8-4 – Specification of invAngle when iTable is equal to 0

| intraPredOrder | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| invAngle | -256 | -293 | -356 | -455 | -630 | -910 | -1638 | -4096 |
| intraPredOrder | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 |
| invAngle | -293 | -356 | -455 | -630 | -910 | -1638 | -4096 | - |

Figure 8-1 illustrates the total 34 intra angles and Table 8-1 specifies the mapping table between intraPredOrder and the angle parameter intraPredAngle when iTable is equal to 1.

Figure 8-3 – Intra prediction angle definition when iTable is equal to 1(informative)

Table 8-10 – Specification of intraPredAngle when iTable is equal to 1

| intraPredOrder | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| intraPredAngle | - | -32 | -26 | -21 | -17 | -13 | -9 | -5 | -2 | - | 2 | 5 | 9 | 13 | 17 | 21 | 26 |
| intraPredOrder | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 |
| intraPredAngle | 32 | -26 | -21 | -17 | -13 | -9 | -5 | -2 | - | 2 | 5 | 9 | 13 | 17 | 21 | 26 | 32 |

Table 8-411 further specifies the mapping table between intraPredOrder and the inverse angle parameter invAngle.

Table 8-5 – Specification of invAngle when iTable is equal to 1

| intraPredOrder | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| invAngle | -256 | -315 | -390 | -482 | -630 | -910 | -1638 | -4096 |
| intraPredOrder | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 |
| invAngle | -315 | -390 | -482 | -630 | -910 | -1638 | -4096 | - |

The reference pixel array refMain[ x ], with x=-nS..2*nS is specified as follows.

……

# Encoder Description changes

## Case1

Figure 5-3 – The 33 intra prediction directions

## Cae2

5.3 Intra Prediction

……

The 33 possible intra prediction directions are illustrated in  below depending on the intra prediction modes of neighboring blocks.

(a)

(b)

Figure 5-4 – The 33 intra prediction directions

# Cross checks

The authors would like to thank Institute for Infocomm Research (I2R) for crosschecking Case1 and Case2, and Intel for crosschecking Case3.

The crosschecking documents No. are E762 from I2R and G481 from Intel.

# Conclusion

This contribution proposes a new look up table for UIP intra prediction. Based on a uniform distribution assumption for the incoming angles, the new entries are derived for the angTable. Simulation shows noticeable coding gains are obtained for the sequences with plentiful diagonal textures by either using the new table only or alternatively using the new and original table. It is suggested to use the new table or alternatively use the new and original table in the HM software.

# Reference

[1] J. Min, S. Lee and et al., “Unification of the Directional Intra Prediction Methods in TMuC”, Doc. JCTVC-D100, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Geneva, Swiss, July 2010

[2] T.K. Tan and F Bossen., “TE5: Results for simplification of unified intra prediction”, Doc. JCTVC-C042, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Guangzhou, China, Oct 2010

# Patent rights declaration(s)

HiSilicon Technologies and Huawei Technologies Co. Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).