| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G379 |
| --- | --- |

| Title: | CE3:Cross-check report of LCU-based adaptive interpolation filter (JCTVC-G258) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Tomonobu Yoshino Sei Naito 2-1-15, Ohara, Fujimino-shi, Saitama, JAPAN | Tel: Email: | +81 49 278 7587 to-yoshino@kddi.com |
| Source: | KDDI Corp. (KDDI R&D Laboratories Inc.) |  |  |

_____________________________

# Abstract

This contribution reports the test results for LCU-based adaptive interpolation filter in CE3.

# Introduction

LCU-based adaptive interpolation filter by NTT is presented in document JCTVC-G258. Testing this approach in CE3 on motion compensation was decided in Torino meeting [1]

This contribution reports the cross-check result of this scheme. The software version was HM4.0. The reference was generated by using the common test conditions [2]. We compiled provided source code on 64-bit WindowsXP environment, and the experiments were performed on the same PCs.

Table 1 Experimental results for RA conditions

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.3% | -0.5% | -0.7% | -1.3% | -0.1% | -0.4% |
| Class B | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.1% | 0.1% | 0.1% | -0.1% | 0.2% | 0.2% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | -0.1% | -0.2% | -0.4% | 0.0% | -0.1% |
|  | -0.1% | -0.1% | -0.2% | -0.4% | 0.0% | -0.1% |
| Enc Time[%] | 103% |  |  | 104% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |
| Mult[%] | 0% |  |  | 0% |  |  |
| Adds[%] | 0% |  |  | 0% |  |  |
| Access[%] | 0% |  |  | 0% |  |  |
| MemBand(2D) Ave.[%] | 0% |  |  | 0% |  |  |
| MemBand(2D) Max.[%] | 5% |  |  | 5% |  |  |
| MemBand(1D) Ave.[%] | 0% |  |  | 0% |  |  |
| MemBand(1D) Max.[%] | 5% |  |  | 5% |  |  |

Table 2 Experimental results for LD-B conditions

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.1% | -0.5% | 0.2% | 0.3% |
| Class C | 0.0% | 0.1% | 0.1% | -0.2% | 0.2% | 0.0% |
| Class D | 0.1% | 0.2% | 0.3% | -0.1% | 0.2% | -0.1% |
| Class E | 0.0% | 0.1% | 0.7% | -1.7% | -0.1% | -0.2% |
| Overall | 0.0% | 0.1% | 0.3% | -0.5% | 0.2% | 0.0% |
|  | 0.0% | 0.1% | 0.3% | -0.5% | 0.2% | 0.1% |
| Enc Time[%] | 103% |  |  | 103% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |
| Mult[%] | 0% |  |  | 0% |  |  |
| Adds[%] | 0% |  |  | 0% |  |  |
| Access[%] | 0% |  |  | 0% |  |  |
| MemBand(2D) Ave.[%] | 0% |  |  | 0% |  |  |
| MemBand(2D) Max.[%] | 6% |  |  | 8% |  |  |
| MemBand(1D) Ave.[%] | 0% |  |  | 0% |  |  |
| MemBand(1D) Max.[%] | 6% |  |  | 8% |  |  |

Table 3 Experimental results for LD-P conditions

|  | Low delay P HE |  |  | Low delay P LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.1% | 0.1% | 0.0% | -2.7% | 0.1% | -0.1% |
| Class C | -0.1% | 0.2% | 0.3% | -0.9% | 0.1% | 0.1% |
| Class D | 0.0% | 0.4% | 0.0% | -0.2% | 0.1% | 0.0% |
| Class E | -0.3% | 0.3% | 0.3% | -5.2% | -0.7% | -0.2% |
| Overall | -0.1% | 0.2% | 0.1% | -2.1% | 0.0% | 0.0% |
|  | -0.1% | 0.3% | 0.1% | -2.1% | 0.0% | -0.1% |
| Enc Time[%] | 103% |  |  | 103% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |
| Mult[%] | 0% |  |  | 0% |  |  |
| Adds[%] | 0% |  |  | 0% |  |  |
| Access[%] | 0% |  |  | 0% |  |  |
| MemBand(2D) Ave.[%] | 0% |  |  | 0% |  |  |
| MemBand(2D) Max.[%] | 5% |  |  | 6% |  |  |
| MemBand(1D) Ave.[%] | 0% |  |  | 0% |  |  |
| MemBand(1D) Max.[%] | 4% |  |  | 6% |  |  |

- JCT-VC, “Core Experiment 3: Motion Compensation”, JCTVC-F903, 6th JCT-VC Meeting, Torino, IT, July. 2011.

- F. Bossen, “Common test conditions and software reference configurations,” JCTVC-F900, 6th. JCT-VC Meeting, Torino, IT, July 2011.

# Conclusions

We have verified results reported in JCTVC-G258. The results of the cross-check matched with NTT's report. The encoding/decoding time also matched closely to those reported by the proponent.