| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G381 WG11 Number: m21943 |
| --- | --- |

| Title: | Nearest placement of Y/Cb/Cr transform coefficients locating at same spatial position |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Youji Shibahara Kyoko Uchibayashi Takahiro Nishi | Tel: Email: | +81-50-3587-2044 shibahara.youji@jp.panasonic.com nishi.takahiro@jp.panasonic.com |
| Source: | Panasonic |  |  |

_____________________________

# Abstract

This contribution proposes a modification about the coding order of transform coefficient for Y, Cb and Cr. In the current HM4.0, the all transform coefficients for each color component in one CU are coded consecutively. One example decoder could be assumed to decode in the order of stream and the decoder would decode all Y coefficients and then decode all Cb coefficients and then decode all Cr coefficients in one CU. In this case, there might be a long time needed to get a first Y/Cb/Cr completed pixel in one CU, especially for large CU, such as CU64x64 having 4 blocks of TU32x32. In order to reduce the large latency, this contribution proposes that the Y/Cb/Cr coefficients constructing same pixel would be coded in the nearest stream order. An experimental result reportedly shows no coding performance drop in average and no difference of encoder and decoder runtime for all intra, random access and low delay configuration.

# Problem Statement

In the HM4.0, the transform coefficient of Y/Cb/Cr in one CU is coded in the order of all Y coefficients and all Cb coefficients and all Cr coefficients. Figure 1 (a) shows an example of one CU having 4 TUs. The typical decoder may need to wait until time slot 9 for completing the residual decoding of set of Y/Cb/Cr at first top-left TU. This long latency may require decoder having larger memory to store the intermediate decoded residual. Figure 2 (a) shows an example of memory for temporally storage. The number followed by Y/Cb/Cr means the position of TU. At time slot 8, the required memory size reaches to maximum and decoded residual image on the temporally memory would be Y1 ~ Y4 and Cb1 ~ Cb4.

# Solutions

## Basic specification of proposal

We propose a change of coding order of Y/Cb/Cr so that all Y/Cb/Cr transform coefficients which construct a pixel at same position should be coded at nearest in the stream order. Figure 1 (b) shows a modified order by our proposal. The first top-left block would be able to have all Y/Cb/Cr transform coefficients (Y1/Cb1/Cr1) at earlier time slot 3. The maximum size of temporally memory could be limited to only Y1 and Cb1 (at time slot 2).

Figure 1: stream order of Y/Cb/Cr

Figure 2: buffer transition

## Additional specification for minimum TU size

Where TU size is MinTuSize, we may need additional specification of coding order. Since the above proposal implicitly assumes that the number of luma transform coefficient block and the number of chroma transform coefficient block are equivalent. However in case of 4:2:0 or 4:2:2 color format and if the TU size equals MinTuSize, those numbers are different and the number of TUs for chroma would be less than the number of TUs for luma.

### Method 1

First proposed order which is not so different to the above stream order is shown in figure 3 (a). While there are 4 luma coefficients, only Y1 is coded at the first and the other remaining Y2~Y3 are coded separately followed by chroma Cb1 and Cr1. The potential issue seems to be requiring non-necessary switching of coefficient decoding process within luma block and chroma block and it might give an impact on throughput for such a small TU.

Figure 3: proposed stream order for MinTuSize

### Method 2

In order to prevent the issue, second proposed order is consecutively coding the luma coefficients locating at same chroma block. An example is shown in figure 3 (b).

# Experimental results

Table 1 shows RD performance and execution time of the software with proposed method 1 relative to HM version 4.0 software. Table 2 shows result for proposed method 2. The simulation is conducted under common condition as specified in JCTVC-F900 [1]. There is no change of BD-rate (and PSNR-rate) for All Intra HE/LC configuration and there is no difference of encoder and decoder runtime.

Table 1: The RD performance of method 1 (anchor: HM version 4)

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time | 100% |  |  | 99% |  |  |
| Dec Time | 100% |  |  | 100% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.2% | 0.2% | 0.0% | 0.2% | -0.1% |
| Class B | 0.0% | 0.0% | 0.1% | 0.0% | -0.1% | 0.1% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% |
| Class D | 0.0% | 0.2% | -0.2% | 0.0% | 0.1% | 0.2% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.1% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time | 99% |  |  | 99% |  |  |
| Dec Time | 99% |  |  | 99% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.1% | -0.2% | 0.0% | 0.0% | 0.1% |
| Class C | 0.0% | 0.1% | -0.1% | 0.0% | 0.2% | 0.0% |
| Class D | 0.0% | 0.6% | 0.0% | 0.0% | 0.2% | 0.1% |
| Class E | 0.0% | -0.1% | -0.5% | -0.1% | -0.1% | -0.6% |
| Overall | 0.0% | 0.2% | -0.2% | 0.0% | 0.1% | -0.1% |
|  | 0.0% | 0.2% | -0.2% | 0.0% | 0.1% | -0.1% |
| Enc Time | 100% |  |  | 100% |  |  |
| Dec Time | 99% |  |  | 100% |  |  |

Table 1: The RD performance of method 2 (anchor: HM version 4)

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time | 100% |  |  | 99% |  |  |
| Dec Time | 100% |  |  | 100% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.2% | -0.2% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.2% |
| Class C | 0.0% | -0.1% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | -0.1% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
| Enc Time | 100% |  |  | 100% |  |  |
| Dec Time | 100% |  |  | 99% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | -0.3% | 0.0% | 0.0% | 0.1% |
| Class C | 0.1% | 0.0% | -0.1% | 0.0% | 0.2% | -0.2% |
| Class D | 0.0% | 0.4% | 0.1% | 0.0% | 0.0% | -0.1% |
| Class E | 0.0% | -0.1% | 0.6% | 0.1% | -0.4% | 0.3% |
| Overall | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
| Enc Time | 100% |  |  | 100% |  |  |
| Dec Time | 100% |  |  | 99% |  |  |

# Conclusion

In this contribution, we proposed a change of transform coefficient order for color components so that the all Y/Cb/Cr components constructing same pixel should be placed nearest in the stream order. It would be able to reduce the latency until getting a first pixel which has all Y/Cb/Cr components and it is thought as to be beneficial for reduction of memory size to store intermediate decoded residual. An experimental result shows that no coding gain change and no runtime change in average for All Intra HE/LC and Random Access HE/LC and Low Delay B HE/LC.

We recommend that this proposal be adopted in HM.

# References

- F. Bossen., “Common conditions and software reference configurations”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-F900, 6th JCT-VC Meeting, Torino, IT, July 2011.

# Patent rights declaration(s)

Panasonic Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Woring draft modification

## Proposed Method 1 (Y1, Cb, Cr, Y2, Y3, Y4 at MinTu)

### coding unit syntax (for method 1)

| coding_unit( x0, y0, log2CUSize ) { | Descriptor |
| --- | --- |
| if( entropy_coding_mode_flag && slice_type != I ) |  |
| skip_flag[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| if( skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0 , log2CUSize ) |  |
| else { |  |
| if( !entropy_coding_mode_flag ) { |  |
| if( slice_type == I && log2CUSize == Log2MinCUSize ) |  |
| intra_part_mode | u(1) |
| } else if( slice_type != I ¦ ¦ log2CUSize = = Log2MinCUSize ) |  |
| pred_type | u(v) ¦ ae(v) |
| x1 = x0 + ( ( 1 << log2CUSize ) >> 1 ) |  |
| y1 = y0 + ( ( 1 << log2CUSize ) >> 1 ) |  |
| x2 = x1 − ( ( 1 << log2CUSize ) >> 2 ) |  |
| y2 = y1 − ( ( 1 << log2CUSize ) >> 2 ) |  |
| x3 = x1 + ( ( 1 << log2CUSize ) >> 2 ) |  |
| y3 = y1 + ( ( 1 << log2CUSize ) >> 2 ) |  |
| if( PartMode == PART_2Nx2N ) { |  |
| prediction_unit( x0, y0 , log2CUSize ) |  |
| } else if( PartMode == PART_2NxN ) { |  |
| prediction_unit( x0, y0 , log2CUSize ) |  |
| prediction_unit( x0, y1 , log2CUSize ) |  |
| } else if( PartMode == PART_Nx2N ) { |  |
| prediction_unit( x0, y0 , log2CUSize ) |  |
| prediction_unit( x1, y0 , log2CUSize ) |  |
| } else if( PartMode == PART_2NxnU ) { |  |
| prediction_unit( x0, y0 , log2CUSize ) |  |
| prediction_unit( x0, y2 , log2CUSize ) |  |
| } else if( PartMode == PART_2NxnD ) { |  |
| prediction_unit( x0, y0 , log2CUSize ) |  |
| prediction_unit( x0, y3 , log2CUSize ) |  |
| } else if( PartMode == PART_nLx2N ) { |  |
| prediction_unit( x0, y0 , log2CUSize ) |  |
| prediction_unit( x2, y0 , log2CUSize ) |  |
| } else if( PartMode == PART_nRx2N ) { |  |
| prediction_unit( x0, y0 , log2CUSize ) |  |
| prediction_unit( x3, y0 , log2CUSize ) |  |
| } else { /* PART_NxN */ |  |
| prediction_unit( x0, y0 , log2CUSize ) |  |
| prediction_unit( x1, y0 , log2CUSize ) |  |
| prediction_unit( x0, y1 , log2CUSize ) |  |
| prediction_unit( x1, y1 , log2CUSize ) |  |
| } |  |
| } |  |
| if( !pcm_flag ) { |  |
| transform_tree( x0, y0, log2CUSize, log2CUSize, 0, 0 ) |  |
| transform_coeff( x0, y0, log2CUSize, log2CUSize, 0, 0 ) |  |
| transform_coeff( x0, y0, log2CUSize, log2CUSize, 0, 1 ) |  |
| transform_coeff( x0, y0, log2CUSize, log2CUSize, 0, 2 ) |  |
| } |  |
| } |  |

### transform coefficient syntax (for method 1)

| transform_coeff( x0, y0, log2TrafoWidth, log2TrafoHeight, trafoDepth, cIdx blk ) { | Descriptor |
| --- | --- |
| if( ( ( cIdx = = 0 && cbf_luma[ x0 ][ y0 ][ trafoDepth ] ) ¦ ¦ ( cIdx = = 1 && cbf_cb[ x0 ][ y0 ][ trafoDepth ] ) ¦ ¦ ( cIdx = = 2 && cbf_cr[ x0 ][ y0 ][ trafoDepth ] ) ) ) { |  |
| if( cu_qp_delta_enabled_flag && !IsCuQpDeltaCoded ) { |  |
| cu_qp_delta | se(v) ¦ ae(v) |
| IsCuQpDeltaCoded = 1 |  |
| } |  |
| if( split_transform_flag[ x0 ][ y0 ][ trafoDepth ] && ( cIdx = = 0 ¦ ¦ log2TrafoSize > Log2MinTrafoSize + 1 ) ) { |  |
| if( InterTUSplitDirection = = 2 ) { |  |
| x1 = x0 + ( ( 1 << log2TrafoWidth ) >> 1 ) |  |
| y1 = y0 |  |
| x2 = x0 |  |
| y2 = y0 + ( ( 1 << log2TrafoHeight ) >> 1 ) |  |
| x3 = x1 |  |
| y3 = y2 |  |
| } else { |  |
| x1 = x0 + ( ( 1 << log2TrafoWidth ) >> 2 ) * InterTUSplitDirection |  |
| y1 = y0 + ( ( 1 << log2TrafoHeight) >> 2 ) * ( 1 − InterTUSplitDirection ) |  |
| x2 = x1 + ( ( 1 << log2TrafoWidth ) >> 2 ) * InterTUSplitDirection |  |
| y2 = y1 + ( ( 1 << log2TrafoHeight) >> 2 ) * ( 1 − InterTUSplitDirection ) |  |
| x3 = x2 + ( ( 1 << log2TrafoWidth ) >> 2 ) * InterTUSplitDirection |  |
| y3 = y2 + ( ( 1 << log2TrafoHeight) >> 2 ) * ( 1 − InterTUSplitDirection ) |  |
| log2TrafoHeight = log2TrafoHeight + 2 * InterTUSplitDirection − 1 |  |
| log2TrafoWidth = log2TrafoWidth − 2 * InterTUSplitDirection + 1 |  |
| } |  |
| transform_coeff( x0, y0, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, cIdx 0 ) |  |
| transform_coeff( x1, y1, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, cIdx 1 ) |  |
| transform_coeff( x2, y2, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, cIdx 2 ) |  |
| transform_coeff( x3, y3, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, cIdx 3 ) |  |
| } else { |  |
| log2TrafoSize = ( ( log2TrafoWidth + log2TrafoHeight ) >> 1 ) − ( cIdx > 0 ? 1 : 2 ) |  |
| log2TrafoSizeC = ( ( log2TrafoSize== Log2MinTrafoSize )? log2TrafoSize : log2TrafoSize – 1 ) |  |
| if ( PredMode = = MODE_INTRA ) { |  |
| if (cIdx = = 0) |  |
| scanIdx = ScanType[ log2TrafoSize – 2 ][ IntraPredMode ] |  |
| else |  |
| scanIdxC = ScanType[ log2TrafoSizeC – 2 ][ IntraPredModeC ] |  |
| } else { |  |
| scanIdx = 0 |  |
| scanIdxC = 0 |  |
| } |  |
| if ( entropy_coding_mode_flag ){ |  |
| if ( scanIdx = = 0 ) |  |
| scanIdx = 3 |  |
| if ( scanIdxC = = 0 ) |  |
| scanIdxC = 3 |  |
| if( cbf_luma[ x0 ][ y0 ][ trafoDepth ] ) |  |
| residual_coding_cabac( x0, y0, log2TrafoSize, trafoDepth, scanIdx, cIdx 0 ) |  |
| if( log2TrafoSize > Log2MinTrafoSize ¦ ¦ blk == 0 ) { |  |
| if( cbf_cb[ x0 ][ y0 ][ trafoDepth ] ) |  |
| residual_coding_cabac( x0, y0, log2TrafoSize, trafoDepth, scanIdxC, 1 ) |  |
| if( cbf_cr[ x0 ][ y0 ][ trafoDepth ] ) |  |
| residual_coding_cabac( x0, y0, log2TrafoSize, trafoDepth, scanIdxC, 2 ) |  |
| } |  |
| }else if ( !entropy_coding_mode_flag ){ |  |
| if( cbf_luma[ x0 ][ y0 ][ trafoDepth ] ) |  |
| residual_coding_cavlc( x0, y0, log2TrafoSize, trafoDepth, cIdx 0 ) |  |
| if( log2TrafoSize > Log2MinTrafoSize ¦ ¦ blk == 0 ) { |  |
| if( cbf_cb[ x0 ][ y0 ][ trafoDepth ] ) |  |
| residual_coding_cavlc( x0, y0, log2TrafoSize, trafoDepth, 1 ) |  |
| if( cbf_cr[ x0 ][ y0 ][ trafoDepth ] ) |  |
| residual_coding_cavlc( x0, y0, log2TrafoSize, trafoDepth, 2 ) |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

## Proposed Method 2 (Y1, Y2, Y3, Y4, Cb, Cr at MinTu)

### coding unit syntax (for method 2)

| coding_unit( x0, y0, log2CUSize ) { | Descriptor |
| --- | --- |
| if( entropy_coding_mode_flag && slice_type != I ) |  |
| skip_flag[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| if( skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0 , log2CUSize ) |  |
| else { |  |
| if( !entropy_coding_mode_flag ) { |  |
| if( slice_type == I && log2CUSize == Log2MinCUSize ) |  |
| intra_part_mode | u(1) |
| } else if( slice_type != I ¦ ¦ log2CUSize = = Log2MinCUSize ) |  |
| pred_type | u(v) ¦ ae(v) |
| x1 = x0 + ( ( 1 << log2CUSize ) >> 1 ) |  |
| y1 = y0 + ( ( 1 << log2CUSize ) >> 1 ) |  |
| x2 = x1 − ( ( 1 << log2CUSize ) >> 2 ) |  |
| y2 = y1 − ( ( 1 << log2CUSize ) >> 2 ) |  |
| x3 = x1 + ( ( 1 << log2CUSize ) >> 2 ) |  |
| y3 = y1 + ( ( 1 << log2CUSize ) >> 2 ) |  |
| if( PartMode == PART_2Nx2N ) { |  |
| prediction_unit( x0, y0 , log2CUSize ) |  |
| } else if( PartMode == PART_2NxN ) { |  |
| prediction_unit( x0, y0 , log2CUSize ) |  |
| prediction_unit( x0, y1 , log2CUSize ) |  |
| } else if( PartMode == PART_Nx2N ) { |  |
| prediction_unit( x0, y0 , log2CUSize ) |  |
| prediction_unit( x1, y0 , log2CUSize ) |  |
| } else if( PartMode == PART_2NxnU ) { |  |
| prediction_unit( x0, y0 , log2CUSize ) |  |
| prediction_unit( x0, y2 , log2CUSize ) |  |
| } else if( PartMode == PART_2NxnD ) { |  |
| prediction_unit( x0, y0 , log2CUSize ) |  |
| prediction_unit( x0, y3 , log2CUSize ) |  |
| } else if( PartMode == PART_nLx2N ) { |  |
| prediction_unit( x0, y0 , log2CUSize ) |  |
| prediction_unit( x2, y0 , log2CUSize ) |  |
| } else if( PartMode == PART_nRx2N ) { |  |
| prediction_unit( x0, y0 , log2CUSize ) |  |
| prediction_unit( x3, y0 , log2CUSize ) |  |
| } else { /* PART_NxN */ |  |
| prediction_unit( x0, y0 , log2CUSize ) |  |
| prediction_unit( x1, y0 , log2CUSize ) |  |
| prediction_unit( x0, y1 , log2CUSize ) |  |
| prediction_unit( x1, y1 , log2CUSize ) |  |
| } |  |
| } |  |
| if( !pcm_flag ) { |  |
| transform_tree( x0, y0, log2CUSize, log2CUSize, 0, 0 ) |  |
| transform_coeff( x0, y0, x0, y0, log2CUSize, log2CUSize, 0, 0 ) |  |
| transform_coeff( x0, y0, log2CUSize, log2CUSize, 0, 1 ) |  |
| transform_coeff( x0, y0, log2CUSize, log2CUSize, 0, 2 ) |  |
| } |  |
| } |  |

### transform coefficient syntax (for method 3)

| transform_coeff( x0, y0, xC, yC, log2TrafoWidth, log2TrafoHeight, trafoDepth, cIdx blk ) { | Descriptor |
| --- | --- |
| if( ( ( cIdx = = 0 && cbf_luma[ x0 ][ y0 ][ trafoDepth ] ) ¦ ¦ ( cIdx = = 1 && cbf_cb[ x0 ][ y0 ][ trafoDepth ] ) ¦ ¦ ( cIdx = = 2 && cbf_cr[ x0 ][ y0 ][ trafoDepth ] ) ) ) { |  |
| if( cu_qp_delta_enabled_flag && !IsCuQpDeltaCoded ) { |  |
| cu_qp_delta | se(v) ¦ ae(v) |
| IsCuQpDeltaCoded = 1 |  |
| } |  |
| if( split_transform_flag[ x0 ][ y0 ][ trafoDepth ] && ( cIdx = = 0 ¦ ¦ log2TrafoSize > Log2MinTrafoSize + 1 ) ) { |  |
| if( InterTUSplitDirection = = 2 ) { |  |
| x1 = x0 + ( ( 1 << log2TrafoWidth ) >> 1 ) |  |
| y1 = y0 |  |
| x2 = x0 |  |
| y2 = y0 + ( ( 1 << log2TrafoHeight ) >> 1 ) |  |
| x3 = x1 |  |
| y3 = y2 |  |
| } else { |  |
| x1 = x0 + ( ( 1 << log2TrafoWidth ) >> 2 ) * InterTUSplitDirection |  |
| y1 = y0 + ( ( 1 << log2TrafoHeight) >> 2 ) * ( 1 − InterTUSplitDirection ) |  |
| x2 = x1 + ( ( 1 << log2TrafoWidth ) >> 2 ) * InterTUSplitDirection |  |
| y2 = y1 + ( ( 1 << log2TrafoHeight) >> 2 ) * ( 1 − InterTUSplitDirection ) |  |
| x3 = x2 + ( ( 1 << log2TrafoWidth ) >> 2 ) * InterTUSplitDirection |  |
| y3 = y2 + ( ( 1 << log2TrafoHeight) >> 2 ) * ( 1 − InterTUSplitDirection ) |  |
| log2TrafoHeight = log2TrafoHeight + 2 * InterTUSplitDirection − 1 |  |
| log2TrafoWidth = log2TrafoWidth − 2 * InterTUSplitDirection + 1 |  |
| } |  |
| transform_coeff( x0, y0, x0, y0, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, cIdx 0) |  |
| transform_coeff( x1, y1, x0, y0, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, cIdx 1) |  |
| transform_coeff( x2, y2, x0, y0, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, cIdx 2) |  |
| transform_coeff( x3, y3, x0, y0, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, cIdx 3) |  |
| } else { |  |
| log2TrafoSize = ( ( log2TrafoWidth + log2TrafoHeight ) >> 1 ) − ( cIdx > 0 ? 1 : 2 ) |  |
| log2TrafoSizeC = ( ( log2TrafoSize== Log2MinTrafoSize )? log2TrafoSize : log2TrafoSize – 1 ) |  |
| if ( PredMode = = MODE_INTRA ) { |  |
| if (cIdx = = 0) |  |
| scanIdx = ScanType[ log2TrafoSize – 2 ][ IntraPredMode ] |  |
| else |  |
| scanIdxC = ScanType[ log2TrafoSizeC – 2 ][ IntraPredModeC ] |  |
| } else { |  |
| scanIdx = 0 |  |
| scanIdxC = 0 |  |
| } |  |
| if ( entropy_coding_mode_flag ){ |  |
| if ( scanIdx = = 0 ) |  |
| scanIdx = 3 |  |
| if ( scanIdxC = = 0 ) |  |
| scanIdxC = 3 |  |
| if( cbf_luma[ x0 ][ y0 ][ trafoDepth ] ) |  |
| residual_coding_cabac( x0, y0, log2TrafoSize, trafoDepth, scanIdx, cIdx 0 ) |  |
| if( log2TrafoSize > Log2MinTrafoSize ) { |  |
| if( cbf_cb[ x0 ][ y0 ][ trafoDepth ] ) |  |
| residual_coding_cabac( x0, y0, log2TrafoSize, trafoDepth, scanIdxC, 1 ) |  |
| if( cbf_cr[ x0 ][ y0 ][ trafoDepth ] ) |  |
| residual_coding_cabac( x0, y0, log2TrafoSize, trafoDepth, scanIdxC, 2 ) |  |
| }else if( log2TrafoSize == Log2MinTrafoSize && blk == 3 ) { |  |
| if( cbf_cb[ xC ][ yC ][ trafoDepth ] ) |  |
| residual_coding_cabac( xC, yC, log2TrafoSize, trafoDepth, scanIdxC, 1 ) |  |
| if( cbf_cr[ xC ][ yC ][ trafoDepth ] ) |  |
| residual_coding_cabac( xC, yC, log2TrafoSize, trafoDepth, scanIdxC, 2 ) |  |
| } |  |
| }else if ( !entropy_coding_mode_flag ){ |  |
| if( cbf_luma[ x0 ][ y0 ][ trafoDepth ] ) |  |
| residual_coding_cavlc( x0, y0, log2TrafoSize, trafoDepth, cIdx 0 ) |  |
| if( log2TrafoSize > Log2MinTrafoSize ) { |  |
| if( cbf_cb[ x0 ][ y0 ][ trafoDepth ] ) |  |
| residual_coding_cavlc( x0, y0, log2TrafoSize, trafoDepth, 1 ) |  |
| if( cbf_cr[ x0 ][ y0 ][ trafoDepth ] ) |  |
| residual_coding_cavlc( x0, y0, log2TrafoSize, trafoDepth, 2 ) |  |
| }else if( log2TrafoSize == Log2MinTrafoSize && blk == 3 ) { |  |
| if( cbf_cb[ xC ][ yC ][ trafoDepth ] ) |  |
| residual_coding_cavlc( xC, yC, log2TrafoSize, trafoDepth, 1 ) |  |
| if( cbf_cr[ xC ][ yC ][ trafoDepth ] ) |  |
| residual_coding_cavlc( xC, yC, log2TrafoSize, trafoDepth, 2 ) |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |