| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G382 |
| --- | --- |

| Title: | CE4-subtest2.2: Quantization with Adaptive Reconstruction Levels |  |  |
| --- | --- | --- | --- |
| Status: | [Input Document to JCT-VC] |  |  |
| Purpose: | [Proposal] |  |  |
| Author(s) or Contact(s): | Xiang Yu Jing Wang Da-ke He Gaelle Martin-Cocher | Email: | xiayu@rim.com wangj@rim.com dhe@rim.com gmartincocher@rim.com |
| Source: | [Research In Motion Ltd., Canada] |  |  |

_____________________________

# Abstract

This document reports results of CE4 subtest 2.2 for evaluating RIM’s proposal F276 about adaptive reconstruction levels (ARL) for four inter settings with RDOQ on or off.  For the RDOQ-on case, the overall  BD-rate averages show 1.0%, 1.7%, 1.6% and 2.2% rate saving for Luma for RAhe, RAloco, LDhe, and LDloco, respectively; With a rate constraint on every temporal level to prevent an undesired bit-allocation effect, most of the rate saving of ARL over the HM4.0 anchor is preserved, as Test-B showed 0.7%, 1.5%, 1.4% and 2.0% rate savings for Luma for RAhe, RAloco, LDhe, and LDloco, respectively.

For the RDOQ-off case, the overall BD-rate averages show 1.9%, 2.0%, 1.3%, and 1.7% rate saving for Luma for RAhe, RAloco, LDhe, and LDloco, respectively. The proposed scheme can be combined with the EEM technique, and the overall BD-rate averages show 1.3%, 1.4%, 1.3% and 1.7% rate savings of a combined scheme of EEM and ARL over an EEM-only scheme.

# Quantization Reconstruction Space Design

As transform coefficients usually assume a Laplacian distribution, a desired reconstruction space for quantization design in image/video coding may be described as [1]:

ĉ = sign(u) ∙ (| u |+p) ∙ s,                       (1)

where ĉ is reconstruction for an index u (an integer as the forward quantization output), s is a step-size parameter, p is a non-zero region offset parameter, and the sign() is defined as

1,  u>0

sign(u) =    0,  u=0                      .

-1, u<0

Quantizers with the above reconstruction space are named nearly-uniform-reconstruction quantizer (NURQ) [1], which can achieve a near-optimal rate distortion coding performance for typical sources such as Laplacian source.

s+p∙s

s

0

p∙sp

s

2s+p∙s

3s+p∙s

4s+p∙s

5s+p∙s

Figure 1. Illustration of the positive half of the reconstruction space for NURQ.

NURQ has been widely used in video coding standards. In H.264/AVC, a special case with p=0 is used with 52 quantization step sizes. Specifically,

ĉ = sign(u) ∙ (| u |) ∙ q,                  (2)

where the quantization step size is selected from the following array:

q[52] = {  0.625,   0.6875, 0.8125,    0.875,   1.0,   1.125,   1.250,   1.375,   1.625,   1.750,   2.0,   2.250,   2.5, 2.75,   3.25,   3.5,   4.0, 4.5,   5.0,   5.5,   6.5,   7.0,   8.0,   9.0,  10.0,  11.0,  13.0,  14.0,  16.0,  18.0,  20.0,  22.0,  26.0,  28.0,  32.0, 36.0,  40.0,  44.0,  52.0,  56.0,  64.0,  72.0,     80.0,  88.0, 104.0, 112.0, 128.0, 144.0, 160.0, 176.0, 208.0,  224.0}.               (3)

q

2q

3q

4q

5q

6q

0

q

Figure 2. Illustration of the positive half of the reconstruction space for H.264/AVC with a step size q.

While in the current HEVC the reconstruction space inherits the H.264/AVC design with p=0, a typical NURQ with a non-zero p had been found effective in previous standards such as H.261, H.263, MPEG-4 part-2, where p=1/2.

ĉ = sign(u) ∙ ( | u | + ½ ) ∙ s.                        (4)

This contribution proposes a NURQ design with its reconstruction space represented based on the 52 step sizes given in (3),

ĉ = sign(u) ∙ (| u | ∙ fs ∙ q + p∙q )           .            (5)

fs∙q+p∙q

fs∙q

0

p∙q

fs∙q

2∙fs∙q+p∙q

2∙fs∙q+p∙q

3∙fs∙q+p∙q

4∙fs∙q+p∙q

Figure 3. Illustration of the positive half of the proposed reconstruction space with (fs, p) based on a given q.

As (5) represents an adaptive selection of the reconstruction levels for quantization, it is named adaptive reconstruction levels (ARL).

# Algorithm Description

To facilitate the discussions, some denotations are introduced as follows:

Define a set of transform coefficients {c (u)j , j=1,…, Nu } corresponding to all coefficients that are quantized to the index of u, as the u-th level, where Nu is the number of samples that fall into this level and c (u)j represents the j-th sample in that set. Denote the average of all samples for the u-th level as ĉ(u),  as

ĉ(u) = (Σj c (u)j )/ Nu  .

Note that the adaptive reconstruction design here is derived based on a given partition. In other words, the partition is assumed to be done, either with RDOQ on or off, before the reconstruction space is derived.

## ARL[2]

### Reconstruction space derivation

The idea of ARL is to find a reconstruction space to minimize the distortion based on a given partition, i.e.,

min   Σu Σj (c (u)j  - ufsq - pq )2                                  (6)

fs ,  p

Essentially, some data sets of (Au = {c (u)j , j=1,…, Nu }, u=1,2,…,M) are collected and are used to derive a reconstruction space represented by (fs ,  p).

In practice, to avoid two-pass coding, statistics collected from one frame is used to derive a reconstruction space, which is then used by another frame at the same temporal layer. Note that in (6) the q is known as the quantization step size used in the procedure when one frame is coded and the statistics is collected. When the obtained reconstruction space is used for coding another frame, the q is needed. In this case, the reconstruction space is described with three quantities, (fs ,  p, q), with fsq specifying the step size parameter and pq defining the non-zero region offset parameter.

### Reconstruction space representation

The 52 given step size may be well utilized to help signal the step size fsq in the obtained reconstruction space. Basically, the step size parameter is compared with the 52 step size and the closest one is chosen to be used as the basis for an effective representation of the reconstruction space.

For example, a reconstruction space is obtained as the step size is 10.8, the non-zero region offset is 2, and qP=23 with qqP=23= 9.0.  Correspondingly, (fs =1.2,  p=0.22,  q=qqP=23= 9.0).

Consider the 52 step sizes in (3). The nearest qP to the step size of 10.8 is qP=25 with qqP=25= 11.0. By using the qP=25, the reconstruction space will be represented as

(fs =0.98,  p=0.18,  q=qqP=25= 11.0), with a smaller dynamic range for fs  and p.

## QP-Rounding[3]

The idea of QP-rounding is to find the optimal step-size parameter fsq, round it to the closest value in (3), and update the QP.

min   Σu Σj (c (u)j  - ufsq )2                                  (7)

fs

Note that the QP-rounding does not requires a syntax change of the current HEVC. Instead, the i-th QP corresponding to a step size qi is transmitted:

| qi - fsq | ≤  | qj - fsq |,

where 0≤j≤51, and qi and  qj take values from (3).

## EEM [1]

It is interesting to describe the EEM algorithm in a similar optimization formula,

minδuAu(δ)c(u)Nu−u⋅q2                                 (8)

where   Au ={c, floor(c/q+ δ)=u} is the partition set for the u-th level as a function of a rounding offset δ. (8) defines a nonlinear optimization problem.

# Experimental Results

The average BD-rate provided in the following tables is calculated using the total bitrate and PSNRAV, where PSNRAV = (6*PSNRY + PSNRU + PSNRV)/8.

## Test-A

ARL vs. HM4.0 anchor, RDOQ-on

|  | Random Access HE |  |  |  | Random Access LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Average | Y | U | V | Average |
| Class A | -1.0% | -4.7% | -4.9% | -1.4% | -1.9% | -7.3% | -7.3% | -2.5% |
| Class B | -0.9% | -4.4% | -4.1% | -1.5% | -1.6% | -5.6% | -5.8% | -2.2% |
| Class C | -1.4% | -4.4% | -4.1% | -1.9% | -2.0% | -5.1% | -4.9% | -2.5% |
| Class D | -0.9% | -3.5% | -3.4% | -1.3% | -1.3% | -4.5% | -4.5% | -1.9% |
| Class E |  |  |  |  |  |  |  |  |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Overall | -1.0% | -4.2% | -4.1% | -1.5% | -1.7% | -5.6% | -5.7% | -2.3% |
|  | -0.7% | -4.1% | -4.0% | -1.3% | -1.6% | -5.5% | -5.5% | -2.2% |
| Enc Time[%] | 100% |  |  |  | 101% |  |  |  |
| Dec Time[%] | 101% |  |  |  | 102% |  |  |  |
|  | Low delay B HE |  |  |  | Low delay B LC |  |  |  |
|  | Y | U | V | Average | Y | U | V | Average |
| Class A |  |  |  |  |  |  |  |  |
| Class B | -1.8% | -5.7% | -5.6% | -2.5% | -2.5% | -6.9% | -7.6% | -3.2% |
| Class C | -2.1% | -5.4% | -5.5% | -2.7% | -3.0% | -6.4% | -6.4% | -3.5% |
| Class D | -1.4% | -3.8% | -4.0% | -1.9% | -2.0% | -4.4% | -4.9% | -2.4% |
| Class E | -0.7% | -2.1% | -2.0% | -0.9% | -1.2% | -4.4% | -4.2% | -1.7% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Overall | -1.6% | -4.5% | -4.5% | -2.1% | -2.2% | -5.7% | -6.0% | -2.8% |
|  | -1.6% | -4.5% | -4.4% | -2.1% | -2.2% | -5.6% | -5.9% | -2.8% |
| Enc Time[%] | 101% |  |  |  | 101% |  |  |  |
| Dec Time[%] | 101% |  |  |  | 103% |  |  |  |

ARL vs. HM4.0 anchor, RDOQ-off

|  | Random Access HE |  |  |  | Random Access LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Average | Y | U | V | Average |
| Class A | -2.7% | 0.5% | 0.3% | -2.4% | -2.1% | 0.1% | 0.5% | -1.9% |
| Class B | -1.8% | -1.0% | -1.3% | -1.7% | -2.0% | -1.0% | -1.2% | -1.8% |
| Class C | -1.6% | -0.9% | -1.2% | -1.5% | -2.0% | -1.2% | -1.4% | -1.9% |
| Class D | -1.4% | -0.7% | -1.0% | -1.3% | -2.0% | -1.1% | -1.4% | -1.9% |
| Class E |  |  |  |  |  |  |  |  |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Overall | -1.9% | -0.5% | -0.8% | -1.7% | -2.0% | -0.8% | -0.9% | -1.9% |
|  | -1.8% | -0.5% | -0.8% | -1.7% | -1.9% | -0.8% | -0.9% | -1.8% |
| Enc Time[%] | 102% |  |  |  | 102% |  |  |  |
| Dec Time[%] | 94% |  |  |  | 95% |  |  |  |
|  | Low delay B HE |  |  |  | Low delay B LC |  |  |  |
|  | Y | U | V | Average | Y | U | V | Average |
| Class A |  |  |  |  |  |  |  |  |
| Class B | -1.8% | -0.5% | -0.6% | -1.6% | -2.0% | -0.7% | -1.0% | -1.8% |
| Class C | -1.5% | -0.3% | -0.3% | -1.3% | -1.9% | -0.9% | -0.8% | -1.7% |
| Class D | -1.3% | -0.5% | -0.5% | -1.2% | -1.7% | -0.8% | -1.3% | -1.6% |
| Class E | -0.3% | 0.1% | 0.0% | -0.2% | -0.7% | -1.3% | 0.0% | -0.7% |
| Class F | #NAME? | #NAME? | #NAME? | #NAME? | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Overall | -1.3% | -0.3% | -0.4% | -1.2% | -1.7% | -0.9% | -0.9% | -1.5% |
|  | -1.3% | -0.3% | -0.4% | -1.2% | -1.6% | -0.9% | -0.9% | -1.5% |
| Enc Time[%] | 101% |  |  |  | 101% |  |  |  |
| Dec Time[%] | 95% |  |  |  | 94% |  |  |  |

## Test-B

ARL with rate-constraints vs. HM4.0 anchor

|  | Random Access HE |  |  |  | Random Access LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Average | Y | U | V | Average |
| Class A | -0.6% | -5.1% | -4.7% | -1.1% | -2.1% | -6.9% | -6.8% | -2.6% |
| Class B | -1.0% | -4.0% | -3.4% | -1.4% | -1.6% | -5.1% | -5.1% | -2.2% |
| Class C | -0.8% | -3.3% | -3.2% | -1.2% | -1.2% | -4.2% | -4.1% | -1.8% |
| Class D | -0.5% | -2.9% | -3.0% | -0.9% | -0.9% | -3.8% | -4.1% | -1.4% |
| Class E |  |  |  |  |  |  |  |  |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Overall | -0.7% | -3.8% | -3.6% | -1.2% | -1.5% | -5.0% | -5.0% | -2.0% |
|  | -0.7% | -3.8% | -3.5% | -1.1% | -1.2% | -5.0% | -4.9% | -1.8% |
| Enc Time[%] | 100% |  |  |  | 99% |  |  |  |
| Dec Time[%] | 99% |  |  |  | 100% |  |  |  |
|  | Low delay B HE |  |  |  | Low delay B LC |  |  |  |
|  | Y | U | V | Average | Y | U | V | Average |
| Class A |  |  |  |  |  |  |  |  |
| Class B | -1.8% | -5.9% | -5.5% | -2.5% | -2.4% | -7.6% | -8.3% | -3.3% |
| Class C | -1.8% | -5.1% | -4.8% | -2.3% | -2.3% | -7.0% | -7.2% | -3.2% |
| Class D | -1.2% | -3.7% | -3.9% | -1.6% | -1.8% | -5.9% | -6.2% | -2.5% |
| Class E | -0.3% | -1.2% | -0.5% | -0.4% | -0.8% | -3.2% | -2.9% | -1.2% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Overall | -1.4% | -4.3% | -4.0% | -1.8% | -2.0% | -6.2% | -6.5% | -2.7% |
|  | -1.4% | -4.2% | -3.9% | -1.8% | -1.9% | -6.1% | -6.2% | -2.7% |
| Enc Time[%] | 101% |  |  |  | 101% |  |  |  |
| Dec Time[%] | 99% |  |  |  | 101% |  |  |  |

## Test-C

ARL vs. Qualcomm QP-rounding

|  | Random Access HE |  |  |  | Random Access LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Average | Y | U | V | Average |
| Class A | 0.0% | -5.4% | -5.3% | -0.6% | -0.1% | -6.1% | -5.8% | -0.8% |
| Class B | -0.3% | -3.9% | -3.5% | -0.9% | -0.3% | -5.1% | -5.0% | -1.0% |
| Class C | -0.7% | -3.6% | -3.2% | -1.2% | -0.6% | -4.3% | -3.9% | -1.2% |
| Class D | -0.3% | -2.8% | -2.6% | -0.7% | -0.3% | -3.7% | -3.6% | -0.8% |
| Class E |  |  |  |  |  |  |  |  |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Overall | -0.3% | -3.9% | -3.6% | -0.9% | -0.3% | -4.8% | -4.6% | -1.0% |
|  | -0.3% | -4.0% | -3.7% | -0.8% | -0.2% | -4.8% | -4.6% | -0.8% |
| Enc Time[%] | 109% |  |  |  | 110% |  |  |  |
| Dec Time[%] | 101% |  |  |  | 102% |  |  |  |
|  | Low delay B HE |  |  |  | Low delay B LC |  |  |  |
|  | Y | U | V | Average | Y | U | V | Average |
| Class A |  |  |  |  |  |  |  |  |
| Class B | -0.6% | -4.8% | -4.4% | -1.2% | -0.2% | -6.5% | -7.4% | -1.2% |
| Class C | -0.9% | -4.4% | -4.4% | -1.5% | -0.4% | -5.7% | -5.5% | -1.3% |
| Class D | -0.2% | -2.9% | -2.8% | -0.6% | -0.1% | -5.1% | -4.9% | -0.8% |
| Class E | -0.3% | -1.8% | -2.2% | -0.6% | 0.0% | -4.3% | -3.8% | -0.6% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Overall | -0.5% | -3.7% | -3.6% | -1.0% | -0.2% | -5.6% | -5.6% | -1.0% |
|  | -0.5% | -3.8% | -3.6% | -1.0% | -0.2% | -5.7% | -5.8% | -1.0% |
| Enc Time[%] | 108% |  |  |  | 108% |  |  |  |
| Dec Time[%] | 101% |  |  |  | 103% |  |  |  |

Note that the encoding/decoding time is not accurate, because the reference data and the tested data are generated on different platforms, as the reference data is obtained by Qualcom and the tested data is generated by us.

## Test-D

ARL with EEM vs. EEM, RDOQ-off

|  | Random Access HE |  |  |  | Random Access LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Average | Y | U | V | Average |
| Class A | -1.8% | 3.6% | 2.0% | -1.3% | -1.3% | 4.8% | 3.3% | -0.8% |
| Class B | -1.4% | 0.5% | 0.2% | -1.1% | -1.5% | 0.5% | 0.2% | -1.2% |
| Class C | -1.3% | 1.7% | 1.8% | -0.8% | -1.5% | 1.2% | 1.5% | -1.1% |
| Class D | -0.9% | 1.8% | 1.8% | -0.4% | -1.1% | 1.3% | 1.5% | -0.7% |
| Class E |  |  |  |  |  |  |  |  |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Overall | -1.3% | 1.8% | 1.4% | -0.9% | -1.4% | 1.9% | 1.5% | -1.0% |
|  | -1.1% | 1.9% | 1.4% | -0.7% | -1.1% | 1.9% | 1.5% | -0.8% |
| Enc Time[%] | 103% |  |  |  | 98% |  |  |  |
| Dec Time[%] | 82% |  |  |  | 80% |  |  |  |
|  | Low delay B HE |  |  |  | Low delay B LC |  |  |  |
|  | Y | U | V | Average | Y | U | V | Average |
| Class A |  |  |  |  |  |  |  |  |
| Class B | -1.6% | -1.6% | -2.3% | -1.6% | -2.1% | -1.9% | -2.4% | -2.0% |
| Class C | -1.8% | -0.7% | -0.2% | -1.5% | -2.1% | -0.7% | -0.1% | -1.8% |
| Class D | -1.1% | -0.5% | -0.9% | -1.0% | -1.4% | -1.1% | -1.2% | -1.3% |
| Class E | -0.4% | -1.3% | -1.1% | -0.5% | -0.8% | -2.4% | -1.9% | -1.0% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Overall | -1.3% | -1.1% | -1.2% | -1.2% | -1.7% | -1.5% | -1.4% | -1.6% |
|  | -1.3% | -1.1% | -1.1% | -1.2% | -1.7% | -1.5% | -1.4% | -1.6% |
| Enc Time[%] | 101% |  |  |  | 97% |  |  |  |
| Dec Time[%] | 82% |  |  |  | 79% |  |  |  |

Note that the encoding/decoding time is not accurate, because the reference data and the tested data are generated on different platforms, as the reference data is obtained by Microsoft and the tested data is generated by us.

# Conclusions

ARL provides consistent gains for both RDOQon and RDOQoff cases without really penalizing anything in the coding process. It is recommended to adopt ARL in HM5.

# Appendix: related syntax

## Clause 7.3.3 Slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| …… |  |
| slice_qp_delta | se(v) |
| RSP_param() |  |
| if( sample_adaptive_offset_enabled_flag ) |  |
| …… |  |
| } |  |

The following table specifies the RSP parameters syntax.

| - RSP_param() { | Descriptor |
| --- | --- |
| RSP_slice_flag | u(1) |
| If(RSP_slice_flag) |  |
| { |  |
| RSP_interY_flag | u(1) |
| if (RSP_interY_flag) |  |
| { |  |
| read_RSP_data(RSP_TYPE_INTERY) |  |
| } |  |
| RSP_interUV_flag | u(1) |
| if (RSP_interUV_flag) |  |
| { |  |
| read_RSP _data(RSP_TYPE_INTERUV) |  |
| } |  |
| RSP_intraY_flag | u(1) |
| if(RSP_intraY_flag) |  |
| { |  |
| read_RSP_data (RSP_TYPE_INTRAY); |  |
| } |  |
| RSP_intraUV_flag | u(1) |
| if(RSP_intraUV_flag) |  |
| { |  |
| read_RSP_data (RSP_TYPE_INTRAUV) |  |
| } |  |
| } |  |
| } |  |
| read_RSP_data (RSP_data_type_id){ |  |
| for(i=1; i<=2;i++){ |  |
| RSP_delta_idx[slice_qp][ RSP_data_type_id][i] | vlc(v) |
| } |  |
| } |  |

Name association to _data_type_id

| RSP_data_type_id | Data type |
| --- | --- |
| 0 | RSP_TYPE_INTERY |
| 1 | RSP_TYPE_INTERUV |
| 2 | RSP_TYPE_INTRAY |
| 3 | RSP_TYPE_INTRAUV |

## The following semantics is to be added to the Clause 7.4.3 Slice header semantics.

RSP_slice_flag specifies whether there are any RSP data transmitted for this slice.

RSP_interUV_flag specifies whether there are RSP for InterUV data.

RSP_intraY_flag specifies whether there are RSP for IntraY data.

RSP_intraUV_flag specifies whether there are RSP for IntraUV data.

RSP_delta_idx[slice_qP][RSP_data_type_id][i] specifies the reconstruction level for a quantization output of value i for a given type of Data with quantization parameter qP.

Four data types are defined for transmitting RSP, i.e., InterY, InterUV, IntraY, and IntraUV. When the parameter RSP_delta_idx[slice_qP][RSP_data_type_id][i] is read from the slice header, the received index RSP_delta_idx[[slice_qP][RSP_data_type_id][i]  is firstly converted to a delta value by the following procedure:

RSPd[[slice_qP][RSP_data_type_id][i] = (RSP_delta_idx[slice_qP][RSP_data_type_id][i] << 2 ) – 32;

Second, the delta value is used to update the corresponding reconstruction level by

RSP[slice_qP][RSP_data_type_id][i] = RSPd[[slice_qP][RSP_data_type_id][i] + (1<<7), 1<=i<=2 ;

When the parameter RSP_delta_idx[slice_qP][RSP_data_type_id][i] is not read from the slice header (for example, when RSP_slice_flag or the corresponding RSP_data_type_flag is not set), RSP[slice_qP][RSP_data_type_id][i] is set to be a default value of (1<<7).

- RSP[slice_qP][RSP_data_type_id][1]: This RSP parameter determines the first reconstruction level, corresponding to the non zero region offset, which is not equal to the reconstruction step size parameter represented by RSP[slice_qP][RSP_data_type_id][2]

## The following semantics change is to be added to Clause 8.5.3 Scaling process for transform coefficients.:

The scaled transform coefficient array dij is derived as follows.

A set of RSP is chosen to be (RSP[slice_qP][RSP_data_type_id][1], RSP[slice_qP][RSP_data_type_id][2]).

After receiving the RSP, the scaled transform coefficients dij is derived as follows:

- dij = (uij×Fa + Fb) >> Fc;					(8-158)

- where,

- Fa = LevelScale(nS)x(nS)[qP%6][i][j]× RSP[slice_qP][RSP_data_type_id][2];

- Fb = (RSP[slice_qP][RSP_data_type_id][1] - RSP[slice_qP][RSP_data_type_id][2])× LevelScale(nS)x(nS)[qP%6][i][j] + iAdd;

- Fc = iShift + 7.

# References

[1] G. Sullivan, “Adaptive quantization encoding technique using an equal expected-value rule”,

JVT-N011.

[2] X. Yu, J. Wang, D. He, and E.h. Yang, “Quantization with Adaptive Reconstruction Levels,”

JCT-VC F276.

[3] X. Wang, R. Joshi, G. Auwera, M. Karczewicz, “Fine granularity QP offset”, JCT-VC F610.

[4] K. Sato, et al., “Description of Core Experiment 4: Quantization”, JCT-VC F904.

# Patent rights declaration(s)

Research In Motion Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).