| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G382 |
| --- | --- |

| Title: | CE4-subtest2.2: Quantization with Adaptive Reconstruction Levels |  |  |
| --- | --- | --- | --- |
| Status: | [Input Document to JCT-VC] |  |  |
| Purpose: | [Proposal] |  |  |
| Author(s) or Contact(s): | Xiang Yu Jing Wang Da-ke He Gaelle Martin-Cocher | Email: | xiayu@rim.com wangj@rim.com dhe@rim.com gmartincocher@rim.com |
| Source: | [Research In Motion Ltd., Canada] |  |  |

_____________________________

# Abstract

This document reports results of CE4 subtest 2.2 for evaluating RIM’s F276 about adaptive reconstruction levels (ARL) for four inter settings with RDOQ on or off for LowQPs, common QPs, and HighQPs.

- For commonQP RDOQ-on, the overall BD-rate averages show 1.0%, 1.7%, 1.6% and 2.2% rate saving for Luma for RAhe, RAloco, LDhe, and LDloco, respectively, over HM4.0 anchor.

With a rate constraint on every temporal level to prevent an undesired bit-allocation effect, most of the rate saving of ARL over the HM4.0 anchor is preserved, as Test-B showed 0.7%, 1.5%, 1.4% and 2.0% rate savings for Luma for RAhe, RAloco, LDhe, and LDloco, respectively.

- For commonQP RDOQ-off, the overall BD-rate averages show 1.9%, 2.0%, 1.3%, and 1.7% rate saving for Luma for RAhe, RAloco, LDhe, and LDloco, respectively, over HM4.0 RDOQoff. The proposed scheme can be combined with the EEM technique, and the overall BD-rate averages show 1.3%, 1.4%, 1.3% and 1.7% rate savings of a combined scheme of EEM and ARL over an EEM-only scheme.

Bit-width analysis is also conducted to improve the proposed method. Without compromising RD performance as shown in the second set test results, bit-width remains within 32bit range.

# Quantization Reconstruction Space Design

As transform coefficients usually assume a Laplacian distribution, a desired reconstruction space for quantization design in image/video coding may be described as [1]:

ĉ = sign(u) ∙ (| u |+p) ∙ s,                       (1)

where ĉ is reconstruction for an index u (an integer as the forward quantization output), s is a step-size parameter, p is a non-zero region offset parameter, and the sign() is defined as

1,  u>0

sign(u) =    0,  u=0                      .

-1, u<0

Quantizers with the above reconstruction space are named nearly-uniform-reconstruction quantizer (NURQ) [1], which can achieve a near-optimal rate distortion coding performance for typical sources such as Laplacian source.

s+p∙s

s

0

p∙sp

s

2s+p∙s

3s+p∙s

4s+p∙s

5s+p∙s

Figure 1. Illustration of the positive half of the reconstruction space for NURQ.

NURQ has been widely used in video coding standards. In H.264/AVC, a special case with p=0 is used with 52 quantization step sizes. Specifically,

ĉ = sign(u) ∙ (| u |) ∙ q,                  (2)

where the quantization step size is selected from the following array:

q[52] = {  0.625,   0.6875, 0.8125,    0.875,   1.0,   1.125,   1.250,   1.375,   1.625,   1.750,   2.0,   2.250,   2.5, 2.75,   3.25,   3.5,   4.0, 4.5,   5.0,   5.5,   6.5,   7.0,   8.0,   9.0,  10.0,  11.0,  13.0,  14.0,  16.0,  18.0,  20.0,  22.0,  26.0,  28.0,  32.0, 36.0,  40.0,  44.0,  52.0,  56.0,  64.0,  72.0,     80.0,  88.0, 104.0, 112.0, 128.0, 144.0, 160.0, 176.0, 208.0,  224.0}.               (3)

q

2q

3q

4q

5q

6q

0

q

Figure 2. Illustration of the positive half of the reconstruction space for H.264/AVC with a step size q.

While in the current HEVC the reconstruction space inherits the H.264/AVC design with p=0, a typical NURQ with a non-zero p had been found effective in previous standards such as H.261, H.263, MPEG-4 part-2, where p=1/2.

ĉ = sign(u) ∙ ( | u | + ½ ) ∙ s.                        (4)

This contribution proposes a NURQ design with its reconstruction space represented based on the 52 step sizes given in (3),

ĉ = sign(u) ∙ (| u | ∙ fs ∙ q + p∙q )           .            (5)

fs∙q+p∙q

fs∙q

0

p∙q

fs∙q

2∙fs∙q+p∙q

2∙fs∙q+p∙q

3∙fs∙q+p∙q

4∙fs∙q+p∙q

Figure 3. Illustration of the positive half of the proposed reconstruction space with (fs, p) based on a given q.

As (5) represents an adaptive selection of the reconstruction levels for quantization, it is named adaptive reconstruction levels (ARL).

# Algorithm Description

To facilitate the discussions, some denotations are introduced as follows:

Define a set of transform coefficients {c (u)j , j=1,…, Nu } corresponding to all coefficients that are quantized to the index of u, as the u-th level, where Nu is the number of samples that fall into this level and c (u)j represents the j-th sample in that set. Denote the average of all samples for the u-th level as ĉ(u),  as

ĉ(u) = (Σj c (u)j )/ Nu  .

Note that the adaptive reconstruction design here is derived based on a given partition. In other words, the partition is assumed to be done, either with RDOQ on or off, before the reconstruction space is derived.

## ARL[2]

### Reconstruction space derivation

The idea of ARL is to find a reconstruction space to minimize the distortion based on a given partition, i.e.,

min   Σu Σj (c (u)j  - ufsq - pq )2                                  (6)

fs ,  p

Essentially, some data sets of (Au = {c (u)j , j=1,…, Nu }, u=1,2,…,M) are collected and are used to derive a reconstruction space represented by (fs ,  p).

In practice, to avoid two-pass coding, statistics collected from one frame is used to derive a reconstruction space, which is then used by another frame at the same temporal layer. Note that in (6) the q is known as the quantization step size used in the procedure when one frame is coded and the statistics is collected. When the obtained reconstruction space is used for coding another frame, the q is needed. In this case, the reconstruction space is described with three quantities, (fs ,  p, q), with fsq specifying the step size parameter and pq defining the non-zero region offset parameter.

### Reconstruction space representation

The 52 given step size may be well utilized to help signal the step size fsq in the obtained reconstruction space. Basically, the step size parameter is compared with the 52 step size and the closest one is chosen to be used as the basis for an effective representation of the reconstruction space.

For example, a reconstruction space is obtained as the step size is 10.8, the non-zero region offset is 2, and qP=23 with qqP=23= 9.0.  Correspondingly, (fs =1.2,  p=0.22,  q=qqP=23= 9.0).

Consider the 52 step sizes in (3). The nearest qP to the step size of 10.8 is qP=25 with qqP=25= 11.0. By using the qP=25, the reconstruction space will be represented as

(fs =0.98,  p=0.18,  q=qqP=25= 11.0), with a smaller dynamic range for fs  and p.

## QP-Rounding[3]

The idea of QP-rounding is to find the optimal step-size parameter fsq, round it to the closest value in (3), and update the QP.

min   Σu Σj (c (u)j  - ufsq )2                                  (7)

fs

Note that the QP-rounding does not requires a syntax change of the current HEVC. Instead, the i-th QP corresponding to a step size qi is transmitted:

| qi - fsq | ≤  | qj - fsq |,

where 0≤j≤51, and qi and  qj take values from (3).

## EEM [1]

It is interesting to describe the EEM algorithm in a similar optimization formula,

minδuAu(δ)c(u)Nu−u⋅q2                                 (8)

where   Au ={c, floor(c/q+ δ)=u} is the partition set for the u-th level as a function of a rounding offset δ. (8) defines a nonlinear optimization problem.

# Experimental Results: Set-1

The average BD-rate provided in the following tables is calculated using the total bitrate and PSNRAV, where PSNRAV = (6*PSNRY + PSNRU + PSNRV)/8.

## Test-A

ARL vs. HM4.0 anchor, RDOQ-on

|  | Random Access HE |  |  |  | Random Access LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Average | Y | U | V | Average |
| Class A | -1.0% | -4.7% | -4.9% | -1.4% | -1.9% | -7.3% | -7.3% | -2.5% |
| Class B | -0.9% | -4.4% | -4.1% | -1.5% | -1.6% | -5.6% | -5.8% | -2.2% |
| Class C | -1.4% | -4.4% | -4.1% | -1.9% | -2.0% | -5.1% | -4.9% | -2.5% |
| Class D | -0.9% | -3.5% | -3.4% | -1.3% | -1.3% | -4.5% | -4.5% | -1.9% |
| Class E |  |  |  |  |  |  |  |  |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Overall | -1.0% | -4.2% | -4.1% | -1.5% | -1.7% | -5.6% | -5.7% | -2.3% |
|  | -0.7% | -4.1% | -4.0% | -1.3% | -1.6% | -5.5% | -5.5% | -2.2% |
| Enc Time[%] | 100% |  |  |  | 101% |  |  |  |
| Dec Time[%] | 101% |  |  |  | 102% |  |  |  |
|  | Low delay B HE |  |  |  | Low delay B LC |  |  |  |
|  | Y | U | V | Average | Y | U | V | Average |
| Class A |  |  |  |  |  |  |  |  |
| Class B | -1.8% | -5.7% | -5.6% | -2.5% | -2.5% | -6.9% | -7.6% | -3.2% |
| Class C | -2.1% | -5.4% | -5.5% | -2.7% | -3.0% | -6.4% | -6.4% | -3.5% |
| Class D | -1.4% | -3.8% | -4.0% | -1.9% | -2.0% | -4.4% | -4.9% | -2.4% |
| Class E | -0.7% | -2.1% | -2.0% | -0.9% | -1.2% | -4.4% | -4.2% | -1.7% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Overall | -1.6% | -4.5% | -4.5% | -2.1% | -2.2% | -5.7% | -6.0% | -2.8% |
|  | -1.6% | -4.5% | -4.4% | -2.1% | -2.2% | -5.6% | -5.9% | -2.8% |
| Enc Time[%] | 101% |  |  |  | 101% |  |  |  |
| Dec Time[%] | 101% |  |  |  | 103% |  |  |  |

ARL vs. HM4.0 anchor, RDOQ-off

|  | Random Access HE |  |  |  | Random Access LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Average | Y | U | V | Average |
| Class A | -2.7% | 0.5% | 0.3% | -2.4% | -2.1% | 0.1% | 0.5% | -1.9% |
| Class B | -1.8% | -1.0% | -1.3% | -1.7% | -2.0% | -1.0% | -1.2% | -1.8% |
| Class C | -1.6% | -0.9% | -1.2% | -1.5% | -2.0% | -1.2% | -1.4% | -1.9% |
| Class D | -1.4% | -0.7% | -1.0% | -1.3% | -2.0% | -1.1% | -1.4% | -1.9% |
| Class E |  |  |  |  |  |  |  |  |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Overall | -1.9% | -0.5% | -0.8% | -1.7% | -2.0% | -0.8% | -0.9% | -1.9% |
|  | -1.8% | -0.5% | -0.8% | -1.7% | -1.9% | -0.8% | -0.9% | -1.8% |
| Enc Time[%] | 102% |  |  |  | 102% |  |  |  |
| Dec Time[%] | 94% |  |  |  | 95% |  |  |  |
|  | Low delay B HE |  |  |  | Low delay B LC |  |  |  |
|  | Y | U | V | Average | Y | U | V | Average |
| Class A |  |  |  |  |  |  |  |  |
| Class B | -1.8% | -0.5% | -0.6% | -1.6% | -2.0% | -0.7% | -1.0% | -1.8% |
| Class C | -1.5% | -0.3% | -0.3% | -1.3% | -1.9% | -0.9% | -0.8% | -1.7% |
| Class D | -1.3% | -0.5% | -0.5% | -1.2% | -1.7% | -0.8% | -1.3% | -1.6% |
| Class E | -0.3% | 0.1% | 0.0% | -0.2% | -0.7% | -1.3% | 0.0% | -0.7% |
| Class F | #NAME? | #NAME? | #NAME? | #NAME? | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Overall | -1.3% | -0.3% | -0.4% | -1.2% | -1.7% | -0.9% | -0.9% | -1.5% |
|  | -1.3% | -0.3% | -0.4% | -1.2% | -1.6% | -0.9% | -0.9% | -1.5% |
| Enc Time[%] | 101% |  |  |  | 101% |  |  |  |
| Dec Time[%] | 95% |  |  |  | 94% |  |  |  |

## Test-B

ARL with rate-constraints vs. HM4.0 anchor

|  | Random Access HE |  |  |  | Random Access LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Average | Y | U | V | Average |
| Class A | -0.6% | -5.1% | -4.7% | -1.1% | -2.1% | -6.9% | -6.8% | -2.6% |
| Class B | -1.0% | -4.0% | -3.4% | -1.4% | -1.6% | -5.1% | -5.1% | -2.2% |
| Class C | -0.8% | -3.3% | -3.2% | -1.2% | -1.2% | -4.2% | -4.1% | -1.8% |
| Class D | -0.5% | -2.9% | -3.0% | -0.9% | -0.9% | -3.8% | -4.1% | -1.4% |
| Class E |  |  |  |  |  |  |  |  |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Overall | -0.7% | -3.8% | -3.6% | -1.2% | -1.5% | -5.0% | -5.0% | -2.0% |
|  | -0.7% | -3.8% | -3.5% | -1.1% | -1.2% | -5.0% | -4.9% | -1.8% |
| Enc Time[%] | 100% |  |  |  | 99% |  |  |  |
| Dec Time[%] | 99% |  |  |  | 100% |  |  |  |
|  | Low delay B HE |  |  |  | Low delay B LC |  |  |  |
|  | Y | U | V | Average | Y | U | V | Average |
| Class A |  |  |  |  |  |  |  |  |
| Class B | -1.8% | -5.9% | -5.5% | -2.5% | -2.4% | -7.6% | -8.3% | -3.3% |
| Class C | -1.8% | -5.1% | -4.8% | -2.3% | -2.3% | -7.0% | -7.2% | -3.2% |
| Class D | -1.2% | -3.7% | -3.9% | -1.6% | -1.8% | -5.9% | -6.2% | -2.5% |
| Class E | -0.3% | -1.2% | -0.5% | -0.4% | -0.8% | -3.2% | -2.9% | -1.2% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Overall | -1.4% | -4.3% | -4.0% | -1.8% | -2.0% | -6.2% | -6.5% | -2.7% |
|  | -1.4% | -4.2% | -3.9% | -1.8% | -1.9% | -6.1% | -6.2% | -2.7% |
| Enc Time[%] | 101% |  |  |  | 101% |  |  |  |
| Dec Time[%] | 99% |  |  |  | 101% |  |  |  |

## Test-C

ARL vs. Qualcomm QP-rounding

|  | Random Access HE |  |  |  | Random Access LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Average | Y | U | V | Average |
| Class A | 0.0% | -5.4% | -5.3% | -0.6% | -0.1% | -6.1% | -5.8% | -0.8% |
| Class B | -0.3% | -3.9% | -3.5% | -0.9% | -0.3% | -5.1% | -5.0% | -1.0% |
| Class C | -0.7% | -3.6% | -3.2% | -1.2% | -0.6% | -4.3% | -3.9% | -1.2% |
| Class D | -0.3% | -2.8% | -2.6% | -0.7% | -0.3% | -3.7% | -3.6% | -0.8% |
| Class E |  |  |  |  |  |  |  |  |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Overall | -0.3% | -3.9% | -3.6% | -0.9% | -0.3% | -4.8% | -4.6% | -1.0% |
|  | -0.3% | -4.0% | -3.7% | -0.8% | -0.2% | -4.8% | -4.6% | -0.8% |
| Enc Time[%] | 109% |  |  |  | 110% |  |  |  |
| Dec Time[%] | 101% |  |  |  | 102% |  |  |  |
|  | Low delay B HE |  |  |  | Low delay B LC |  |  |  |
|  | Y | U | V | Average | Y | U | V | Average |
| Class A |  |  |  |  |  |  |  |  |
| Class B | -0.6% | -4.8% | -4.4% | -1.2% | -0.2% | -6.5% | -7.4% | -1.2% |
| Class C | -0.9% | -4.4% | -4.4% | -1.5% | -0.4% | -5.7% | -5.5% | -1.3% |
| Class D | -0.2% | -2.9% | -2.8% | -0.6% | -0.1% | -5.1% | -4.9% | -0.8% |
| Class E | -0.3% | -1.8% | -2.2% | -0.6% | 0.0% | -4.3% | -3.8% | -0.6% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Overall | -0.5% | -3.7% | -3.6% | -1.0% | -0.2% | -5.6% | -5.6% | -1.0% |
|  | -0.5% | -3.8% | -3.6% | -1.0% | -0.2% | -5.7% | -5.8% | -1.0% |
| Enc Time[%] | 108% |  |  |  | 108% |  |  |  |
| Dec Time[%] | 101% |  |  |  | 103% |  |  |  |

Note that the encoding/decoding time is not accurate, because the reference data and the tested data are generated on different platforms, as the reference data is obtained by Qualcom and the tested data is generated by us.

## Test-D

ARL with EEM vs. EEM, RDOQ-off

|  | Random Access HE |  |  |  | Random Access LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Average | Y | U | V | Average |
| Class A | -1.8% | 3.6% | 2.0% | -1.3% | -1.3% | 4.8% | 3.3% | -0.8% |
| Class B | -1.4% | 0.5% | 0.2% | -1.1% | -1.5% | 0.5% | 0.2% | -1.2% |
| Class C | -1.3% | 1.7% | 1.8% | -0.8% | -1.5% | 1.2% | 1.5% | -1.1% |
| Class D | -0.9% | 1.8% | 1.8% | -0.4% | -1.1% | 1.3% | 1.5% | -0.7% |
| Class E |  |  |  |  |  |  |  |  |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Overall | -1.3% | 1.8% | 1.4% | -0.9% | -1.4% | 1.9% | 1.5% | -1.0% |
|  | -1.1% | 1.9% | 1.4% | -0.7% | -1.1% | 1.9% | 1.5% | -0.8% |
| Enc Time[%] | 103% |  |  |  | 98% |  |  |  |
| Dec Time[%] | 82% |  |  |  | 80% |  |  |  |
|  | Low delay B HE |  |  |  | Low delay B LC |  |  |  |
|  | Y | U | V | Average | Y | U | V | Average |
| Class A |  |  |  |  |  |  |  |  |
| Class B | -1.6% | -1.6% | -2.3% | -1.6% | -2.1% | -1.9% | -2.4% | -2.0% |
| Class C | -1.8% | -0.7% | -0.2% | -1.5% | -2.1% | -0.7% | -0.1% | -1.8% |
| Class D | -1.1% | -0.5% | -0.9% | -1.0% | -1.4% | -1.1% | -1.2% | -1.3% |
| Class E | -0.4% | -1.3% | -1.1% | -0.5% | -0.8% | -2.4% | -1.9% | -1.0% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Overall | -1.3% | -1.1% | -1.2% | -1.2% | -1.7% | -1.5% | -1.4% | -1.6% |
|  | -1.3% | -1.1% | -1.1% | -1.2% | -1.7% | -1.5% | -1.4% | -1.6% |
| Enc Time[%] | 101% |  |  |  | 97% |  |  |  |
| Dec Time[%] | 82% |  |  |  | 79% |  |  |  |

Note that the encoding/decoding time is not accurate, because the reference data and the tested data are generated on different platforms, as the reference data is obtained by Microsoft and the tested data is generated by us.

## Non-CE4 Tests

It was raised in cross-check G403 that ARL does not perform well in low QP settings  and provides worst performances than EEM. Initially the implementation of ARL along with EEM was done in a way to demonstrate that gain of ARL were cumulative with EEM gains for common condition QP value, but the integration was not performed to demonstrate the value of ARL in low of high QP value. In this revised version, a change to the ARL encoder was performed, to demonstrate that ARL performs well for low QP settings ( RDOQ on and Off) and high QP settings (RDOQ off, in order to better compare with EEM).  Results for ARL compared to EEM results in low and high QP value are also provided.

It should be noted that for common condition the results are practically unchanged. Please note that only class C and D results are available at this moment. Complete results will be updated in the near future.

### RDOQ on with common QP and lowQP

- Some changes on the encoder. Decoder remains the same as our Oct. 12 release. In summary, we observe gain for both low QP and common QP cases. The gain for common QP cases is almost the same as before.

- ARL vs. HM4.0 anchor, RDOQ-on, Low QP (2, 7, 12, 17)

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class C | -0.4% | -3.1% | -3.2% | -0.9% | -3.0% | -3.1% |
| Class D | -0.4% | -3.1% | -3.4% | -0.9% | -3.1% | -3.1% |
| Class E |  |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
|  | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class C | -0.1% | -1.3% | -1.5% | -0.4% | -2.0% | -2.3% |
| Class D | -0.1% | -1.5% | -1.7% | -0.5% | -2.5% | -2.7% |
| Class E | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
|  | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |

- ARL vs. HM4.0 anchor, RDOQ-on, Common QP (22, 27, 32, 37)

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class C | -1.5% | -4.1% | -4.0% | -2.0% | -5.0% | -4.8% |
| Class D | -0.8% | -3.4% | -3.2% | -1.4% | -4.4% | -4.4% |
| Class E |  |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
|  | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class C | -2.1% | -4.6% | -4.6% | -3.0% | -5.9% | -5.9% |
| Class D | -1.5% | -4.1% | -3.8% | -2.0% | -5.2% | -5.4% |
| Class E | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
|  | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |

- ARL vs. QP-rounding (as C’ in JCTVC-G403), RDOQ-on, low QP (2, 7, 12, 17)

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class C | -1.8% | -1.8% | -1.8% | -1.6% | -1.9% | -2.0% |
| Class D | -2.3% | -2.4% | -2.9% | -1.7% | -1.8% | -2.2% |
| Class E |  |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
|  | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class C | -2.1% | -0.5% | 0.0% | -1.9% | -1.6% | -1.7% |
| Class D | -2.0% | -0.6% | -0.6% | -2.1% | -1.7% | -1.9% |
| Class E | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
|  | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |

- ARL vs. QP-rounding (as C’ in JCTVC-G403), RDOQ-on, Common QP (22, 27, 32, 37)

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class C | -0.6% | -2.7% | -2.7% | -0.5% | -3.6% | -3.3% |
| Class D | -0.3% | -2.4% | -2.0% | -0.3% | -3.4% | -3.3% |
| Class E |  |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
|  | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class C | -0.8% | -2.6% | -2.8% | -0.4% | -4.4% | -4.1% |
| Class D | -0.4% | -2.6% | -2.3% | 0.0% | -4.7% | -4.3% |
| Class E | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
|  | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |

### RDOQ off with lowQP, commonQP and highQP

Similar to the RDOQ-on case, some changes are made to the encoder. Decoder remains the same as the Oct. 12 release. In summary, for lowQP cases ARL loses for HE settings but gains for LC setting when compared with EEM; for both commonQP and highQP, ARL achieves rate savings over EEM.

- HM4.0, RDOQ-off, ARL vs. EEM, Low QP (2, 7, 12, 17)

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class C | 0.9% | 0.0% | -0.2% | -0.8% | -1.0% | -0.7% |
| Class D | 0.7% | -0.1% | 0.0% | -0.6% | -1.4% | -0.9% |
| Class E |  |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
|  | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class C | 2.0% | 0.0% | -1.8% | 0.0% | -0.6% | -1.4% |
| Class D | 1.2% | -0.2% | -1.2% | -0.3% | -1.3% | -1.9% |
| Class E | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
|  | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |

- HM4.0, RDOQ-off, ARL vs. HM4.0, Low QP (2, 7, 12, 17)

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class C | -5.0% | -2.7% | -2.1% | -5.2% | -2.7% | -2.5% |
| Class D | -5.3% | -2.4% | -2.0% | -5.9% | -3.0% | -3.0% |
| Class E |  |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
|  | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class C | -5.2% | -2.6% | -1.8% | -5.5% | -2.7% | -2.2% |
| Class D | -5.7% | -2.3% | -2.0% | -6.0% | -2.7% | -2.5% |
| Class E | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
|  | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |

- HM4.0, RDOQ-off, ARL vs. EEM, Common QP (22, 27, 32, 37)

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class C | -1.4% | 1.1% | 1.4% | -1.5% | 0.5% | 1.1% |
| Class D | -0.9% | 1.6% | 1.5% | -1.0% | 1.0% | 1.5% |
| Class E |  |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
|  | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class C | -1.8% | -0.7% | -0.1% | -2.0% | -0.8% | -0.1% |
| Class D | -1.2% | -0.1% | -1.0% | -1.3% | -1.2% | -0.9% |
| Class E | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
|  | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |

- HM4.0, RDOQ-off, ARL vs. EEM, High QP (38, 41, 44, 47)

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class C | -1.7% | 0.8% | 1.8% | -1.7% | 1.1% | 0.8% |
| Class D | -1.1% | 1.2% | 1.3% | -1.0% | 2.3% | 2.0% |
| Class E |  |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
|  | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class C | -1.5% | 1.5% | 1.3% | -1.3% | 0.6% | 1.2% |
| Class D | -1.0% | -0.3% | 0.6% | -0.6% | 2.2% | 1.4% |
| Class E | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
|  | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |

# Experimental Results: Set-2

The software deQ process for the experiment results Set-1 did not satisfy worse-case hardware bit-width requirement. Thus, the deQ is modified. Specifically, the deQ for Set-1 is as follows,

If level[i][j] >= 0

coeffQ[i][j] = ( level[i][j] *(IQ_s << (QP/6)) + offset ) >> (M-1+DB+7)

elseif  level[i][j] < 0

coeffQ[i][j] = ( level[i][j] *(IQ_s<< (QP/6)) – offset ) >> (M-1+DB+7)

where

offset = (1<<(M-2+DB)) +IQ_o[QP%6]

For the worse-case where level is assumed with 16bit QP takes the largest there may be ALU overflow. This is solved by changing the deQ to the following:

- IQ_s[QP%6] = (IQ[QP%6]*(128+delta_s)+64)  >> 7

- IQ_o[QP%6] = (IQ[QP%6]*delta_o+64) >> 7

Slice level computations: IQ_s: 6*4 ADD for (InterY, InterUV, IntraY, IntraUV)  + 4 ADD for (128+delta_s). 6*4 MULT for (InterY, InterUV, IntraY, IntraUV). IQ_o: 6*4 MULT, 6*4 ADD.

Table size: 2*6*4 for (InterY, InterUV, IntraY, IntraUV)*7bits

Dequantizer equations are as follows:

- coeffQ[i][j] = sgn(level[i][j]) * ( abs(level[i][j]) * IQ _s + IQ_o ) << (QP/6)  >> (M-1+DB)

With this update, the dynamic range is 32signed bit.

Thus, tests are carried out to verify the impact of the change on the coding performance.

Partial new results for Test-A, ARL vs HM4.0 anchor is shown in JCTVC-ARLnewvsHM40_TestA-RDOQon.xls, and the summary is copied in the following:

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -1.0% | -4.8% | -4.8% | -1.8% | -7.3% | -7.3% |
| Class B | -0.9% | -4.5% | -4.1% | -1.6% | -5.6% | -5.8% |
| Class C | -1.5% | -4.2% | -4.0% | -2.0% | -5.3% | -5.0% |
| Class D | -0.8% | -3.5% | -3.4% | -1.3% | -4.4% | -4.4% |
| Class E |  |  |  |  |  |  |
| Overall | -1.0% | -4.2% | -4.1% | -1.7% | -5.6% | -5.7% |
|  | -0.7% | -4.1% | -3.9% | -1.6% | -5.5% | -5.5% |
| Enc Time[%] | 99% |  |  | 103% |  |  |
| Dec Time[%] | 98% |  |  | 101% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -1.9% | -5.6% | -5.7% | -2.5% | -7.0% | -7.6% |
| Class C | -2.1% | -5.3% | -5.0% | -2.9% | -6.3% | -6.4% |
| Class D | -1.4% | -4.1% | -4.4% | -1.9% | -4.7% | -4.5% |
| Class E | -0.6% | -1.8% | -1.0% | -1.2% | -4.3% | -3.8% |
| Overall | -1.6% | -4.5% | -4.3% | -2.2% | -5.8% | -5.8% |
|  | -1.6% | -4.4% | -4.3% | -2.2% | -5.7% | -5.6% |
| Enc Time[%] | 100% |  |  | 105% |  |  |
| Dec Time[%] | 97% |  |  | 100% |  |  |

Compared with the original results in Sec 3.1 Test-A, the percentage difference for luma in most case is at the second digit after the decimal, the percentage difference for chroma in most cases is at the first digit.

# Coding features enabled by the scale and the offsest

The scale allows a finer control of the quantization step size, which can find a good use on rate control, besides improving the RD coding performance.

For the dequantization offset, it allows adaptive decoder-side quantization dead-zone. As such, the encoder has the flexibility to process small transform coefficients differently from others. One application is to preserve film grain [5]. Though film grain is not a major concern for SDTV or low-resolution videos, it becomes much noticeable as video applications go to the high-end such as in cinema and HD images [5].

# Conclusions

ARL gains are naturally combinable with gains provided by encoder-only techniques for quantization. The additional gain of ARL over RDOQ+QProunding at common QPs is of 0.2~0.5% on luma and 4% on chroma at common QPs; the additional gain of ARL over RDOQ+QProunding at low QPs is about 2% (partial results in S3.5.2) on luma. In the cases where RDOQ is off, though ARL’s rate distortion performance is slightly worse than EEM at low QPs, ARL provides over 1% gain over EEM at common Qps and high QPs. Overall, ARL provides consistent gains for both RDOQ on and RDOQ off cases with little or no cost in the coding process. It is recommended to adopt ARL in HM5.

# Appendix: related syntax

## Clause 7.3.3 Slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| …… |  |
| slice_qp_delta | se(v) |
| RSP_param() |  |
| if( sample_adaptive_offset_enabled_flag ) |  |
| …… |  |
| } |  |

The following table specifies the RSP parameters syntax.

| - RSP_param() { | Descriptor |
| --- | --- |
| RSP_slice_flag | u(1) |
| If(RSP_slice_flag) { |  |
| for(i=0; i<=3;i++){ |  |
| RSP_data_type_flag[i] | u(1) |
| if (RSP_data_type_flag[i]) { |  |
| read_RSP_data(i) |  |
| } |  |
| } |  |
| } |  |
| } |  |
| read_RSP_data (RSP_data_type_id){ |  |
| for(i=1; i<=2;i++){ |  |
| RSP_delta_idx[slice_qp][ RSP_data_type_id][i] | vlc(v) |
| } |  |
| } |  |

Name association to RSP_data_type_id

| RSP_data_type_id | Data type |
| --- | --- |
| 0 | RSP_TYPE_INTERY |
| 1 | RSP_TYPE_INTERUV |
| 2 | RSP_TYPE_INTRAY |
| 3 | RSP_TYPE_INTRAUV |

## The following semantics is to be added to the Clause 7.4.3 Slice header semantics.

RSP_slice_flag specifies whether there are any RSP data transmitted for this slice.

RSP_data_type_flag[i] specifies whether there are RSP transmitted for data with RSP_data_type_id = i.

RSP_delta_idx[slice_qP][RSP_data_type_id][i] specifies the value used for reconstructing the i-th RSP for a given type of Data with quantization parameter qP.

## Derivation process for RSP

The i-th RSP for a given type of Data with quantization parameter qP is derived from RSP_delta_idx[slice_qP][RSP_data_type_id][i] as follows:

RSP[slice_qP][RSP_data_type_id][i] = (RSP_delta_idx[slice_qP][RSP_data_type_id][i] << 2 ) + 96, 1<=i<=2 ;

When the parameter RSP_delta_idx[slice_qP][RSP_data_type_id][i] is not read from the slice header, RSP[slice_qP][RSP_data_type_id][i] is set to be a default value of 128.

## The following semantics change is to be added to Clause 8.5.3 Scaling process for transform coefficient.:

After receiving delta_s and a delta_o, for each of IntraY, InterY, IntraUV, Inter UV, IQ_s and IQ_o are calculated as follows:

IQ_s[QP%6] = (IQ[QP%6]*(128+delta_s)+64)  >> 7

IQ_o[QP%6] = (IQ[QP%6]*delta_o+64) >> 7

Dequantizer equations are as follows:

coeffQ[i][j] = sgn(level[i][j]) * ( abs(level[i][j]) * IQ _s + IQ_o ) << (QP/6)  >> (M-1+DB)

1,   level>0

where sgn(level)=  0,   level =0

-1,  level<0

# References

[1] G. Sullivan, “Adaptive quantization encoding technique using an equal expected-value rule”,

JVT-N011.

[2] X. Yu, J. Wang, D. He, and E.h. Yang, “Quantization with Adaptive Reconstruction Levels,”

JCT-VC F276.

[3] X. Wang, R. Joshi, G. Auwera, M. Karczewicz, “Fine granularity QP offset”, JCT-VC F610.

[4] K. Sato, et al., “Description of Core Experiment 4: Quantization”, JCT-VC F904.

[5] Thomas Wedi and Steffen Wittmann,  “Quantization with an Adaptive Dead Zone Size for H.264/AVC

FRExt”, JVT-K026.

# Patent rights declaration(s)

Research In Motion Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).