| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G387 |
| --- | --- |

| Title: | Parallel Merge/skip Mode for HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal, |  |  |
| Author(s) or Contact(s): | Xing Wen, Oscar C. Au, Wei Dai, Chao Pang, Feng Zou, Jingjing Dai, Xingyu Zhang Department of Electronic and Computer Engineering HKUST Clear Water Bay, Kowloon Hong Kong | Tel: Email: | +852-66261845 wxxab@ust.hk eeau@ust.hk |
| Source: | Hong Kong University of Science and Technology |  |  |

_____________________________

# Abstract

The current HEVC merge/skip is copying motion parameters to the current PU from a candidate list which consists of spatial and temporal neighbouring PUs. However, it is hard for parallel encoding and decoding due to the data dependency. Furthermore, different shapes and position of PU would result in different definition of candidate lists; this would lead to potentially extra hardware cost and not easy to be efficiently implemented by the hardware. Base on JCTVC-F069 [1], we proposed 5 schemes for parallel merge/skip mode. The scheme 1 uses identical candidate list for all the PUs inside certain motion estimation regions (MER). Scheme 2 only use unified spatial candidate with original temporal candidate in HM4.0. Scheme 3 use same candidate list as Scheme 2 after re-order. Scheme 4 use same candidate list for all PU as in scheme 2 except the PU located along the boundary of the MER. Scheme 5 is based on full possible candidate set and it is identical to JCTVC-G164 from LG. Simulation results reveal that Scheme 2 achieve an average gain of 3.4%, 3.5%, 4.1% and 4.4% in RA-HE, RA-LC, LB-HE and LB-LC for 32x32 block level parallel ME when compared to the modified PME HM4.0 design; and the Scheme 4 achieve an average gain of 3.7%, 3.9%, 4.7% and 5.5% in RA-HE, RA-LC, LB-HE and LB-LC for 32x32 block level parallel ME when compared to the modified PME HM4.0 design. This proposal is crosschecked by Microsoft.

# Introduction

In current HEVC, merge/skip mode design [2] is highly sequential and introduces dependency among neighboring PUs. It leads to difficulty for the motion estimation on the encoder side for the pipelined architecture, and also prevents the decoder side from achieving parallel motion compensation.

Figure 1. Illustration of the merge/skip MVP candidate list of HM4.0

Figure 1 illustrates the candidate list construction of the merge/skip mode defined in HM4.0. For the merge/skip mode of the current PU, a total of five neighboring PUs (i.e. left (A), above (B), above-right (C), bottom-left (D) and above-left (E)s neighboring PUs) and temporally co-located PUs (right-bottom (2) and center (2)) are involved to form the merge/skip mode candidate list. The reference index for temporal TMVP derivation also depends on the availability and reference index of neighbors of the current PU.

# Problem Statement

The existing merge/skip is not hardware-friendly due to two reasons. First, the special varying definition of candidate list results in highly sequential merge/skip checking. Figure 2 gives an example of certain CU. With the current HM4.0 design, at encoder side, only the PUs whose top-left overlap with the top-left of current CU can check their merge/skip in parallel (highlighted in green color). The other possible PU partitions need to wait until the PUs in green color to finish motion estimation process. This is all based on the spatial varying definition of the candidate list which changes with the size and location of the current PU.

Secondly, in currently HM4.0 design, the size of the basic merge/skip unit is from 64x64 down to 4x4 blocks and it is fixed (decided by the LCU size and MaxPartitionDepth). However in certain situation and video content, it may be waste of computing power and time for a encoder to check all the possible merge/skip candidates from all the possible PU partitions. It may lead to irregular date flow and huge memory access (reference pixel), since the definition of the candidate lists are different for different PU partitions.

Figure 2. For a block of 2N x 2N with all possible PU partitions

Thirdly, at decoder side, it is difficult to do the motion compensation in parallel according to the current design. For example, if one CU contain more than PUs, they must do the motion compensation sequentially, since the spatial varying definition of candidate list causes data dependency problem between neighbouring PUs.

# Proposed Algorithm

## Part 1: High level syntax

In JCTVC-F069 [2], a slice level syntax element (log2_parallel_merge_level_minus2) is signaled to signal the parallel merge/skip level. Based on the value of this syntax element, a LCU is divided into a number of non-overlapped motion estimation regions (MER) of square shape. As described in JCTVC-F069 [2], a high level syntax element log2_parallel_merge_level_minus2 is added to Picture parameter set (PPS) RBSP syntax as shown in table 1 to indicate the parallel depth of merge/skip.

Table 1 Modified pic_parameter_set_rbsp()

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pic_parameter_set_id | ue(v) |
| seq_parameter_set_id | ue(v) |
| entropy_coding_mode_flag | u(1) |
| num_temporal_layer_switching_point_flags | ue(v) |
| for( i = 0; i < num_temporal_layer_switching_point_flags; i++ ) |  |
| temporal_layer_switching_point_flag[ i ] | u(1) |
| num_ref_idx_l0_default_active_minus1 | ue(v) |
| num_ref_idx_l1_default_active_minus1 | ue(v) |
| pic_init_qp_minus26 /* relative to 26 */ | se(v) |
| constrained_intra_pred_flag | u(1) |
| shared_pps_info_enabled_flag | u(1) |
| if( shared_pps_info_enabled_flag ) |  |
| if( adaptive_loop_filter_enabled_flag ) |  |
| alf_param( ) |  |
| if( cu_qp_delta_enabled_flag ) |  |
| max_cu_qp_delta_depth | u(4) |
| log2_parallel_merge_level_minus2 | ue(v) |
| rbsp_trailing_bits( ) |  |
| } |  |

The LCU is divided into non-overlapped square regions, called motion estimation regions (MER). All the PU located in same MR can be processed in parallel. The size of MER is defined by the log2_parallel_merge_level_minus2, as shown in Table 2.

Table 2 Parallel level defined by log2_parallel_merge_level_minus2

| log2_parallel_merge_level_minus2 | MER size | Remark |
| --- | --- | --- |
| 0 | 4x4 | Sequential merge/skip mode for all PUs in a LCU because minimum PU size allowed by HEVC is 4x4 |
| 1 | 8x8 | Parallel merge/skip mode search allowed for all PUs inside an 8x8 block |
| 2 | 16x16 | Parallel merge/skip mode search allowed for all PUs inside a 16x16 block |
| 3 | 32x32 | Parallel merge/skip mode search allowed for all PUs inside a 32x32 block |
| 4 | 64x64 | Parallel merge/skip mode search allowed for all PUs inside a 64x64 block |

## Proposed scheme 1 (ISC+ITC):

### Candidate list derivation

Our proposed Scheme 1 uses the same candidate list for all PUs (with any size) in the same MER. So each MER derives the candidate list once, and only once, and uses this list for all PUs inside this MER. The candidate list of MER in the proposed scheme 1 is derived as follows:

- Based on log2_parallel_merge_level_minus2, divide the LCU into one or more non-overlapped MERs. The number of MER for different merge_depth are listed in Table 2. .

- Generate the spatial candidate list for each MER, and all PUs of any sizes in the same MER use the same spatial candidate (called identical spatial candidate, or ISC) list. For example, in Figure 3, the spatial candidate list of MER0 is generated from A, B, C, D and E. The spatial candidate lists for PU0 and PU1 in MER0 are same.

- Generate the temporal candidate list for each MER to make sure all PUs inside the same MER use identical temporal candidates (called identical temporal candidate, or ITC).

Figure 3 Scheme 1: unified candidate list for PUs inside same MER

### Complexity analysis

To analyze the complexity of of the proposed Scheme 1, we assume the worst case in which one MER is subdivided into PUs of size 4x4.

In HM4.0, each 4x4 PU has its own candidate list with up to 5 candidate MVPs. In the worst case, one LCU needs to access memory 256x5 times for reference pixel data during the motion estimation, fetching one 4x4 block at a time. And it may need to store and access TMVP for each 4x4 PU to form the candidate list.

Table 3 Complexity of original Merge/skip for HM 4.0

|  | LCU size = 64x64 |
| --- | --- |
| # of 4x4 PU | 256 |
| Memory access for reference pixel | 256x5=1280, fetching 4x4 at a time |
| Memory access for MV | 5*256= 1280 |

For Scheme 1, with the help of the ISC and ITC, we can reduce the memory access significantly. Also, at decoder side, this scheme can simplify the logic to derive the candidate list for each PU since it only require one time derivation for all PU inside certain MER.

Table 4 Complexity of the Scheme 1

|  | MER size = 64x64 | MER size = 32x32 | MER size = 16x16 | MER size = 8x8 |
| --- | --- | --- | --- | --- |
| # of 4x4 PU | 256 | 64 | 16 | 4 |
| Memory access for reference pixel | 5, fetching 64x64 at a time | 5, fetching 32x32 at a time | 5, fetching 16x16 at a time | 5, fetching 8x8 at a time |
| Memory access for MV | 5 | 5 | 5 | 5 |

## Proposed scheme 2 (ISC+ Orig TMVP in HM4.0):

### Candidate list derivation

Based on Scheme 1, we further propose Scheme 2 which only uses ISC but keep TMVP same as original HM4.0 (i.e. TMVP can be different for different PU). This is because the original TMVP in HM4.0 typically has better prediction accuracy and did not cause data dependency problem. So removing ITC should give better coding performance than Scheme 1.

### Complexity analysis

Table 5 Complexity of the Scheme 2

|  | MER size = 64x64 | MER size = 32x32 | MER size = 16x16 | MER size = 8x8 |
| --- | --- | --- | --- | --- |
| # of 4x4 PU | 256 | 64 | 16 | 4 |
| Memory access for reference pixel | 260, fetching 64x64 for 4 times and 4x4 for 256 times | 68, fetching 32x32 for 4 times and 4x4 for 64 times | 20, fetching 16x16 for 4 times and 4x4 for 16 times | 8, fetching 8x8 for 4 times and 4x4 for 4 times |
| Memory access for MV | 260 | 68 | 20 | 8 |

## Proposed scheme 3(ISC+ Orig TMVP in HM4.0 + Re-order):

### Candidate list derivation

In all our schemes (Schemes 1 to 5), we use truncated unary code for binarization of the indices of Merge/skip, as proposed in JCTVC-F470 [?] and accepted in Torino meeting. For example, the codewords for list size = 5 are shown in Table 6. with unary code, the order of the items in the candidate list would affect the RD performance.

Table 6 Codeword of Merge_idx for list size = 5

| Merge_idx | bin | Scheme 2 (MER0, PU0) | Scheme 3 (MER0, PU0) |
| --- | --- | --- | --- |
| 0 | 0 | Left | Above left |
| 1 | 10 | Above | Left |
| 2 | 110 | Above right | Above |
| 3 | 1110 | Left bottom | Above right |
| 4 | 1111 | TMVP | TMVP |

The indices of the candidate MV predictors in the Merge/skip mode in Scheme 2 use the same order as HM 4.0, as shown above. Although the order is the same, the candidate MV is not the same. Their order are the same, in the sense that both HM4.0 and Scheme 2 give the same index 0 to the Left MV, index 1 to the Above MV, and so on. But the definition of Left MV is different in HM4.0 an Scheme 2. The Left MV in HM4.0 is closer to the current MER than that in Scheme 2. Thus the MV prediction tends to be more accurate in HM4.0.  This can cause potential RD drops. So based on Scheme 2, we introduce Scheme 3 which is basically Scheme 2 with a re-ordering method as follows:

- The candidate list of MER is same as Scheme 2.

- The order of the candidate list is based on spatial distance. The one is closer to top-left position of current PU would have higher order. For example, in Figure 3, the order of candidate list of PU4 would be {A, E, C}.

Figure 3 Scheme 1: unified candidate list for PUs inside same MER

### Complexity analysis

The complexity of Scheme 3 is similar to Scheme 2, but plus the re-ordering process. The re-order process can be implemented by look-up table based on the spatial position of current PU.

## Proposed scheme 4 （ISC+ Orig TMVP +Re-order）:

### Candidate list derivation

In our experiments, we found that the major RD loss comes from the PUs located alone the edge of the MER. So based on Scheme 3, we further propose Scheme 4, in which the candidate lists of boundary PU are derived based on their locations:

- For each MER, the set of possible special candidates are fixed. For example, the set of possible spatial candidates for MER0 including 7 neighboring PUs in Fig .4 is {A, B, C, D, E, F, G}

- If the top-left corner of current PU is located along the edge of the MER (PUcol < 1 || PUrow < 1), choose corresponding MVs from the set of possible candidates based on their spatial distance to current PU. For example, the candidate list of PU0 is {F, C, D, E, TMVP}. The candidate list of PU1 is {A, E TMVP, C, D}. The candidate list is formed based on PU locations.

- If the top-left corner of current PU is not located along the edge of the MER (PUcol > 0 || PUrow >0), the candidate list of current PU is same as scheme 1 and 2. For example, in Fig .4, the candidate list of PU5 and PU6 is {A, C, E, TMVP} in MER 3

Figure 4 Scheme 4: unified candidate list + adaptive list for PUs inside same MER with 7 possible candidates set

### Complexity analysis

In the proposed scheme 4, we choose the possible candidate set size to be 7, as shown in Fig .5 {A, B, C, D, E, F, G}. However Scheme 4 actually can work with arbitrary size of possible candidate set. For example, the candidate set size can be consisted of all neighboring PUs along the edge of MER, which means 8*2+3=19 MVs for MER size = 32x32, or 16*2+3=35 MVs for MER size = 64x64.

Fig .5 gives an example in which the size of MER is 32x32 and size of possible candidate set is 19 and all PUs are in size of 4x4. According to the Scheme 4 descriptions, the PU in yellow color would use ISC as the proposed Scheme 2. But the PUs in the color of blue, green and pink are the PU located along the edges of current MER and their candidate lists are formed from possible candidate set based on their own spatial locations and sizes. If the size of possible candidate size is 19, the PUs along the edge would have their own candidate lists. For example, the candidate list of PU1 is left, top, top-right, left-bottom PUs and TMVP of PU1; the candidate list of PU2 is left PU of PU 1, top, top-right PUs of PU2, left-bottom PU of PU1 and TMVP of PU2. The Full possible candidate sets results in better RD performance compared our proposed Scheme 1, 2, and 3. However, the full possible candidate sets would increase the memory access and increase the cost of hardware.

Figure 5 Example of Scheme 4 with different possible candidates set (MER size = 32x32, all PUs with size 4x4)

Table 7 shows the complexity for proposed Scheme 4 with full possible candidate set.

Table 7 Complexity of the Scheme 4 with full possible candidate set (worst case)

|  | MER size = 64x64 | MER size = 32x32 |
| --- | --- | --- |
| # of 4x4 PU | 256 | 64 |
| Memory access for reference pixel | 4+4*31+256=384 | 4+4*15+64=128 |
| Memory access for MV | 35+256=291 | 19+64=83 |

Table 8 shows the complexity for proposed Scheme 4 with possible candidate size of 7 (shown as Fig .4). Compare to Scheme 4 with full sets, the Scheme 4 with possible candidate set size of 7 can save over 30% memory access for reference pixel data and about 10% memory access for MV. The most important is that the memory loading for Scheme 4 with possible candidate set size of 7 is much more regular than Scheme 4 with full possible candidate set. For Scheme 4 with possible candidate set size of 7, the hardware implementation can load 7 times of reference pixels data with size of the MER, and each time corresponding to one candidate in the candidate set. However, the Scheme 4 with the full set loads the reference pixels data based on 4x4 PU (worst case) and it candidate lists, which would increase the memory access time significantly.

Table 8 Complexity of the Scheme 4 with possible candidate set size of 7 (worst case)

|  | MER size = 64x64 | MER size = 32x32 |
| --- | --- | --- |
| # of 4x4 PU | 256 | 64 |
| Memory access for reference pixel | 7+256=263 | 7+64=71 |
| Memory access for MV | 7+256=263 | 7+64=71 |

## Proposed scheme 5 （Scheme4+full possible candidate set）:

Scheme 5 is the special case of Scheme 4 using the full possible candidate set, which means all PUs around MER edges are treated as available. Note that Scheme 5 is actually the same method as LG described in JCTVC-G164. The code and simulation results are verified by LG.

### Candidate list derivation

  - Same as Scheme 4, the candidate list is formed based on PU locations.

  - If the top-left corner of current PU is located along the edge of the MER (PUcol < 1 || PUrow < 1), choose corresponding MVs from the possible candidates based on their spatial distance to current PU. For example, the candidate list of PU0 is left, above, above-right, left-bottom, TMVP (as defined in HM4.0). For PU 1, the candidate list is A, B, TMVP, C, D.

  - If the top-left corner of current PU is located inside the MER, they would use ISC as described in Scheme 4.

- Figure 6 Example of Scheme 5

# Simulation Results

We have compared the proposed Scheme 1 to 4 to both JCTVC-F069 and modified HM4.0 software PME anchor from CE 9 mailing list. TI has offered the modified PME HM4.0 software in which the PME is performed at 2N x 2N block level (N = 64, 32, 16 and 8, LCU size is fixed to 64x64). The simulation result of the modified PME compared to original HM4.0 is shown in Table 9.

Table 9 BD-rate increase of modified PME HM4.0 software compared to HM4.0 anchor

| Parallel ME level 2N x 2N (LCU = 64x64) | RA-HE (%) | RA-LC (%) | LB-HE (%) | LB-LC (%) |
| --- | --- | --- | --- | --- |
| 64 x 64 | 6.8 | 7.7 | 8.1 | 10.9 |
| 32 x 32 | 5.0 | 5.3 | 6.7 | 7.8 |
| 16 x 16 | 2.4 | 2.3 | 3.4 | 3.3 |
| 8 x 8 | 0.2 | 0.2 | 0.4 | 0.3 |

Following the common testing conditions specified in [3], simulations with different MER size are carried out to show the effectiveness of the proposed methods.. Note that results of Class F are not count in the “overall” results.

For MER size =64x64,

- Scheme 1 achieve an average gain of 3.6%, 4.3%, 3.4%, and 5.3% in RA-HE, RA-LC, LB-HE and LB-LC compared to modified HM4.0 software PME anchor. It achieves an average loss of 0.4%, 0.2%, 1.0% and 0.7% in RA-HE, RA-LC, LB-HE and LB-LC compared to JCTVC-F069.

- Scheme 2 achieve an average gain of 4.0%, 4.8%, 3.9%, and 5.8% in RA-HE, RA-LC, LB-HE and LB-LC compared to modified HM4.0 software PME anchor. It achieves an average loss of 0.0%, -0.2%, 0.5% and 0.1% in RA-HE, RA-LC, LB-HE and LB-LC compared to JCTVC-F069.

- Scheme 3 achieve an average gain of 4.2%, 4.8%, 4.1%, and 4.4% in RA-HE, RA-LC, LB-HE and LB-LC compared to modified HM4.0 software PME anchor. It achieves an average loss of  -0.2%, -0.3%, 0.2% and -0.1% in RA-HE, RA-LC, LB-HE and LB-LC compared to JCTVC-F069.

- Scheme 4 achieve an average gain of 4.4%, 5.1%, 4.6%, and 6.5% in RA-HE, RA-LC, LB-HE and LB-LC compared to modified HM4.0 software PME anchor. It achieves an average loss of  -0.4%, -0.6%, -0.3% and -0.7% in RA-HE, RA-LC, LB-HE and LB-LC compared to JCTVC-F069.

For MER size =32x32,

- Scheme 1 achieve an average gain of 3.1%, 3.4%, 3.7%, and 4.1% in RA-HE, RA-LC, LB-HE and LB-LC compared to modified HM4.0 software PME anchor. It achieves an average loss of 0.3%, 0.1%, 0.8% and 0.3% in RA-HE, RA-LC, LB-HE and LB-LC compared to JCTVC-F069.

- Scheme 2 achieve an average gain of 3.3%, 3.6%, 4.0%, and 4.4% in RA-HE, RA-LC, LB-HE and LB-LC compared to modified HM4.0 software PME anchor. It achieves an average loss of 0.0%, -0.1%, 0.4% and 0.0% in RA-HE, RA-LC, LB-HE and LB-LC compared to JCTVC-F069.

- Scheme 3 achieve an average gain of 3.4%, 3.5%, 4.1%, and 4.4% in RA-HE, RA-LC, LB-HE and LB-LC compared to modified HM4.0 software PME anchor. It achieves an average loss of 0.0%, -0.1%, 0.4% and -0.1% in RA-HE, RA-LC, LB-HE and LB-LC compared to JCTVC-F069.

- Scheme 4 achieve an average gain of 4.4%, 5.1%, 4.6%, and 6.5% in RA-HE, RA-LC, LB-HE and LB-LC compared to modified HM4.0 software PME anchor. It achieves an average loss of  -0.4%, -0.6%, -0.3% and -0.7% in RA-HE, RA-LC, LB-HE and LB-LC compared to JCTVC-F069.

For MER size =16x16,

- Scheme 1 achieve an average gain of 1.7%, 1.6%, 2.1%, and 2.1% in RA-HE, RA-LC, LB-HE and LB-LC compared to modified HM4.0 software PME anchor. It achieves an average loss of 0.2%, 0.1%, 0.5% and 0.4% in RA-HE, RA-LC, LB-HE and LB-LC compared to JCTVC-F069.

- Scheme 2 achieve an average gain of 1.8%, 1.7%, 2.3%, and 2.3% in RA-HE, RA-LC, LB-HE and LB-LC compared to modified HM4.0 software PME anchor. It achieves an average loss of 0.1%, 0.0%, 0.4% and 0.3% in RA-HE, RA-LC, LB-HE and LB-LC compared to JCTVC-F069.

- Scheme 3 achieve an average gain of 1.7%, 1.6%, 2.2%, and 2.0% in RA-HE, RA-LC, LB-HE and LB-LC compared to modified HM4.0 software PME anchor. It achieves an average loss of 0.2%, 0.2%, 0.5% and 0.5% in RA-HE, RA-LC, LB-HE and LB-LC compared to JCTVC-F069.

Table 9 Simulation results of Scheme 1 compared to PME

|  | MER size = 64x64 | MER size = 32x32 | MER size = 16x16 |
| --- | --- | --- | --- |
| RA-HE | -3.6% | -3.1% | -1.7% |
| RA-LC | -4.3% | -3.4% | -1.6% |
| LB-HE | -3.4% | -3.7% | -2.1% |
| LB-LC | -5.3% | -4.6% | -2.1% |

Table 10 Simulation results of Scheme 1 compared to JCTVC-069

|  | MER size = 64x64 | MER size = 32x32 | MER size = 16x16 |
| --- | --- | --- | --- |
| RA-HE | 0.4% | 0.3% | 0.2% |
| RA-LC | 0.2% | 0.1% | 0.1% |
| LB-HE | 1.0% | 0.8% | 0.5% |
| LB-LC | 0.7% | 0.5% | 0.4% |

Table 11 Simulation results of Scheme 2 compared to PME

|  | MER size = 64x64 | MER size = 32x32 | MER size = 16x16 |
| --- | --- | --- | --- |
| RA-HE | -4.0% | -3.3% | -1.8% |
| RA-LC | -4.8% | -3.6% | -1.7% |
| LB-HE | -3.9% | -4.0% | -2.3% |
| LB-LC | -5.8% | -4.4% | -2.3% |

Table 12 Simulation results of Scheme 2 compared to JCTVC-069

|  | MER size = 64x64 | MER size = 32x32 | MER size = 16x16 |
| --- | --- | --- | --- |
| RA-HE | 0.0% | 0.0% | 0.1% |
| RA-LC | -0.2% | -0.1% | 0.0% |
| LB-HE | 0.5% | 0.4% | 0.4% |
| LB-LC | 0.1% | 0.0% | 0.3% |

Table 13 Simulation results of Scheme 3 compared to PME

|  | MER size = 64x64 | MER size = 32x32 | MER size = 16x16 |
| --- | --- | --- | --- |
| RA-HE | -4.2% | -3.4% | -1.7% |
| RA-LC | -4.8% | -3.5% | -1.6% |
| LB-HE | -4.1% | -4.1% | -2.2% |
| LB-LC | -5.3% | -4.4% | -2.0% |

Table 14 Simulation results of Scheme 3 compared to JCTVC-069

|  | MER size = 64x64 | MER size = 32x32 | MER size = 16x16 |
| --- | --- | --- | --- |
| RA-HE | -0.2% | 0.0% | 0.2% |
| RA-LC | -0.3% | -0.1% | 0.2% |
| LB-HE | 0.2% | 0.4% | 0.5% |
| LB-LC | -0.1% | -0.1% | 0.5% |

Table 15 Simulation results of Scheme 4 with possible candidate set size of 7 compared to PME

|  | MER size = 64x64 | MER size = 32x32 |
| --- | --- | --- |
| RA-HE | -4.4% | -3.7% |
| RA-LC | -5.1% | -3.9% |
| LB-HE | -4.6% | -4.7% |
| LB-LC | -6.5% | -5.5% |

Table 16 Simulation results of Scheme 4 with possible candidate set size of 7 compared to JCTVC-F069

|  | MER size = 64x64 | MER size = 32x32 |
| --- | --- | --- |
| RA-HE | -0.4% | -0.3% |
| RA-LC | -0.6% | -0.4% |
| LB-HE | -0.3% | -0.3% |
| LB-LC | -0.7% | -0.5% |

# References

[1]	M. Zhou, “Parallelized merge/skip mode for HEVC,” JCT-VC Document, JCTVC-F069, Torino, July 2011.

[2]	T. Wiegand, B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, “WD4: Working Draft 4 of High-Efficiency Video Coding,” JCT-VC Document, JCTVC-E803, Torino, July 2011.

[3]	F. Bossen, “Common test conditions and software reference configurations,” JCT-VC Document, JCTVC-F900, Torino, July 2011.

# Patent rights declaration(s)

Hong Kong University of Science and Technology may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).