| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G388 WG11 Number: m21951 |
| --- | --- |

| Title: | CE12: Cross-check of Mitsubishi’s deblocking filter JCTVC-F175/Gxxx |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Tomoo Yamakage Saori Asaka | Email: | tomoo.yamakage@toshiba.co.jp |
| Source: | TOSHIBA Corporation |  |  |

_____________________________

# Abstract

Cross-check results of JCTVC-Gxxx (CE12: Signaling of boundary filtering strength of deblocking filter for intra) are reported.  Coding efficiency results are matched except one test case for the anchor, which would be caused by a bug (ticket #203).  Relative encoding and decoding time are similar to those of proponents’.  Source code was reviewed and confirmed the code was consistent with the description in JCTVC-Gxxx.  Since CE12 plans to conduct an informal subjective viewing, it is suggested to postpone the decision until the viewing results become available.

# Introduction

This contribution is a report of cross-check results of JCTVC-Gxxx, which is a contribution to CE12.

Cross-check is performed under the CE12 test conditions and additional higher Qp range conditions that refer to JCTVC-F900 and JCTVC-F912.  Coding efficiency results are matched with the proponent’s results except one test case for the anchor (i.e., results of the proposal are exactly matched).  Relative encoding and decoding time are similar to those of proponent’s.

Source code was reviewed and confirmed that it was consistent with the description in JCTVC-Gxxx.

# Experimental Results

Detailed results in Excel sheet are attached.  The following tables are the summary of experimental results:

Table 1  Common test conditions

| Test case | High Efficiency |  |  |  | Low Complexity |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | IO | RA | LDB | LDP | IO | RA | LDB | LDP |
| Class A | 0.0% | 0.0% |  |  | 0.0% | 0.1% |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
| Class D | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
| Class E | 0.0% |  | 0.2% | 0.1% | 0.0% |  | 0.0% | 0.1% |
| Average | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
| Enc time | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 100% |
| Dec time | 100% | 100% | 100% | 99% | 101% | 101% | 101% | 100% |

Table 2  Higher Qp test conditions

| Test case |  | High Efficiency |  |  |  |  |  |  |  | Low Complexity |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | IO |  | RA |  | LDB |  | LDP |  | IO |  | RA |  | LDB |  | LDP |  |  |
| Class A |  | -0.2% |  | -0.2% |  |  |  |  |  | 0.1% |  | 0.1% |  |  |  |  |  |
| Class B |  | -0.1% |  | -0.1% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.1% |  |
| Class C |  | -0.1% |  | 0.1% |  | 0.2% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.1% |  | 2.1% |  |
| Class D |  | -0.1% |  | 0.4% |  | 0.5% |  | 0.1% |  | 0.0% |  | 0.2% |  | 0.1% |  | 0.3% |  |
| Class E |  | -0.1% |  |  |  | 0.3% |  | -0.2% |  | 0.0% |  |  |  | 0.1% |  | 0.3% |  |
| Average |  | -0.1% |  | 0.0% |  | 0.2% |  | 0.0% |  | 0.0% |  | 0.1% |  | 0.1% |  | 0.7% |  |
| Enc time |  | 100% |  | 100% |  | 100% |  | 100% |  | 100% |  | 100% |  | 100% |  | 100% |  |
| Dec time |  | 100% |  | 100% |  | 100% |  | 99% |  | 101% |  | 101% |  | 101% |  | 99% |  |

For a higher Qp test case (SteamLocomotive,HE-RA,QP42), there is a mismatch of the anchor between proponent’s result and cross-checker’s result.  This would be caused by a bug in HM4.0 (ticket #203).

There is no mismatch between encoder and both decoders.  Relative encoding and decoding time are similar to those of proponent’s.

# Conclusion

This contribution shows verification results of JCTVC-Gxxx. Coding efficiency results are confirmed.  No mismatch between encoder and decoder is observed.  Relative encoding and decoding time are also confirmed.  Source code was reviewed.  Since CE12 plans to conduct an informal subjective viewing, it is suggested to postpone the decision until the viewing results become available.

# Patent rights declaration(s)

TOSHIBA Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).