| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-Gxxx |
| --- | --- |

| Title: | CE5: CAVLC coding table modification |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | CE Report |  |  |
| Author(s) or Contact(s): | Seongwan Kim, Jaeho Lee, Sangyoun Lee Yonsei University Chanyul Kim Youngo Park Jeonghoon Park Samsung | Tel: Email: Tel: Email: | +82-2-2123-5768 knauer@yonsei.ac.kr jhlee82@yonsei.ac.kr syleee@yonsei.ac.kr +82-31-279-8826 dionism@samsung.com youngo.park@samsung.com jeonghoon@samsung.com |
| Source: | Yonsei University and Samsung Electronics Co., Ltd |  |  |

_____________________________

# Abstract

In this contribution, CAVLC coding table modification, VLC table modification in JCTVC-F408, is reported. Simulation results show that coding gains are -0.1%, -0.1% and -0.1% for all intra, random access and low delay(B) configurations respectively.

# Introduction

In JCTVC-F408, CAVLC coding table modification is proposed. When vlcNum is equal 8, the mapping rule between cn and codeword is generalized to available for maxrun > 3. In other vlcNum(vlcNum != 8), one parameter k is considered to determine whether golomb or exp-golomb is used for mapping between cn and codeword.

In run-mode, for determining k, three thresholds are used according to transform sizes, trailing ones, intra or inter, and luma or chroma. In level-mode, parameter k is determined from table, according to vlcNum, and luma or chroma.

# Simulation results

Based on test conditions specified in [2], simulations are performed using all three low complexity configurations. The detailed results are available in the attached Excel file.

Table 1 shows the summarized results of the proposed coding scheme. The performance improvements are -0.1%, -0.1%, -0.1% in all intra, low delay and random access configurations.

Table 1

|  | All Intra LC |  |  | Random Access LC |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.1% | -0.3% | -0.4% | 0.0% | 0.2% | -0.3% |  |  |  |
| Class B | -0.2% | -0.3% | -0.3% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | 0.2% |
| Class C | -0.1% | -0.2% | -0.2% | -0.1% | -0.2% | 0.0% | -0.1% | -0.1% | -0.2% |
| Class D | -0.1% | -0.1% | -0.1% | -0.1% | 0.2% | -0.1% | -0.1% | 0.1% | -0.1% |
| Class E | -0.1% | 0.0% | 0.0% |  |  |  | 0.0% | 0.8% | -0.1% |
| Overall | -0.1% | -0.2% | -0.2% | -0.1% | 0.0% | -0.1% | -0.1% | 0.1% | 0.0% |
| Enc Time[%] | 103% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  | 102% |  |  |

# Conclusion

This contribution reports CE results for CAVLC improvements. From the results, we recommend the proposed scheme be adopted into HM.

# References

[1] Seongwan Kim, Jaeho Lee, Sangyoun Lee, Jianle Chen, Jeonghoon Park, “CE5: Run and level mode coding improvement in CAVLC”, JCTVC-F408, Torino, Italy, July 2011

[2] Xianglin Wang, Ping Wu, Chanyul Kim, “CAVLC entropy coding improvements”, JCTVC-F905, Torino, Italy, July 2011

# Patent rights declaration(s)

Yonsei University and Samsung Electronics Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).