| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G390 |
| --- | --- |

| Title: | CE3: MC boundary filter (tool test 7) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kenji Kondo and Teruhiko Suzuki Sony City Osaki, 2-10-1 Osaki Shinagawa-ku, Tokyo Japan | Tel: Email: | +81-50-3750-2744 kenji.kondo@jp.sony.com teruhikos@jp.sony.com |
| Source: | SONY Corporation |  |  |

_____________________________

# Abstract

This contribution reports results of MC boundary filter (MBF) which is studied in core experiment (CE) 3[1]. To reduce complexity both memory bandwidth (b/w) and computation, the MBF uses different filter coefficients to MC block boundary. With the proposed method, the memory b/w can be reduced -44% and -17% for worst and actual in common test condition [3]. The computation of multiplications can be reduced -28% and -9% for worst and actual case. The impact for coding efficiency are 0.3%, 0.2%, 0.3%, 0.1%, 0.3%, and 0.2% for RA_HE, RA_LC, LD_HE, LD_LC, LDP_HE and LDP_LC. The combination MBF with a restriction of PU 8x4 and 4x8 bi-prediction is additionally tested. The worst memory b/w can be reduced -58%. The impact for coding efficiency are 0.4%, 0.4%, 0.6%, 0.4%, 0.1%, and 0.1% for RA_HE, RA_LC, LD_HE, LD_LC, LDP_HE and LDP_LC.

# Introduction

In general LSI codec system, the decoded picture buffer is implemented as external memory. This memory bandwidth between codec LSI and external memory is critical issue for consumer products because of memory chip cost and power consumption. MC process needs to access to external memory. So, it is important to reduce read access at MC process. In this contribution, we propose MC interpolation filter to reduce this memory b/w. This proposal can reduce also computation of MC interpolation process.

# An algorithm for memory b/w reduction

In order to reduce memory b/w and computation, we proposed the MC boundary filter in Turin meeting [2]. Basic algorithm is introduced in [2].

## Design new filter with compact support

In order to reduce memory bandwidth and computation, we designed new filter. The filter has compact support in MC. Figure 1 shows the example of interpolation. A light blue box means reference pixel. A green box is interpolation pixel. In usual, interpolation filter interpolates white boxes (a, b, or c) around center when interpolation filter has 8 tap such as DCT IF 8 tap. Proposal filter with compact support interpolates these green boxes (a, b, or c) from 8 light blue pixels.

Figure 1: newly designed filter

## MC process with compact support

We can reduce input pixel by use newly designed filter. The example is shown in Figure 2. In this figure, four interpolation pixels are created such as a part of 4x4 MC blocks. In usual case, interpolation filter needs 11 reference pixels to calculate FIR. But, if previous filter is used, couple of pixels at left and right side becomes unnecessary as shown in figure. Gray boxes mean the unnecessary pixel. For interpolation pixel around center, a conventional filter such as DCT-IF is used.

Figure 2: Interpolation in limited input

We show example of interpolation process using new designed filter in Error: Reference source not found. It is called reduction Case-C. Where, light blue boxes are full pel, yellow boxes are vertical quarter pel and pink boxes are MC block with 16 pixels. Dash line surrounds necessary pixels for DCT IF 8 tap. Solid line surrounds necessary pixels for proposal. It can reduce necessary pixels at block boundary by using proposal filter from 11x11 to 9x9 block size.

## More compact support for reduction

Furthermore, we can reduce pixel of boundary using same concept. The example is shown Figure 3. To reduce 4 pixels of boundaries in 4x4 blocks, the filter has to have 7 tap (not 8 tap) because filter needs opposite pixel if it has 8 tap. Although this restriction is not needed for over 8x8 block, current our frame work always uses this 7 tap filter to boundary for all block size to reduce controlling complexity.

Figure 3: Interpolation in more limited input

Using this method, we show example of reduction case 2. In this example, vertical necessary pixels are reduced than horizontal. In general, pixel values are allocated horizontally in external memory and accessed as chunk data corresponds with number of bus bit. From this implementation, to reduce vertical can gets more efficiency.

As we can see that proposal filter can reduce input pixels of MC process. In this example, those are reduced from 11x11 to 11x7 block size.

## Filter coefficients

In this section, actual filter coefficients are shown. For luma interpolation, the filter coefficients are the same as [2]. In this contribution, the chroma filter is newly proposed.

### Case-A for luma interpolation

Table 1 Filter coefficients of 1st line of boundary

| Position | Filter coefficients |
| --- | --- |
| Integer | 0, 0, 0, 64, 0, 0, 0, 0, 0 |
| 1/4 pel | 0, 0, -5, 54, 21, -9, 5, -3, 1 |
| 2/4 pel | 0, 0, -6, 36, 44, -15, 8, -4, 1 |
| 3/4 pel | 0, 0, -3, 16, 59, -12, 6, -3, 1 |

Table 2 Filter coefficients of 2nd line of boundary

| Position | Filter coefficients |
| --- | --- |
| Integer | 0, 0, 0, 64, 0, 0, 0, 0, 0 |
| 1/4 pel | 0, 2, -9, 56, 20, -8, 4, -2, 1 |
| 2/4 pel | 0, 3, -10, 39, 42, -14, 7, -4, 1 |
| 3/4 pel | 0, 2, -6, 18, 58, -11, 5, -3, 1 |

### Case-C for luma interpolation

Table 3 Filter coefficients of 1st line of boundary

| Position | Filter coefficients |
| --- | --- |
| Integer | 0, 0, 0, 64, 0, 0, 0, 0 |
| 1/4 pel | 0, 2, -9, 56, 20, -8, 4, -1 |
| 2/4 pel | 0, 3, -10, 39, 41, -13, 6, -2 |
| 3/4 pel | 0, 2, -6, 18, 58, -11, 4, -1 |

### Case-A and Case-C for chroma interpolation

Table 4 Filter coefficients of 1st line of boundary

| Position | Filter coefficients |
| --- | --- |
| Integer | 0, 64, 0, 0 |
| 1/8 pel | 0, 56, 10, -2 |
| 2/8 pel | 0, 48, 21, -5 |
| 3/8 pel | 0, 38, 33, -7 |
| 4/8 pel | 0, 29, 43, -8 |
| 5/8 pel | 0, 20, 52, -8 |
| 6/8 pel | 0, 13, 58, -7 |
| 7/8 pel | 0, 6, 62, -4 |

# Experiment

We prepared three test cases (A, B and C). Case-A replaces filter coefficients for 2 lines of boundary. The Case-C replaces filter coefficients for 1 line of boundary. So, Case-A can reduce complexity than Case-C. Case-B is prepared to know a complexity of motion estimation (ME) by using only DCTIF 8 tap.

In order to investigate the combination with restriction, we test to restrict PU 8x4 and 4x8 bi-prediction based on case C.

## Test condition and machine platform

Experiment was carried out based on HM software version 4.0 as follow CE3 mandate. And encoding time and decoding time were measured on same platform machines that spec is Core i7 CPU 3.2GHz, and have 8 GB memory. And the Linux 64 bit (called Cent OS) is installed in the platform machine.

For encode, cluster that consists of above identical platform was used. For decode, only 1 computer in the cluster was used, and decoding time was measured without yuv output.

## Coding condition

High coding efficiency anchor:

Random access configuration

Low delay configuration

“P only” (GPB off case)

Low complexity anchor:

Random access configuration

Low delay configuration

“P only” (GPB off case)

Common test condition is specified in JCTVC-E700 [3].

## De-coupling ME and MC

We tested a de-coupling ME and MC that is recommended in CE3. This means to be not used MBF during motion search and mode decision of 2nd step. In motion search and the mode decision, only DCTIF 8 tap is used. This is tested as Case-B.

# Test results

Simulation results are shown in Table 5 for Case-A.

Table 5: Simulation result for Case-A

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | 0.3% | 0.2% | 0.1% | 0.5% | 0.4% |
| Class B | 0.1% | 0.3% | 0.4% | 0.1% | 0.4% | 0.4% |
| Class C | 0.3% | 0.4% | 0.5% | 0.3% | 0.4% | 0.5% |
| Class D | 0.6% | 0.5% | 0.7% | 0.6% | 0.7% | 1.0% |
| Class E |  |  |  |  |  |  |
| Overall | 0.3% | 0.4% | 0.4% | 0.2% | 0.5% | 0.6% |
|  | 0.3% | 0.4% | 0.4% | 0.2% | 0.5% | 0.6% |
| Enc Time[%] | 105% |  |  | 105% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |
| Mult[%] | -8% |  |  | -8% |  |  |
| Adds[%] | -9% |  |  | -9% |  |  |
| Access[%] | -16% |  |  | -16% |  |  |
| MemBand(2D) Ave.[%] | -16% |  |  | -17% |  |  |
| MemBand(2D) Max.[%] | -11% |  |  | -9% |  |  |
| MemBand(1D) Ave.[%] | -16% |  |  | -17% |  |  |
| MemBand(1D) Max.[%] | -10% |  |  | -9% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.1% | 0.4% | 0.4% | 0.0% | 0.5% | 0.6% |
| Class C | 0.3% | 0.4% | 0.6% | 0.2% | 0.9% | 0.7% |
| Class D | 0.5% | 1.2% | 0.6% | 0.4% | 1.6% | 1.5% |
| Class E | 0.2% | 0.3% | 0.1% | -0.1% | 0.4% | -0.1% |
| Overall | 0.3% | 0.6% | 0.5% | 0.1% | 0.9% | 0.7% |
|  | 0.3% | 0.5% | 0.4% | 0.1% | 0.9% | 0.7% |
| Enc Time[%] | 105% |  |  | 105% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |
| Mult[%] | -9% |  |  | -9% |  |  |
| Adds[%] | -10% |  |  | -10% |  |  |
| Access[%] | -17% |  |  | -17% |  |  |
| MemBand(2D) Ave.[%] | -16% |  |  | -17% |  |  |
| MemBand(2D) Max.[%] | -6% |  |  | -8% |  |  |
| MemBand(1D) Ave.[%] | -17% |  |  | -17% |  |  |
| MemBand(1D) Max.[%] | -5% |  |  | -7% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.1% | 0.3% | 0.6% | 0.0% | 0.7% | 0.6% |
| Class C | 0.3% | 0.7% | 0.7% | 0.3% | 0.7% | 1.0% |
| Class D | 0.6% | 1.1% | 1.0% | 0.5% | 1.5% | 1.5% |
| Class E | 0.1% | -0.2% | -0.4% | 0.0% | -0.1% | -0.9% |
| Overall | 0.3% | 0.5% | 0.5% | 0.2% | 0.7% | 0.6% |
|  | 0.3% | 0.5% | 0.5% | 0.2% | 0.7% | 0.6% |
| Enc Time[%] | 104% |  |  | 104% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |
| Mult[%] | -10% |  |  | -10% |  |  |
| Adds[%] | -10% |  |  | -10% |  |  |
| Access[%] | -18% |  |  | -18% |  |  |
| MemBand(2D) Ave.[%] | -17% |  |  | -18% |  |  |
| MemBand(2D) Max.[%] | -10% |  |  | -10% |  |  |
| MemBand(1D) Ave.[%] | -18% |  |  | -18% |  |  |
| MemBand(1D) Max.[%] | -10% |  |  | -9% |  |  |

Simulation results are shown in Table 6 for Case-C.

Table 6: Simulation result for Case-C

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | 0.2% | 0.2% | 0.1% | 0.5% | 0.4% |
| Class B | 0.0% | 0.3% | 0.3% | 0.0% | 0.3% | 0.4% |
| Class C | 0.0% | 0.3% | 0.4% | 0.0% | 0.5% | 0.4% |
| Class D | 0.0% | 0.5% | 0.6% | -0.1% | 0.4% | 0.8% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.3% | 0.4% | 0.0% | 0.4% | 0.5% |
|  | 0.0% | 0.3% | 0.4% | 0.0% | 0.4% | 0.5% |
| Enc Time[%] | 106% |  |  | 107% |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  |
| Mult[%] | -4% |  |  | -4% |  |  |
| Adds[%] | -4% |  |  | -4% |  |  |
| Access[%] | -11% |  |  | -11% |  |  |
| MemBand(2D) Ave.[%] | -12% |  |  | -12% |  |  |
| MemBand(2D) Max.[%] | -6% |  |  | -5% |  |  |
| MemBand(1D) Ave.[%] | -12% |  |  | -13% |  |  |
| MemBand(1D) Max.[%] | -7% |  |  | -5% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.2% | 0.2% | 0.0% | 0.3% | 0.5% |
| Class C | 0.0% | 0.4% | 0.5% | -0.1% | 0.7% | 0.6% |
| Class D | -0.1% | 0.8% | 1.1% | -0.1% | 1.0% | 1.4% |
| Class E | 0.2% | 0.2% | 0.4% | 0.0% | 0.2% | -0.6% |
| Overall | 0.0% | 0.4% | 0.5% | -0.1% | 0.6% | 0.5% |
|  | 0.0% | 0.4% | 0.5% | -0.1% | 0.6% | 0.6% |
| Enc Time[%] | 106% |  |  | 107% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |
| Mult[%] | -5% |  |  | -5% |  |  |
| Adds[%] | -5% |  |  | -5% |  |  |
| Access[%] | -12% |  |  | -12% |  |  |
| MemBand(2D) Ave.[%] | -12% |  |  | -13% |  |  |
| MemBand(2D) Max.[%] | -5% |  |  | -7% |  |  |
| MemBand(1D) Ave.[%] | -12% |  |  | -13% |  |  |
| MemBand(1D) Max.[%] | -4% |  |  | -6% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.3% | 0.3% | 0.1% | 0.6% | 0.5% |
| Class C | 0.0% | 0.5% | 0.7% | 0.1% | 0.8% | 1.0% |
| Class D | 0.1% | 0.8% | 1.0% | 0.0% | 1.0% | 1.0% |
| Class E | 0.2% | -0.4% | -0.4% | 0.2% | -0.8% | -0.7% |
| Overall | 0.1% | 0.3% | 0.4% | 0.1% | 0.5% | 0.5% |
|  | 0.1% | 0.3% | 0.4% | 0.1% | 0.5% | 0.5% |
| Enc Time[%] | 105% |  |  | 106% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |
| Mult[%] | -5% |  |  | -5% |  |  |
| Adds[%] | -5% |  |  | -5% |  |  |
| Access[%] | -12% |  |  | -12% |  |  |
| MemBand(2D) Ave.[%] | -13% |  |  | -13% |  |  |
| MemBand(2D) Max.[%] | -6% |  |  | -5% |  |  |
| MemBand(1D) Ave.[%] | -13% |  |  | -14% |  |  |
| MemBand(1D) Max.[%] | -7% |  |  | -5% |  |  |

## De-coupling ME and MC

Simulation results are shown in Table 7 for Case-B. In this table, run time is not incorrect because of implementation issue.

Table 7: Simulation result for Case-B

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | 0.3% | 0.1% | 0.1% | 0.6% | 0.7% |
| Class B | 0.1% | 0.3% | 0.3% | 0.1% | 0.4% | 0.4% |
| Class C | 0.3% | 0.4% | 0.5% | 0.3% | 0.5% | 0.6% |
| Class D | 0.7% | 0.7% | 0.7% | 0.7% | 0.8% | 1.1% |
| Class E |  |  |  |  |  |  |
| Overall | 0.3% | 0.4% | 0.4% | 0.3% | 0.6% | 0.7% |
|  | 0.3% | 0.4% | 0.4% | 0.3% | 0.6% | 0.7% |
| Enc Time[%] | 119% |  |  | 120% |  |  |
| Dec Time[%] | 103% |  |  | 103% |  |  |
| Mult[%] | -8% |  |  | -8% |  |  |
| Adds[%] | -8% |  |  | -8% |  |  |
| Access[%] | -16% |  |  | -16% |  |  |
| MemBand(2D) Ave.[%] | -16% |  |  | -17% |  |  |
| MemBand(2D) Max.[%] | -9% |  |  | -8% |  |  |
| MemBand(1D) Ave.[%] | -16% |  |  | -17% |  |  |
| MemBand(1D) Max.[%] | -10% |  |  | -8% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.1% | 0.2% | 0.4% | 0.0% | 0.5% | 0.5% |
| Class C | 0.3% | 0.5% | 0.6% | 0.2% | 0.9% | 0.7% |
| Class D | 0.5% | 1.3% | 1.1% | 0.5% | 1.4% | 1.6% |
| Class E | 0.1% | -0.3% | 0.3% | -0.1% | 0.1% | 0.1% |
| Overall | 0.3% | 0.5% | 0.6% | 0.2% | 0.8% | 0.7% |
|  | 0.3% | 0.4% | 0.6% | 0.2% | 0.7% | 0.8% |
| Enc Time[%] | 116% |  |  | 117% |  |  |
| Dec Time[%] | 103% |  |  | 104% |  |  |
| Mult[%] | -9% |  |  | -9% |  |  |
| Adds[%] | -10% |  |  | -9% |  |  |
| Access[%] | -17% |  |  | -17% |  |  |
| MemBand(2D) Ave.[%] | -16% |  |  | -17% |  |  |
| MemBand(2D) Max.[%] | -5% |  |  | -9% |  |  |
| MemBand(1D) Ave.[%] | -17% |  |  | -17% |  |  |
| MemBand(1D) Max.[%] | -4% |  |  | -7% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.1% | 0.6% | 0.7% | 0.1% | 0.6% | 0.5% |
| Class C | 0.4% | 0.6% | 0.7% | 0.3% | 0.7% | 0.9% |
| Class D | 0.7% | 1.2% | 1.2% | 0.6% | 1.2% | 1.5% |
| Class E | 0.1% | -0.5% | -0.1% | 0.0% | -0.2% | -0.3% |
| Overall | 0.3% | 0.5% | 0.7% | 0.3% | 0.6% | 0.7% |
|  | 0.3% | 0.5% | 0.7% | 0.3% | 0.6% | 0.8% |
| Enc Time[%] | 119% |  |  | 119% |  |  |
| Dec Time[%] | 102% |  |  | 102% |  |  |
| Mult[%] | -10% |  |  | -10% |  |  |
| Adds[%] | -10% |  |  | -10% |  |  |
| Access[%] | -18% |  |  | -18% |  |  |
| MemBand(2D) Ave.[%] | -17% |  |  | -18% |  |  |
| MemBand(2D) Max.[%] | -8% |  |  | -9% |  |  |
| MemBand(1D) Ave.[%] | -18% |  |  | -18% |  |  |
| MemBand(1D) Max.[%] | -8% |  |  | -10% |  |  |

## Test case-C with restriction

The simulation results are shown in Table 8.

Table 8 Results for case c with restriction

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.2% | 0.3% | 0.3% | 0.2% | 0.8% | 0.7% |
| Class B | 0.2% | 0.5% | 0.5% | 0.2% | 0.5% | 0.6% |
| Class C | 0.4% | 0.7% | 0.8% | 0.4% | 0.8% | 0.9% |
| Class D | 0.6% | 0.9% | 1.1% | 0.6% | 1.1% | 1.3% |
| Class E |  |  |  |  |  |  |
| Overall | 0.4% | 0.6% | 0.6% | 0.4% | 0.8% | 0.8% |
|  | 0.4% | 0.6% | 0.7% | 0.4% | 0.8% | 0.8% |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 102% |  |  | 103% |  |  |
| Mult[%] | -4% |  |  | -5% |  |  |
| Adds[%] | -5% |  |  | -5% |  |  |
| Access[%] | -12% |  |  | -12% |  |  |
| MemBand(2D) Ave.[%] | -13% |  |  | -14% |  |  |
| MemBand(2D) Max.[%] | -6% |  |  | -5% |  |  |
| MemBand(1D) Ave.[%] | -13% |  |  | -14% |  |  |
| MemBand(1D) Max.[%] | -6% |  |  | -5% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.3% | 0.2% | 0.5% | 0.2% | 0.8% | 0.9% |
| Class C | 0.6% | 1.0% | 1.1% | 0.5% | 1.3% | 1.1% |
| Class D | 0.8% | 1.5% | 1.7% | 0.6% | 1.9% | 1.9% |
| Class E | 0.6% | 0.3% | 0.4% | 0.2% | -0.1% | -0.4% |
| Overall | 0.6% | 0.7% | 0.9% | 0.4% | 1.0% | 1.0% |
|  | 0.6% | 0.8% | 0.9% | 0.4% | 1.0% | 1.0% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 103% |  |  | 103% |  |  |
| Mult[%] | -6% |  |  | -5% |  |  |
| Adds[%] | -6% |  |  | -5% |  |  |
| Access[%] | -13% |  |  | -12% |  |  |
| MemBand(2D) Ave.[%] | -14% |  |  | -14% |  |  |
| MemBand(2D) Max.[%] | -4% |  |  | -7% |  |  |
| MemBand(1D) Ave.[%] | -14% |  |  | -14% |  |  |
| MemBand(1D) Max.[%] | -3% |  |  | -6% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.3% | 0.3% | 0.1% | 0.6% | 0.5% |
| Class C | 0.0% | 0.5% | 0.7% | 0.1% | 0.8% | 1.0% |
| Class D | 0.1% | 0.8% | 1.0% | 0.0% | 1.0% | 1.0% |
| Class E | 0.2% | -0.4% | -0.4% | 0.2% | -0.8% | -0.7% |
| Overall | 0.1% | 0.3% | 0.4% | 0.1% | 0.5% | 0.5% |
|  | 0.1% | 0.3% | 0.4% | 0.1% | 0.5% | 0.5% |
| Enc Time[%] | 104% |  |  | 106% |  |  |
| Dec Time[%] | 103% |  |  | 105% |  |  |
| Mult[%] | -5% |  |  | -5% |  |  |
| Adds[%] | -5% |  |  | -5% |  |  |
| Access[%] | -12% |  |  | -12% |  |  |
| MemBand(2D) Ave.[%] | -13% |  |  | -13% |  |  |
| MemBand(2D) Max.[%] | -6% |  |  | -5% |  |  |
| MemBand(1D) Ave.[%] | -13% |  |  | -14% |  |  |
| MemBand(1D) Max.[%] | -7% |  |  | -5% |  |  |

# Theoretical complexity analysis

We investigated theoretical complexity analysis using CE3’s excel sheet. The summary of complexity analysis is shown in Table 9. We can see that complexity both b/w and operation is decreasing. The memory b/w of Case-A significantly decreases to 44% for 2D mapping bandwidth.

Table 9: Summary of complexity analysis

|  |  | Case-A and B | Case-C | Case-C with restriction |
| --- | --- | --- | --- | --- |
| Theoretically worst | Memory Bandwidth for 2D mapping | -44% | -38% | -58% |
|  | Memory Bandwidth for 1D mapping | -33% | -25% | -59% |
|  | Multiplication | -28% | -21% | -33% |
|  | Addition | -27% | -20% | -33% |

# Subjective check

The subjective check was carried out as follow [1]. The visual quality was the same with anchor. Those still images are included in this contribution.

# Conclusion

We tested the MC boundary filter which is one of tools in CE3. This contribution reported that the complexity both memory bandwidth and computation can be reduced. For memory bandwidth, it is reduced -44% and -17% in worst case and actual case. For computation cost of multiplication, it is reduced -28% and -9% in worst case and actual case. The impact for coding efficiency is 0.3%, 0.2%, 0.3%, 0.1%, 0.3%, and 0.2% for RA_HE, RA_LC, LD_HE, LD_LC, LDP_HE and LDP_LC.

# References

[1] “Core Experiment 3: Motion Compensation”, JCTVC-F903, July 2011

[2] “CE3: Results on MC boundary filter (tool 4)” JCTVC-F100, 6th. JCT-VC Meeting, Torino, July 2011

[3] F. Bossen, “Common HM test conditions and software reference configurations,” JCTVC-F900, 6th. JCT-VC Meeting, Torino, IT, 14-22 July, 2011.

# Patent rights declaration(s)

SONY corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).