| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G392 |
| --- | --- |

| Title: | Non-CE3: Report on a restriction for small block |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kenji Kondo and Teruhiko Suzuki Sony City Osaki, 2-10-1 Osaki Shinagawa-ku, Tokyo Japan | Tel: Email: | +81-50-3750-2744 kenji.kondo@jp.sony.com teruhikos@jp.sony.com |
| Source: | SONY |  |  |

_____________________________

# Abstract

This contribution reports the results of restriction for small block. To cut worst case complexity, this contribution tested three restrictions that small PU size is restricted without decoder change. The case-A is that PU size 8x4, 4x8 and 8x8 for bi-prediction are restricted. The case-B is that PU size 8x4 and 4x8 are restricted. The case-C is that 8x4 and 4x8 for bi-prediction are restricted. With the restriction, it is shown that the impact of coding efficiency is 2.2%, 1.4%, and 0.3% for case-A, B, and C.

# Introduction

In Torino meeting, We pointed out concern of memory bandwidth for high resolution image such as 4K2K in [3]. This contribution studies a restriction for motion compensation to reduce worst memory bandwidth.

We show memory bandwidth for each PU in Figure 1. In this graph, the memory alignment is assumed as 8x2 mapping. And, vertical means number of input pixel per one output pixel. In legend, prediction mode, horizontal size, vertical size and sub pel position are shown. Where “efgijkpqr” is sub pel position, it needs 2d-filtering horizontally and vertically and “abc” and “dhn” need 1d-filtering horizontally or vertically. As shown the figure, the bandwidth increases when small PU size, bi-prediction and 2d-filtering are chosen.

Especially, large image size such as 4K2K application consumes memory bandwidth. If we guarantee worst memory bandwidth, more cost will be needed.

If we want to reduce worst case, a restriction is one of ideas. For example, in H.264/AVC standard, the usage of 8x4 and 4x8 bi-prediction are restricted if the level 3.1 and more than over 3.1 is used. In current HM design, interpolation filter uses 8 tap FIR filter for luma and 4 tap for chroma. H.264/AVC uses 6 tap and 2 tap for luma and chroma. So memory bandwidth of HEVC increases than H.264/AVC.

This contribution tests a restriction to know the impact. In the test, the test condition as same as Core experiment-3 is used.

Figure 1 Memory bandwidth for PU level

# Experiment

We prepared three test cases (A, B and C) as shown in Table 1. Case-B restricts usage of PU 8x4 and 4x8. Case-A restricts usage of bi-prediction 8x8 more than case-B. For case-C, PU 8x4 and 4x8 of bi-prediction are restricted.

Table 1 test case

|  |  | Restriction case |  |  |
| --- | --- | --- | --- | --- |
| Pred. mode | PU size | Case-A | Case-B | Case-C |
| uni | 8x4 | Restricted | Restricted |  |
| uni | 4x8 | Restricted | Restricted |  |
| bi | 8x4 | Restricted | Restricted | Restricted |
| bi | 4x8 | Restricted | Restricted | Restricted |
| bi | 8x8 | Restricted |  |  |

## Test condition and machine platform

Experiment was carried out based on HM software version 4.0 as follow CE3 mandate. And encoding time and decoding time were measured on same platform machines that spec is Core i7 CPU 3.2GHz, and have 8 GB memory. And the Linux 64 bit (called Cent OS) is installed in the platform machine.

For encode, cluster that consists of above identical platform was used. For decode, only 1 computer in the cluster was used, and decoding time was measured without yuv output.

## Coding condition

High coding efficiency anchor:

Random access configuration

Low delay configuration

“P only” (GPB off case)

Low complexity anchor:

Random access configuration

Low delay configuration

“P only” (GPB off case)

Common test condition is specified in [4].

# Test results

## Case-A

Simulation results are shown in Table 2 for Case-A. Average BD% for Case-A is 2.2%.

Table 2: Simulation result for Case-A

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 1.0% | 1.3% | 1.0% | 1.0% | 1.8% | 1.6% |
| Class B | 1.5% | 1.1% | 0.9% | 1.4% | 1.2% | 1.3% |
| Class C | 2.4% | 2.6% | 2.9% | 2.4% | 2.7% | 3.0% |
| Class D | 3.5% | 3.6% | 3.6% | 3.5% | 3.4% | 3.8% |
| Class E |  |  |  |  |  |  |
| Class F | #REF! | #REF! | #REF! | #REF! | #REF! | #REF! |
| Overall | 2.1% | 2.1% | 2.0% | 2.0% | 2.2% | 2.4% |
|  | 2.1% | 2.1% | 2.0% | 2.0% | 2.2% | 2.4% |
| Enc Time[%] | 92% |  |  | 86% |  |  |
| Dec Time[%] | 98% |  |  | 98% |  |  |
| Mult[%] | -1% |  |  | -1% |  |  |
| Adds[%] | -1% |  |  | -1% |  |  |
| Access[%] | -3% |  |  | -3% |  |  |
| MemBand(2D) Ave.[%] | -5% |  |  | -5% |  |  |
| MemBand(2D) Max.Frm[%] | 7% |  |  | 7% |  |  |
| MemBand(2D) Max.CU[%] | -45% |  |  | -50% |  |  |
| MemBand(1D) Ave.[%] | -5% |  |  | -5% |  |  |
| MemBand(1D) Max.Frm[%] | 8% |  |  | 7% |  |  |
| MemBand(1D) Max.CU[%] | -45% |  |  | -49% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 1.9% | 0.6% | 0.1% | 1.8% | 1.6% | 1.9% |
| Class C | 3.4% | 3.3% | 3.5% | 3.2% | 3.5% | 3.8% |
| Class D | 4.6% | 4.4% | 4.6% | 4.6% | 4.5% | 4.8% |
| Class E | 1.5% | 0.6% | 1.7% | 1.3% | 1.0% | 0.6% |
| Class F | #REF! | #REF! | #REF! | #REF! | #REF! | #REF! |
| Overall | 2.9% | 2.2% | 2.4% | 2.8% | 2.7% | 2.9% |
|  | 2.9% | 2.2% | 2.3% | 2.8% | 2.7% | 2.9% |
| Enc Time[%] | 87% |  |  | 84% |  |  |
| Dec Time[%] | 97% |  |  | 97% |  |  |
| Mult[%] | -2% |  |  | -2% |  |  |
| Adds[%] | -2% |  |  | -2% |  |  |
| Access[%] | -5% |  |  | -4% |  |  |
| MemBand(2D) Ave.[%] | -8% |  |  | -7% |  |  |
| MemBand(2D) Max.Frm[%] | 1% |  |  | 2% |  |  |
| MemBand(2D) Max.CU[%] | -49% |  |  | -48% |  |  |
| MemBand(1D) Ave.[%] | -7% |  |  | -7% |  |  |
| MemBand(1D) Max.Frm[%] | 0% |  |  | 2% |  |  |
| MemBand(1D) Max.CU[%] | -48% |  |  | -46% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 1.0% | 0.9% | 1.0% | 0.8% | 1.1% | 0.8% |
| Class C | 2.1% | 2.3% | 2.4% | 1.7% | 1.9% | 2.0% |
| Class D | 3.7% | 3.6% | 3.6% | 3.3% | 3.7% | 3.3% |
| Class E | 0.7% | 0.1% | 0.4% | 0.5% | -0.3% | 0.2% |
| Class F | #REF! | #REF! | #REF! | #REF! | #REF! | #REF! |
| Overall | 1.9% | 1.8% | 1.9% | 1.6% | 1.7% | 1.6% |
|  | 1.9% | 1.8% | 1.8% | 1.6% | 1.7% | 1.6% |
| Enc Time[%] | 94% |  |  | 95% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |
| Mult[%] | 0% |  |  | 0% |  |  |
| Adds[%] | 0% |  |  | 0% |  |  |
| Access[%] | -2% |  |  | -2% |  |  |
| MemBand(2D) Ave.[%] | -4% |  |  | -4% |  |  |
| MemBand(2D) Max.Frm[%] | 1% |  |  | 2% |  |  |
| MemBand(2D) Max.CU[%] | -12% |  |  | -30% |  |  |
| MemBand(1D) Ave.[%] | -4% |  |  | -3% |  |  |
| MemBand(1D) Max.Frm[%] | 1% |  |  | 2% |  |  |
| MemBand(1D) Max.CU[%] | -12% |  |  | -30% |  |  |

## Case-B

The simulation results are shown in Table 3. The average BD% is 1.4%.

Table 3: Simulation result for Case-B

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.5% | 0.8% | 0.5% | 0.5% | 1.0% | 0.7% |
| Class B | 0.6% | 0.6% | 0.6% | 0.5% | 0.5% | 0.6% |
| Class C | 1.4% | 1.5% | 1.8% | 1.2% | 1.5% | 1.6% |
| Class D | 2.3% | 2.5% | 2.4% | 2.1% | 2.2% | 2.4% |
| Class E |  |  |  |  |  |  |
| Class F | #REF! | #REF! | #REF! | #REF! | #REF! | #REF! |
| Overall | 1.2% | 1.3% | 1.3% | 1.0% | 1.2% | 1.3% |
|  | 1.2% | 1.3% | 1.3% | 1.0% | 1.3% | 1.3% |
| Enc Time[%] | 95% |  |  | 91% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
| Mult[%] | 0% |  |  | 0% |  |  |
| Adds[%] | 0% |  |  | 0% |  |  |
| Access[%] | -1% |  |  | -1% |  |  |
| MemBand(2D) Ave.[%] | -2% |  |  | -2% |  |  |
| MemBand(2D) Max.Frm[%] | 6% |  |  | 10% |  |  |
| MemBand(2D) Max.CU[%] | -11% |  |  | -12% |  |  |
| MemBand(1D) Ave.[%] | -2% |  |  | -2% |  |  |
| MemBand(1D) Max.Frm[%] | 6% |  |  | 10% |  |  |
| MemBand(1D) Max.CU[%] | -11% |  |  | -18% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.7% | 0.6% | 0.6% | 0.6% | 0.7% | 0.9% |
| Class C | 1.8% | 2.0% | 2.0% | 1.5% | 1.7% | 1.7% |
| Class D | 2.9% | 3.4% | 3.3% | 2.7% | 3.0% | 3.3% |
| Class E | 0.6% | 0.5% | 1.1% | 0.4% | 0.8% | 0.2% |
| Class F | #REF! | #REF! | #REF! | #REF! | #REF! | #REF! |
| Overall | 1.5% | 1.6% | 1.7% | 1.3% | 1.5% | 1.5% |
|  | 1.5% | 1.7% | 1.7% | 1.3% | 1.5% | 1.6% |
| Enc Time[%] | 93% |  |  | 92% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
| Mult[%] | 0% |  |  | 0% |  |  |
| Adds[%] | 0% |  |  | 0% |  |  |
| Access[%] | -1% |  |  | -1% |  |  |
| MemBand(2D) Ave.[%] | -2% |  |  | -2% |  |  |
| MemBand(2D) Max.Frm[%] | 7% |  |  | 8% |  |  |
| MemBand(2D) Max.CU[%] | -14% |  |  | -22% |  |  |
| MemBand(1D) Ave.[%] | -2% |  |  | -2% |  |  |
| MemBand(1D) Max.Frm[%] | 8% |  |  | 8% |  |  |
| MemBand(1D) Max.CU[%] | -13% |  |  | -22% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 1.0% | 0.9% | 1.0% | 0.8% | 1.1% | 0.8% |
| Class C | 2.1% | 2.3% | 2.4% | 1.7% | 1.9% | 2.0% |
| Class D | 3.7% | 3.6% | 3.6% | 3.3% | 3.7% | 3.3% |
| Class E | 0.7% | 0.1% | 0.4% | 0.5% | -0.3% | 0.2% |
| Class F | #REF! | #REF! | #REF! | #REF! | #REF! | #REF! |
| Overall | 1.9% | 1.8% | 1.9% | 1.6% | 1.7% | 1.6% |
|  | 1.9% | 1.8% | 1.8% | 1.6% | 1.7% | 1.6% |
| Enc Time[%] | 98% |  |  | 93% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |
| Mult[%] | 0% |  |  | 0% |  |  |
| Adds[%] | 0% |  |  | 0% |  |  |
| Access[%] | -2% |  |  | -2% |  |  |
| MemBand(2D) Ave.[%] | -4% |  |  | -4% |  |  |
| MemBand(2D) Max.Frm[%] | 1% |  |  | 2% |  |  |
| MemBand(2D) Max.CU[%] | -12% |  |  | -30% |  |  |
| MemBand(1D) Ave.[%] | -4% |  |  | -3% |  |  |
| MemBand(1D) Max.Frm[%] | 1% |  |  | 2% |  |  |
| MemBand(1D) Max.CU[%] | -12% |  |  | -30% |  |  |

## Case C

The simulation results are shown in Table 4. The average BD% is 0.3%.

Table 4 Simulation results for case-C

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.2% | 0.1% | -0.1% | 0.2% | 0.4% | 0.2% |
| Class B | 0.2% | 0.2% | 0.2% | 0.2% | 0.1% | 0.2% |
| Class C | 0.4% | 0.3% | 0.5% | 0.4% | 0.4% | 0.4% |
| Class D | 0.6% | 0.6% | 0.6% | 0.7% | 0.5% | 0.6% |
| Class E |  |  |  |  |  |  |
| Class F | #REF! | #REF! | #REF! | #REF! | #REF! | #REF! |
| Overall | 0.3% | 0.3% | 0.3% | 0.3% | 0.3% | 0.3% |
|  | 0.3% | 0.3% | 0.3% | 0.3% | 0.3% | 0.3% |
| Enc Time[%] | 99% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
| Mult[%] | 0% |  |  | 0% |  |  |
| Adds[%] | 0% |  |  | 0% |  |  |
| Access[%] | -1% |  |  | -1% |  |  |
| MemBand(2D) Ave.[%] | -2% |  |  | -2% |  |  |
| MemBand(2D) Max.Frm[%] | 4% |  |  | 5% |  |  |
| MemBand(2D) Max.CU[%] | -11% |  |  | -18% |  |  |
| MemBand(1D) Ave.[%] | -1% |  |  | -1% |  |  |
| MemBand(1D) Max.Frm[%] | 5% |  |  | 4% |  |  |
| MemBand(1D) Max.CU[%] | -11% |  |  | -18% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.3% | 0.0% | -0.1% | 0.3% | 0.3% | 0.3% |
| Class C | 0.6% | 0.7% | 0.5% | 0.5% | 0.7% | 0.4% |
| Class D | 0.9% | 1.0% | 0.8% | 0.8% | 0.8% | 0.4% |
| Class E | 0.4% | -0.6% | 1.0% | 0.2% | 0.6% | -0.2% |
| Class F | #REF! | #REF! | #REF! | #REF! | #REF! | #REF! |
| Overall | 0.5% | 0.3% | 0.5% | 0.4% | 0.6% | 0.3% |
|  | 0.5% | 0.3% | 0.4% | 0.4% | 0.5% | 0.3% |
| Enc Time[%] | 103% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |
| Mult[%] | -1% |  |  | -1% |  |  |
| Adds[%] | -1% |  |  | -1% |  |  |
| Access[%] | -2% |  |  | -1% |  |  |
| MemBand(2D) Ave.[%] | -2% |  |  | -2% |  |  |
| MemBand(2D) Max.Frm[%] | 4% |  |  | 4% |  |  |
| MemBand(2D) Max.CU[%] | -14% |  |  | -22% |  |  |
| MemBand(1D) Ave.[%] | -2% |  |  | -2% |  |  |
| MemBand(1D) Max.Frm[%] | 4% |  |  | 3% |  |  |
| MemBand(1D) Max.CU[%] | -13% |  |  | -22% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | #REF! | #REF! | #REF! | #REF! | #REF! | #REF! |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 106% |  |  | 105% |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  |
| Mult[%] | 0% |  |  | 0% |  |  |
| Adds[%] | 0% |  |  | 0% |  |  |
| Access[%] | 0% |  |  | 0% |  |  |
| MemBand(2D) Ave.[%] | 0% |  |  | 0% |  |  |
| MemBand(2D) Max.Frm[%] | 0% |  |  | 0% |  |  |
| MemBand(2D) Max.CU[%] | 0% |  |  | 0% |  |  |
| MemBand(1D) Ave.[%] | 0% |  |  | 0% |  |  |
| MemBand(1D) Max.Frm[%] | 0% |  |  | 0% |  |  |
| MemBand(1D) Max.CU[%] | 0% |  |  | 0% |  |  |

# Theoretical complexity analysis

We investigated theoretical complexity analysis using CE3’s excel sheet. The summary of complexity analysis is shown in Table 5. We can see that complexity both b/w and operation is decreasing. The memory b/w of case-A significantly decreases -52% for 2D mapping bandwidth.

Table 5: Summary of worst complexity analysis

|  |  | Test case |  |  |
| --- | --- | --- | --- | --- |
|  |  | Case-A | Case-B | Case-C |
| Worst case analysis | Memory Bandwidth for 2D mapping | -52% | -36% | -36% |
|  | Memory Bandwidth for 1D mapping | -47% | -31% | -31% |
|  | Multiplication | -22% | -22% | -22% |
|  | Addition | -22% | -22% | -22% |

The worst complexity of case-B is the same with case-C because PU8x8 for bi-prediction is worst.

# Conclusion

We tested the restriction for MC. This contribution reported that the restriction can reduce worst case complexity both memory bandwidth and computation. For memory bandwidth, it is significantly reduced -52% and -36% for case-A and B. For computation cost of multiplication, it is reduced -22%. The negative impact for coding efficiency is 2.2% and 1.4% for case-A and B.

# References

[1] “Core Experiment 3: Motion Compensation”, JCTVC-F903, July 2011

- [2] F. Bossen, “Common HM test conditions and software reference configurations,” JCTVC-F900, 6th. JCT-VC Meeting, Torino, IT, 14-22 July, 2011.

[3] T. Suzuki, “Study of memory bandwidth of HEVC Motion Compensation”, JCTVC-F487, 6th JCT-VC Meeting, Torino, IT, 14-22 July, 20011

# Patent rights declaration(s)

SONY corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).