| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G393 |
| --- | --- |

| Title: | CE3: Cross check for Toshiba's proposal (tool 2) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kenji Kondo and Teruhiko Suzuki Sony City Osaki, 2-10-1 Osaki Shinagawa-ku, Tokyo Japan | Tel: Email: | +81-50-3750-2744 kenji.kondo@jp.sony.com teruhikos@jp.sony.com |
| Source: | SONY |  |  |

_____________________________

# Abstract

This contribution reports cross-verification results of CE3 tool 2[1] described in JCTVC-GXXX. Cross-verification task has been successfully completed, and it is confirmed that the BD-rate results match those provided by proponent.

# Software analysis

The following modifications are confirmed through the analysis of software provided by Toshiba:

- Switch filter coefficients according to the sub pel position in an MC reference block:

  - If a sample position to be interpolated locates on ‘e’, ‘g’, ‘p’ and ‘r’ sub pel position, proposed 7 tap filter is used. These ‘e’, ‘g’, ‘p’ and ‘r’ sub pel position need to generate quarter pel horizontally and vertically.

  - Otherwise, a sample value is generated by using the HM DCTIF filter.

# Visual quality

For visual quality, BQMall and PartyScene for P-only without SAO are checked. I felt that the proposal is the same with anchor for visual quality. The still images are included in this contribution.

# Experiment

## Test condition and machine platform

Experiment was carried out based on HM software version 4.0 as follow CE3 mandate. And encoding time and decoding time were measured on same platform machines that spec is Core i7 CPU 3.2GHz, and have 8 GB memory. And the Linux 64 bit (called Cent OS) is installed in the platform machine.

For encode, cluster that consists of above identical platform was used. For decode, only 1 computer in the cluster was used, and decoding time was measured without yuv output.

## Coding condition

High coding efficiency anchor:

Random access configuration

Low delay configuration

“P only” (GPB off case)

Low complexity anchor:

Random access configuration

Low delay configuration

“P only” (GPB off case)

Common test condition is specified in [2].

# Conclusion

It is confirmed that the CE3 tool 2 results reported by proponent are accurate.

# References

[1] “Core Experiment 3: Motion Compensation”, JCTVC-F903, July 2011

- [2] F. Bossen, “Common HM test conditions and software reference configurations,” JCTVC-F900, 6th. JCT-VC Meeting, Torino, IT, 14-22 July, 2011.

# Patent rights declaration(s)

SONY corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).