| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G395 |
| --- | --- |

| Title: | CE3: Cross check for eBrisk's proposal (tool 6) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kenji Kondo and Teruhiko Suzuki Sony City Osaki, 2-10-1 Osaki Shinagawa-ku, Tokyo Japan | Tel: Email: | +81-50-3750-2744 kenji.kondo@jp.sony.com teruhikos@jp.sony.com |
| Source: | SONY |  |  |

_____________________________

# Abstract

This contribution reports cross-verification results of CE3 tool 6[1] described in JCTVC-GXXX. Cross-verification task has been successfully completed, and it is confirmed that the BD-rate results match those provided by proponent.

# Software analysis

The following modifications are confirmed through the analysis of software provided by eBrisk:

- AIF on/off can be controlled for each L0/L1.

  - If AIF is off, conventional filter coefficients of current HM are used.

  - Chroma is fixed filter.

- Tap length is a variable. But the control information is not sent. Decoder can know it by check filter coefficients. It can reduce a complexity in average.

# Visual quality

For visual quality, BQMall and PartyScene for P-only without SAO are checked. We checked both movie and still image. For movie sequence, I felt that the proposal is the same degree with anchor for visual quality. For still image, the ringing noise of proposal AIF was suppressed. Those still images are included in this contribution.

# Experiment

## Test condition and machine platform

Experiment was carried out based on HM software version 4.0 as follow CE3 mandate. And encoding time and decoding time were measured on same platform machines that spec is Core i7 CPU 3.2GHz, and have 8 GB memory. And the Linux 64 bit (called Cent OS) is installed in the platform machine.

For encode, cluster that consists of above identical platform was used. For decode, only 1 computer in the cluster was used, and decoding time was measured without yuv output.

## Coding condition

High coding efficiency anchor:

Random access configuration

Low delay configuration

“P only” (GPB off case)

Low complexity anchor:

Random access configuration

Low delay configuration

“P only” (GPB off case)

Common test condition is specified in [2].

# Conclusion

It is confirmed that the CE3 tool 6results reported by proponent are accurate.

# References

[1] “Core Experiment 3: Motion Compensation”, JCTVC-F903, July 2011

- [2] F. Bossen, “Common HM test conditions and software reference configurations,” JCTVC-F900, 6th. JCT-VC Meeting, Torino, IT, 14-22 July, 2011.

# Patent rights declaration(s)

SONY corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).