| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 19-30 November, 2011 | Document: JCTVC-G396 M21959 |
| --- | --- |

| Title: | Non-CE9: reordering of merge candidate |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chul Keun Kim Yongjoon Jeon Byeongmoon Jeon #221 Yangjae-dong, Seoucho-gu, Seoul 137-130, Korea | Email: | chulkeun.kim@lge.com |
| Source: | LG |  |  |

_____________________________

# Abstract

In this contribution, two methods for reordering the merge candidates are presented. The first method is applied only for 2Nx2N PUs of square shape. The first method (Proposed method 1) swaps A1 and B1 candidate order in the merge list if the propose condition is satisfied. It is reportedly shown in the experimental results that 0.1~0.2% BD rate reduction is achieved without increasing encoding and decoding time with the proposed method 1. The second method (Proposed method 2) is applied for the second PUs of 2NxN, 2NxnU, 2NxnD, Nx2N, nLx2N and nRx2N partitions of rectangular shape. The proposed method 2 uses the MVP candidate which belongs to the first PU of those rectangular partitions for creating the combined bi-pred. candidates even though this candidate shall not exist in the initial list due to the avoiding check operation. It is reportedly shown in the experimental results that 0.1~0.2% BD rate reduction is achieved without increasing encoding and decoding time with the proposed method 2. In addition, the proposed methods are tested with the new anchor in which the avoiding check operation is removed. It is reportedly shown from the simulation results that the proposed 2 achieves 0.0~0.1% BD rate saving without encoding/decoding time increase relative to the new anchor and the combination of the proposed method 1 and the proposed method 2 achieves 0.1~0.2% BD rate saving. The encoding and decoding time is almost same as the anchor.

# Proposed method

## Proposed method 1: Merge list reordering for square PUs (2Nx2N)

In HM 4.0, spatial merge candidate order is as follows.

- A1, B1, B0, A0, B2

We propose to swap A1 and B1 if neighboring MVs are vertically correlated rather than horizontally correlated as follows.

- B1, A1, B0, A0, B2

The proposed swapping can be applied only if all of the following conditions are satisfied.

- PartMode == PART_2Nx2N

- A1, B1, B2 are available and not intra-coded.

The following pseudo code represents the proposed method.

if ( DV < DH )

- Swap A1 and B1

- Fig. 1 The candidates used for the proposed method 1

The variables DV and DH represent the distance between mvLXA1 and mvLXB2 and the distance between mvLXB1 and mvLXB2 respectively.

Depending on slice type, the variables DV and DH are derived as follows.

If B slice,

- DV = DistL0V + DistL1V

- DH = DistL0H + DistL1H

If P slice,

- DV = DistL0V

- DH = DistL0H

,where

- DistLXV = Abs(mvLXA1[0] – mvLXB2[0]) + Abs(mvLXA1[1] – mvLXB2[1])

- DistLXH = Abs(mvLXB1[0] – mvLXB2[0]) + Abs(mvLXB1[1] – mvLXB2[1])

## Proposed Method 2: Merge list reordering for rectangular PUs

In the second PUs (PU1 in Fig. 2) of 2NxN, 2NxnU, 2NxnD, Nx2N, nLx2N and nRx2N partitions, if a MVP candidate has same motion (mvLX, refIdxLX and predFlagLX) with the candidate which belongs to the first PU (PU0 of Fig. 2), the MVP candidate is avoided to be added to the merge list. This comparison is called as ‘avoiding check’ in this contribution. The candidate which belongs to the first PU also shall not be added to the list due to this avoiding check operation.

Fig. 2 Example of 2NxN partition

The proposed method 2 is applied only if PartIdx is equal to 1 and PartMode is PART_2NxN, PART_2NxnU, PART_2NxnD, PART_Nx2N, PART_nLx2N or PART_nRx2N.

In the proposed method 2, it is proposed to use the MVP candidate which belongs to the first PU (B1 in Fig. 2) for creating the combined bi-pred. candidates even though the candidate shall not exist in the initial merge list. This is illustrated in Fig. 3. In Fig. 3, B1 shall not be in the initial merge list due to the avoiding check. However, we use B1 for the combined bi-pred. candidates.

- Fig. 3. Proposed method

In the combined bi-pred. candidate generation process, the MVP candidate which belongs to the first PU can be combined only with the first entry of the merge list. Therefore just two new combined bi-pred. candidates are introduced by the proposed method 2. This process is illustrated in Fig. 4.

- Fig. 4. The combined bi-pred. candidates created by the proposed method (yellow ones)

Note that the maximum number for the combined bi-pred. candidates is 5 which is same as the current HM. The difference between the current HM and the proposed method 2 is that when decoding the second PUs of 2NxN, 2NxnU, 2NxnD, Nx2N, nLx2N and nRx2N partitions, different combined bi-pred. candidates can be made. This is illustrated in Fig. 5. In this figure, ‘X’ represents the candidate which belongs to the first PU of those rectangular partitions. If the PU is the second PU of 2NxN, 2NxnU, 2NxnD, Nx2N, nLx2N and nRx2N partitions, the entries of 0 to 4 can be used. Otherwise, the entries of 2 to 6 can be used. In this case, the entries used for the combined bi-pred. candidates are same as the current HM.

Fig. 5 Specification of l0CandIdx and l1CandIdx in the proposed method 2.

# Simulation result

For the simulation, HM4.0 MrgEncFix is used as the anchor.

1. Results for Proposed method 1

Anchor : HM4.0 MrgEncFix

Tested : HM4.0 MrgEncFix + Proposed method 1

As shown in Table 1, 0.1%, 0.0%, 0.2%, and 0.1% BD rate reductions are achieved for RA-HE, RA-LC, LB-HE, and LB-LC relative to the anchor. The encoding and decoding time is almost same as the anchor.

Table 1. Results for proposed method 1

2. Results for Proposed method 2

Anchor : HM4.0 MrgEncFix

Tested : HM4.0 MrgEncFix + Proposed method 2

As shown in Table 2, 0.1%, 0.1%, 0.1%, and 0.2% BD rate reductions are achieved for RA-HE, RA-LC, LB-HE, and LB-LC relative to the anchor. The encoding and decoding time is almost same as the anchor.

Table 2. Results for proposed method 2

3. Additional tests

At this meeting, there are several proposals which propose to remove the avoiding check operation to simplify the current merge/skip mode. If this simplification is applied, the performance of the proposed method 2 could be affected. To measure the performance of the proposed methods under the HM with this simplification, the following experiments have carried out.

1. Simplification
In this experiment, the avoiding check operation is removed. However, when decoding the second PU of 2NxN, 2NxnU, 2NxnD, Nx2N, nLx2N and nRx2N partitions, the MVP candidate which belongs to the first PU of those partitions is avoided to be added to the merge list not to make the PU predicted forming 2Nx2N block, which has already covered by the 2Nx2N mode. This is same as what is proposed in JCTVC-G681. However, in this experiment, HM4.0 MrgEncFix is used while JCTVC-G681 used HM4.0.

Anchor : HM4.0 MrgEncFix

Tested : HM4.0 MrgEncFix + ‘simplification’

Table 3. Results for “Simplification” relative to HM4.0 MrgEncFix.

As shown in Table 3, negligible loss is occurred by the simplification. The encoding and decoding time is almost same as the anchor.

2. Simplification + Proposed method 2

The proposed method 2 has simulated with the HM4.0 MrgEncFix with the simplification.

In the simplification, when decoding the second PU of 2NxN, 2NxnU, 2NxnD, Nx2N, nLx2N and nRx2N partitions, the MVP candidate which belongs to the first PU shall not exist in the initial merge list after the 1st pruning process since the candidate is avoided to be added in the merge list by the simplification. However, the proposed method 2 uses the candidate for creating two combined bi-pred. candidates which are created by the combination of the candidate and the first entry of the initial list.

Table 4 shows the results of the tested scheme relative to the HM4.0 with the simplification. For this experiment, “HM4.0 MrgEncFix + Simplification” is used as the anchor

Anchor : HM4.0 MrgEncFix + ‘simplification’

Tested : HM4.0 MrgEncFix + ‘simplification’ + Proposed method 2

Table 4. Results for “Simplification + Proposed method 2” relative to “Simplification”.

As shown in Table 4, the proposed method 2 provides 0.0%, 0.1%, 0.1%, and 0.1% BD gains for RA-HE, RA-LC, LB-HE, and LB-LC to the simplification without any encoding/decoding time increase. Note that the proposed method 2 does not introduce additional complexity compared to the current HM.

3. Simplification + Proposed method 2 +Proposed method 1

The proposed method 1 which is proposed for the PUs of square shape is also tested with the simplification and the proposed method 2 which are for the PUs of rectangular shape. The purpose of this experiment is to show that the gain of the proposed method 1 is additive to the other proposals. As shown in Table 5, the encoding and decoding time is same as the anchor.

Anchor : HM4.0 MrgEncFix + ‘simplification’

Tested : HM4.0 MrgEncFix + ‘simplification’ + Proposed method 2 + Proposed method 1

Table 5. Results for “Simplification + Proposed method 2 +Proposed method 1” relative to “Simplification”.

# Conclusion

In this contribution, two reordering methods for the merge list are proposed 1) reordering of B1 candidate (to the first position within the list) for 2Nx2N PUs of square shape 2) using the avoided candidate which belongs to the first PU for creating the combined bi-pred. candidates when decoding the second PUs of 2NxN, 2NxnU, 2NxnD, Nx2N, nLx2N and nRx2N partitions. The simulation shows that both methods achieve average 0.1~0.2% BD rate reduction relative to the HM4.0 MrgEncFix without increasing encoding/decoding time. In addition, from the simulation results for the additional testes, it is revealed that the proposed methods provide similar coding gain when the simplification is used. Therefore it is recommended the proposed methods to be adopted in the next version of HM.

# Patent rights declaration(s)

LG Electronics  may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Required Changes in Working Draft

The WD changes according to the proposed method 1, proposed method 2, and the simplification are as follows. The changes related to the proposals are highlighted in yellow.

## Required Changes in Working Draft for proposed method 1

##### 8.4.2.1.1 Derivation process for luma motion vectors for merge mode

4. The merging candidate list, mergeCandList, is constructed as follows.

  - A1, if availableFlagA1 is equal to 1

  - B1, if availableFlagB1 is equal to 1

  - B0, if availableFlagB0 is equal to 1

  - A0, if availableFlagA0 is equal to 1

  - B2, if availableFlagB2 is equal to 1

  - Col, if availableFlagCol is equal to 1

- The following process is invoked to swap A1and B1 if PartMode of the current prediction unit is PART_2NxN and availableFlagA1, availableFlagB1, and availableFlagB2 are equal to 1.

- DistVert and DistHorz are derived as specified below.

  - If the current slice is B slice,

  - DistVert = DistVertL0 + DistVertL1, DistHorz = DistHorzL0 + DistHorzL1

  - Otherwise if the current slice is P slice,

  - DistVert = DistVertL0, DistHorz = DistHorzL0

- where,

  - DistVertLX = Abs(mvLXA1[0] – mvLXB2[0]) + Abs(mvLXA1[1] – mvLXB2[1])

  - DistHorzLX = Abs(mvLXB1[0] – mvLXB2[0]) + Abs(mvLXB1[1] – mvLXB2[1])

- If DistVert is smaller than DistHorz, the merging candidates A1and B1 in mergeCandList are swapped.

- When merging candidates have the same motion vectors and the same reference indices, the merging candidates are removed from the list except the merging candidate which has the smallest order in the mergeCandList.

…

## Required Changes in Working Draft for proposed method 2

8. 4.1.2.1 Derivation process for luma motion vectors for merge mode

Inputs of this process are

- a luma location ( xP, yP ) of the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture,

- variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH,

- a variable PartIdx specifying the index of the current prediction unit within the current coding unit.

Outputs of this process are

–    the luma motion vectors mvL0 and mvL1,

–    the reference indices refIdxL0 and refIdxL1,

–    the prediction list utilization flags predFlagL0 and predFlagL1.

The motion vectors mvL0 and mvL1, the reference indices refIdxL0 and refIdxL1, and the prediction utilization flags predFlagL0 and predFlagL1 are derived as specified by the following ordered steps:

- The derivation process for merging candidates from neighboring prediction unit partitions in subclause 8.4.2.1.2 is invoked with luma location ( xP, yP ), the width and the height of the prediction unit nPSW and nPSH and the partition index PartIdx as inputs and the output is assigned to the availability flags availableFlagN, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N and the motion vectors mvL0N and mvL1N with N being replaced by A0, A1, B0, B1 or B2.

- The derivation process of reference indices for temporal merging candidate in subclause 8.4.2.1.3 is invoked with luma location ( xP, yP ), nPSW, nPSH as the inputs and the output is directly assigned to refIdxLX.

- The derivation process for temporal luma motion vector prediction in subclause 8.4.2.1.9 is invoked with luma location ( xP, yP ), refIdxLX as the inputs and with the output being the availability flag availableFlagLXCol and the temporal motion vector mvLXCol. The variables availableFlagCol and predFlagLXCol (with X being 0 or 1, respectively) are derived as specified below.

availableFlagCol = availableFlagL0Col || availableFlagL1Col                                            (8-71)

predFlagLXCol = availableFlagLXCol                                                                               (8-72)

- The merging candidate list, mergeCandList, is constructed as follows.

  - A1, if availableFlagA1 is equal to 1

  - B1, if availableFlagB1 is equal to 1

  - B0, if availableFlagB0 is equal to 1

  - A0, if availableFlagA0 is equal to 1

  - B2, if availableFlagB2 is equal to 1

  - Col, if availableFlagCol is equal to 1

- When merging candidates have the same motion vectors and the same reference indices, the merging candidates are removed from the list except the merging candidate which has the smallest order in the mergeCandList.

- The variable numMergeCand and numOrigMergeCand are set to the number of merging candidates in the mergeCandList and the variable maxNumMergeCand is set to 5.

- When slice_type is equal to B, the following applies.

  - If PartIdx is equal to 1, the merging candidate mergeCandX is set as follows.

      - If PartMode of the current prediction unit is PART_2NxN or PART_2NxnU or PART_2NxnD, mergeCandX is set to B1.

      - Otherwise if PartMode of the current prediction unit is PART_Nx2N or PART_nLx2N or PART_nRx2N, mergeCandX is set to A1.

  - The derivation process for combined bi-predictive merging candidates specified in subclause 8.4.2.1.4 is invoked with mergeCandList, mergeCandX, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N, the motion vectors mvL0N and mvL1N of every candidate N being in mergeCandList, numMergeCand, numOrigMergeCand and maxNumMergeCand given as input and the output is assigned to mergeCandList, numMergeCand, the reference indices refIdxL0combCandk and refIdxL1combCandk, the prediction list utilization flags predFlagL0combCandk and predFlagL1combCandk and the motion vectors mvL0combCandk and mvL1combCandk of every new candidate combCandk being added in mergeCandList. The number of candidates being added numCombMergeCand is set equal to ( numMergeCand – numOrigMergeCand ). When numCombMergeCand is greater than 0, k ranges from 0 to numCombMergeCand − 1, inclusive.

…

8. 4.1.2.1 Derivation process for combined bi-predictive merging candidates

Inputs of this process are

- a merging candidate list mergeCandList,

- a merging candidate mergeCandX

- reference indices refIdxL0N and refIdxL1N of every candidate N being in mergeCandList,

- prediction list utilization flags predFlagL0N and predFlagL1N of every candidate N being in mergeCandList,

- motion vectors mvL0N and mvL1N of every candidate N being in mergeCandList,

- the number of elements numMergeCand within mergeCandList,

- the number of elements numOrigMergeCand within the mergeCandList after the spatial and temporal merge candidate derivation process,

- the maximum number of elements maxNumMergeCand within the mergeCandList.

Outputs of this process are

- the merging candidate list mergeCandList,

- the number of elements numMergeCand within mergeCandList.

- reference indices refIdxL0combCandk and refIdxL1combCandk of every new candidate combCandk being added in mergeCandList during the invokation of this process,

- prediction list utilization flags predFlagL0combCandk and predFlagL1combCandk of every new candidate combCandk being added in mergeCandList during the invokation of this process,

- motion vectors mvL0combCandk and mvL1combCandk of every new candidate combCandk being added in mergeCandList during the invokation of this process,

The function RefPicOrderCnt( refidx, LX ) specifies the value of PicOrderCnt of the picture that is the reference picture RefPicListX[ refidx ] of the current picture with X being 0 or 1. PicOrderCnt of the reference picture shall be maintained until the picture is marked as “non-existing”. The variable PicOrderCnt specifies the PicOrderCnt of the current picture.

[Ed. (BB): PicOrderCnt(.) needs to be defined. Currently the Decoding process for picture order count is not described]

When numOrigMergeCand is greater than 1 and less than maxNumMergeCand, the variable numInputMergeCand is set to numMergeCand, the variables combIdx and combCnt are set to 0, the variable combStop is set to false and the following steps are repeated until combStop is equal to true.

If PartIdx is equal to 1 and PartMode is not PART_2Nx2N or PART_NxN, the variable combIdx is set equal to 0. Otherwise combIdx is set equal to 2.

- The variables l0CandIdx and l1CandIdx are derived using combIdx as specified in Table 8-11.

- The following assignments are made with l0Cand being the candidate at position l0CandIdx and l1Cand being the candidate at position l1CandIdx in the merging candidate list mergeCandList ( l0Cand = mergeCandList[ l0CandIdx ] , l1Cand = mergeCandList[ l1CandIdx ] ). If lXCandIdx is ‘NA’ in Table 8-11 with X being 0 or 1, mergeCandX is assigned to lXCand.

- When all of the following conditions are true,

    - predFlagL0l0Cand = = 1

    - predFlagL1l1Cand = = 1

    - RefPicOrderCnt( refIdxL0l0Cand, L0 ) != RefPicOrderCnt( refIdxL1l1Cand, L1 ) | | mvL0l0Cand != mvL1l1Cand

the following applies.

…

    - When no candidate in mergeCandList has the same list0 motion vector and reference index as l0Cand and the same list1 motion vector and reference index as l1Cand, the candidate combCandk with k equal to ( numMergeCand − numInputMergeCand ) is added at the end of mergeCandList ( mergeCandList[ numMergeCand ] = combCandk ) and the reference indices, the prediction list utilization flags and the motion vectors of combCandk are dervied as follows and numMergeCand is incremented by 1.

refIdxL0combCandk = refIdxL0l0Cand                                                                    (8-92)

refIdxL1combCandk = refIdxL1l1Cand                                                                    (8-93)

predFlagL0combCandk = 1                                                                                       (8-94)

predFlagL1combCandk = 1                                                                                       (8-95)

mvL0combCandk[ 0 ] = mvL0l0Cand[ 0 ]                                                               (8-96)

mvL0combCandk[ 1 ] = mvL0l0Cand[ 1 ]                                                               (8-97)

mvL1combCandk[ 0 ] = mvL1l1Cand[ 0 ]                                                               (8-98)

mvL1combCandk[ 1 ] = mvL1l1Cand[ 1 ]                                                               (8-99)

numMergeCand = numMergeCand + 1                                                                    (8-100)

    - The variable combCnt is incremented by 1.

- The variable combIdx is incremented by 1.

- When combIdx is equal to ( numOrigMergeCand * ( numOrigMergeCand − 1 ) ) or numMergeCand is equal to maxNumMergeCand or combCnt is equal to 5, combStop is set to true.

Table 8-11 – Specification of l0CandIdx and l1CandIdx

| combIdx | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| l0CandIdx | NA | 0 | 0 | 1 | 0 | 2 | 1 | 2 | 0 | 3 | 1 | 3 | 2 | 3 |
| l1CandIdx | 0 | NA | 1 | 0 | 2 | 0 | 2 | 1 | 3 | 0 | 3 | 1 | 3 | 2 |

| combIdx | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| l0CandIdx | 0 | 1 | 0 | 2 | 1 | 2 | 0 | 3 | 1 | 3 | 2 | 3 |
| l1CandIdx | 1 | 0 | 2 | 0 | 2 | 1 | 3 | 0 | 3 | 1 | 3 | 2 |

## Required Changes in Working Draft for the simplification

8. 4.1.2.2 Derivation process for spatial merging candidates

Inputs to this process are

- a luma location ( xP, yP ) specifying the top-left luma sample of the current prediction unit relative to the top-left sample of the current picture,

- variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH,

- a variable PartIdx specifying the index of the current prediction unit within the current coding unit.

Outputs of this process are (with N being replaced by A0, A1, B0, B1 or B2 and with X being replaced by 0 or 1)

- the availability flags availableFlagN of the neighbouring prediction units,

- the reference indices refIdxLXN of the neighbouring prediction units,

- the prediction list utilization flags predFlagLXN of the neighbouring prediction units,

- the motion vectors mvLXN of the neighbouring prediction units.

For the derivation of availableFlagN, with N being A0, A1, B0, B1 or B2 and ( xN, yN ) being ( xP – 1,  yP + nPSH ), ( xP − 1,  yP + nPSH − 1 ), ( xP + nPSW,  yP – 1 ), ( xP + nPSW − 1,  yP – 1 ) or ( xP – 1,  yP – 1 ), the following applies.

–    If one of the following conditions is true, the availableFlagN is set equal to 0, both components mvLXN are set equal to 0, refIdxLXN and predFlagLX[ xN, yN ] of the prediction unit covering luma location ( xN, yN ) are assigned respectively to mvLXN, refIdxLXN and predFlagLXN.

  - N is equal to B2 and availableFlagA0 + availableFlagA1 + availableFlagB0 + availableFlagB1 is equal to 4.

  - The prediction unit covering luma location ( xN, yN ) is not available or PredMode is MODE_INTRA.

  - PartMode of the current prediction unit is PART_2NxN or PART_2NxnU or PART_2NxnD and PartIdx is equal to 1 and N is equal to B1

  - PartMode of the current prediction unit is PART_Nx2N or PART_nLx2N or PART_nRx2N and PartIdx is equal to 1 and N is equal to A1

- PartMode of the current prediction unit is PART_2NxN or PART_2NxnU or PART_2NxnD and PartIdx is equal to 1 and the prediction units covering luma location ( xP,  yP – 1 ) ( PartIdx = 0 ) and luma location ( xN, yN ) (Cand. N) have identical motion parameters:

  - mvLX[ xP,  yP – 1 ]  = =  mvLX[ xN, yN ]

  - refIdxLX[ xP,  yP – 1 ]  = =  refIdxLX[ xN, yN ]

  - predFlagLX[ xP,  yP – 1 ]  = =  predFlagLX[ xN, yN ]

- PartMode of the current prediction unit is PART_Nx2N or PART_nLx2N or PART_nRx2N and PartIdx is equal to 1 and the prediction units covering luma location ( xP – 1,  yP ) ( PartIdx = 0 ) and luma location ( xN, yN ) (Cand. N) have identical motion parameters:

  - mvLX[ xP – 1,  yP ]  = =  mvLX[ xN, yN ]

  - refIdxLX[ xP – 1,  yP ]  = =  refIdxLX[ xN, yN ]

  - predFlagLX[ xP – 1,  yP ]  = =  predFlagLX[ xN, yN ]

- PartMode of the current prediction unit is PART_NxN and PartIdx is equal to 3 and the prediction units covering luma location ( xP – 1,  yP ) ( PartIdx = 2 ) and luma location ( xP – 1,  yP – 1 ) ( PartIdx = 0 )  have identical motion parameters:

  - mvLX[ xP – 1,  yP ]  = =  mvLX[ xP – 1,  yP – 1 ]

  - refIdxLX[ xP – 1,  yP ]  = =  refIdxLX[ xP – 1,  yP – 1 ]

  - predFlagLX[ xP – 1,  yP ]  = =  predFlagLX[ xP – 1,  yP – 1 ]

and the prediction units covering luma location ( xP,  yP – 1 ) ( PartIdx = 1 ) and luma location ( xN, yN ) (Cand. N) have identical motion parameters:

  - mvLX[ xP,  yP – 1 ]  = =  mvLX[ xN, yN ]

  - refIdxLX[ xP,  yP – 1 ]  = =  refIdxLX[ xN, yN ]

  - predFlagLX[ xP,  yP – 1 ]  = =  predFlagLX[ xN, yN ]

- PartMode of the current prediction unit is PART_NxN and PartIdx is equal to 3 and the prediction units covering luma location ( xP,  yP – 1 ) ( PartIdx = 1 ) and luma location ( xP – 1,  yP – 1 ) ( PartIdx = 0 ) have identical motion parameters:

  - mvLX[ xP,  yP – 1 ]  = =  mvLX[ xP – 1,  yP – 1 ]

  - refIdxLX[ xP,  yP – 1 ]  = =  refIdxLX[ xP – 1,  yP – 1 ]

  - predFlagLX[ xP,  yP – 1 ]  = =  predFlagLX[ xP – 1,  yP – 1 ]

and the prediction units covering luma location ( xP – 1,  yP ) ( PartIdx = 2 ) and luma location ( xN, yN ) (Cand. N) have identical motion parameters:

  - mvLX[ xP – 1,  yP ]  = =  mvLX[ xN, yN ]

  - refIdxLX[ xP – 1,  yP ]  = =  refIdxLX[ xN, yN ]

  - predFlagLX[ xP – 1,  yP ]  = =  predFlagLX[ xN, yN ]

–    Otherwise, availableFlagN is set equal to 1 and the variables mvLX[ xN, yN ], refIdxLX[ xN, yN ] and predFlagLX[ xN, yN ] of the prediction unit covering luma location ( xN, yN ) are assigned respectively to mvLXN, refIdxLXN and predFlagLXN.