| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 19-30 November, 2011 | Document: JCTVC-G396 M21959 |
| --- | --- |

| Title: | Non-CE9: swapping of merge candidate |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chul Keun Kim Yongjoon Jeon Byeongmoon Jeon #221 Yangjae-dong, Seoucho-gu, Seoul 137-130, Korea | Email: | chulkeun.kim@lge.com |
| Source: | LG |  |  |

_____________________________

# Abstract

The current merge/skip list order is {A1, B1, B0, A0, B2}. This contribution proposes to swap the two MVP candidates A1 and B1 if the neighbouring MVs are vertically correlated rather than horizontally correlated. The proposed swapping is applied only when PartMode is PART_2Nx2N and the three neighbouring MVP candidates A1, B1, and B2 are available. Simulation results show that the proposed swapping method achieves 0.1%, 0.0%, 0.2%, and 0.1% BD rate reduction for RA-HE, RA-LC, LB-HE, and LB-LC configurations respectively while encoding time remains same as the anchor (HM4.0 with bug-fix).

# Proposed method

In HM 4.0, spatial merge candidate order is as follows.

- A1, B1, B0, A0, B2

We propose to swap A1 and B1 if neighboring MVs are vertically correlated rather than horizontally correlated as follows.

- B1, A1, B0, A0, B2

The proposed swapping can be applied only if all of the following conditions are satisfied.

- PartMode == PART_2Nx2N

- A1, B1, B2 are available and not intra-coded.

The following pseudo code represents the proposed method.

if ( DV < DH )

- Swap A1 and B1

- Fig. 1 The candidates used for the proposed swapping

The variables DV and DH represent the distances between the distance between mvLXA1 and mvLXB2 and the distance between mvLXB1 and mvLXB2 respectively.

Depending on slice type, the variables DV and DH are derived as follows.

If B slice,

- DV = DistL0V + DistL1V

- DH = DistL0H + DistL1H

If P slice,

- DV = DistL0V

- DH = DistL0H

,where

- DistLXV = Abs(mvLXA1[0] – mvLXB2[0]) + Abs(mvLXA1[1] – mvLXB2[1])

- DistLXH = Abs(mvLXB1[0] – mvLXB2[0]) + Abs(mvLXB1[1] – mvLXB2[1])

# Simulation result

In the simulations, the anchor is HM4.0 with bug-fix (HM4.0 MrgEncFix) which is released by Samsung on CE13 reflector. The proposed method is implemented in this software. It is revealed from the results of Table 1 that the proposed swapping achieves 0.1%, 0.0%, 0.2%, and 0.1% BD rate reduction for RA-HE, RA-LC, LB-HE, and LB-LC configurations respectively while encoding time remains same as the anchor (HM4.0 with bug-fix).

Table 1. HM4.0 MrgEncFix vs Proposed method

| Reference: HM 4.0 MRG_ENC_FIX |  |  |  | Tested: Proposed method |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | -0.2% | -0.2% | 0.0% | 0.1% | -0.1% |
| Class B | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% | -0.1% |
| Class D | -0.1% | -0.1% | 0.0% | 0.0% | -0.2% | -0.1% |
| Class E |  |  |  |  |  |  |
| Overall | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% | -0.1% |
|  | -0.1% | -0.1% | 0.0% | 0.0% | -0.1% | -0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 98% |  |  | 101% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.1% | 0.1% | -0.1% | 0.0% | -0.1% | -0.4% |
| Class C | -0.1% | -0.2% | -0.2% | 0.0% | 0.2% | 0.2% |
| Class D | -0.1% | 0.2% | -0.2% | -0.1% | 0.0% | 0.1% |
| Class E | -0.4% | -0.1% | -1.0% | -0.2% | 0.4% | -0.2% |
| Overall | -0.2% | 0.0% | -0.3% | -0.1% | 0.1% | -0.1% |
|  | -0.2% | 0.0% | -0.3% | -0.1% | 0.0% | -0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |

# Conclusion

In this contribution, a swapping of A1 and B1 merge candidates is proposed. The simulation results revealed that the proposed swapping improves coding efficiency without any increase of encoding time. Therefore it is recommended to adopt the proposed swapping method in the next version of HM.

# Patent rights declaration(s)

LG Electronics  may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Required Changes in Working Draft

The required changes are marked in yellow.

##### 8.4.2.1.1 Derivation process for luma motion vectors for merge mode

…

- The merging candidate list, mergeCandList, is constructed as follows.

  - A1, if availableFlagA1 is equal to 1

  - B1, if availableFlagB1 is equal to 1

  - B0, if availableFlagB0 is equal to 1

  - A0, if availableFlagA0 is equal to 1

  - B2, if availableFlagB2 is equal to 1

  - Col, if availableFlagCol is equal to 1

- The following process is invoked to swap A1and B1 if PartMode of the current prediction unit is PART_2NxN and availableFlagA1, availableFlagB1, and availableFlagB2 are equal to 1.

- DistVert and DistHorz are derived as specified below.

  - If the current slice is B slice,

  - DistVert = DistVertL0 + DistVertL1, DistHorz = DistHorzL0 + DistHorzL1

  - Otherwise if the current slice is P slice,

  - DistVert = DistVertL0, DistHorz = DistHorzL0

- where,

  - DistVertLX = Abs(mvLXA1[0] – mvLXB2[0]) + Abs(mvLXA1[1] – mvLXB2[1])

  - DistHorzLX = Abs(mvLXB1[0] – mvLXB2[0]) + Abs(mvLXB1[1] – mvLXB2[1])

- If DistVert is smaller than DistHorz, the merging candidates A1and B1 in mergeCandList are swapped.

- When merging candidates have the same motion vectors and the same reference indices, the merging candidates are removed from the list except the merging candidate which has the smallest order in the mergeCandList.

…