| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G397_r2 |
| --- | --- |

| Title: | Non-CE9/Non-CE13: Simplification of adding new merge candidates |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Bin Li Jizheng Xu Houqiang Li | Tel: Email: |  |
| Source: | University of Science and Technology of China Microsoft Corp. |  |  |

_____________________________

# Abstract

The parsing robustness issue was solved in JCTVC-F470 by a method similar to coding full index of merge candidate. For some other syntax elements, the parsing dependency of the surrounding information is also removed. One most important part of JCTVC-F470 is to compensate the loss introduced by coding full merge index. Some new merge candidates are also added to the set of merge candidates, which was adopted in the latest HM design. For each new added merge candidate, the duplication checking is also performed. This contribution evaluates the benefits of each individual step and proposes to remove some steps to simplify the design.

# Introduction

The parsing robustness issue was solved in JCTVC-F470 [1] by a method similar to coding full index of merge candidate. For some other syntax elements, the parsing dependency of the surrounding information is also removed. One most important part of JCTVC-F470 is to compensate the loss introduced by coding full merge index. Some new merge candidates are also added to the set of merge candidates. For each new added merge candidate, the duplication checking is also performed. This contribution presents the benefits of each individual step.

# Test Results

JCTVC-F470 added new merge candidates by three steps,

Step 1, add combined merge candidates;

Step 2, add non-scaled merge candidates;

Step 3, add zero merge candidates.

For each new added merge candidate, the duplication checking is also performed. The following experiments show the results of disabling each step.

The summary results are shown in Table 1.

Table 1 disabling each step in adding new merge candidates (Y BD-Rate)

|  | RA_HE | RA_LC | LB_HE | LB_LC | LP_HE | LP_LC |
| --- | --- | --- | --- | --- | --- | --- |
| dis all | 0.6% | 0.9% | 1.2% | 2.2% | 0.3% | 0.5% |
| dis step 1 | 0.3% | 0.5% | 1.0% | 1.8% | 0.0% | 0.0% |
| dis step 2 | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| dis step 3 | 0.1% | 0.1% | 0.3% | 0.3% | 0.3% | 0.5% |
| no duplication checking all | 0.1% | 0.1% | 0.1% | 0.2% | 0.1% | 0.1% |
| no duplication checking 1 | 0.1% | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% |
| no duplication checking 2 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| no duplication checking 3 | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% |

It can be seen that, removing the duplication checking for step 2 has no noticeable impact on the performance. Even removing all the duplication checking processes only brings about 0.1% loss. The impact of removing the entire step 2 is also negligible.

# Text Modifications

1.	Text for disabling step 2.

8.4.2.1.1

7.

…

The derivation process for non-scaled bi-predictive merging candidates specified in subclause Error: Reference source not found is invoked with mergeCandList, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N, the motion vectors mvL0N and mvL1N of every candidate N being in mergeCandList, numMergeCand, numOrigMergeCand and maxNumMergeCand given as input and the output is assigned to mergeCandList, numMergeCand, the reference indices refIdxL0nscaleCandl and refIdxL1nscaleCandl, the prediction list utilization flags predFlagL0nscaleCandl and predFlagL1nscaleCandl, the motion vectors mvL0nscaleCandl and mvL1nscaleCandl of every new candidate nscaleCandl being added in mergeCandList. The number of candidates being added numNscaleMergeCand is set equal to ( numMergeCand – numOrigMergeCand – numCombMergeCand ). When numNscaleMergeCand is greater than 0, l ranges from 0 to numNscaleMergeCand – 1, inclusive.

…

And deleting the entire subclause 8.4.2.1.5

2.	Text for disabling duplication checking.

8.4.2.1.4

When no candidate in mergeCandList has the same list0 motion vector and reference index as l0Cand and the same list1 motion vector and reference index as l1Cand, the candidate combCandk with k equal to ( numMergeCand − numInputMergeCand ) is added at the end of mergeCandList ( mergeCandList[ numMergeCand ] = combCandk ) and the reference indices, the prediction list utilization flags and the motion vectors of combCandk are dervied as follows and numMergeCand is incremented by 1.

8.4.2.1.5

When no candidate in mergeCandList has a list0 motion vector equal to listX motion vector of origCand, a list1 motion vector equal to the listX motion vector of origCand multiplied by 1 and list0 and list1 reference indices equal to the listX reference index of origCand, the candidate nscaleCandl with l equal to ( numMergeCand − numInputMergeCand )is added at the end of mergeCandList ( mergeCandList[ numMergeCand ] = nscaleCandl ) and the reference indices, the prediction list utilization flags and the motion vectors of nscaleCandl are dervied as follows and numMergeCand is incremented by 1.

8.4.2.1.6

no candidate in mergeCandList has a list0 motion vector equal to (0,0) and reference index equal to zeroIdx or (zeroCnt is equal to 2 and zeroIdx is equal to 0),

no candidate in mergeCandList has a list0 motion vector equal to (0,0), a list0 reference index equal to zeroIdx, a list1 motion vector equal to (0,0) and a list1 reference index equal to zeroIdx or (zeroCnt is equal to 2 and zeroIdx is equal to 0)

3.	Text for only disabling duplication checking in step 2.

8.4.2.1.5

When no candidate in mergeCandList has a list0 motion vector equal to listX motion vector of origCand, a list1 motion vector equal to the listX motion vector of origCand multiplied by 1 and list0 and list1 reference indices equal to the listX reference index of origCand, the candidate nscaleCandl with l equal to ( numMergeCand − numInputMergeCand )is added at the end of mergeCandList ( mergeCandList[ numMergeCand ] = nscaleCandl ) and the reference indices, the prediction list utilization flags and the motion vectors of nscaleCandl are dervied as follows and numMergeCand is incremented by 1.

# References

- JCT-VC, “Parsing Robustness for Merge/AMVP”, JCTVC-F470, JCT-VC Meeting, Torino, IT, July, 2011.

# Patent rights declaration(s)

University of Science and Technology of China and Microsoft Corp. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).