| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G398_r1 |
| --- | --- |

| Title: | High-level Syntax: Marking process for non-TMVP pictures |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Bin Li Jizheng Xu Houqiang Li | Tel: Email: |  |
| Source: | University of Science and Technology of China Microsoft Corp. |  |  |

_____________________________

# Abstract

In the current HM design, MVP and merge candidates may be incorrect when packet loss exists. The error may influence motion vectors of all the following pictures. Disabling some pictures’ TMVP (non-TMVP pictures) may stop the error propagation for motion vectors. However, it cannot be always guaranteed that the error can be stopped with non-TMVP pictures. This contribution presents a mechanism to make sure that the error can be stopped after non-TMVP pictures.

# Introduction

The parsing robustness issue has been solved in the latest version of HM, HM-4.0. But only the correct syntax elements are guaranteed, the MVP and merge candidates may also be incorrect when packet loss exists. It has agreed that adding one PPS flag to disable TMVP. One aim of disabling TMVP is to guarantee the motion information of the non-TMVP picture itself is correct. For the current default referencing structure, the motion vector in the pictures after that non-TMVP picture in output order will also be correct if no more packet loss exists. But unfortunately it may not work for structures that are different from the common test configurations.

This contribution provides a mechanism to guarantee correct motion vector after the non-TMVP picture for any referencing structure by adding a marking process before decoding the first picture with an output order greater than the current non-TMVP picture.

# Solution

The PPS level flag to disable TMVP can guarantee the correct motion vector for the pictures with output order greater than that non-TMVP picture for the default HM referencing structure. But it is not always true for any flexible referencing structure. For example, picture n is non-TMVP picture with correct reconstructed motion vector. For the first picture with greater output order, picture n+x (x>0), if picture n is the first reference picture in both lists (list 0 and list 1), the motion vector in that picture will be correct too. But we have multiple reference frames and flexible referencing structures; it cannot always be guaranteed that picture n is the first reference picture in both lists (list 0 and list 1). So the motion vector in the pictures with greater output order than picture may also be incorrect. The situation is similar to the other pictures with output order greater than picture n.

We propose to mark all the unreliable motion vectors not used for motion vector predictor before decoding the first picture following the non-TMVP picture with an output order greater than that non-TMVP picture. The motion vector after that non-TMVP picture in output order will be correct. An alternative method is before decoding the non-TMVP picture, mark all the pictures in DPB “unused for temporal motion vector precition”, which is more simpler.

The RD performance of non-TMVP and the error concealment results with correct motion information have already been shown in [1][2], the text modification will be shown in the next part. It should be noticed that for the default HM coding structure, additional marking process will not bring any performance difference compared to the case of simply disabling the TMVP in that non-TMVP picture. The RD performance of adding the marking process is the same with that of just inserting some non-TMVP pictures. The text and decoding process modifications are shown in the next part.

We also propose to move that flag (enable_temporal_mvp_flag) to APS, because for the purpose of truncating incorrect motion vector and not bringing too much performance loss, some pictures may not use TMVP while the others pictures may use.

An alternative method is adding a new NAL unit type, named coded slice of non-TMVP picture. The decoding process and TMVP usage is similar to the case above.

The benefit of the new NAL unit type is for the purpose of random access for motion vectors, which benefits applications that only need to access motion information of a bitstream, e.g., activity detection, shot detection. If new NAL unit type is added, the non-TMVP flag is not necessary anymore.

# Text Modification

The text modification is based on JCTVC-F803_d5.

It has been adopted that to “put a flag in PPS to disable temporal MV predictor”. These flags can also be used to indicate non-TMVP picture. But those flags are missing currently.

3

non-TMVP picture: A coded picture with enable_temporal_mvp_flag equals to 0. TMVP is not used in non-TMVP pictures. Non-TMVP picture with temporal_id equals to 0 also indicates the marking process will occur before decoding the first picture with an output order greater than the current non-TMVP picture.

7.3.2.2

| pic_init_qp_minus26 /* relative to 26 */ | se(v) |
| --- | --- |
| constrained_intra_pred_flag | u(1) |
| enable_temporal_mvp_flag | u(1) |
| slice_granularity | u(2) |

7.4.2.2

enable_temporal_mvp_flag specifies whether temporal motion vector predictor can be used or not. If enable_temporal_mvp_flag is equal to 1, temporal motion vector predictors are used; otherwise (enable_temporal_mvp_flag is equal to 0), the temporal motion vector predictors are not used. enable_temporal_mvp_flag equals to 0 together with temporal_id equals to 0 also indicates the marking process will occur before decoding the first picture with an output order greater than the current non-TMVP picture.

8.2.4 Decoded temporal information marking process

Solution 1

This process is invoked at the beginning of the decoding process for each slice.

If the current POC is greater than the last_non_TMVP_POC (last_non_TMVP_POC is initialized with the maximum POC number before decoding the whole sequence) and the temporal_id of the current picture is 0, all the pictures in DPB except the picture with the POC equals to last_non_TMVP_POC are marked as “unused for temporal motion vector prediction”. last_non_TMVP_POC is set to be equal to maximum POC number after the marking process.

If the enable_temporal_mvp_flag of the current picture is 0 and the temporal_id of the current picture is 0, last_non_TMVP_POC is set to equal to the current POC.

An alternative solution is that the marking process takes place immediately before decoding a picture.

Solution 2

This process is invoked after decoding of each picture.

If the enable_temporal_mvp_flag of the current picture is 0 and the temporal_id of the current picture is 0, all the pictures in DPB is marked as “unused for temporal motion vector prediction”.

This solution is a little simpler than the previous one but will bring a little performance loss (for RA cases) than that one.

Solution 1 and Solution 2 provide identical coding results when non-TMVP picture is inserted into the bit-stream every 8 pictures in HM-4.0.

A bug report, it seems that the syntax element of collocated_from_l0_flag in HM-4.0 has just the opposite semantics.

WRITE_FLAG( pcSlice->getColDir(), "collocated_from_l0_flag" );

TComPic *pColPic = getSlice()->getRefPic( RefPicList(getSlice()->isInterB() ? getSlice()->getColDir() : 0), 0);

When pcSlice->getColDir() == 1, collocated block is from list 1 instead of list 0.

An simple fix is WRITE_FLAG( pcSlice->getColDir() ? 0 : 1, "collocated_from_l0_flag" ); at both encoder and decoder side.

8.4.2.1.9

…

The variables mvLXCol and availableFlagLXCol are derived as follows.

- If colPu is coded in an intra prediction mode or colPu is unavailable or the picture of colPu is marked as “unused for temporal motion vector prediction” or the current picture is non-TMVP picture, both components of mvLXCol are set equal to 0 and availableFlagLXCol is set equal to 0.

…

# References

- JCT-VC, “An investigation on robust parsing”, JCTVC-E148, JCT-VC Meeting, Geneva, CH, March, 2011.

- JCT-VC, “High-level Syntax: Temporal Information Decoding Refresh”, JCTVC-F201, JCT-VC Meeting, Torino, IT, July, 2011.

# Patent rights declaration(s)

University of Science and Technology of China and Microsoft Corp. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).