| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G401 |
| --- | --- |

| Title: | RDO with weighted distortion in HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Bin Li Gary J. Sullivan Jizheng Xu | Tel: Email: | +1 425 882 8080 garysull@microsoft.com |
| Source: | University of Science and Technology of China Microsoft Corp. |  |  |

_____________________________

# Abstract

For analysis of coding performance, situations are sometimes encountered where the rate-distortion behaviour is substantially different between the luma and chroma components. One approach that has recently been suggested for overall compression effect evaluation is to use a simple combined PSNR metric – e.g., PSNRAVGYUV = (6*PSNRAVGY+PSNRAVGU+PSNRAVGV)/8, as suggested at the Daegu meeting (and probably also previously) and as used in JCTVC-F386. This contribution suggests to consider this approach to be a useful step forward. However, it also advocates caution when considering experiments for which the chroma BD results are changing much more significantly and in the opposite direction with respect to the luma results – especially when chroma improves and luma degrades. Indeed, it is reported that bit allocation effects alone can cause apparent coding gain. To illustrate this issue, the contribution reports the results of some preliminary experiments in which the use of a simple weighting in the R-D mode decision criterion can reportedly result in significant apparent combined BD coding gain (0.8% in the HE LD case) when the luma BD fidelity is degrading (by 1.4%) and the chroma BD fidelity is improving (by about 11%). Furthermore, the chroma QP can be changed by the syntax element of chroma_qp_index_offset (although this syntax element is reported to be missing currently from the draft and HM software). According to the reported experimental results, if the chroma QP value is decreased by 3, up to 5.5% combined BD-Rate gain can reportedly be obtained for HE LD case. If the chroma QP is further decreased, more measured gain in terms of the combined BD-Rate measure may also be obtained.

# Experiment description

The RDO decision method in HM-3.0 is basically characterized by the following decision method

min(SSEY+SSEU+SSEV)+λR

In HM-4.0, after adopting the modification proposed in JCTVC-F386, the RDO is

min(SSEY+ω(SSEU+SSEV))+λR

where ω is calculated according to the QP difference between luma and chroma, which generally results in a weighting parameter ω that is greater than or equal to 1.

In our first set of reported experiment results, the weight of chroma distortion is further enlarged, and our optimization target is

min(SSEY+1.5∗ω(SSEU+SSEV))∗67+λR

where 67 is used to normalize the total distortion considering 4:2:0 sample format.

It should be noticed that the weight was changed only for the parts of the encoding process where the distortion of different color components is considered together (such as mode decision). When making decision for chroma only, such as RDOQ for chroma and ALF, SAO decision for chroma, the same method as in JCTVC-F386 was used.

# Experimental Results

The summary results of the introduced method are shown in Table1.

The average PSNR was calculated by PSNRAVGYUV = (6*PSNRAVGY+PSNRAVGU+PSNRAVGV)/8. Bjøntegaard delta averages are reported in the tables below, with the column identified as “Average” using this combined PSNR equation.

Table 1 Summary results of weighted distortion calculation

|  | All Intra HE |  |  |  | All Intra LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Average | Y | U | V | Average |
| Class A | 0.2% | -2.4% | -2.4% | -0.1% | 0.3% | -2.1% | -2.3% | 0.0% |
| Class B | 0.3% | -2.6% | -2.7% | -0.2% | 0.2% | -2.2% | -2.1% | -0.2% |
| Class C | 0.3% | -1.9% | -2.0% | -0.1% | 0.2% | -1.7% | -1.7% | -0.1% |
| Class D | 0.2% | -1.6% | -1.7% | -0.1% | 0.2% | -1.3% | -1.4% | -0.1% |
| Class E | 0.3% | -2.7% | -2.7% | -0.3% | 0.2% | -2.0% | -2.0% | -0.2% |
| Overall | 0.3% | -2.3% | -2.3% | -0.1% | 0.2% | -1.9% | -1.9% | -0.1% |
| EncT | 100% |  |  |  | 99% |  |  |  |
| DecT | 99% |  |  |  | 99% |  |  |  |

|  | Random Access HE |  |  |  | Random Access LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Average | Y | U | V | Average |
| Class A | 0.9% | -9.4% | -9.3% | -0.4% | 1.3% | -9.4% | -9.1% | 0.1% |
| Class B | 1.0% | -8.7% | -8.3% | -0.7% | 1.0% | -7.8% | -7.4% | -0.5% |
| Class C | 0.9% | -6.9% | -6.9% | -0.5% | 0.9% | -6.1% | -6.2% | -0.3% |
| Class D | 0.8% | -6.4% | -7.1% | -0.5% | 0.8% | -6.4% | -6.5% | -0.5% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | 0.9% | -7.9% | -7.9% | -0.6% | 1.0% | -7.4% | -7.3% | -0.3% |
| EncT | 101% |  |  |  | 101% |  |  |  |
| DecT | 100% |  |  |  | 99% |  |  |  |

|  | Low delay B HE |  |  |  | Low delay B LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Average | Y | U | V | Average |
| Class A |  |  |  |  |  |  |  |  |
| Class B | 1.6% | -12.5% | -12.2% | -0.8% | 1.7% | -10.7% | -10.1% | -0.3% |
| Class C | 1.6% | -9.6% | -9.9% | -0.5% | 1.6% | -8.2% | -8.8% | -0.2% |
| Class D | 1.4% | -9.7% | -10.2% | -0.5% | 1.3% | -8.5% | -8.9% | -0.4% |
| Class E | 0.5% | -11.7% | -11.6% | -1.6% | 0.7% | -11.3% | -10.7% | -1.2% |
| Overall | 1.4% | -10.9% | -11.0% | -0.8% | 1.4% | -9.6% | -9.6% | -0.5% |
| EncT | 100% |  |  |  | 100% |  |  |  |
| DecT | 99% |  |  |  | 98% |  |  |  |

Furthermore, the case of decreasing the chroma QP was also tested. Only the chroma QP was changed, and the RDO is performed with the method proposed in JCTVC-F386. The chroma QP was modified by

QP_chroma_new = Clip3( 0, QP_chroma_old − X, 51 ). The cases of X = 1, 2, 3 were tested and the summary results are shown below.

Table 2 Summary results of changing chroma QP

AI_HE

|  | Chroma QP − 1 |  |  |  | Chroma QP − 2 |  |  |  | Chroma QP − 3 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Avg. | Y | U | V | Avg. | Y | U | V | Avg. |
| A | 1.6% | -11.2% | -11.3% | -0.4% | 3.5% | -20.7% | -20.9% | -0.5% | 5.8% | -29.9% | -30.1% | -0.6% |
| B | 2.0% | -13.2% | -13.7% | -0.9% | 4.4% | -24.4% | -25.1% | -1.4% | 7.4% | -34.7% | -35.2% | -1.7% |
| C | 1.9% | -11.2% | -11.2% | -0.6% | 4.1% | -20.7% | -20.7% | -1.0% | 6.7% | -30.0% | -29.9% | -1.3% |
| D | 1.7% | -10.9% | -10.9% | -0.5% | 3.7% | -20.2% | -20.3% | -0.9% | 6.0% | -29.2% | -29.4% | -1.2% |
| E | 1.1% | -12.3% | -12.2% | -1.4% | 2.3% | -22.6% | -22.5% | -2.5% | 3.8% | -31.5% | -31.4% | -3.4% |
| All | 1.7% | -11.8% | -11.9% | -0.7% | 3.7% | -21.8% | -22.0% | -1.2% | 6.1% | -31.2% | -31.4% | -1.6% |
| EncT | 100% |  |  |  | 101% |  |  |  | 102% |  |  |  |
| DecT | 100% |  |  |  | 101% |  |  |  | 103% |  |  |  |

AI_LC

|  | Chroma QP − 1 |  |  |  | Chroma QP − 2 |  |  |  | Chroma QP − 3 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Avg. | Y | U | V | Avg. | Y | U | V | Avg. |
| A | 1.6% | -10.8% | -11.0% | -0.2% | 3.4% | -20.2% | -20.5% | -0.1% | 5.7% | -29.4% | -29.7% | 0.0% |
| B | 2.0% | -12.9% | -13.3% | -0.6% | 4.3% | -23.7% | -24.3% | -1.0% | 7.2% | -33.7% | -34.3% | -1.1% |
| C | 1.9% | -11.0% | -11.1% | -0.5% | 4.1% | -20.5% | -20.5% | -0.7% | 6.6% | -29.7% | -29.6% | -1.0% |
| D | 1.8% | -10.9% | -11.0% | -0.4% | 3.7% | -20.4% | -20.6% | -0.7% | 6.0% | -29.6% | -29.7% | -1.0% |
| E | 1.0% | -11.9% | -12.0% | -1.1% | 2.3% | -22.3% | -22.0% | -1.9% | 3.7% | -30.7% | -30.7% | -2.5% |
| All | 1.7% | -11.6% | -11.7% | -0.5% | 3.7% | -21.5% | -21.7% | -0.8% | 6.0% | -30.8% | -31.0% | -1.1% |
| EncT | 101% |  |  |  | 101% |  |  |  | 102% |  |  |  |
| DecT | 100% |  |  |  | 102% |  |  |  | 103% |  |  |  |

RA_HE

|  | Chroma QP − 1 |  |  |  | Chroma QP − 2 |  |  |  | Chroma QP − 3 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Avg. | Y | U | V | Avg. | Y | U | V | Avg. |
| A | 1.2% | -18.8% | -19.0% | -1.4% | 2.7% | -33.3% | -33.2% | -2.5% | 4.7% | -45.0% | -45.1% | -3.3% |
| B | 1.6% | -18.1% | -18.2% | -2.1% | 3.7% | -32.0% | -32.4% | -3.8% | 6.2% | -43.4% | -44.0% | -5.1% |
| C | 1.7% | -14.4% | -14.3% | -1.4% | 3.6% | -26.3% | -26.1% | -2.6% | 5.8% | -36.7% | -36.5% | -3.6% |
| D | 1.4% | -14.8% | -15.0% | -1.6% | 3.0% | -26.8% | -26.9% | -3.0% | 4.9% | -37.5% | -37.4% | -4.2% |
| E |  |  |  |  |  |  |  |  |  |  |  |  |
| All | 1.5% | -16.6% | -16.7% | -1.7% | 3.3% | -29.8% | -29.8% | -3.0% | 5.4% | -40.8% | -41.0% | -4.1% |
| EncT | 100% |  |  |  | 101% |  |  |  | 101% |  |  |  |
| DecT | 101% |  |  |  | 102% |  |  |  | 102% |  |  |  |

RA_LC

|  | Chroma QP − 1 |  |  |  | Chroma QP − 2 |  |  |  | Chroma QP − 3 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Avg. | Y | U | V | Avg. | Y | U | V | Avg. |
| A | 1.2% | -17.7% | -18.0% | -1.1% | 2.7% | -31.8% | -32.4% | -2.0% | 4.7% | -43.6% | -44.5% | -2.5% |
| B | 1.6% | -16.9% | -17.0% | -1.7% | 3.8% | -30.6% | -30.9% | -3.0% | 6.3% | -41.6% | -42.3% | -4.0% |
| C | 1.6% | -13.9% | -13.8% | -1.2% | 3.6% | -25.7% | -25.5% | -2.2% | 5.9% | -35.8% | -35.7% | -2.9% |
| D | 1.4% | -14.5% | -14.2% | -1.4% | 3.0% | -27.0% | -26.7% | -2.7% | 5.0% | -37.4% | -37.0% | -3.7% |
| E |  |  |  |  |  |  |  |  |  |  |  |  |
| All | 1.5% | -15.8% | -15.9% | -1.4% | 3.3% | -28.9% | -29.0% | -2.5% | 5.5% | -39.7% | -40.0% | -3.3% |
| EncT | 100% |  |  |  | 101% |  |  |  | 101% |  |  |  |
| DecT | 101% |  |  |  | 102% |  |  |  | 103% |  |  |  |

LB_HE

|  | Chroma QP − 1 |  |  |  | Chroma QP − 2 |  |  |  | Chroma QP − 3 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Avg. | Y | U | V | Avg. | Y | U | V | Avg. |
| A |  |  |  |  |  |  |  |  |  |  |  |  |
| B | 1.5% | -19.7% | -19.8% | -2.3% | 3.4% | -34.5% | -34.5% | -4.2% | 6.0% | -45.9% | -46.1% | -5.6% |
| C | 1.6% | -15.7% | -16.1% | -1.8% | 3.4% | -28.6% | -28.5% | -3.2% | 5.7% | -39.0% | -39.2% | -4.3% |
| D | 1.3% | -16.0% | -16.4% | -1.8% | 2.8% | -29.7% | -29.7% | -3.5% | 4.6% | -40.4% | -40.5% | -4.8% |
| E | 0.6% | -17.9% | -18.3% | -2.8% | 1.2% | -33.1% | -31.9% | -5.3% | 2.0% | -44.9% | -44.3% | -7.7% |
| All | 1.3% | -17.5% | -17.7% | -2.1% | 2.9% | -31.6% | -31.3% | -4.0% | 4.8% | -42.6% | -42.6% | -5.5% |
| EncT | 100% |  |  |  | 101% |  |  |  | 101% |  |  |  |
| DecT | 102% |  |  |  | 102% |  |  |  | 103% |  |  |  |

LB_LC

|  | Chroma QP − 1 |  |  |  | Chroma QP − 2 |  |  |  | Chroma QP − 3 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Avg. | Y | U | V | Avg. | Y | U | V | Avg. |
| A |  |  |  |  |  |  |  |  |  |  |  |  |
| B | 1.5% | -17.7% | -17.7% | -1.8% | 3.6% | -32.2% | -32.4% | -3.3% | 6.2% | -43.3% | -43.6% | -4.3% |
| C | 1.6% | -14.5% | -14.9% | -1.4% | 3.7% | -27.3% | -27.6% | -2.5% | 6.0% | -37.5% | -37.7% | -3.4% |
| D | 1.4% | -14.9% | -15.2% | -1.5% | 3.0% | -28.8% | -29.2% | -3.0% | 4.9% | -39.4% | -39.4% | -4.1% |
| E | 0.4% | -17.7% | -17.1% | -2.5% | 1.2% | -31.6% | -30.9% | -4.5% | 2.1% | -43.8% | -43.5% | -6.4% |
| All | 1.3% | -16.2% | -16.3% | -1.8% | 3.0% | -30.0% | -30.1% | -3.3% | 5.0% | -41.0% | -41.1% | -4.4% |
| EncT | 100% |  |  |  | 101% |  |  |  | 101% |  |  |  |
| DecT | 102% |  |  |  | 103% |  |  |  | 104% |  |  |  |

The changing of encoding and decoding time is mainly caused by the changing of bitrate.

It can be seen that

- Changing the weight of distortion of the different color components can bring some measured BD-Rate gain if the combined PSNR is used.

- If the QP of chroma is decreased (for H.264/MPEG-4 AVC, this is non-normative change with the help of chroma_qp_index_offset in PPS. But currently, we do not have that syntax element in either WD4 or HM4, so the chroma QP is hard coded in our tests), more measured gain can be obtained.

# References

- JCT-VC, “Chroma RD cost computation in HM3.0”, JCTVC-F386, JCT-VC Meeting, Torino, IT, July, 2011.