| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G402 |
| --- | --- |

| Title: | CE5: Cross-check of CAVLC Adaptation using Difference Counter (JCTVC-G310) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Bin Li Jizheng Xu | Tel: Email: |  |
| Source: | University of Science and Technology of China Microsoft Corp. |  |  |

_____________________________

# Abstract

This document is a crosscheck report of CE5 specified in JCTVC-F905, difference counter (JCTVC-G310). It is reported that the proposed difference counter brings negligible impact on RD performance, about 0.0%, 0.0%, 0.0%, and 0.0% BD-Rate for AI_LC, RA_LC, LB_LC, and LP_LC respectively, and that these measurements match the results reported by the proponent. Cody study is also provided to analyze the properties of the difference counter scheme.

# Introduction

This document reports crosscheck result of CE 5 [1], difference counter, JCTVC-G310 [3]. The executable file was compiled by Microsoft Visual Studio 2010, 64 bit. The tests were run with the following running environment.

| Platform | CPU | Memory | Compiler |
| --- | --- | --- | --- |
| Windows Server 2008 R2 64 bit | 2xIntel Xeon L5420@2.5Ghz | 16GB | VS2010 |

# Result

Our results (AI_LC, RA_LC, and LB_LC) match with those from Sharp [3]. The summary results are as follows.

Anchor: HM-4.0, Test: HM-4.0 + difference counter

Table 1 Summary result of difference counter (Y BD-rate)

|  | AI_LC | RA_LC | LB_LC | LP_LC |
| --- | --- | --- | --- | --- |
| Class A | 0.0% | 0.0% |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.1% | 0.0% |
| Class E | 0.0% |  | -0.1% | 0.0% |
| All | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time | 99% | 102% | 102% | 101% |
| Dec Time | 100% | 100% | 100% | 100% |

# Code study

The code was implemented the same as the proponent described. Some important lines are listed below.

if ( uiCodeIdx < uiCounterNum )

{

if (pucTableCounter[uiCodeIdx] == 0)

{

bSwapping = true;

}

else

{

pucTableCounter[uiCodeIdx]--;

}

if (uiCodeIdx+1 < uiCounterNum && pucTableCounter[uiCodeIdx+1] < 3)

{

pucTableCounter[uiCodeIdx+1] ++;

}

}

else

{

bSwapping = true;

}

In the previous design, pucTableCounter stores the actual number of one code word comes. When pucTableCounter[uiCodeIdx] >= pucTableCounter[uiPrevCodeIdx], the two code words are swapped in the code word table. It has been proved that swapping according to the actual number of the counter is helpful to improve the performance.

In the proposed difference counter method, instead of recording the actual number of each code word, the counter difference between two code words are recorded. When the difference counter equals to 0, two code words are swapped.

The first question is that, in the previous design, the swapping condition is “>=”, which means that when the current counter and the previous counter having the same value, but because the current counter is nearer, the swapping takes place. It is the case of updating counter firstly and determining swapping or not secondly. But in the design of the difference counter, it is determining swapping or not firstly but updating counter secondly. In such a case, the swapping does not take place when the two counters are equal (difference counter equals to 0 after updating). So it is necessary to also test the case of updating counter before determining swapping or not.

The second question is the difference counter itself. Although the difference counter does not lead to significant difference in terms of RD performance compared with the previous design of actual counter, the difference counter does not record the actual number anymore and may lead to worse results in some cases. Let us take the following case as an example.

Suppose there are eight (or more) code words in one group, one the first 4 code words are recorded by the counter. That is the common case in the HM-4.0. We only consider the first 4 code words for simple. After the initialization, the code word table and the difference counter table are as below.

| code word | 0 | 1 | 2 | 3 |
| --- | --- | --- | --- | --- |
| diff counter | 0 | 0 | 0 | 0 |

Suppose “1” comes, the two tables will be changed to

| code word | 1 | 0 | 2 | 3 |
| --- | --- | --- | --- | --- |
| diff counter | 0 | 0 | 1 | 0 |

And then “0” comes, the two tables will be

| code word | 0 | 1 | 2 | 3 |
| --- | --- | --- | --- | --- |
| diff counter | 0 | 0 | 2 | 0 |

And then “2” comes, the two tables will be

| code word | 0 | 1 | 2 | 3 |
| --- | --- | --- | --- | --- |
| diff counter | 0 | 0 | 1 | 1 |

Then “2” comes, the two tables will be

| code word | 0 | 1 | 2 | 3 |
| --- | --- | --- | --- | --- |
| diff counter | 0 | 0 | 0 | 2 |

It can be seen that, although “2” comes more times than “0” and “1”, it cannot be listed in the front of the table. Even more symbols of 1022 come, “2” cannot be listed in the front of the table either. In the last status of the above tables, the different counter 0 in red does not represent the actual relationship between code word “1” and “2”.

Fortunately, the old design does not have this kind of disadvantage. The tables below demonstrate what will happen with old design.

After the initialization, the code word table and the difference counter table are as below.

| code word | 0 | 1 | 2 | 3 |
| --- | --- | --- | --- | --- |
| Counter | 0 | 0 | 0 | 0 |

Suppose “1” comes, the two tables will be changed to

| code word | 1 | 0 | 2 | 3 |
| --- | --- | --- | --- | --- |
| Counter | 1 | 0 | 0 | 0 |

And then “0” comes, the two tables will be

| code word | 0 | 1 | 2 | 3 |
| --- | --- | --- | --- | --- |
| Counter | 1 | 1 | 0 | 0 |

And then “2” comes, the two tables will be

| code word | 0 | 2 | 1 | 3 |
| --- | --- | --- | --- | --- |
| Counter | 1 | 1 | 1 | 0 |

Then “2” comes, the two tables will be

| code word | 2 | 0 | 1 | 3 |
| --- | --- | --- | --- | --- |
| Counter | 2 | 1 | 1 | 0 |

The code word table lists as expected and the counter also represents the actual relationship among the code words.

So, it can be seen that difference counter cannot always represent the actual relationship among the code words, while the previous design of actual counter does not have this disadvantage.

# References

- JCT-VC, “Description of Core Experiment 5: CAVLC Entropy Coding Improvements”, JCTVC-F905, JCT-VC Meeting, Torino, IT, July, 2011.

- JCT-VC, “CAVLC Adaptation using Difference Counter”, JCTVC-F395, JCT-VC Meeting, Torino, IT, July, 2011.

- JCT-VC, “CE5: CAVLC Adaptation using Difference Counte”, JCTVC-G310, JCT-VC Meeting, Geneva, CH, Nov. 2011.