| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G403_r2 |
| --- | --- |

| Title: | CE4-Subtest-2 Cross-check of Adaptive Reconstruction Levels for Quantization Design (JCTVC-G382) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Bin Li Jizheng Xu Gary J. Sullivan | Tel: Email: | +1 425 882 8080 garysull@microsoft.com |
| Source: | University of Science and Technology of China Microsoft Corp. |  |  |

_____________________________

# Abstract

This document is a cross-check report for CE4-subtest2.2 specified in JCTVC-F904: quantization with adaptive reconstruction levels (JCTVC-G382). It is reported that the proposed adaptive de-quantization offset algorithm brings about 1.0%, 1.7%, 1.6%, and 2.2% in luma bit rate savings for RA_HE, RA_LC, LB_HE, and LB_LC respectively, and that these measurements match the results reported by the proponent. In addition to the tests performed according to the CE plan additional testing results and code study are also provided to analyze the properties of the adaptive de-quantization proposal and its use by the encoder. It is reported that most of the measured gain is actually from non-normative modifications rather than from the effect of the normative aspects of the proposal. It is also reported that the current implementation does not seem to work well when the QP is small. The effects on luma and chroma are also observed and reported to be somewhat different. It is observed that the proposed scheme seems to change the QP in a regular way similar to the hierarchical prediction structure. Further analysis to exclude the influence of QP/lambda changes shows that by only changing QPs in HM4.0 accordingly (by applying the same QPs and lambda setting from the proposed scheme in JCTVC-G382, which currently needs to run the proposed encoder first before using HM4.0. But conceptually it only use the statistics of the pictures coded before the current picture to be code.) can achieve even better gains in Luma than what’s proposed in JCTVC-G382. Thus, it is still not clear how many gains can be obtained beyond non-normative changes.

# Introduction

This document reports cross-check result for CE 4.2.2 [1], quantization with adaptive reconstruction levels, JCTVC-G382 [2]. The executable file was compiled by Microsoft Visual Studio 2010, 64 bit. The tests were run with the following running environment.

| Platform | CPU | Memory | Compiler |
| --- | --- | --- | --- |
| Windows Server 2008 R2 64 bit | 2xIntel Xeon L5420@2.5Ghz | 16GB | VS2010 |

# Result

Four tests required by CE4.2 were performed. Our results (Table 1~4) match with those from RIM [2]. The summary results are as follows.

- Test CE4.2.2.A

Anchor: HM-4.0, Test: ARL

Table 1 Summary result of CE4.2.2.A (Y BD-rate)

|  | RA_HE | RA_LC | LB_HE | LB_LC |
| --- | --- | --- | --- | --- |
| Class A | -1.0% | -1.9% |  |  |
| Class B | -0.9% | -1.6% | -1.8% | -2.5% |
| Class C | -1.4% | -2.0% | -2.1% | -3.0% |
| Class D | -0.9% | -1.3% | -1.4% | -2.0% |
| Class E |  |  | -0.7% | -1.2% |
| All | -1.0% | -1.7% | -1.6% | -2.2% |
| Enc Time | 100% | 101% | 100% | 101% |
| Dec Time | 100% | 101% | 100% | 100% |

- Test CE4.2.2.B

Anchor: HM-4.0, Test: ARL with bit rate constraint

Table 2 Summary result of CE4.2.2.B (Y BD-rate)

|  | RA_HE | RA_LC | LB_HE | LB_LC |
| --- | --- | --- | --- | --- |
| Class A | -0.6% | -2.1% |  |  |
| Class B | -1.0% | -1.6% | -1.8% | -2.4% |
| Class C | -0.8% | -1.2% | -1.8% | -2.3% |
| Class D | -0.5% | -0.9% | -1.2% | -1.8% |
| Class E |  |  | -0.3% | -0.8% |
| All | -0.7% | -1.5% | -1.4% | -2.0% |
| Enc Time | 100% | 102% | 100% | 102% |
| Dec Time | 101% | 102% | 101% | 101% |

- Test CE4.2.2.C

Anchor: QP Rounding, Test: ARL

Table 3 Summary result of CE4.2.2.C (Y BD-rate)

|  | RA_HE | RA_LC | LB_HE | LB_LC |
| --- | --- | --- | --- | --- |
| Class A | 0.0% | -0.1% |  |  |
| Class B | -0.3% | -0.3% | -0.6% | -0.2% |
| Class C | -0.7% | -0.6% | -0.9% | -0.4% |
| Class D | -0.3% | -0.3% | -0.2% | -0.1% |
| Class E |  |  | -0.3% | 0.0% |
| All | -0.3% | -0.3% | -0.5% | -0.2% |
| Enc Time | 99% | 100% | 99% | 100% |
| Dec Time | 100% | 100% | 100% | 100% |

It seems that for the results in Table 1, most of the gain comes from QP rounding rather than ARL.

- Test CE4.2.2.D

Anchor: RDOQ off EEM on, Test: RDOQ off EEM on ARL

Table 4 Summary result of CE4.2.2.D (Y BD-rate)

|  | RA_HE | RA_LC | LB_HE | LB_LC |
| --- | --- | --- | --- | --- |
| Class A | -1.8% | -1.3% |  |  |
| Class B | -1.4% | -1.5% | -1.6% | -2.1% |
| Class C | -1.3% | -1.5% | -1.8% | -2.1% |
| Class D | -0.9% | -1.1% | -1.1% | -1.4% |
| Class E |  |  | -0.4% | -0.8% |
| All | -1.3% | -1.4% | -1.3% | -1.7% |
| Enc Time | 102% | 101% | 101% | 101% |
| Dec Time | 100% | 101% | 100% | 100% |

Besides the tests required by the CE, some more tests were also performed.

- Test CE4.2.2.C’

Anchor: modified QP rounding (encoder only actions, no normative changes compared with HM-4.0), Test: ARL

Table 5 Summary result of CE4.2.2.C’ (Y BD-rate)

|  | RA_HE | RA_LC | LB_HE | LB_LC |
| --- | --- | --- | --- | --- |
| Class A | 0.0% | 0.2% |  |  |
| Class B | -0.2% | -0.2% | -0.6% | 0.0% |
| Class C | -0.6% | -0.5% | -0.7% | -0.3% |
| Class D | -0.3% | -0.3% | -0.3% | 0.0% |
| Class E |  |  | -0.5% | -0.2% |
| All | -0.3% | -0.2% | -0.5% | -0.1% |
| Enc Time | 99% | 100% | 99% | 100% |
| Dec Time | 100% | 100% | 100% | 99% |

It seems that compared with the improved QP rounding, which includes only non-normative changes, the gain of ARL is very small, even smaller than the results of CE4.2.2.C.

Some tests using small QP values were also performed.

- Test CE4.2.2.A (low QP)

Anchor: HM-4.0 anchor with low QP (2~17), Test: ARL with low QP (2~17)

Table 6 Summary result of CE4.2.2.A (low QP) (Y BD-rate)

|  | RA_HE | RA_LC | LB_HE | LB_LC |
| --- | --- | --- | --- | --- |
| Class A | 1.6% | 2.6% |  |  |
| Class B | 2.2% | 1.7% | 3.3% | 3.8% |
| Class C | 1.5% | 1.2% | 2.6% | 2.8% |
| Class D | 1.3% | 0.5% | 1.9% | 1.8% |
| Class E |  |  | 0.7% | 2.2% |
| All | 1.7% | 1.5% | 2.3% | 2.7% |
| Enc Time | 99% | 98% | 100% | 99% |
| Dec Time | 99% | 98% | 98% | 98% |

The decreasing of the decoding time was mainly caused by the decreasing of the bit rate. Please note that the average BD-Rate of the U and V components is gain in this test.

- Test CE4.2.2.C’ (low QP)

Anchor: modified QP rounding (encoder only actions) with low QP (2~17), Test: ARL with low QP (2~17)

Table 7 Summary result of CE4.2.2.C’ (low QP) (Y BD-rate)

|  | RA_HE | RA_LC | LB_HE | LB_LC |
| --- | --- | --- | --- | --- |
| Class A | -1.2% | -0.1% |  |  |
| Class B | 1.0% | 1.5% | 1.3% | 2.5% |
| Class C | 0.4% | 0.9% | 0.9% | 1.6% |
| Class D | -0.4% | 0.2% | 0.4% | 0.7% |
| Class E |  |  | 0.6% | 1.7% |
| All | 0.0% | 0.7% | 0.9% | 1.7% |
| Enc Time | 100% | 99% | 100% | 98% |
| Dec Time | 100% | 96% | 98% | 97% |

Please note that the average BD-Rate of U and V components is gain in this test.

From Table 6 and Table 7, it seems that the proposed ARL does not work well when QP is small. Comparing with HM-4.0 anchor or modified QP rounding, Y BD-Rate loss was obtained.

- Test CE4.2.2.D (low QP)

Anchor: RDOQ off EEM on with low QP (2~17), Test: RDOQ off EEM on ARL with low QP (2~17)

Table 8 Summary result of CE4.2.2.D (low QP) (Y BD-rate)

|  | RA_HE | RA_LC | LB_HE | LB_LC |
| --- | --- | --- | --- | --- |
| Class A | 3.3% | 3.5% |  |  |
| Class B | 1.7% | 0.2% | 2.7% | 1.4% |
| Class C | 2.0% | 0.5% | 3.0% | 1.2% |
| Class D | 2.1% | 1.0% | 2.3% | 0.9% |
| Class E |  |  | -0.3% | 0.0% |
| All | 2.2% | 1.2% | 2.1% | 1.0% |
| Enc Time | 100% | 101% | 100% | 101% |
| Dec Time | 103% | 102% | 101% | 99% |

# Code study

The code was studied and there are some issues to be explained in details.

- 7 additional bits are required when doing the dequantization.

- The default value of

m_AllARL     [Qp][type][0] = defaultvalue ;

is 1. And m_AllARL[Qp][type][0] is the delta QP.

Which means if there are not enough coefficients to update m_AllARL[Qp][type][0], the QP used in the proposed ARL is always greater that of HM-4.0.

The decreasing of bit rate and checking some of logs confirmed that conclusion that the effective QP used in the ARL proposal is greater than with HM-4.0.

- It seems that the gain on the U and V components is larger than that of Y. Is it caused by the different quantization relationship between Y and UV? For HM-4.0, the quantization parameter of UV is determined by that of Y. But for ARL, the quantization step size offset of UV is signaled independently. It seems that the larger gain on UV may be caused by the smaller quantization step size of UV

Looking into the code and results more details, there are two parameters having an impact on the dequantization process. For example, when the coefficient > 0,

iCoeffQ = ( piQCoef[n] * scale + iAdd + iAddD ) >> iShift;

iAddD is the signaled offset, and scale *= m_ARLarrayPnt[2], where m_ARLarrayPnt[2] is another important parameter relative to the quantization step size. It can been seen that small m_ARLarrayPnt[2] is equal to small quantization step size; vice versa. When m_ARLarrayPnt[2] == 128, the quantization step size is equal to that of the original HM-4.0. Some of the average m_ARLarrayPnt[2] of different color components were analyzed to verify the conclusion. Kimono LB_HE was selected to be tested. The average value of m_ARLarrayPnt[2] are listed in Table 9.

Table 9 Average m_ARLarrayPnt[2] of Kimono LB_HE

| QP | Inter Y | Inter UV | Intra Y | Intra UV |
| --- | --- | --- | --- | --- |
| 22 | 133 | 120 | 149 | 132 |
| 27 | 132 | 125 | 142 | 126 |
| 32 | 131 | 128 | 133 | 128 |
| 37 | 130 | 126 | 130 | 128 |

It can be seen that, comparing to the quantization step size of HM-4.0 of the same QP, the quantization step size of Y is larger while the quantization step size of UV is smaller. That may also be part of the explanation of the different behavior of Y and U/V.

- On the issue of QP setting between CE4.2.2.A and CE4.2.2.C

In CE4.2.2.C, the benefit of normative parts is analyzed. The anchor of CE4.2.2.C is HM-4.0 with QP rounding only (non-normative modifications), and the test is the same with CE4.2.2.A, i.e. proposed ARL based on QP rounding. The QP setting of BasketballPass QP22, LB_HE was checked in details. Test A is the test of CE4.2.2.A (proposed ARL), and Test C is the anchor of CE4.2.2.C (original CE4.2.2.C, not the version modified by us).

Table 10 QP in BasketballPass, QP22, LB_HE

|  |  | test A |  |  | test C |  |
| --- | --- | --- | --- | --- | --- | --- |
| POC | nQP | QP | difference |  | QP | difference |
| 40 | 23 | 24 | 1 |  | 24 | 1 |
| 41 | 25 | 27 | 2 |  | 26 | 1 |
| 42 | 24 | 26 | 2 |  | 25 | 1 |
| 43 | 25 | 27 | 2 |  | 26 | 1 |
| 44 | 23 | 24 | 1 |  | 24 | 1 |
| 45 | 25 | 27 | 2 |  | 26 | 1 |
| 46 | 24 | 26 | 2 |  | 25 | 1 |
| 47 | 25 | 27 | 2 |  | 26 | 1 |
| 48 | 23 | 24 | 1 |  | 24 | 1 |
| 49 | 25 | 27 | 2 |  | 26 | 1 |
| 50 | 24 | 26 | 2 |  | 25 | 1 |
| 51 | 25 | 27 | 2 |  | 26 | 1 |
| 52 | 23 | 24 | 1 |  | 24 | 1 |
| 53 | 25 | 27 | 2 |  | 26 | 1 |
| 54 | 24 | 26 | 2 |  | 25 | 1 |
| 55 | 25 | 27 | 2 |  | 26 | 1 |
| 56 | 23 | 24 | 1 |  | 24 | 1 |
| 57 | 25 | 27 | 2 |  | 26 | 1 |
| 58 | 24 | 26 | 2 |  | 25 | 1 |
| 59 | 25 | 27 | 2 |  | 26 | 1 |
| 60 | 23 | 24 | 1 |  | 24 | 1 |
| 61 | 25 | 27 | 2 |  | 26 | 1 |
| 62 | 24 | 26 | 2 |  | 25 | 1 |
| 63 | 25 | 27 | 2 |  | 26 | 1 |
| 64 | 23 | 24 | 1 |  | 24 | 1 |
| 65 | 25 | 27 | 2 |  | 26 | 1 |
| 66 | 24 | 26 | 2 |  | 25 | 1 |
| 67 | 25 | 27 | 2 |  | 26 | 1 |
| 68 | 23 | 24 | 1 |  | 24 | 1 |
| 69 | 25 | 27 | 2 |  | 26 | 1 |
| 70 | 24 | 26 | 2 |  | 25 | 1 |
| 71 | 25 | 27 | 2 |  | 26 | 1 |
| 72 | 23 | 24 | 1 |  | 24 | 1 |
| 73 | 25 | 28 | 3 |  | 26 | 1 |
| 74 | 24 | 26 | 2 |  | 25 | 1 |
| 75 | 25 | 28 | 3 |  | 26 | 1 |
| 76 | 23 | 24 | 1 |  | 24 | 1 |
| 77 | 25 | 28 | 3 |  | 26 | 1 |
| 78 | 24 | 27 | 3 |  | 25 | 1 |
| 79 | 25 | 28 | 3 |  | 26 | 1 |
| 80 | 23 | 24 | 1 |  | 24 | 1 |

The nQP is the nominal QP, which is calculated according to the parameter of RateGOPSize, and used to determine lambda. The QP is the actual QP used during the coding, which is determined by QP rounding. There are following observations:

- The QP is always larger than nQP.

- The QP in Test A is quite different than that in Test C. It can be seen that the QP difference in Test C is always 1, while that in Test A is 1 for pictures with POC%4==0, some larger values for pictures with POC%4!=0. As we known that, the pictures with POC%4==0 are much more important than the others. It can be seen that the bit allocation is quite different in CE4.2.2.A and CE4.2.2.C.

Here are the similar results with BasketballDrill, QP22, RA_HE,

Table 11 QP in BasketballDrill QP22 RA_HE

|  |  | test A |  |  | test C |  |
| --- | --- | --- | --- | --- | --- | --- |
| POC | nQP | QP | Difference |  | QP | difference |
| 48 | 22 | 22 | 0 |  | 22 | 0 |
| 44 | 24 | 25 | 1 |  | 25 | 1 |
| 42 | 25 | 27 | 2 |  | 26 | 1 |
| 46 | 25 | 27 | 2 |  | 26 | 1 |
| 41 | 26 | 31 | 5 |  | 27 | 1 |
| 43 | 26 | 31 | 5 |  | 27 | 1 |
| 45 | 26 | 31 | 5 |  | 27 | 1 |
| 47 | 26 | 31 | 5 |  | 27 | 1 |
| 56 | 23 | 24 | 1 |  | 24 | 1 |
| 52 | 24 | 25 | 1 |  | 25 | 1 |
| 50 | 25 | 27 | 2 |  | 26 | 1 |
| 54 | 25 | 27 | 2 |  | 26 | 1 |
| 49 | 26 | 31 | 5 |  | 27 | 1 |
| 51 | 26 | 31 | 5 |  | 27 | 1 |
| 53 | 26 | 31 | 5 |  | 27 | 1 |
| 55 | 26 | 31 | 5 |  | 27 | 1 |
| 64 | 23 | 24 | 1 |  | 24 | 1 |
| 60 | 24 | 25 | 1 |  | 25 | 1 |
| 58 | 25 | 27 | 2 |  | 26 | 1 |
| 62 | 25 | 27 | 2 |  | 26 | 1 |
| 57 | 26 | 31 | 5 |  | 27 | 1 |
| 59 | 26 | 31 | 5 |  | 27 | 1 |
| 61 | 26 | 31 | 5 |  | 27 | 1 |
| 63 | 26 | 31 | 5 |  | 27 | 1 |
| 72 | 23 | 24 | 1 |  | 24 | 1 |
| 68 | 24 | 25 | 1 |  | 25 | 1 |
| 66 | 25 | 27 | 2 |  | 26 | 1 |
| 70 | 25 | 27 | 2 |  | 26 | 1 |
| 65 | 26 | 31 | 5 |  | 27 | 1 |
| 67 | 26 | 31 | 5 |  | 27 | 1 |
| 69 | 26 | 31 | 5 |  | 27 | 1 |
| 71 | 26 | 31 | 5 |  | 27 | 1 |
| 80 | 23 | 24 | 1 |  | 24 | 1 |
| 76 | 24 | 25 | 1 |  | 25 | 1 |
| 74 | 25 | 27 | 2 |  | 26 | 1 |
| 78 | 25 | 27 | 2 |  | 26 | 1 |
| 73 | 26 | 31 | 5 |  | 27 | 1 |
| 75 | 26 | 31 | 5 |  | 27 | 1 |
| 77 | 26 | 31 | 5 |  | 27 | 1 |
| 79 | 26 | 31 | 5 |  | 27 | 1 |
| 88 | 23 | 24 | 1 |  | 24 | 1 |
| 84 | 24 | 25 | 1 |  | 25 | 1 |
| 82 | 25 | 27 | 2 |  | 26 | 1 |
| 86 | 25 | 27 | 2 |  | 26 | 1 |
| 81 | 26 | 31 | 5 |  | 27 | 1 |
| 83 | 26 | 31 | 5 |  | 27 | 1 |
| 85 | 26 | 31 | 5 |  | 28 | 2 |
| 87 | 26 | 31 | 5 |  | 28 | 2 |
| 96 | 22 | 22 | 0 |  | 22 | 0 |

It shows a more interesting behavior than that in the LB_HE results. The results are from the second I frame (POC 48) to the third I frame (POC 96). The QPs for I frames are not allowed to be changed. Table 11 shows that for RA cases, the QPs in Test A are quite different from those in Test C. It can be seen that most QPs in Test C changed by 1, while that in Test A, regular pattern of QP differences was observed with hierarchical levels. For level 0 and 1, QP difference (QP−nQP) is 1, level 2 is 2, and level 3 is 5.

We chose the above sequences for analysis because in the results of CE4.2.2.A, in RA_HE, the luma BD-Rate of BasketballDrill is of the second largest. The reason why we do not choose the sequence with the largest luma BD-Rate (which is with the sequence Nebuta) is that it seems that behavior of Nebuta is quite different from the other sequences, (it was pointed out in the 6th JCT-VC meeting, that the BD-Rate using piecewise cubic method and cubic method is quite different). For LB_HE, BasketPass shows the third largest luma BD-Rate gain among all testing sequences. The first and the second are Kimono and BasketballDrill. There is a scene changing in Kimono. BasketballDrill has already been selected for RA_HE. Thus we chose the sequence with the third largest luma BD-Rate gain, BasketballPass.

The reason we chose the point with QP 22 is that, from the sheet of “Plot”, we observed that for both of these two sequences, the gain is much larger at the high bit rate point, so we chose the point of QP 22 to study.

Another aspect we want to explain the POC range we chose. Because as the QP rounding offset is determined adaptively, we should not choose the first few pictures of the sequence. So we skipped the first few pictures.

What we are interested to know is that, why the QP setting in CE4.2.2.A and CE4.2.2.C are quite different? And how much CE4.2.2.A may benefit from this kind of difference?

We also observed that the impact of using different QP setting cannot negligible, even that the lambda values used in RDO remain unchanged. According to our understanding, the QP and lambda both have impact on bit allocation. Generally speaking, if the lambda is unchanged and the QP is changed, i.e. QP is decreased, more bits will be allocated; and vice versa.
If the QP is unchanged but the lambda is changed, i.e. lambda is increased, less bits will be allocated; and vice versa.
The results of CE4.2.2.A and CE4.2.2.B can be used as evidence. In CE4.2.2.A, lambda is kept unchanged but most QPs are changed larger, so the bit rate becomes smaller. In CE4.2.2.B, both the lambda and QPs are changed but almost to the opposite direction, so the bitrate is kept almost unchanged in that test.

We also tested it in HM-4.0. The anchor is original HM-4.0, and the test is HM-4.0 with modified QP. Only the QP is modified, the lambda is kept the same with HM-4.0. The QP is modified after the lambda is determined according to

if ( POC % 4 == 0 ) iQP += 0;
else if ( POC % 4 == 2 ) iQP += 1;
else                 iQP += 2;

With this kind of modification, for class D sequences, 0.4% bits saving for RA_HE, and 0.9% bits saving for LB_HE can be obtained.

Table 12 The benefit of changing QP without changing lambda in HM-4.0

|  | Random Access HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class D | -0.4% | -0.2% | -0.3% |
|  | Low delay B HE |  |  |
|  | Y | U | V |
| Class D | -0.9% | -0.8% | -0.5% |

The BD-rate saving by changing QP but without changing lambda is not small. So it is still not clear compared with CE4.2.2.C, how many gains are from changing QP in CE4.2.2.A.

According to our understanding, comparing CE4.2.2.A and CE4.2.2.B, the impact of bit allocation can be excluded. It is very difficult to say the gain in CE4.2.2.B comes from normative changes or non-normative changes.

Comparing CE4.2.2.A and CE4.2.2.C, the impact of non-normative changes can be excluded. But it is also very difficult to say whether the gain comes from bit re-allocation. From Table 12, we can know that changing QP can bring bit saving.

As shown in Table 10 and 11, we want to say that as the QP is quite different in CE4.2.2.A and CE4.2.2.C, CE4.2.2.A benefits from the bit allocation (pictures with POC%4!=0 have relative larger QPs in CE4.2.2.A than that in CE4.2.2.C).

In conclusion, it is not clear whether the proposed normative change is efficient or not if the bit allocation issue and the non-normative issues are excluded together. We suggest that the comparison between the normative change and the non-normative change should also be performed with the rate constraint in order to clearly identify the gain brought by the normative change.

# Further investigation on the bit allocation issue

To further investigate the impact of bit allocation, we did some more tests, with only non-normative changes based on HM-4.0. It is very difficult to exactly match the bit rate of CE4.2.2.A. So we chose a simpler method, i.e. matching QP and lambda for each picture. According to our knowledge, if both the QP and the lambda are identical, the bit rate drifting at each temporal level will not be very far.

The QP of each picture is changed according to the encoding results of CE4.2.2.A. (Each picture will have the same QP with the CE4.2.2.A; actually, the encoding log of CE4.2.2.A is used as another input parameter at the encoder side. The QP is modified according to what is read for the log of CE4.2.2.A.) In such a case, the QP and lambda in the test below and CE4.2.2.A are identical. For the results in Table 13, the anchor is original HM-4.0, and the test is HM-4.0 + QP modifying.

Table 13 Encoder only QP modifying VS. HM-4.0

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -1.2% | -1.4% | -1.4% | -2.2% | -2.6% | -3.0% |
| Class B | -1.0% | -2.0% | -1.9% | -1.8% | -2.0% | -2.3% |
| Class C | -1.5% | -2.6% | -2.4% | -2.2% | -2.5% | -2.4% |
| Class D | -1.0% | -1.9% | -2.0% | -1.5% | -2.0% | -1.8% |
| Class E |  |  |  |  |  |  |
| Overall | -1.1% | -2.0% | -1.9% | -1.9% | -2.2% | -2.4% |
|  | -0.8% | -1.7% | -1.7% | -1.9% | -2.1% | -2.3% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -1.9% | -2.9% | -2.9% | -2.8% | -1.9% | -2.0% |
| Class C | -2.1% | -2.9% | -3.0% | -3.1% | -2.3% | -2.4% |
| Class D | -1.6% | -2.4% | -2.5% | -2.3% | -1.2% | -1.6% |
| Class E | -0.7% | -1.0% | -0.6% | -1.3% | -1.1% | -1.1% |
| Overall | -1.6% | -2.4% | -2.4% | -2.5% | -1.7% | -1.8% |
|  | -1.6% | -2.4% | -2.3% | -2.5% | -1.5% | -1.5% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |

To compare the result of encoder only QP modifying and ARL, we use HM-4.0 + QP modifying as anchor, and the results in CE4.2.2.A as test, as shown in Table 14.

Table 14 Test C’’ - CE4.2.2.A VS. Encoder only QP modifying

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.2% | -3.3% | -3.5% | 0.3% | -4.8% | -4.4% |
| Class B | 0.1% | -2.4% | -2.2% | 0.3% | -3.7% | -3.6% |
| Class C | 0.1% | -1.9% | -1.7% | 0.2% | -2.7% | -2.5% |
| Class D | 0.1% | -1.6% | -1.4% | 0.2% | -2.6% | -2.7% |
| Class E |  |  |  |  |  |  |
| Overall | 0.1% | -2.3% | -2.2% | 0.3% | -3.5% | -3.3% |
|  | 0.2% | -2.4% | -2.3% | 0.3% | -3.4% | -3.3% |
| Enc Time[%] | 101% |  |  | 102% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.1% | -2.9% | -2.7% | 0.4% | -5.1% | -5.7% |
| Class C | 0.0% | -2.5% | -2.6% | 0.2% | -4.2% | -4.1% |
| Class D | 0.2% | -1.4% | -1.5% | 0.3% | -3.1% | -3.2% |
| Class E | 0.0% | -1.1% | -1.5% | 0.1% | -3.3% | -3.2% |
| Overall | 0.1% | -2.1% | -2.2% | 0.2% | -4.0% | -4.2% |
|  | 0.1% | -2.1% | -2.1% | 0.2% | -4.2% | -4.4% |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |

It should be noted that most of the bit rates for each temporal of the anchor and the test in Table 14 do not exactly match. But as shown in the attached excel file, the bit rate range for each temporal level does not drift much. Although the bit rate of each temporal level does not exactly match, we think this difference is not too large and as they used the identical QP and lambda setting, the comparison is relatively fair.

From the above table we found that

- For luma, non-normative changes work a little better than CE4.2.2.A

- For chroma, non-normative changes work a little worse than CE4.2.2.A.

Maybe different de-quantization step sizes (even with the same QP) may be part of the reasons (see Table 9). Comparing the number of luma loss and chroma gain, these kinds of results can be recognized as bits shifting, as already discussed in JCTVC-G401 [3].

From the above analysis, it can be concluded that most gain of CE4.2.2.A, or even a better gain, can be obtained by non-normative changes, e.g., changing QPs, under the common testing conditions. The best performance we have observed, in terms of Luma gains, is achieved by HM4.0 with QPs according to those used in CE4.2.2.A.

# References

- JCT-VC, “Description of Core Experiment 4: Quantization”, JCTVC-F904, JCT-VC Meeting, Torino, IT, July, 2011.

- JCT-VC, “CE4 Subtest-2 Adaptive Reconstruction Levels for Quantization Design”, JCTVC-G382, JCT-VC Meeting, Geneva, CH, Nov. 2011.

- JCT-VC, “RDO with weighted distortion in HEVC”, JCTVC-G401, JCT-VC Meeting, Geneva, CH, Nov. 2011.