| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G404 |
| --- | --- |

| Title: | CE9: Cross-check of parallelized merge/skip mode (JCTVC-G085) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Bin Li Jizheng Xu | Tel: Email: |  |
| Source: | University of Science and Technology of China Microsoft Corp. |  |  |

_____________________________

# Abstract

This document is a crosscheck report of CE9 specified in JCTVC-F909, parallelized merge/skip mode (JCTVC-G085). The results we obtained match the results reported by the proponent.

# Introduction

This document reports crosscheck result of CE 9 [1], parallelized merge/skip mode, JCTVC-G085 [3]. The executable file was compiled by Microsoft Visual Studio 2010, 64 bit. The tests were run with the following running environment.

| Platform | CPU | Memory | Compiler |
| --- | --- | --- | --- |
| Windows Server 2008 R2 64 bit | 2xIntel Xeon L5420@2.5Ghz | 16GB | VS2010 |

# Result

Our results match with those from TI [3]. The summary results are as follows.

Table 1 Modified Anchor (Y BD-rate)

|  | RA_HE | RA_LC | LB_HE | LB_LC |
| --- | --- | --- | --- | --- |
| Class A | -0.3% | -0.2% |  |  |
| Class B | -0.3% | -0.2% | -0.3% | -0.3% |
| Class C | -0.2% | -0.2% | -0.3% | -0.2% |
| Class D | -0.2% | -0.1% | -0.3% | -0.2% |
| Class E |  |  | -0.6% | -0.4% |
| All | -0.2% | -0.2% | -0.4% | -0.3% |
| Enc Time | 113% | 113% | 116% | 116% |
| Dec Time | 101% | 100% | 100% | 100% |

Table 2 Modified Anchor Vs. PMRG_8_a) (Y BD-rate)

|  | RA_HE | RA_LC | LB_HE | LB_LC |
| --- | --- | --- | --- | --- |
| Class A | 0.1% | 0.1% |  |  |
| Class B | 0.2% | 0.1% | 0.2% | 0.2% |
| Class C | 0.3% | 0.3% | 0.5% | 0.4% |
| Class D | 0.4% | 0.4% | 0.6% | 0.6% |
| Class E |  |  | 0.2% | 0.2% |
| All | 0.2% | 0.2% | 0.4% | 0.3% |
| Enc Time | 99% | 100% | 100% | 100% |
| Dec Time | 98% | 99% | 98% | 98% |

Table 3 PMRG_8_a) Vs. b) (Y BD-rate)

|  | RA_HE | RA_LC | LB_HE | LB_LC |
| --- | --- | --- | --- | --- |
| Class A | -0.1% | -0.1% |  |  |
| Class B | -0.2% | -0.1% | -0.2% | -0.2% |
| Class C | -0.3% | -0.3% | -0.5% | -0.4% |
| Class D | -0.4% | -0.4% | -0.5% | -0.5% |
| Class E |  |  | -0.1% | -0.1% |
| All | -0.2% | -0.2% | -0.3% | -0.3% |
| Enc Time | 100% | 100% | 100% | 100% |
| Dec Time | 100% | 100% | 101% | 101% |

Table 4 Modified Anchor Vs. PMRG_16_a) (Y BD-rate)

|  | RA_HE | RA_LC | LB_HE | LB_LC |
| --- | --- | --- | --- | --- |
| Class A | 1.6% | 1.5% |  |  |
| Class B | 2.6% | 2.5% | 3.2% | 3.2% |
| Class C | 2.6% | 2.5% | 3.7% | 3.5% |
| Class D | 2.7% | 2.8% | 3.6% | 3.9% |
| Class E |  |  | 3.0% | 2.6% |
| All | 2.4% | 2.3% | 3.4% | 3.3% |
| Enc Time | 95% | 96% | 97% | 98% |
| Dec Time | 96% | 97% | 96% | 96% |

Table 5 PMRG_16_a) Vs. b) (Y BD-rate)

|  | RA_HE | RA_LC | LB_HE | LB_LC |
| --- | --- | --- | --- | --- |
| Class A | -1.3% | -1.1% |  |  |
| Class B | -2.2% | -2.0% | -2.7% | -2.5% |
| Class C | -2.0% | -1.8% | -2.7% | -2.6% |
| Class D | -2.0% | -2.0% | -2.5% | -2.7% |
| Class E |  |  | -2.6% | -2.2% |
| All | -1.9% | -1.8% | -2.6% | -2.5% |
| Enc Time | 103% | 103% | 102% | 102% |
| Dec Time | 101% | 101% | 102% | 103% |

# References

- JCT-VC, “Description of Core Experiment 9: MV Coding and Skip/Merge Operations”, JCTVC-F909, JCT-VC Meeting, Torino, IT, July, 2011.

- JCT-VC, “Parallelized merge/skip mode for HEVC”, JCTVC-F069, JCT-VC Meeting, Torino, IT, July, 2011.

- JCT-VC, “CE9: Test results on parallelized merge/skip mode”, JCTVC-G085, JCT-VC Meeting, Geneva, CH, Nov. 2011.