| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G406 |
| --- | --- |

| Title: | Non-CE9: Improvement in temporal candidate of merge mode and AMVP (JCTVC-G343) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Bin Li Jizheng Xu | Tel: Email: |  |
| Source: | University of Science and Technology of China Microsoft Corp. |  |  |

_____________________________

# Abstract

This document is a crosscheck report of median TMVP from HIT (JCTVC-G343). It is reported that the proposed median TMVP brings a little improvement on RD performance, about −0.1%, −0.1%, −0.2%, −0.2%, −0.2%, and −0.3% BD-Rate for RA_HE, RA_LC, LB_HE, LB_LC, LP_HE and LP_LC respectively of solution A; And for solution B, the BD-Rate is also about −0.1%, −0.1%, −0.2%, −0.2%, −0.2%, and −0.3%. And that these measurements match the results reported by the proponent.

# Introduction

This document reports crosscheck result of median TMVP from HIT, JCTVC-G343 [1]. The executable file was compiled by Microsoft Visual Studio 2010, 64 bit. The tests were run with the following running environment.

| Platform | CPU | Memory | Compiler |
| --- | --- | --- | --- |
| Windows Server 2008 R2 64 bit | 2xIntel Xeon L5420@2.5Ghz | 16GB | VS2010 |

# Result

Our results match those from the proponent. The summary results are shown in Table 1 and 2.

Table 1 Summary results of median TMVP, solution A (Y BD-Rate)

|  | RA_HE | RA_LC | LB_HE | LB_LC | LP_HE | LP_LC |
| --- | --- | --- | --- | --- | --- | --- |
| Class A | -0.1% | -0.1% |  |  |  |  |
| Class B | -0.1% | -0.1% | -0.2% | -0.2% | -0.2% | -0.3% |
| Class C | -0.1% | -0.1% | -0.2% | -0.2% | -0.2% | -0.2% |
| Class D | -0.1% | -0.1% | -0.2% | -0.2% | -0.1% | -0.2% |
| Class E |  |  | -0.2% | -0.2% | -0.2% | -0.3% |
| Overall | -0.1% | -0.1% | -0.2% | -0.2% | -0.2% | -0.3% |
| EncT. | 100% | 100% | 100% | 101% | 100% | 100% |
| DecT. | 101% | 102% | 101% | 102% | 101% | 102% |

Table 2 Summary results of median TMVP, solution B (Y BD-Rate)

|  | RA_HE | RA_LC | LB_HE | LB_LC | LP_HE | LP_LC |
| --- | --- | --- | --- | --- | --- | --- |
| Class A | -0.1% | -0.1% |  |  |  |  |
| Class B | -0.1% | -0.1% | -0.2% | -0.2% | -0.2% | -0.3% |
| Class C | -0.1% | -0.1% | -0.2% | -0.2% | -0.2% | -0.2% |
| Class D | -0.1% | -0.2% | -0.2% | -0.2% | -0.1% | -0.2% |
| Class E |  |  | -0.2% | -0.2% | -0.2% | -0.3% |
| Overall | -0.1% | -0.1% | -0.2% | -0.2% | -0.2% | -0.3% |
| EncT. | 100% | 100% | 100% | 100% | 100% | 100% |
| DecT. | 101% | 102% | 101% | 102% | 101% | 101% |

# References

- JCT-VC, “CE9: Improvement in temporal candidate of merge mode and AMVP”, JCTVC-G343, JCT-VC Meeting, Geneva, CH, Nov. 2011.