| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G409 |
| --- | --- |

| Title: | CE12, Subset 1: Report of Deblocking for Large Size Blocks |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhongbo Shi Xiaoyan Sun Jizheng Xu | Tel: Email: | {xysun, jzxu}@microsoft.com |
| Source: | University of Science and Technology of China Microsoft |  |  |

_____________________________

# Abstract

This document presents a deblocking scheme for large size blocks to improve the visual quality of HEVC decoded videos. For large smooth regions with small variation, an extra smoothing deblocking mode is introduced to suppress the visually severe blocking artifacts. It is observed that the proposed method can reduce blocking artifacts in smooth regions, which are usually more visible to human eyes.

# Introduction

Visual artifacts are more visible to human eyes in smooth regions. It is also the case for blocking artifacts. Although compared to AVC, the current HEVC scheme can yield much better decoded videos, the blocking artifacts are still annoying, especially at low bit-rates and in smooth region. Thus, we propose to use a stronger deblocking filter for blocks with large block size and in smooth region.

The scheme described in this document is based on what we have done in [1], [2] and [3]. The difference is that we have integrated the proposed scheme into HM4.0 software and fine-tuned the parameters. The relationship between this tool and parallel deblocking is also investigated and the line buffer used for deblocking is adjusted and limited considering the hardware implementation.

# Algorithm description

There are two deblocking mode in the proposed scheme, the HEVC deblocking mode (Mode A), which performs exactly the same filtering as in the current HEVC deblocking, and the extra smoothing deblocking mode (Mode B), which performs a strong and long distance filtering across certain transform unit (TU) boundaries. The details of Mode B are introduced as follows.

## Variance calculation for 16x16 subblocks

The variances of 16x16 subblocks in a LCU (64x64) will be calculated as following way and used for deblocking mode decision as described in subsection 2.2. Similar to the parallel deblocking decision proposed in [4] by Panasonic, all these variances are calculated based on the non-filtered frame and stored as parameters of a LCU. This will ensure the parallel deblocking decision is also supported in Mode B.

Let Pij i,j = 0,1,2…15 denote the pixels in one 16x16 subblock, and the variance of the subblock is calculated as:

Boundary_Mean   = ∑ P0,i + ∑ Pi,0 + ∑ P15,i + ∑ Pi,15   i = 0,1,2…1;

Variance_Subblock = ∑abs (Pi,j – Boundary_Mean )  >> 7;

when i = 0,2,4…14, j = 0, 2,4…14;

when i = 1,3,5…15, j = 1,3,5…15;

The variances of all sixteen 16x16 subblocks in a LCU will be stored as parameters.

## Deblocking Mode decision

There are two deblocking mods in the proposed scheme: The HEVC deblocking mode( Mode A ) and the extra smoothintg deblocking mode ( Mode B), which is supposed to be applied in large smooth regions with small variation. In the proposed scheme, TU boundaries satisfy the following conditions will be processed with deblocking Mode B.

- The two adjacent TU’s related to the TU boundary are both in intra-coded CU’s or The two adjacent TU’s related to the TU boundary are both in inter-coded CU’s and The partition size of CU’s are 2Nx2N;

- The smaller transform size between the two TU's is larger than a certain value (set to 8 in the proposed scheme);

- The larger transform size between the two TU's is larger than a certain value (set to 16 in the proposed scheme);

- The sum of Variance_Subblock from the two subblocks related to the TU boundary is smaller than a certain threshold ( set to 2 for I-frame and 1 for B/P-frame in the proposed scheme );

- All the pixels at the TU boundaries should be processed as strong filtering in Mode A;

Other TU boundaries will be processed with deblocking Mode A.

## Filtering Processing in Mode B

For deblocking Mode B, the filtering is performed along lines perpendicular to the TU boundary. As shown in Fig. 1, suppose  are the pixels in the same row or column of two adjacent TU’s.

Fig. 1. Extra smoothing deblocking (Mode B). A vertical boundary of a luminance block is taken for example.  are the pixels to be filtered. In this example, the filtered value of q4 is obtained by averaging .

For the horizontal filtering (vertical edge), their values are modified as

When QPLuma <=30

,

When QPLuma > 30

,

And the following conditions should also be satisfied:

(1) The p12 to p15 used in above functions are replaced by p11.

(2) The q12 to q15 used in above functions are replaced by q11.

These two conditions limit the max number of pixels used in filtering to 12 (e.g. P0…P11 and q0 … q11).

For the vertical filtering, we do not increasing the line buffer from upper CU considering the hard ware implementation. So the vertical filtering processing is performed as

When QP<= 30

When QP>30

We only perform the filtering processing in Mode B to the luminance signal. For chroma signal the filtering processing is as the same as that in Mode A.

As described, we limit the number of pixels used for horizontal filtering to 12 (e.g. P0…P11 and q0 … q11) at most and the number of pixels used for vertical filtering to 8 at most. This will ensure that there is no overlapped pixel between two neighboring edge filtered by no matter Mode A or Mode B as shown in Fig2.

Fig.2

In Fig2, The vertical edge 1 is the edge between one 32x32TU and 16x16TU and filtered by Mode B. The vertical edge 2 is the edge between two 16x16TUs and filtered by Mode A. There is no overlapped pixel using for the two edges.

Hence the Mode B also supports the LCU-based parallel deblocking proposed in [5] by Sony. The only difference is the latency between two LCU increases slightly (from 4 lines to 12 lines) because more pixels are buffered for horizontal filtering in Mode B.

Fig.3

Besides, we also limit the line buffer from upper CU. In Mode B, the number of pixels from upper CU is 4 and the number of pixels from current CU is 8 at most as shown in Fig3. This will ensure there is no additional line buffer required from upper filtered CU compared to the original Mode A.

This is a heavy and long distance smoothing across the TU boundary and pixels in large smooth regions are preferred to be involved. For deblocking Mode B, no boundary strength and filter thresholds are needed.

Once the deblocking mode for each TU boundary is determined, the two different deblocking modes are simultaneously employed in a raster-scan order at the TU level for both luminance and chroma components. The reconstructed frame is obtained in a single deblocking pass.

# Compression performance discussion

Experiments are done with HM4.0 software platform. We compared our results with HM4.0 anchors.

## Objective compression performance

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | -0.1% | -0.1% | -0.1% | 0.3% | 0.0% |
| Class B | 0.0% | -0.1% | 0.1% | 0.0% | 0.0% | 0.1% |
| Class C | 0.1% | -0.1% | 0.1% | 0.0% | 0.0% | -0.1% |
| Class D | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | -0.1% | 0.1% | 0.0% | 0.1% | 0.0% |
|  | 0.0% | -0.1% | 0.1% | 0.0% | 0.1% | 0.0% |
| Enc Time[%] | 107% |  |  | 102% |  |  |
| Dec Time[%] | 102% |  |  | 104% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | -0.2% | -0.2% | 0.0% | -0.1% | -0.1% |
| Class C | 0.0% | 0.0% | -0.1% | 0.0% | 0.1% | -0.4% |
| Class D | 0.0% | 0.2% | 0.2% | 0.0% | -0.1% | -0.3% |
| Class E | 0.0% | 0.1% | 0.8% | 0.0% | 0.3% | 0.2% |
| Overall | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | -0.2% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.2% |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 102% |  |  | 104% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.2% | 0.1% | 0.0% | 0.1% | 0.0% |
| Class C | 0.0% | 0.2% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | -0.1% | -0.3% | 0.0% | -0.2% | -0.1% |
| Class E | 0.1% | 0.1% | -0.1% | 0.0% | -0.5% | -0.4% |
| Overall | 0.0% | 0.1% | 0.0% | 0.0% | -0.1% | -0.1% |
|  | 0.0% | 0.1% | -0.1% | 0.0% | -0.1% | -0.1% |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 102% |  |  | 104% |  |  |

## Subjective compression performance

We observed slight improvements (less blocking artifacts) for Kimono (sky and face area) and Parkscene (trunk area), for all_intra, low_delay and random_access at low bit-rates (Qp37, and sometimes Qp32).

Some examples taken from the required sequences included in the visual test of CE12 are shown below.

Kimono_1920x1080_24, All intra, QP = 37, frame #5, cropped. Left: anchor, right: proposed

Kimono_1920x1080_24, All intra, QP = 37, frame #16, cropped. Left: anchor, right: proposed

# Closing remarks

This document presents the proposed deblocking scheme for large-size blocks in the current HEVC. Less visual artifacts are observed, while the complexity and the objective quality changes are margional.

# References

[1] Z. Shi, Z. Xiong, X. Sun, J. Xu, “CE8, Subset 1: Report of Content-Adaptive Deblocking,”JCTVC-D137, Jan.2011, Daegu, KR.

[2] Z. Shi, Z. Xiong, X. Sun, J. Xu, “CE12, Subset 1: Report of Deblocking For Large Size Blocks,”JCTVC-E144, March.2011, Geneva, CH.

[3] Z. Shi, X. Sun, J. Xu, “CE12, Subset 1: Report of Deblocking For Large Size Blocks,”JCTVC-F198, July.2011, Torino, IT. 
[4] M. Narroschke, T. Wedi, “CE12, Subset 2: Results for parallel deblocking filter decisions,” JCTVC-E224, March.2011, Geneva, CH.

[5] M. Ideda, J. Tanaka, T. Suzuki, “CE12, Subset2: Parallel deblcking filter”, JCTVC-E181, March.2011, Geneva, CH.

# Patent rights declaration(s)

University of Science and Technology of China, and Microsoft Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Appendix: the text for working draft

### 8.6.1 Deblocking filter process

6.	The derivation process of the boundary filtering strength specified in subclause 8.6.1.3 is invoked with the luma location ( xC, yC ), the coding unit size log2CUSize, horEdgeFlags and verEdgeFlags as inputs and an array of size (2)x(nS)x(nS), bS as output.

7.	The derivation process of the boundary filtering mode specidfied in subclause 8.6.1.4 is involded with the luma location (xC,yC), the coding unit size log2CUSize, horEdgeFlags, and verEdgeFlags as inputs and an array of size (2)x(nS)x(nS), FM(Filtering Mode) as output.

8.	The filtering process for coding unit specified in subclause 8.6.1.5 are invoked with the luma location ( xC, yC ) specifying the top-left luma sample of the current coding unit relative to the top left luma sample of the current picture, the coding unit size log2CUSize ,the array bS and the array FM as inputs and the modified reconstructued picture as output.

#### 8.6.1.4 Derivation process of filtering mode

Inputs of this process are:

–	a luma location ( xC, yC ) specifying the top-left luma sample of the current coding unit relative to the top-left luma sample of the current picture,

–	a variable log2CUSize specifying the size of the current coding unit,

–	a two-dimensional arrays of size (nS)x(nS), horEdgeFlags and verEdgeFlags.

Output of this process is an array of size (2)x(nS)x(nS), FM specifying the filtering mode.

Let ( xEk, yEj ) with k = 0..nE-1 and j = 0..nE-1 specify a set of edge sample locations where nE is set equal to ( ( 1 << log2CUSize ) >> 2 ), xE0 = 0, yE0 = 0, xEk+1 = xEk + 4 and yEj+1 = yEj + 4.

For ( xEk, yEj ) with k %4  is equal to  0 or j%4 is equal to 0, the following applies.

- If horEdgeFlags[ xEk ][ yEj ] is equal to 1,

- Set sample p0 = recPicture[ xC + xEk ][ yC + yEj – 1 ] and q0 = recPicture[ xC + xEk ][ yC + yEj ].

- The variable filterDir is set equal to 1.

- Otherwise, if verEdgeFlags[ xEk ][ yEj ] is equal to 1,

- Set sample p0 = recPicture[ xC + xEk – 1 ][ yC + yEj ] and q0 = recPicture[ xC + xEk ][ yC + yEj ].

- The variable filterDir is set equal to 0.

- Depending on the value of filterDir, the variable FM[ filterDir ][ xEk ][ yEj ] is derived as follows.

- If the following conditions are ture, the variable FM[ filterDir ][ xEk+i ][ yEj+h ], i,h = 0,..,3, is set equal to 1, specifying the filtering mode is ModeB.

- The  transform unit TU0 which containts q0 and the trandform unit TU1 which contarints p0 are both in intra-   code coding unit or in inter-code coding unit with 2Nx2N prediction unit.

- The smaller transform size between TU0 and TU1 is larger than 8.

- The larger thransform size between TU0 and TU1 is larger than 16.

- The variable boundaryMeanP and boundaryMeanQ are derived as follows.

- If horEdgeFlags[ xEk ][ yEj ] is equal to 1,

- Set sample pmn and qmn as follows,

- pmn = recPicture[ xC + xEk + m ][ yC + yEj – 1 – n ]   m, n = 0,1,…,15.

- qmn = recPicture[ xC + xEk  + m ][ yC + yEj + n ]  m, n = 0,1,…,15.

- Otherwise, If verEdgeFlags[ xEk ][ yEj ] is equal to 1,

- Set sample pmn and qmn as follows,

- pmn = recPicture[ xC + xEk – 1 – m ][ yC + yEj + n ]   m, n = 0,1,…,15.

- qmn = recPicture[ xC + xEk  + m ][ yC + yEj + n ]  m, n = 0,1,…,15.

- boundaryMeanP = ( ∑pm0  + ∑ p0n + ∑ p15n+ ∑ pm15) >> 6 m, n = 0,1,…,15.

- boundaryMeanQ = (∑qm0  + ∑q0n + ∑q15n + ∑qm15) >>6 m, n = 0,1,…,15.

- The variable boundaryVarP and boundaryVarQ are derived as follows.

- If horEdgeFlags[ xEk ][ yEj ] is equal to 1,

- Set sample pmn and qmn as follows,

- pmn = recPicture[ xC + xEk + m ][ yC + yEj – 1 – n ]   m, n = 0,1,…,15.

- qmn = recPicture[ xC + xEk  + m ][ yC + yEj + n ]  m, n = 0,1,…,15.

- Otherwise, If verEdgeFlags[ xEk ][ yEj ] is equal to 1,

- Set sample pmn and qmn as follows,

- pmn = recPicture[ xC + xEk – 1 – m ][ yC + yEj + n ]   m, n = 0,1,…,15.

- qmn = recPicture[ xC + xEk  + m ][ yC + yEj + n ]  m, n = 0,1,…,15.

- boundaryVarP = ( ∑ abs ( pmn – boundaryMeanP ))>>7

- When m = 0,2,4..14 n = 0, 2, 4…14;  when m = 1,3,5..15 n = 1,3,5…,15.

- boundaryVarQ = ( ∑ abs ( qmn – boundaryMeanQ ))>>7

- When m = 0,2,4..14 n = 0, 2, 4…14;  when m = 1,3,5..15 n = 1,3,5…,15.

- The sum of boundaryVarP and boundaryVarQ is smaller than 2.

- Otherwise, the variable FM[ filterDir ][ xEk+i ][ yEj+h ], i,h = 0,..,3, is set equal to 0.

#### 8.6.1.5 Filtering process for coding unit

Inputs of this process are:

–	a luma location ( xC, yC ) specifying the top-left luma sample of the current coding unit relative to the top left luma sample of the current picture,

–	a variable log2CUSize specifying the coding unit size,

–	an array bS specifying the boundary filtering strength.

–	an array bFM specifying the boundary filtering strength.

Output of this process is:

–	modified reconstruction of the picture.

The filtering process for luma edges in the current coding unit consists of the following ordered steps:

- The variable nD is set equal to 1 << ( log2CUSize – 3 ).

- All elements of the three-dimensional array of size (2)x(nD)x(nD), dEdge are initialized to zero.

- All elements of the three-dimensional array of size (2)x(nD)x(1<<log2CUSize), dSample are initialized to zero.

- All elements of the three-dimensional array of size (2)x(nD)x(nD), bStrength ae initialized to zero.

- All elements of the three-dimensional array of size (2)x(nD)x(nD), bFilterMode are initialized to zero.

- For xDk set equal to xC+( k << 3 ), k=0..nD – 1, the following applies:

- For yDm set equal to yC+( m << 3 ), m=0..nD – 1, the following ordered steps apply:

    - Boundary filtering strength bSVer is derived as follows:

bSVer = Max( bS[ 0 ][ xDk ][ yDm + i ] ) for i = 0..7 	(8-1)

    - bStrength[1][k][m] is set equal to bSVer.

    - bFilterMode[1][k][m] is set equal to bFM[ 0 ][ xDk ][ yDm ].

    - Decision process for luma block edge in subclause Error: Reference source not found is invoked with the luma location of the coding unit ( xC, yC ), the luma location of the block ( xDk, yDm ), a variable verticalEdgeFlag set equal to 1, and the boundary filtering strength bSVer as inputs and the decision dEdge[1][k][m] and an array dS of size (8) as outputs.

    - dSample[1][k][(m<<3)+i] is set equal to dS[i] for i=0..7.

    - Boundary filtering strength bSHor is derived as follows:

bSHor = Max( bS[ 1 ][ xDk + i ][ yDm ] ) for i = 0..7 	(8-2)

    - bStrength[0][k][m] is set equal to bSHor.

    - bFilterMode[0][k][m] is set equal to bFM[ 1 ][ xDk ][ yDm ].

    - Decision process for luma block edge in subclause Error: Reference source not found is invoked with the luma location of the coding unit ( xC, yC ), the luma location of the block ( xDk, yDm ), a variable verticalEdgeFlag set equal to 0, the boundary filtering strength bSHor as inputs, the decision dEdge[0][k][m] and an array dS of size (8) as outputs.

    - dSample[0][m][(k<<3)+i] is set equal to dS[i] for i=0..7.

- For xDk set equal to xC+( k << 3 ), k=0..nD - 1, the following applies:

- For yDm set equal to yB+( m << 3 ), m=0..nD – 1, the following ordered steps apply:

    - dS[i] is set equal to dSample[1][k][(m<<3)+i] for i=0..7.

    - Filtering process for luma block edge in subclause  is invoked with the luma location of the coding unit ( xC, yC ), the luma location of the block ( xDk, yDm ), a variable verticalEdgeFlag set equal to 1, the boundary filtering strength bStrength[1][k][m], the decision dEdge[1][k][m], the array of size (8), dS and the filter mode bFilterMode[1][k][m] as inputs and the modified luma picture buffer as outputs.

- For yDm set equal to yC+( m << 3 ), m=0..nD - 1, the following applies:

- For xDk set equal to xC+( k << 3 ), k=0..nD – 1, the following ordered steps apply:

    - If xDk is equal to 0, the parameter xPOS is set equal to 1. If xDk is equal to xB+( (nD-1)<<3), xPOS is set equal to 2. If xDk is equal to xB+( (nD-2)<<3), xPOS is set equal to 3. Otherwize xPOS is set to 0.

    - dS[i] is set equal to dSample[0][m][ (k << 3) + i ] for i = 0..7.

    - Filtering process for luma block edge in subclause  is invoked with the luma location of the coding unit ( xC, yC ), the luma location of the block ( xDk, yDm ), a variable verticalEdgeFlag set equal to 0, the boundary filtering strength bStrength[0][k][m], the decision dEdge[0][k][m], and the array of size (8), dS, the filter mode bFilterMode[0][k][m], xPOS, dSL[m][], dEL[m][], bSL[m][],  tCL[m][] , bFML[m][] and bCuSizeL, as inputs and the modified luma picture buffer as output.

- The elements of the two dimensional array of size (12)x(nD), dSL are set as follows.

If  Log2CUSize is equal to 3:

dSL[m][i] , i = 0…3 are set to dSample[0][m][ (k << 3) +j], j = 4…7 respectively.

Other elements in dSL[m][i] are set to 0;

If  Log2CUSize is more than 3:

dSL[m][i] , i = 0…7 are set to dSample[0][m][ (k << 3) +j], j = 0…7 respectively.

dSL[m][i] , i = 9…11 are set to dSample[0][m][ ((k-1) << 3) +j], j = 4…7  respectively.

- The elements of the array of size (2)x(nD), dEL are set as follows.

dEL[m][1] is set equal to dEdge[0][k][m].

dEL[m][0] is set equal to dEdge[0][k-1][m].

- The elements of the array of size (2)x(nD), bSL are set as follows.

bSL[m][1] is set equal to bStrength[0][k][m].

bSL[m][0] is set equal to bStrength[0][k-1][m].

- The elements of the array of size (2)x(nD), tC are set as follows.

If bSL[m][1] is greater than 2, the variable tC is specified as Error: Reference source not found with luma quantization parameter Clip3(0, 55, qPL + 2 ) as input, Otherwise (bSL[m][1] is equal or less than 2), the variable tC is specified as Error: Reference source not found with luma quantization parameter qPL as input.

tCL[m][1] is set equal to tc.

If bSL[m][0] is greater than 2, the variable tC is specified as Error: Reference source not found with luma quantization parameter Clip3(0, 55, qPL + 2 ) as input,  Otherwise (bSL[m][0] is equal or less than 2), the variable tC is specified as Error: Reference source not found with luma quantization parameter qPL as input.

tCL[m][0] is set equal to equal to tc.

- The elements of the array of size (2)x(nD), bFML are set as follows.

bFML[m][0] is set equal to bFilterMode[0][k][m].

bFML[m][1] is set equal to bFilterMode[0][k-1][m].

- The variable bCuSizeL is set as follows. If Log2CUSize is equal to 3, bCuSizeL is set equal to 0, otherwise bCuSizeL is set equal to 1.

The filtering process for chroma edges in the current coding unit consists of the following ordered steps:

……

##### 8.6.5.2 Filtering process for luma block edge

Inputs of this process are:

–	a luma location ( xC, yC ) specifying the top-left luma sample of the current coding unit relative to the top left luma sample of the current picture,

–	a luma location ( xB, yB ) specifying the top-left luma sample of the current block relative to the top left luma sample of the current coding unit,

–	a variable verticalEdgeFlag,

–	a variable bS specifying the boundary filtering strength,

–	a variable dE containing a decision,

–	one-dimensional array of size (8), dS containing decisions,

–	an variable FM specifying the filtring mode,

–	 two-dimensional array of (1), bSL,

–	 two-dimensional array of (1), tCL,

–	two-demensinal array of (1) bFML,

–	a variable bCuSizeL

Output of this process is:

–	modified reconstruction of the picture.

Let s’ represent the luma sample array recPictureL of the current picture.

Depending on pcm_flag, a variable β is specified as follows:

–	If pcm_flag is equal to 1, the variables β is specified as Error: Reference source not found with luma quantization parameter 0 as input.

–	Otherwise, the variables β is specified as Error: Reference source not found with luma quantization parameter qPL as input.

A variable tC is specified as follows:

–	If bS is greater than 2, the variable tC is specified as Error: Reference source not found with luma quantization parameter ( qPL + 4 ) as input,

–	Otherwise (bS is equal or less than 2), the variable tC is specified as Error: Reference source not found with luma quantization parameter qPL as input.

Depending on verticalEdgeFlag, the following applies:

–	If verticalEdgeFlag is equal to 1, the following ordered steps apply:

Depending on FM and dS[k], the following orederd steps apply:

- If the FM is equal to 0 or the FM is equal to 1 and any elemnt in dS[k] is 0, k=0…7, the          following ordered steps apply:

- The sample values pi,k and qi,k with i = 0..3 and k = 0..7 are derived as follows:

qi,k = s’[ xC + xB +i, yC + yB + k ]

pi,k = s’[ xC + xB – i – 1, yC + yB + k ]

- If dE is not equal to 0, for each sample location ( xC + xB, yC + yB + k ), k = 0..7, the following ordered steps apply:

- The filtering process for a luma sample specified in subclause Error: Reference source not found is invoked with sample values pi,k, qi,k with i = 0..3, the decision dS[k], variables dEp1 and dEq1, the boundary filtering strength bS and the variable tC as inputs and the number of filtered samples nDp and nDq from each side of the block boundary, and the filtered sample values pi’ and qj’ as outputs.

- The filtered sample values pi’ and qj’ with i = 0..nDp – 1, j = 0..nDq – 1 replace the corresponding samples inside the sample array s’ as follows:

s’[ xC + xB +i, yC + yB + k ] = qi’

s’[ xC + xB – i – 1, yC + yB + k ] = pi’

−  Otherwise( FM is eqanl to 1 and all elements in dS[k] are 1), the following ordered sptes apply:

- The sample values pi,k and qi,k with i = 0..N-1 and k = 0..7 are derived as follows:

qi,k = s’[ xC + xB +i, yC + yB + k ]

pi,k = s’[ xC + xB – i – 1, yC + yB + k ]

N is decided by qPL, when qPL <= 30, N is equal to 8, otherwise N is equal to 12.

- If dE is not equal to 0, for each sample location ( xC + xB, yC + yB + k ), k = 0..7, the following ordered steps apply:

- The filtering process for a luma sample specified in subclause 8.6.1.5.7 is invoked with sample values pi,k, qi,k with i = 0..N-1, and the filtered sample values pi’ and qi’ as outputs.

- The filtered sample values pi’ and qi’ with i = 0..N – 1 replace the corresponding samples inside the sample array s’ as follows:

s’[ xC + xB +i, yC + yB + k ] = qi’

s’[ xC + xB – i – 1, yC + yB + k ] = pi’

–	Otherwise (verticalEdgeFlag is equal to 0), the following ordered steps apply:

- Depending on xPOS , Log2CUSize and bCuSizeL, the parameter ks and  ke are set as follows:

-  If xPOS is equal to 0:

- If bCuSizeL is equal to 0, the ks is set to – 4, otherwise ks is set to – 12.

- If Log2CUSize is equal to 3 or 4, the ke is set to 3, otherwise ke is set to 7.

-  If xPOS is equal to 2:

The filtering processing in this subsection is skipped.

-  If xPOS is equal to 3:

The  ks is set to 0 and the ke is set to 3;

-  Otherwize, the ks is set to 0 and the ke is set to 7.

- The sample values pi,k and qj,k with i = 0..3, j = 0..7 and k = ks..ke are derived as follows:

qj,k = s’[ xC + xB +k, yC + yB + j ]

pi,k = s’[ xC + xB +k, yC + yB – i – 1 ]

- If ks is equal to -4 and dEL[1] is not equal to 0, for each sample location ( xC + xB + k, yC + yB ), k = -4  ..-1,the following ordered steps apply:

- If the bFML[1] is equal to 0 or the bFML[1] is equal to 1 and any elemnt in dSL[k][1] is 0,    k=0…7, the following ordered steps apply:

- The filtering process for a luma sample specified in subclause Error: Reference source not found is invoked with sample values pi,k, qi,k with i = 0..3, decision dSL[-k-1], variables dEp1 and dEq1, the boundary filtering strength bSL and the variable tCL as inputs and the number of filtered samples nDp and nDq from each side of the block boundary and the filtered sample values pi’ and qj’ as outputs.

- The filtered sample values pi’ and qj’ with i = 0..nDp – 1, j = 0..nDq – 1 replace the corresponding samples inside the sample array s’ as follows:

s’[ xC + xB +k, yC + yB + i ] = qi’

s’[ xC + xB +k, yC + yB – i – 1 ] = pi’

- Otherwise (bFML[1] is eqanl to 1 and all elements in dSL[k][1] are 1), the following          ordered sptes apply:

- The filtering process for a luma sample specified in subclause Error: Reference source not found is invoked with sample values pi,k, qj,k with i = 0..3 and j = 0…7, and the filtered sample values pi’ and qi’ as outputs.

- The filtered sample values pi’ and qi’ with i = 0..3 and j = 0…N-1 replace the corresponding samples inside the sample array s’ as follows:

s’[ xC + xB +i, yC + yB + k ] = qi’

s’[ xC + xB – i – 1, yC + yB + k ] = pi’

The variable N is set as follows. If qPL is equal or less than 30, N is equal to 4; otherwise N is equal to 8.

- If ks is equal to less than -4 and dEL[1] is not equal to 0, for each sample location ( xC + xB + k, yC + yB ), k = - 8 ... - 1,the following ordered steps apply:

- If the bFML[1] is equal to 0 or the bFML[1] is equal to 1 and any elemnt in dSL[k][1] is 0, k=0…7, the following ordered steps apply:

- The filtering process for a luma sample specified in subclause Error: Reference source not found is invoked with sample values pi,k, qi,k with i = 0..3, decision dSL[-k-1], variables dEp1 and dEq1, the boundary filtering strength bSL and the variable tCL as inputs and the number of filtered samples nDp and nDq from each side of the block boundary and the filtered sample values pi’ and qj’ as outputs.

- The filtered sample values pi’ and qj’ with i = 0..nDp – 1, j = 0..nDq – 1 replace the corresponding samples inside the sample array s’ as follows:

s’[ xC + xB +k, yC + yB + i ] = qi’

s’[ xC + xB +k, yC + yB – i – 1 ] = pi’

- Otherwise (bFML[1] is eqanl to 1 and all elements in dSL[k][1] are 1), the following ordered    sptes apply:

- The filtering process for a luma sample specified in subclause Error: Reference source not found is invoked with sample values pi,k, qj,k with i = 0..3and j = 0…7, and the filtered sample values pi’ and qi’ as outputs.

- The filtered sample values pi’ and qi’ with i = 0..3 and j = 0…N-1 replace the corresponding samples inside the sample array s’ as follows:

s’[ xC + xB +i, yC + yB + k ] = qi’	                                                                                          (8-3)

s’[ xC + xB – i – 1, yC + yB + k ] = pi’

The variable N is set as follows. If qPL is equal or less than 30, N is equal to 4;     otherwise N is equal to 8.

- If ks is equal to less than -4 and dEL[1] is not equal to 0, for each sample location ( xC + xB + k, yC + yB ), k = - 12  ..-  9,the following ordered steps apply:

- If the bFML[0] is equal to 0 or the bFML[0] is equal to 1 and any elemnt in dSL[k][0] is 0,   k=0…7, the following ordered steps apply:

- The filtering process for a luma sample specified in subclause Error: Reference source not found is invoked with sample values pi,k, qi,k with i = 0..3, decision dSL[-k-1], variables dEp1 and dEq1, the boundary filtering strength bSL and the variable tCL as inputs and the number of filtered samples nDp and nDq from each side of the block boundary and the filtered sample values pi’ and qj’ as outputs.

- The filtered sample values pi’ and qj’ with i = 0..nDp – 1, j = 0..nDq – 1 replace the corresponding samples inside the sample array s’ as follows:

s’[ xC + xB +k, yC + yB + i ] = qi’

s’[ xC + xB +k, yC + yB – i – 1 ] = pi’

- Otherwise (bFML[1] is eqanl to 1 and all elements in dSL[k][1] are 1), the following ordered  sptes apply:

- The filtering process for a luma sample specified in subclause Error: Reference source not found is invoked with sample values pi,k, qj,k with i = 0..3and j = 0…7, the filtered sample values pi’ and qi’ as outputs.

- The filtered sample values pi’ and qi’ with i = 0..3 and j = 0…N-1 replace the corresponding samples inside the sample array s’ as follows:

s’[ xC + xB +i, yC + yB + k ] = qi’

s’[ xC + xB – i – 1, yC + yB + k ] = pi’

The variable N is set as follows. If qPL is equal or less than 30, N is equal to 4; otherwise N is equal to 8.

- If dE is not equal to 0, for each sample location ( xC + xB + k, yC + yB ), k = 0.. ke , the following ordered steps apply:

- If the FM is equal to 0 or the FM is equal to 1 and any elemnt in dS[k] is 0, k=0…7, the following ordered steps apply:

- The filtering process for a luma sample specified in subclause Error: Reference source not found is invoked with sample values pi,k, qi,k with i = 0..3, decision dSL[-k-1], variables dEp1 and dEq1, the boundary filtering strength bSL and the variable tCL as inputs and the number of filtered samples nDp and nDq from each side of the block boundary and the filtered sample values pi’ and qj’ as outputs.

- The filtered sample values pi’ and qj’ with i = 0..nDp – 1, j = 0..nDq – 1 replace the    corresponding samples inside the sample array s’ as follows:

s’[ xC + xB +k, yC + yB + i ] = qi’

s’[ xC + xB +k, yC + yB – i – 1 ] = pi’

- Otherwise( FM is eqanl to 1 and all elements in dS[k] are 1), the following ordered sptes  apply:

- The filtering process for a luma sample specified in subclause Error: Reference source not found is invoked with sample values pi,k, qj,k with i = 0..3and j = 0…7, and the number of filtered samples nD and the filtered sample values pi’ and qi’ as outputs.

- The filtered sample values pi’ and qi’ with i = 0..3 and j = 0…N-1 replace the corresponding samples inside the sample array s’ as follows:

s’[ xC + xB +i, yC + yB + k ] = qi’

s’[ xC + xB – i – 1, yC + yB + k ] = pi’

The variable N is set as follows. If qPL is equal or less than 30, N is equal to 4; otherwise N is equal to 8.

##### 8.6.1.5.7 Horizanl Luma Filtering process for large-size blocks

Inputs of this process are:

–	sample values, pi and qi with i = 0… N – 1,

Output of this process is:

–	filtered sample values, pi’ and qi’ with i = 0… N – 1,

- If qPL is equal or less than 30, the following ordered steps apply:

pi’ = ( ∑ pn + ∑ qm ) / N,  n = 0,..,i+N/2, m = 0,.., N/2 – i – 2

qi’ = ( ∑ pn + ∑ qm ) / N,  n = 0,.., N/2 – i – 1, m = 0,..,i+N/2 – 1

- Otherwise, the following ordered steps apply:

p0’ =  ( ∑ pn + ∑ qm + 8 ) >> 4, n = 0…8, m = 0…6

p1’ =  ( ∑ pn + ∑ qm + 8 ) >> 4, n = 0…9, m = 0…5

p2’ =  ( ∑ pn + ∑ qm + 8 ) >> 4, n = 0…10, m = 0…4

p3’ =  ( ∑ pn + ∑ qm + 8 ) >> 4, n = 0…11, m = 0…3

p4’ =  ( p11 + ∑ pn + ∑ qm + 8 ) >> 4, n = 0…11, m = 0…2

p5’ =  ( 2*p11 + ∑ pn + ∑ qm + 8 ) >> 4, n = 0…11, m = 0…1

p6’ =  ( 3*p11 + ∑ pn + q0 + 8 ) >> 4, n = 0…11

p7’ =  ( 4*p11 + ∑ pn + 8 ) >> 4, n = 0…11

q0’=  ( ∑ pn + ∑ qm + 8 ) >> 4, n = 0…7, m = 0…7

q1’=  ( ∑ pn + ∑ qm + 8 ) >> 4, n = 0…6, m = 0…8

q2’=  ( ∑ pn + ∑ qm + 8 ) >> 4, n = 0…5, m = 0…9

q3’=  ( ∑ pn + ∑ qm + 8 ) >> 4, n = 0…4, m = 0…10

q4’=  ( ∑ pn + ∑ qm + 8 ) >> 4, n = 0…3, m = 0…11

q5’=  ( ∑ pn + ∑ qm + q11 + 8 ) >> 4, n = 0…2, m = 0…11

q6’=  ( ∑ pn + ∑ qm + 2*q11 + 8 ) >> 4, n = 0…1, m = 0…11

q7’=  (  p0 + ∑ qm + 3*q11 + 8 ) >> 4, m = 0…11

The variagble N is decided by qPL, when qPL <= 30, N is equal to 8, otherwise N is equal to 12.

Each of the filtered sample values, pi’ with i = 0…nDp-1, is substituted by the corresponding input sample value pi if all of the following conditions are true.

–	pi is a sample of an I_PCM block.

–	pcm_loop_filter_disable_flag value is equal to 1.

Similarly, each of the filtered sample values, qj’ with j = 0…nDq-1, is substituted by the corresponding input sample value qj if all of the following conditions are true.

–	qj is a sample of an I_PCM block.

–	pcm_loop_filter_disable_flag value is equal to 1.

[Ed. (WJ): for PCM case, deblocking filter applies first and the filtered pixels are restored. Rather than this, it’s better to skip the filtering itself for PCM samples since first filtering is actually not needed.]

##### 8.6.1.5.8 Vertical Luma Filtering process for large-size blocks

Inputs of this process are:

–	sample values, pi and qj with i = 0… 3 and j = 0…7,

Output of this process is:

–	filtered sample values, pi’ and qj’ with i = 0… 3 and j = 0…N-1,

- If qPL is equal or less than 30, the following ordered steps apply:

p0’ =  ( p3 + ∑ pn + ∑ qm + 4 ) >> 3, n = 0…3, m = 0…2

p1’ =  ( 2*p3 + ∑ pn + ∑ qm + 4 ) >> 3, n = 0…3, m = 0…1

p2’ =  ( 3*p3 + ∑ pn +  q0 + 4 ) >> 3, n = 0…3

p3’ =  ( 4*p3 + ∑ pn + 4 ) >> 3, n = 0…3

q0’=  ( ∑ pn + ∑ qm + 4 ) >> 3, n = 0…3, m = 0…3

q1’=  ( ∑ pn + ∑ qm + q3 + 4 ) >> 3, n = 0…2, m = 0…3

q2’=  ( ∑ pn + ∑ qm + 2*q3 + 4 ) >> 3, n = 0…1, m = 0…3

q3’=  (  p0  + ∑ qm + 3*q3 + 4 ) >> 3, m = 0…3

- Otherwise, the the following ordered steps apply:

p0’ =  ( 5*p3 + ∑ pn + ∑ qm + 8 ) >> 4, n = 0…3, m = 0…6

p1’ =  ( 6*p3 + ∑ pn + ∑ qm + 8 ) >> 4, n = 0…3, m = 0…5

p2’ =  ( 7*p3 + ∑ pn + ∑ qm + 8 ) >> 4, n = 0…3, m = 0…4

p3’ =  ( 8*p3 + ∑ pn + ∑ qm + 8 ) >> 4, n = 0…3, m = 0…3

q0’=   ( 4*p3 + ∑ pn + ∑ qm + 8 ) >> 4, n = 0…3, m = 0…7

q1’=   ( 3*p3 + ∑ pn + ∑ qm + q7 + 8 ) >> 4, n = 0…3, m = 0…7

q2’=   ( 2*p3 + ∑ pn + ∑ qm + 2*q7 + 8 ) >> 4, n = 0…3, m = 0…7

q3’=   ( p3 + ∑ pn + ∑ qm + 3*q7 + 8 ) >> 4, n = 0…3, m = 0…7

q4’=   ( ∑ pn + ∑ qm + 4*q7 + 8 ) >> 4, n = 0…3, m = 0…7

q5’=   ( ∑ pn + ∑ qm + 5*q7 + 8 ) >> 4, n = 0…2, m = 0…7

q6’=   ( ∑ pn + ∑ qm + 6*q7 + 8 ) >> 4, n = 0…1, m = 0…7

q7’=   (  p0 + ∑ qm + 7*q7 + 8 ) >> 4, m = 0…7

The variable N is set as follows. If qPL is equal or less than 30, N is equal to 4; otherwise N is equal  to 8.

Each of the filtered sample values, pi’ with i = 0…nDp-1, is substituted by the corresponding input sample value pi if all of the following conditions are true.

–	pi is a sample of an I_PCM block.

–	pcm_loop_filter_disable_flag value is equal to 1.

Similarly, each of the filtered sample values, qj’ with j = 0..nDq-1, is substituted by the corresponding input sample value qj if all of the following conditions are true.

–	qj is a sample of an I_PCM block.

–	pcm_loop_filter_disable_flag value is equal to 1.

[Ed. (WJ): for PCM case, deblocking filter applies first and the filtered pixels are restored. Rather than this, it’s better to skip the filtering itself for PCM samples since first filtering is actually not needed.]