| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G413 |
| --- | --- |

| Title: | Modified probability update and table removal for multi-parameter CABAC update (F254) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | C. Rosewarne 1 Thomas Holt Drive, North Ryde, NSW 2113, Australia. | Tel: Email: | +61 2 9805 2844 chris.rosewarne@cisra.canon.com.au |
|  | M. Maeda 30-2, Shimomaruko 3-chome, Ohta-ku, Tokyo 146-8501, Japan. | Tel: Email: | +81-3-3758-2111 maeda.mitsuru@canon.co.jp |
| Source: | Canon Information Systems Research Australia (CiSRA), Canon Inc. |  |  |

_____________________________

# Abstract

This contribution presents a method of updating probability estimates of a context model that enables the substitution of the look-up table required for determining an offset for the range update with a function. The intention of removing the look-up table is to reduce area for hardware implementations. The intention of the modification to the probability estimate updating method is to remove one operation from the substituted function, which we assert provides a timing benefit for hardware implementation. When implemented on the multi-parameter version of JCTVC-F254 integrated in HM-4.0, the proposed technique has a 0.01% increase in IA-HE, a 0.02% increase in RA-HE, a 0.00% effect on LB-HE and a 0.01% increase in LP-HE.

# Introduction

The JCTVC-F254 contribution by Samsung introduces a new scheme for determining the probability update mechanism. The new update mechanism introduces a look-up table to convert the probability estimate into a threshold for comparison within the arithmetic encoder or decoder. This contribution presents a method for replacing the look-up table with a multiplication operation and altering the probability update mechanism such that a clipping operation, which was previously embedded in the look-up table contents, no longer needs to be performed when the multiplication operation is in use. This modification results in a small reduction in the dynamic range of each probability estimate, however the performance impact is asserted to be negligible and the hardware implementation is asserted to be improved firstly, by removal of the look-up table and secondly, by removing the clipping operation from the critical logic path in the range register.

# Modified probability update method

In CABAC, it is necessary to determine a comparison threshold for the arithmetic decode operation. This comparison threshold is determined by subtracting an offset from a range value. The offset is dependent on the current range value and the probability estimate (for F254, this may be either a single probability estimate per context or two probability estimates per context). Among other changes, two alterations made in F254 with respect to CABAC.  Firstly, the ‘valMPS’ concept was removed, so now each probability estimate covers the entire [0, 1] interval, instead of [0, 0.5]. Secondly, the probability states are now evenly spaced over the [0, 1] interval, instead of mapped to probabilities via a mapping function having an exponential decay. Accordingly, the probability state may now be considered as a quantized probability estimate.

The F254 implementation determines an offset (“uiLPS”) value by performing a table look-up to a two dimensional table. The first dimension index is created by truncating the combined probability from 15-bits to 9-bits. The second dimension index is created by truncating the 9-bit range value to 6-bits (the MSB is required to always be ‘1’ and is ignored). The table contains 9-bit entries, resulting in an overall table size of 288Kb. Although it is possible to reduce the table size by increasing the truncation of the probability estimate or the range value, this would reduce the coding efficiency gains observed in F254, while still leaving a relatively large table in the design. From a hardware implementation perspective, look-up tables are generally only preferred if they model a function that would be difficult to implement otherwise, as area is a key cost driver.

This contribution firstly replaces the look-up table with a 6-bit by 9-bit multiplication. The 9-bit truncated probability estimate is inverted prior to being provided to the multiplier. The 15-bit multiplier output is truncated back to a 9-bit offset value. We assert that a multiplier of this size is much simpler to implement than an equivalent table.

This contribution secondly introduces a minimum clipping operation into the updateLPS function for updating the probability estimates in the context model.

The previous updateLPS function is defined as:

iP0 -= iP0 >> ALPHA0

iP1 -= iP1 >> ALPHA1

The proposed updateLPS function is defined as:

iP0 = max(iP0 – iP0 >> ALPHA0, 2<<6)

iP0 = max(iP1 – iP1 >> ALPHA1, 2<<6)

The purpose of the probability estimate clipping operation is to restrict the lower bound of the truncated probability. After inversion, this restricts the maximum offset value to maintain a required minimum difference between range and offset (δ in F254).

# Experiment Results

Results are presented in Table 1 and Table 2 below. Refer to the accompanying spreadsheet for results with greater precision. The CE1_F254_MULTI_source software (which implements F254 in HM-4.0 and was provided to CE1 participants) was used for the evaluation. Execution took place on a Linux cluster using GCC 3.4.6.

Table 1. Results for Modified probability update for multi-parameter CABAC update (relative to HM-4.0).

|  | All Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | -1.0% | -0.8% | -0.9% |
| Class B | -0.9% | -1.2% | -1.2% |
| Class C | -0.9% | -1.2% | -1.3% |
| Class D | -0.9% | -1.7% | -1.7% |
| Class E | -0.9% | -2.0% | -1.9% |
| Class F | -0.6% | -0.9% | -0.8% |
| Overall (A-F) | -0.9% | -1.3% | -1.3% |
|  | -0.9% | -1.3% | -1.3% |
| Overall (A-E) | -0.9% | -1.4% | -1.4% |
|  | -0.9% | -1.3% | -1.4% |
| Enc Time[%] | 104% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Random Access HE |  |  |
|  | Y | U | V |
| Class A | -1.1% | -0.3% | -0.2% |
| Class B | -0.9% | -0.8% | -0.8% |
| Class C | -0.9% | -0.9% | -1.0% |
| Class D | -0.8% | -1.0% | -1.6% |
| Class E |  |  |  |
| Class F | -0.5% | -0.5% | -0.5% |
| Overall (A-F) | -0.9% | -0.7% | -0.8% |
|  | -0.8% | -0.7% | -0.9% |
| Overall (A-E) | -0.9% | -0.8% | -0.9% |
|  | -0.9% | -0.8% | -0.9% |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay B HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | -0.8% | -0.9% | -1.2% |
| Class C | -0.7% | -0.8% | -0.9% |
| Class D | -0.8% | -1.2% | -1.5% |
| Class E | -1.0% | -2.5% | -1.3% |
| Class F | -0.5% | -0.9% | -0.1% |
| Overall (A-F) | -0.8% | -1.2% | -1.0% |
|  | -0.8% | -1.1% | -0.9% |
| Overall (A-E) | -0.8% | -1.3% | -1.2% |
|  | -0.8% | -1.2% | -1.1% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay P HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | -0.8% | -0.7% | -0.6% |
| Class C | -0.8% | -0.6% | -0.7% |
| Class D | -0.7% | -0.9% | -0.2% |
| Class E | -1.0% | -2.4% | -2.0% |
| Class F | -0.4% | -0.3% | -0.3% |
| Overall (A-F) | -0.7% | -0.9% | -0.7% |
|  | -0.7% | -0.8% | -0.7% |
| Overall (A-E) | -0.8% | -1.0% | -0.8% |
|  | -0.8% | -1.0% | -0.8% |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 102% |  |  |

Table 2. Results for Modified probability update for multi-parameter CABAC update (relative to HM-4.0 with F254 with multi-source).

|  | All Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.0% | -0.1% | -0.2% |
| Class B | 0.0% | -0.1% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | -0.2% | -0.2% |
| Class F | 0.0% | 0.0% | 0.0% |
| Overall (A-F) | 0.0% | -0.1% | -0.1% |
|  | 0.0% | -0.1% | -0.1% |
| Overall (A-E) | 0.0% | -0.1% | -0.1% |
|  | 0.0% | -0.1% | -0.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Random Access HE |  |  |
|  | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | -0.1% | -0.1% |
| Class C | 0.0% | 0.0% | -0.1% |
| Class D | 0.0% | 0.1% | -0.3% |
| Class E |  |  |  |
| Class F | 0.0% | -0.1% | 0.1% |
| Overall (A-F) | 0.0% | 0.0% | -0.1% |
|  | 0.0% | 0.0% | -0.1% |
| Overall (A-E) | 0.0% | 0.0% | -0.1% |
|  | 0.0% | 0.0% | -0.2% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay B HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.0% | -0.1% | -0.1% |
| Class C | 0.0% | -0.1% | 0.2% |
| Class D | 0.0% | -0.1% | -0.5% |
| Class E | -0.1% | -0.5% | -0.9% |
| Class F | 0.0% | -0.3% | 0.0% |
| Overall (A-F) | 0.0% | -0.2% | -0.2% |
|  | 0.0% | -0.2% | -0.1% |
| Overall (A-E) | 0.0% | -0.2% | -0.3% |
|  | 0.0% | -0.2% | -0.2% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Low delay P HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.0% | -0.1% | -0.5% |
| Class C | 0.0% | -0.2% | 0.0% |
| Class D | 0.0% | -0.6% | 0.4% |
| Class E | 0.0% | -1.0% | -1.1% |
| Class F | 0.0% | -0.3% | -0.4% |
| Overall (A-F) | 0.0% | -0.4% | -0.3% |
|  | 0.0% | -0.4% | -0.2% |
| Overall (A-E) | 0.0% | -0.4% | -0.2% |
|  | 0.0% | -0.5% | -0.2% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |

# Summary

The presented scheme enables removal of the uiLPS look-up table for a saving of 288Kb. The modified updateLPS function removes one clamp operation from the uiLPS function and inserts this operation into the probability estimate update operation. The result is minimal impact on coding efficiency.

# References

- A.Alshin, E.Alshina, “Multi-parameter probability up-date for CABAC”, JCTVC-F254, 6th JCT-VC Meeting, Torino, IT, July 2011.

# Patent rights declaration(s)

CiSRA may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Canon Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).