| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, 21-30 November, 2011 | Document: JCTVC-G417 |
| --- | --- |

| Title: | Improved coding of CU-level information in CAVLC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | 1Sang Yong Yi, 2Haechul Choi, 3Sung-Chang Lim, 1Jae-Gon Kim, 3Hui Yong Kim, 3Jinho Lee, 3Jin Soo Choi 1Korea Aerospace University, 100 Hanggongdae-gil, Hwajeon-dong, Deogyang-gu, Goyang-si, Gyeonggi-do, 412-791, Korea 2Hanbat University, Deogmyeong-dong, Yuseong-gu, Daejeon, 305-719, Korea 3ETRI, 218 Gajeong-ro, Yuseong-gu, Daejeon, 305-700, Korea | Tel: Email: | +82-2-300-0414 {ysyysy1921, jgkim} @kau.ac.kr +82-42-821-1149 choihc@hanbat.ac.kr +82-42-860-5772 {sclim, hykim5, jinosoul, jschoi} @etri.re.kr |
| Source: | Korea Aerospace University, Hanbat University, and ETRI |  |  |

_____________________________

# Abstract

This contribution presents a modification of syntax element named cu_split_pred_part_mode in CAVLC. In HM4.0, cu_split_pred_part_mode includes various CU-level syntax elements and a PU-level syntax element of merge_flag for joint coding. This contribution proposes to exclude merge_flag from cu_split_pred_part_mode. It is reported that the average BD-rate of the proposed method are -0.1%, -0.3%, and -0.3% for RA-LC, LB-LC, and LP-LC, respectively, without additional complexity increase. By using the proposed method, the table size for cu_split_pred_part_mode can be reduced and the PU-level syntax can be simplified.

# Introduction

In HM4.0, cu_split_pred_part_mode includes CU-level syntax elements of split_coding_unit_flag, skip_flag, prediction mode, partition type, and a PU-level syntax element of merge_flag for joint coding of those syntax elements. Therefore, merge_flag is encoded/decoded in CU-level or PU-level depending on partition type. In order to encode/decode merge_flag in PU-level only, excluding merge_flag from cu_split_pred_part mode is proposed in this contribution.

# Proposed method

In HM4.0, cu_split_pred_part_mode is specified in three tables depending on the CU size and whether inter NxN prediction is allowed [2]. In this contribution, merge_flag for MODE_INTER and PART_2Nx2N is simply excluded from cu_split_pred_part_mode in CAVLC. Thus, the proposed method removes a line with merge_flag equal to 1 in each table. By removing merge_flag from cu_split_pred_part_mode, the table size for cu_split_pred_part_mode is reduced. According to the CU size and allowInterNxN flag, three modified tables are defined as the following Table 2, 4, and 6, where merge_flag is excluded.

Table 1 cu_split_pred_part_mode table when CUSize > SCU in HM4.0

| - cu_split_pred_ part_mode | - split_coding_unit_flag | - skip_flag | - merge_flag | - PredMode | PartMode |
| --- | --- | --- | --- | --- | --- |
| - 0 | - 1 | - - | - - | - - | - |
| - 1 | - 0 | - 1 | - - | - MODE_SKIP | PART_2Nx2N |
| - 2 | - 0 | - 0 | - 1 | - MODE_INTER | PART_2Nx2N |
| - 3 | - 0 | - 0 | - 0 | - MODE_INTER | PART_2Nx2N |
| - 4 | - 0 | - - | - - | - MODE_INTER | PART_2NxN |
| - 5 | - 0 | - - | - - | - MODE_INTER | PART_Nx2N |
| - 6 | - 0 | - - | - - | - MODE_INTRA | PART_2Nx2N |
| - 7 | - 0 | - - | - - | - MODE_INTER | PART_2NxnU |
| - 8 | - 0 | - - | - - | - MODE_INTER | PART_2NxnD |
| - 9 | - 0 | - - | - - | - MODE_INTER | PART_nLx2N |
| - 10 | - 0 | - - | - - | - MODE_INTER | PART_nRx2N |

Table 2 Proposed cu_split_pred_part_mode table when CUSize > SCU

| - cu_split_pred_ part_mode | - split_coding_unit_flag | - skip_flag | - PredMode | PartMode |
| --- | --- | --- | --- | --- |
| - 0 | - 1 | - - | - - | - |
| - 1 | - 0 | - 1 | - MODE_SKIP | PART_2Nx2N |
| - 2 | - 0 | - 0 | - MODE_INTER | PART_2Nx2N |
| - 3 | - 0 | - - | - MODE_INTER | PART_2NxN |
| - 4 | - 0 | - - | - MODE_INTER | PART_Nx2N |
| - 5 | - 0 | - - | - MODE_INTRA | PART_2Nx2N |
| - 6 | - 0 | - - | - MODE_INTER | PART_2NxnU |
| - 7 | - 0 | - - | - MODE_INTER | PART_2NxnD |
| - 8 | - 0 | - - | - MODE_INTER | PART_nLx2N |
| - 9 | - 0 | - - | - MODE_INTER | PART_nRx2N |

Table 3 cu_split_pred_part_mode table when CUSize == SCU and allowInterNxN is true in HM4.0

| - cu_split_pred_ part_mode | - split_coding_ unit_flag | - skip_flag | - merge_flag | - PredMode | PartMode |
| --- | --- | --- | --- | --- | --- |
| - 0 | - 0 | - 1 | - - | - MODE_SKIP | PART_2Nx2N |
| - 1 | - 0 | - 0 | - 1 | - MODE_INTER | PART_2Nx2N |
| - 2 | - 0 | - 0 | - 0 | - MODE_INTER | PART_2Nx2N |
| - 3 | - 0 | - - | - - | - MODE_INTER | PART_2NxN |
| - 4 | - 0 | - - | - - | - MODE_INTER | PART_Nx2N |
| - 5 (escape symbol) | - 0 | - - | - - | - MODE_INTRA | PART_2Nx2N |
|  |  |  |  | - MODE_INTRA | PART_NxN |
|  |  |  |  | - MODE_INTER | PART_NXN |

Table 4 Proposed cu_split_pred_part_mode table when CUSize == SCU and allowInterNxN is true

| - cu_split_pred_ part_mode | - split_coding_ unit_flag | - skip_flag | - PredMode | PartMode |
| --- | --- | --- | --- | --- |
| - 0 | - 0 | - 1 | - MODE_SKIP | PART_2Nx2N |
| - 1 | - 0 | - 0 | - MODE_INTER | PART_2Nx2N |
| - 2 | - 0 | - - | - MODE_INTER | PART_2NxN |
| - 3 | - 0 | - - | - MODE_INTER | PART_Nx2N |
| - 4 (escape symbol) | - 0 | - - | - MODE_INTRA | PART_2Nx2N |
|  |  |  | - MODE_INTRA | PART_NxN |
|  |  |  | - MODE_INTER | PART_NXN |

Table 5 cu_split_pred_part_mode table when CUSize == SCU and allowInterNxN is false in HM4.0

| - cu_split_pred_ part_mode | - split_coding_ unit_flag | - skip_flag | - merge_flag | - PredMode | PartMode |
| --- | --- | --- | --- | --- | --- |
| - 0 | - 0 | - 1 | - - | - MODE_SKIP | PART_2Nx2N |
| - 1 | - 0 | - 0 | - 1 | - MODE_INTER | PART_2Nx2N |
| - 2 | - 0 | - 0 | - 0 | - MODE_INTER | PART_2Nx2N |
| - 3 | - 0 | - - | - - | - MODE_INTER | PART_2NxN |
| - 4 | - 0 | - - | - - | - MODE_INTER | PART_Nx2N |
| - 5 (escape symbol) | - 0 | - - | - - | - MODE_INTRA | PART_2Nx2N |
|  |  |  |  | - MODE_INTRA | PART_NxN |

Table 6 Proposed cu_split_pred_part_mode table when CUSize == SCU and allowInterNxN is false

| - cu_split_pred_ part_mode | - split_coding_ unit_flag | - skip_flag | - PredMode | PartMode |
| --- | --- | --- | --- | --- |
| - 0 | - 0 | - 1 | - MODE_SKIP | PART_2Nx2N |
| - 1 | - 0 | - 0 | - MODE_INTER | PART_2Nx2N |
| - 2 | - 0 | - - | - MODE_INTER | PART_2NxN |
| - 3 | - 0 | - - | - MODE_INTER | PART_Nx2N |
| - 4 (escape symbol) | - 0 | - - | - MODE_INTRA | PART_2Nx2N |
|  |  |  | - MODE_INTRA | PART_NxN |

In HM4.0, there is a condition check with regard to merge_flag in PU-level syntax as in Table 7. Since merge_flag for MODE_INTER and PART_2Nx2N is excluded from cu_split_pred_part_mode, the PU- level syntax is simplified by removing the condition check as in Table 7. Thus, there is no dependency on entropy_coding_mode and partition type in the parsing of merge_flag.

Table 7 Modified prediction unit syntax for merge_flag

| prediction_unit( x0, y0 , log2CUSize ) { | Descriptor |
| --- | --- |
| if( skip_flag[ x0 ][ y0 ] ) { |  |
| merge_idx[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } else if( PredMode = = MODE_INTRA ) { |  |
| ... |  |
| } else { /* MODE_INTER */ |  |
| if( entropy_coding_mode_flag ¦¦ PartMode != PART_2Nx2N ) |  |
| merge_flag[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| if( merge_flag[ x0 ][ y0 ] ) { |  |
| merge_idx[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } else { |  |
| ... |  |

# Experimental Results

The proposed method was implemented in HM4.0rc1. The experiments are conducted with 3 test configurations as specified in JCTVC-F900 [1]. Table 8 shows the summary result of the proposed method compared to HM4.0rc1 anchor. In the case of random access, the proposed method shows BD-rate reduction of 0.1% on average. The proposed method also provides the average BD-rate reduction of 0.3% for both cases of low delay B and low delay P. From the experimental results, the proposed method achieves higher coding efficiency than HM4.0rc1 for RA-LC, LB-LC and LP-LC without additional encoding/decoding complexity increase.

Table 8 Summary result of the proposed method

|  | Random Access LC |  |  | Low delay B LC |  |  | Low delay P LC |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.3% | 0.1% |  |  |  |  |  |  |
| Class B | -0.1% | -0.1% | 0.0% | -0.1% | -0.3% | -0.1% | -0.3% | -0.1% | -0.4% |
| Class C | -0.2% | -0.1% | -0.1% | -0.3% | -0.2% | -0.5% | -0.3% | -0.1% | -0.2% |
| Class D | -0.2% | -0.1% | -0.1% | -0.4% | -0.5% | -0.5% | -0.3% | -0.1% | -0.9% |
| Class E |  |  |  | -0.4% | -0.1% | -0.4% | -0.4% | 0.0% | -1.0% |
| Overall | -0.1% | 0.0% | 0.0% | -0.3% | -0.3% | -0.4% | -0.3% | -0.1% | -0.6% |
|  | -0.1% | 0.0% | 0.0% | -0.3% | -0.3% | -0.4% | -0.3% | -0.1% | -0.6% |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |

# Conclusions

This contribution proposes to exclude merge_flag from cu_split_pred_part_mode. The experimental results show that the average BD-rate of the proposed method are -0.1%, -0.3%, and -0.3% for RA-LC, LB-LC and LP-LC, respectively, without additional complexity increase. By using the proposed method, the table size for cu_split_pred_part_mode can be reduced and the prediction unit level syntax can be also simplified. It is recommended that the proposed method be integrated into the next version of HM.

# Reference

- F.Bossen, "Common test conditions and software reference configurations," JCTVC-F900, July 2011, Torino, IT

- JCT-VC, “High Efficiency Video Coding (HEVC) text specification Working Draft 4,” JCTVC-F803, JCT-VC Meeting, Torino, IT, 14-22 July, 2011.

# Patent rights declaration(s)

Korea Aerospace University may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Hanbat University may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

ETRI may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# WD changes

(changes relative to WD4-v5 are marked with yellow)

7.3.7	Prediction unit syntax

| prediction_unit( x0, y0, log2CUSize ) { | Descriptor |
| --- | --- |
| if( skip_flag[ x0 ][ y0 ] ) { |  |
| merge_idx[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } else if( PredMode = = MODE_INTRA ) { |  |
| if( PartMode == PART_2Nx2N && log2CUSize >= Log2MinIPCMCUSize ) |  |
| pcm_flag | u(1) ¦ ae(v) |
| if( pcm_flag ) { |  |
| while ( !byte_aligned( ) ) |  |
| pcm_alignment_zero_bit | u(v) |
| for( i = 0; i < 1 << ( log2CUSize << 1 ); i++ ) |  |
| pcm_sample_luma[ i ] | u(v) |
| for( i = 0; i < ( 1 << ( log2CUSize << 1 ) ) >> 1; i++ ) |  |
| pcm_sample_chroma[ i ] | u(v) |
| } else { |  |
| prev_intra_luma_pred_flag[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| if( prev_intra_luma_pred_flag[ x0 ][ y0 ] ) |  |
| mpm_idx[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| else |  |
| rem_intra_luma_pred_mode[ x0 ][ y0 ] | ce(v) ¦ ae(v) |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| SignaledAsChromaDC = ( chroma_pred_from_luma_enabled_flag ? intra_chroma_pred_mode[ x0 ][ y0 ] == 3 : intra_chroma_pred_mode[ x0 ][ y0 ] == 2 ) |  |
| } |  |
| } else { /* MODE_INTER */ |  |
| if( entropy_coding_mode_flag ¦¦ PartMode != PART_2Nx2N ) |  |
| merge_flag[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| if( merge_flag[ x0 ][ y0 ] ) { |  |
| merge_idx[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } else { |  |
| if( slice_type = = B ) { |  |
| if( !entropy_coding_mode_flag ) { |  |
| combined_inter_pred_ref_idx | ue(v) |
| if( combined_inter_pred_ref_idx == MaxPredRef ) |  |
| inter_pred_flag[ x0 ][ y0 ] | ue(v) |
| } else |  |
| inter_pred_flag[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| if( inter_pred_flag[ x0 ][ y0 ] = = Pred_LC ) { |  |
| if( num_ref_idx_lc_active_minus1 > 0 ) { |  |
| if( !entropy_coding_mode_flag ) { |  |
| if( combined_inter_pred_ref_idx == MaxPredRef ) |  |
| ref_idx_lc_minus4[ x0 ][ y0 ] | ue(v) |
| } else |  |
| ref_idx_lc[ x0 ][ y0 ] | ae(v) |
| } |  |
| if( entropy_coding_mode_flag ) { |  |
| mvd_coding_cabac(mvd_lc[ x0 ][ y0 ][ 0 ], mvd_lc[ x0 ][ y0 ][ 1 ]) |  |
| } else { |  |
| mvd_lc[ x0 ][ y0 ][ 0 ] | se(v) |
| mvd_lc[ x0 ][ y0 ][ 1 ] | se(v) |
| } |  |
| mvp_idx_lc[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| else { /* Pred_L0 or Pred_BI */ |  |
| if( num_ref_idx_l0_active_minus1 > 0 ) { |  |
| if( !entropy_coding_mode_flag ) { |  |
| if( combined_inter_pred_ref_idx == MaxPredRef ) |  |
| ref_idx_l0_minusX[ x0 ][ y0 ] | ue(v) |
| } else |  |
| ref_idx_l0_minusX[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| if( entropy_coding_mode_flag ) { |  |
| mvd_coding_cabac(mvd_l0[ x0 ][ y0 ][ 0 ], mvd_l0[ x0 ][ y0 ][ 1 ]) |  |
| } else { |  |
| mvd_l0[ x0 ][ y0 ][ 0 ] | se(v) |
| mvd_l0[ x0 ][ y0 ][ 1 ] | se(v) |
| mvp_idx_l0[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| if( inter_pred_flag[ x0 ][ y0 ] = = Pred_BI ) { |  |
| if( num_ref_idx_l1_active_minus1 > 0 ) { |  |
| if( !entropy_coding_mode_flag ) { |  |
| if( combined_inter_pred_ref_idx == MaxPredRef ) |  |
| ref_idx_l1_minusX[ x0 ][ y0 ] | ue(v) |
| } else |  |
| ref_idx_l1[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| if( entropy_coding_mode_flag ) { |  |
| mvd_coding_cabac(mvd_l1[ x0 ][ y0 ][ 0 ], mvd_l1[ x0 ][ y0 ][ 1 ]) |  |
| } else { |  |
| mvd_l1[ x0 ][ y0 ][ 0 ] | se(v) |
| mvd_l1[ x0 ][ y0 ][ 1 ] | se(v) |
| } |  |
| mvp_idx_l1[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |

7.4.5	Coding tree semantics

cu_split_pred_part_mode[ x0 ][ y0 ] specifies split_coding_unit_flag and when the coding unit is not split the skip_flag[ x0 ][ y0 ], the merge_flag[ x0 ][ y0 ], PredMode and PartMode of a coding unit. The array indices x0 and y0 specify the location ( x0, y0 ) of the top-left luma sample of the coding unit relative to the top-left luma sample of the picture.

The variable allowInterNxN is specified as follows.

- If log2CUSize is greater than 3 or inter_4x4_enabled_flag is equal to 1, allowInterNxN is set to true.

- Otherwise (log2CUSize is equal to 3 and inter_4x4_enabled_flag is equal to 0), allowInterNxN is set to false.

Table 7-9 – Specification of cu_split_pred_part_mode when log2CUSize is greater than Log2MinCUSize

| - cu_split_pred_ part_mode | - split_coding_unit_flag | - skip_flag | - merge_flag | - PredMode | PartMode |
| --- | --- | --- | --- | --- | --- |
| - 0 | - 1 | - - | - - | - - | - |
| - 1 | - 0 | - 1 | - - | - MODE_SKIP | PART_2Nx2N |
| - 2 | - 0 | - 0 | - 1 | - MODE_INTER | PART_2Nx2N |
| - 3 | - 0 | - 0 | - 0 | - MODE_INTER | PART_2Nx2N |
| - 43 | - 0 | - - | - - | - MODE_INTER | PART_2NxN |
| - 54 | - 0 | - - | - - | - MODE_INTER | PART_Nx2N |
| - 65 | - 0 | - - | - - | - MODE_INTRA | PART_2Nx2N |
| - 76 | - 0 | - - | - - | - MODE_INTER | PART_2NxnU |
| - 87 | - 0 | - - | - - | - MODE_INTER | PART_2NxnD |
| - 98 | - 0 | - - | - - | - MODE_INTER | PART_nLx2N |
| - 109 | - 0 | - - | - - | - MODE_INTER | PART_nRx2N |

Table 7-10 – Specification of cu_split_pred_part_mode when log2CUSize is equal to Log2MinCUSize and allowInterNxN is true

| - cu_split_pred_ part_mode | - split_coding_ unit_flag | - skip_flag | - merge_flag | - PredMode | PartMode |
| --- | --- | --- | --- | --- | --- |
| - 0 | - 0 | - 1 | - - | - MODE_SKIP | PART_2Nx2N |
| - 1 | - 0 | - 0 | - 1 | - MODE_INTER | PART_2Nx2N |
| - 2 | - 0 | - 0 | - 0 | - MODE_INTER | PART_2Nx2N |
| - 32 | - 0 | - - | - - | - MODE_INTER | PART_2NxN |
| - 43 | - 0 | - - | - - | - MODE_INTER | PART_Nx2N |
| - 54 (escape symbol) | - 0 | - - | - - | - MODE_INTRA | PART_2Nx2N |
|  |  |  |  | - MODE_INTRA | PART_NxN |
|  |  |  |  | - MODE_INTER | PART_NXN |

Table 7-11 – Specification of cu_split_pred_part_mode when log2CUSize is equal to Log2MinCUSize and allowInterNxN is equal to false

| - cu_split_pred_ part_mode | - split_coding_ unit_flag | - skip_flag | - merge_flag | - PredMode | PartMode |
| --- | --- | --- | --- | --- | --- |
| - 0 | - 0 | - 1 | - - | - MODE_SKIP | PART_2Nx2N |
| - 1 | - 0 | - 0 | - 1 | - MODE_INTER | PART_2Nx2N |
| - 2 | - 0 | - 0 | - 0 | - MODE_INTER | PART_2Nx2N |
| - 32 | - 0 | - - | - - | - MODE_INTER | PART_2NxN |
| - 43 | - 0 | - - | - - | - MODE_INTER | PART_Nx2N |
| - 54 (escape symbol) | - 0 | - - | - - | - MODE_INTRA | PART_2Nx2N |
|  |  |  |  | - MODE_INTRA | PART_NxN |

9.2.2.4	Initialisation process for splitPredPartModeTable

Outputs of this process are initial values of the variable array splitPredPartModeTable and associated counters.

The variable array splitPredPartModeTable is initialized as specified in Table 9-12 .

Table 9-12 – Specification of splitPredPartModeTable[cuDepth][codeNum]

|  | - codeNum |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - cuDepth | - 0 | - 1 | - 2 | - 3 | - 4 | - 5 | - 6 | - 7 | - 8 | - 9 | - 10 |
| - 0 | - 0 | - 1 | - 2 | - 3 | - 4 | - 5 | - 6 | - 7 | - 8 | - 9 | - 10 |
| - 1 | - 0 | - 1 | - 2 | - 3 | - 4 | - 5 | - 6 | - 7 | - 8 | - 9 | - 10 |
| - 2 | - 0 | - 1 | - 2 | - 3 | - 4 | - 5 | - 6 | - 7 | - 8 | - 9 | - 10 |
| - 3 | - 1 | - 2 | - 3 | - 4 | - 5 | - 0 | - 6 | - 7 | - 8 | - 9 | - 10 |

For each cuDepth with a value from 0 to (Log2MaxCUSize – Log2MinCUSize), there are four counters used for codeword index adaptive mapping. These counters are initialized according to subclause Error: Reference source not found.

9.2.4.5	Parsing process for cu_split_pred_part_mode

This process is invoked when entropy_coding_mode_flag is equal to 0 for parsing syntax element cu_split_pred_part_mode in subclause Error: Reference source not found.

Inputs to this process are bits from slice data, splitPredPartModeTable.

Output of this process is the syntax element cu_split_pred_part_mode.

The value of cu_split_pred_part_mode is derived as follows.

- If log2CUSize is greater than Log2MinCUSize, the parsing process described in subclause Error: Reference source not found is invoked with vlcNum of 14 and cMax of 9 as inputs and the variable codeNum as output.

- Otherwise, the parsing process described in subclause Error: Reference source not found is invoked with vlcNum of 14 and cMax of 4 as inputs and the variable codeNum as output.

- The syntax element cu_split_pred_part_mode is derived as

cu_split_pred_part_mode = splitPredPartModeTable[Log2MaxCUSize – log2CUSize][codeNum] 	(9-1)

- If log2CUSize is equal to Log2MinCUSize and cu_split_part_mode has a value of 4, PredMode and PartMode are determined as follows

- Read one bit and assign its value to a variable b

- If b is equal to 1, set PredMode to MODE_INTRA and PartMode to PART_2Nx2N

- Otherwise if inter_4x4_enabled_flag is equal to 0 and log2CUSize is equal to 3, set PredMode to MODE_INTRA and PartMode to PART_NxN

- Otherwise, read the next one bit and assign its value to the variable b.

    - If b is equal to 1, set PredMode to MODE_INTRA and PartMode to PART_NxN

    - Otherwise, set PredMode to MODE_INTER and PartMode to PART_NxN

- The variable array splitPredPartModeTable[Log2MaxCUSize – log2CUSize] is updated by invoking process Error: Reference source not found with splitPredPartModeTable[Log2MaxCUSize – log2CUSize], codeNum and a value of 4 as inputs.