| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G418 |
| --- | --- |

| Title: | Simplification of intra prediction mode mapping method |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jinho Lee, Sung-Chang Lim, Hui Yong Kim, Jin Soo Choi. Realistic Media Research Team, ETRI 218 Gajeongno, Yuseong-gu Daejeon, South Korea | Tel: Email: | +82-42-860-5656 jinosoul@etri.re.kr sclim@etri.re.kr hykim5@etri.re.kr jschoi@etri.re.kr |
| Source: | ETRI (Electronics and Telecommunications Research Institute) |  |  |

_____________________________

# Abstract

This contribution presents simplification of intra prediction mode mapping method. The proposed method replaces neighbouring prediction modes by 'Intra_Planar' when 'IntraPredMode' of the a neighbouring block is greater than or equal to 'intraPredModeNum' of current block. It is reported that the average BD-rate loss of the proposed method is 0.04% in Intra, both of High-Efficiency (HE) and Low-Complexity (LC).

# Introduction

WD4.0[1] supports various number of intra prediction modes according to the block size, i.e., 4 for 64x64, 35 for 32x32, 16x16, 8x8, and 18 for 4x4. In the derivation process for luma intra prediction mode in WD4.0, 'candModeList' is derived from neighbouring blocks. Then, if 'IntraPredMode' of a neighbouring block is greater than or equal to 'intraPredModeNum' of current block, the 'IntraPredMode' is set equal to an another mode by mapping table as represented in Table 1 [1]. Thus, when current block size is 64x64 or 4x4, 'mapIntraPredMode4' and 'mapIntraPredMode10' are used, respectively.

Table 1 : Specification of intra mapping table

| value | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| mapIntraPredMode4[ value ] | 0 | 1 | 2 | 3 | 3 | 3 | 1 | 3 | 2 | 2 | 3 | 3 | 1 | 1 | 3 | 3 | 2 |  |
| mapIntraPredMode10[ value ] | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 3 | 3 | 3 | 3 | 3 | 3 | 3 |  |
| value | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 |
| mapIntraPredMode4[ value ] | 2 | 3 | 3 | 3 | 3 | 1 | 1 | 1 | 3 | 3 | 3 | 3 | 2 | 2 | 2 | 2 | 3 | 3 |
| mapIntraPredMode10[ value ] | 3 | 3 | 4 | 5 | 5 | 1 | 1 | 6 | 6 | 7 | 4 | 8 | 8 | 2 | 2 | 9 | 9 | 3 |

# Proposed methods

The intra prediction mode mapping table of WD 4.0 is restrictively used, i.e., when the 'IntraPredMode' of a neighbouring block is greater than or equal to 'intraPredModeNum' of current block, and the cases occurs when the current block size is 64x64 or 4x4.

This contribution proposes a simple mapping method of intra prediction mode. When the case occurs we replaced the 'IntraPredMode' of a neighbouring block by 'Intra_Planar' (mode 0) mode rather than using the mapping table as illustrated in Fig 1. It does not require a storage to store the mapping table, and mapping table selection (mapIntraPredMode4 or mapIntraPredMode10) according to the current block size is not needed. In addition, WD text is simplified as described in Annex.

Figure 1 : Flowchart of WD4.0 and proposed method.

# Simulation results

The proposed method was performed based on HM4.0rc1 software under common test condition [2]. The experiments are conducted on the platform of Intel Core i7 CPU 990X @ 3.46GHz with 24GB RAM. The method is cross-checked by Sonly (JCTVC-G870).

Table 2 shows the performance of the proposed method.

Table 2 : Performance of the proposed method.

# Conclusions

The proposed intra prediction mode mapping method simplifies 'candModeList' derivation process without visible coding loss. We suggest the simplified method to be adopted into the HM.

# Patent rights declaration(s)

ETRI (Electronics and Telecommunications Research Institute) may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

- JCT-VC, “High Efficiency Video Coding (HEVC) text specification Working Draft 4”, JCTVC-F803, JCT-VC Meeting, Torino, July 2011.

- Frank Bossen, "Common test conditions and software reference configurations," JCTVC-F900, July 2011.

# Annex - Decoding process (Based on WD4_d5)

8.3.1 	Derivation process for luma intra prediction mode

...

Table 8-1 – Specification of intraPredModeNum

| log2PUSize | intraPredModeNum |
| --- | --- |
| 2 | 18 |
| 3 | 35 |
| 4 | 35 |
| 5 | 35 |
| 6 | 4 |

Table 8-3 specifies the mapping table used for converting the number of intra prediction modes.

Table 8-3 – Specification of mapIntraPredMode4 and mapIntraPredMode10

| value | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| mapIntraPredMode4[ value ] | 0 | 1 | 2 | 3 | 3 | 3 | 1 | 3 | 2 | 2 | 3 | 3 | 1 | 1 | 3 | 3 | 2 |  |
| mapIntraPredMode10[ value ] | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 3 | 3 | 3 | 3 | 3 | 3 | 3 |  |
| value | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 |
| mapIntraPredMode4[ value ] | 2 | 3 | 3 | 3 | 3 | 1 | 1 | 1 | 3 | 3 | 3 | 3 | 2 | 2 | 2 | 2 | 3 | 3 |
| mapIntraPredMode10[ value ] | 3 | 3 | 4 | 5 | 5 | 1 | 1 | 6 | 6 | 7 | 4 | 8 | 8 | 2 | 2 | 9 | 9 | 3 |

...

- The derivation process for neighbouring treeblocks specified in subclause XXX with ( xB,  yB ) given as input and the output is assigned to tbAddrA and tbAddrB specifying the treeblock addresses of treeblocks covering ( xBA,  yBA ) and ( xBB, yBB ) respectively where ( xBA,  yBA ) is set equal to ( xB-1,  yB ) and ( xBB,  yBB ) is set equal to ( xB,  yB-1 ).

- For N being either replaced A or B, the variables intraPredModeN are derived as follows.

- If the treeblock with address tbAddrN is not available, intraPredModeN is set equal to -1

- Otherwise, if the coding unit covering ( xBN,  yBN ) is not coded as intra mode, intraPredModeN is set equal to Intra_DC,

- Otherwise, intraPredModeN is set equal to IntraPredMode[ xBN ][ yBN ], where IntraPredMode is the variable array assigned to the coding unit covering the luma location ( xBN, yBN ).

- For N being either replaced A or B, the variables candIntraPredModeN are derived as follows.

- If intraPredModeN is greater than or equal to intraPredModeNum

- If intraPredModeNum is equal to 4 then candIntraPredModeN is set equal to mapIntraPredMode4[ intraPredModeN ]

- Otherwise candIntraPredModeN is set equal to mapIntraPredMode10[ intraPredModeN ].

- candIntraPredModeN is set equal to Intra_Planar

Otherwise, candIntraPredModeN is set equal to intraPredModeN