| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G419 |
| --- | --- |

| Title: | Inconsistency of intra LM mode between HM and WD |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jinho Lee, Sung-Chang Lim, Hui Yong Kim, Jin Soo Choi. Realistic Media Research Team, ETRI 218 Gajeongno, Yuseong-gu Daejeon, South Korea | Tel: Email: | +82-42-860-5656 jinosoul@etri.re.kr sclim@etri.re.kr hykim5@etri.re.kr jschoi@etri.re.kr |
| Source: | ETRI (Electronics and Telecommunications Research Institute) |  |  |

_____________________________

# Abstract

This contribution reports inconsistency of intra LM mode between HM and WD. In the reference sample substitution process of WD 4.0, one neighboring row and one neighboring column are padded by available samples. Intra LM mode uses one row and 2nd left column of reconstructed luma blocks. Therefore, if left block is unavailable, LM mode cannot use 2nd left column. Currently, WD4.0 has no description for that case, and HM4.0 uses 1st left column instead of 2nd left column in LM mode. In this contribution, revised WD text is proposed to get rid of the inconsistency between HM and WD. In addition, a simplified LM mode is proposed to keep consistency of padding process. It is reported that the average BD-rate loss of the proposed method is 0.1%, 0.8%, and 0.7% for Y, U, and V in AI-HE, respectively.

# Introduction

In the padding process, i.e., reference sample substitution process of WD4.0 [1], availability checking of neighboring blocks is performed, and unavailable reference samples are padded by available neighboring samples. Therefore, all the reference samples illustrated in Fig. 1 (a) becomes available, and all the intra prediction modes can be performed without availability checking of neighboring blocks.

In WD4.0, intra LM mode uses reconstructed luma samples to predict a chroma block. For the left neighboring samples of a luma block, 2nd column is used as illustrated in Fig. 1 (b).

However, when the left block is unavailable, the 2nd left column also becomes unavailable since there is no padding process for the 2nd left column. There is no description with regard to this issue in WD4.0, and HM4.0 uses 1st left column instead of 2nd left column since the 1st left column is always available by the reference sample padding process.

(a) Reference sample substitution              (b) Intra LM mode

Fig. 1. Reference sample substitution and intra LM mode in WD4.0

# Modification of WD 4.0

The mismatch problem between HM and WD can be solved by modifying WD to fit in HM. For this solution, the reference sample padding process and Intra_FromLuma prediction should be revised. In the reference sample padding process, the 1st left column of reconstructed luma block should be replaced by the 2nd left column when intra prediction mode is equal to Intra_FromLuma.

In addition, 'recSamplesL' of LM prediction should be split into neighboring samples and inner samples of a block as follows:

pY’[ x, y ] = ( pL [ 2x-1, 2y+1 ] + 2*pL [ 2x, 2y+1 ] + pL [ 2x+1, 2y+1 ] + 2 ) >> 2, with x=0..nS-1, y = -1

pY’[ x, y ] = ( pL [ 2x+1, 2y ] + pL [ 2x+1, 2y+1 ] ) >> 1, with x=-1, y = 0..nS-1

pY’[ x, y ] = ( recSamplesL[ 2x, 2y ] + recSamplesL[ 2x, 2y+1 ] ) >> 1, with x=0..nS-1, y = 0..nS-1

where, pL[ x, y ] and recSamplesL[ x, y ] denote neighboring sample and inner sample of reconstructed luma block, respectively. More details of WD text are described in Annex (8.1).

# Additional issue on consistency of padding process

By current padding process of HM4.0, availability checking of neighboring blocks is not required in the intra prediction process except LM mode. Currently, LM mode requires different padding process for luma block from the conventional method since the 2nd left column is used as shown in Fig. 1. (b).

To have consistent padding process, a simplified method of LM mode is proposed. The proposed method uses 1st left column rather than 2nd left column as illustrated in Fig. 2. Therefore, padding process for luma block is not required in LM mode since the reference samples of reconstructed luma block are already padded. However, the coding efficiency of intra LM mode would be degraded, since chroma sampling ratio is not considered in this method.

Fig. 2. Proposed intra LM mode

# Simulation result

The proposed method was performed based on HM4.0rc1 software under common test condition [2]. The simulation is conducted on the platform of Intel Core i7 CPU 990X @ 3.46GHz with 24GB RAM. And the proposed method is cross-checked by Samsung (JCTVC-G886).

Table 1 shows the coding efficiency of the proposed method. And, the coding efficiency of LM mode is represented in JCTVC-G511 [3] as shown in Table 2. The loss of the proposed method is negligible comparing to the performance of LM mode itself.

Table 1. Performance of the proposed method

Table 2. Performance of LM mode (JCTVC-G511)

Table 3 shows the coding efficiency of the proposed method based on HM4.0 with alpha 7bit-bugfix version. This simulation is cross-checked by Sony (JCTVC-G1037).

Table 3. Performance of the proposed method (HM4.0 with bugfix)

# Conclusion

In this contribution, in order to get rid of the inconsistency of intra LM mode between HM and WD, a modified WD text is proposed. In addition, a change of the neighboring left reference sample position of luma block in LM mode is proposed to keep consistency of padding process. About 0.1% of luma BD-rate increase and 0.7% of chroma BD-rate increase are obtained by the proposed method.

# Patent rights declaration(s)

ETRI (Electronics and Telecommunications Research Institute) may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

- JCT-VC, “High Efficiency Video Coding (HEVC) text specification Working Draft 4”, JCTVC-F803, JCT-VC Meeting, Torino, July 2011.

- Frank Bossen, "Common test conditions and software reference configurations," JCTVC-F900, JCT-VC Meeting, Torino, July 2011.

- Kazushi Sato, “Performance Evaluation of Luma-based Chroma Intra Prediction”, JCTVC-G511, JCT-VC Meeting, Geneva, November 2011.

# Annex - Decoding process (Based on WD4_d5)

## Modification of WD4.0 to solve mismatch between HM and WD

- 8.3.3.1.1  Reference sample substitution process for intra sample prediction

Inputs to this process are the reference samples p[ x, y ] with x = -1, y = -1..nS*2-1 and x = 0..nS*2-1, y = -1 for intra sample prediction.

If intra prediction mode is equal to Intra_FromLuma and p[x, y] is luma reconstructed sample, p[x-1, y] is assigned to p[x, y] with x=-1, y=0..nS*2-1.

Outputs of this process are the modified reference samples p[ x, y ] with x = -1, y = -1..nS*2-1 and x = 0..nS*2-1, y = -1 for intra sample prediction.

The values of the samples p[ x, y ] with x = -1, y = -1..nS*2-1 and x = 0..nS*2-1, y = -1 are modified as follows:

- If all samples p[ x, y ] with x = -1, y = -1..nS*2-1 and x = 0..nS*2-1, y = -1 are marked as “not available for intra prediction,” the value ( 1 << ( BitDepthY - 1 ) ) is substituted for the values of all samples p[ x, y ].

- Otherwise (at least one but not all samples p[ x, y ] are marked as “not available for intra prediction”), the following steps are performed for each sample p[ x0, y0 ] with x0 = -1, y0 = -1..nS*2-1 and x0 = 0..nS*2-1, y0 = -1 marked as "not available for intra prediction":

- Let the variables q and r be initially set to -1.

- The variable q is modified as follows:

  - If x0 is equal to -1, a reference sample p[ x, y ] marked as “available for intra prediction” is identified by searching sequentially among p[ x, y ] starting from x = -1, y = y0+1 to x = -1, y = nS*2-1. As soon as a sample marked as “available for intra prediction” is found, the search is terminated and the value of p[ x, y ] is assigned to q. [Ed.: (WJ) is there any elegant way rather than searching?]

  - Otherwise (x0 is not equal to -1), a sample marked as “available for intra prediction” is identified among the samples p[ x, y ] by searching sequentially starting from x = x0-1, y = -1 to x = -1, y = -1. When a sample marked as “available for intra prediction” is not found among p[ x, y ] with x = x0-1..-1, y = -1, the search is continued among p[ x, y ] sequentially starting from x = -1, y = 0 to x = -1, y = nS*2-1. As soon as a sample marked as “available for intra prediction” is found, the search is terminated and the value of p[ x, y ] is assigned to q. [Ed.: (WJ) is there any elegant way rather than searching?]

- The variable r is modified as follows:

  - If x0 is equal to -1, a sample marked as “available for intra prediction” is identified among the samples p[ x, y ] by searching sequentially starting from x = -1, y = y0-1 to x = -1, y = -1. When a sample marked as “available for intra prediction” is not found among p[ x, y ] with x = -1, y = y0-1..-1, the search is continued among p[ x, y ] sequentially starting from x = 0, y = -1 to x = nS*2-1, y = -1. As soon as a sample marked as “available for intra prediction” is found, the search is terminated and the value of p[ x, y ] is assigned to r. [Ed.: (WJ) is there any elegant way rather than searching?]

  - Otherwise (x0 is not equal to -1), a sample marked as “available for intra prediction” is identified among the samples p[ x, y ] by searching sequentially starting from x = x0+1, y = -1 to x = nS*2-1, y = -1. As soon as a sample marked as “available for intra prediction” is found, the search is terminated and the value of p[ x, y ] is assigned to r. [Ed.: (WJ) is there any elegant way rather than searching?]

- The value of the sample p[ x0, y0 ] is modified as follows:

  - If q is not equal to -1 and r is not equal to -1, the value ( ( q + r + 1 ) >> 1 ) is substituted for the value of p[ x0, y0 ].

  - Otherwise, if q is not equal to -1, the value of q is substituted for the value of p[ x0, y0 ].

  - Otherwise (q is equal to -1 and r is not equal to -1), the value of r is substituted for the value of p[ x0, y0 ].

All samples p[ x, y ] with x = -1, y = -1..nS*2-1 and x = 0..nS*2-1, y = -1 are marked as “available for intra prediction.”

...

- 8.3.3.1.8  Specification of Intra_FromLuma prediction mode

Inputs to this process are:

–	a sample location ( xB, yB ) specifying the top-left sample of the current block relative to the top-left sample of the current picture,

–	neighbouring samples of chroma p[ x, y ], with x, y = -1..2*nS-1,

–	neighbouring samples of luma pL[ x, y ], with x, y = -1..4*nS-1,

–	a variable nS specifying the prediction size.

Output of this process is:

–	predicted samples predSamples[ x, y ], with x, y =0..nS-1.

This intra prediction mode is invoked when intraPredMode is equal to 35.

The values of the prediction samples predSamples[ x, y ], with x, y = 0..nS-1, are derived as the following ordered steps:

- Variable k3 and the sample array pY’ are derived as:

k3 = Max( 0, BitDepthC + log2( nS ) – 14 )		(8-1)

pY’[ x, y ] = ( recSamplesL pL [ 2x-1, 2y+1 ] + 
2*recSamplesL pL [ 2x, 2y+1 ] + recSamplesL pL [ 2x+1, 2y+1 ] + 2 ) >> 2, with x=0..nS-1, y = -1	(8-2)

pY’[ x, y ] = ( pL [ 2x+1, 2y ] + pL [ 2x+1, 2y+1 ] ) >> 1, with x=-1, y = 0..nS-1

pY’[ x, y ] = ( recSamplesL[ 2x, 2y ] + recSamplesL[ 2x, 2y+1 ] ) >> 1, with x=-1 0..nS-1, y = 0..nS-1	(8-3)

- Variables L, C, LL, LC and k2 are derived as follows:

L = 		(8-4)

C = 		(8-5)

LL = 	(8-6)

LC = 	(8-7)

k2 = log2( (2*nS) >> k3 )		(8-8)

## Modification of intra LM mode (proposed method)

- 8.3.3.1.8  Specification of Intra_FromLuma prediction mode

Inputs to this process are:

–	a sample location ( xB, yB ) specifying the top-left sample of the current block relative to the top-left sample of the current picture,

–	neighbouring samples of chroma p[ x, y ], with x, y = -1..2*nS-1,

–	neighbouring samples of luma pL[ x, y ], with x, y = -1..4*nS-1,

–	a variable nS specifying the prediction size.

Output of this process is:

–	predicted samples predSamples[ x, y ], with x, y =0..nS-1.

This intra prediction mode is invoked when intraPredMode is equal to 35.

The values of the prediction samples predSamples[ x, y ], with x, y = 0..nS-1, are derived as the following ordered steps:

- Variable k3 and the sample array pY’ are derived as:

k3 = Max( 0, BitDepthC + log2( nS ) – 14 )		(8-9)

pY’[ x, y ] = ( recSamplesL pL [ 2x-1, 2y+1 ] + 
2*recSamplesL pL [ 2x, 2y+1 ] + recSamplesL pL [ 2x+1, 2y+1 ] + 2 ) >> 2, with x=0..nS-1, y = -1	(8-10)

pY’[ x, y ] = ( pL [ 2x+1, 2y ] + pL [ 2x+1, 2y+1 ] ) >> 1, with x=-1, y = 0..nS-1

pY’[ x, y ] = ( recSamplesL[ 2x, 2y ] + recSamplesL[ 2x, 2y+1 ] ) >> 1, with x=-1 0..nS-1, y = 0..nS-1	(8-11)

- Variables L, C, LL, LC and k2 are derived as follows:

L = 		(8-12)

C = 		(8-13)

LL = 	(8-14)

LC = 	(8-15)

k2 = log2( (2*nS) >> k3 )		(8-16)

...