| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, 21-30 November, 2011 | Document: JCTVC-G421 |
| --- | --- |

| Title: | CE9: Results of Experiments MRG_ID02, MRG_ID03, and MRG_ID04 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hui Yong Kim, *Kyung Yong Kim, *Sang Min Kim, *Gwang Hoon Park, Sung-Chang Lim, Jinho Lee ETRI, 218 Gajeong-Ro, Yuseong-Gu, Daejeon, 305-700, Korea *Media Lab., Kyung Hee University 1, Seochun, Kiheung, Youngin, Gyonggi, 446-701, Korea | Tel: Email: | +82-42-860-5069 {hykim5, sclim, jinosoul} @etri.re.kr +82-31-201-3680 {*kimky, *jephiros, *ghpark} @khu.ac.kr |
| Source: | ETRI and Kyung Hee University |  |  |

_____________________________

# Abstract

This contribution reports the results of CE9 experiment MRG_ID02, MRG_ID03, and MRG_ID04. Experimental results showed that an average loss of 0.1% ~ 0.2% for Low Delay test condition is observed.

# Experimental Description

In this experiment, the PUs spatially neighboring to the current PU are searched for a non-zero motion vector when the identical motion is recognized. More specifically, list0 MVs are searched for this purpose. If such a MV is found, it is used as mvL1Col. If the MVs are still the same, the prediction mode is inferred to uni-prediction rather than bi-prediction. This method corresponds to test MRG_ID02 of CE9 [1]. At 6th JCT-VC meeting, a participant suggested to, instead of looking for a MV that is non-zero, look for one that is different from what would otherwise be inferred. If no MV is found, the prediction mode is inferred to uni-prediction rather than bi-prediction. This method corresponds to MRG_ID03 of CE9. Related to MRG_ID02 and MRG_ID03 tests, the following two ways of handling the refIdxL1Col are investigated: 1) keep the refIdxL1Col and search for such a MV that the reference picture of which is the same as the one pointed by refIdxL1Col, 2) ignore the original refIdxL1Col, search for a MV, then assign the list1 reference index pointing to the reference picture of the found MV as refIdxL1Col. These two variations are identified by “A” and “B”, respectively.

Combination of the MRG_ID02 and MRG_ID03 are tested. These combinatory tests correspond to test MRG_ID04.

# Simulation results

Common test conditions specified in JCTVC-F900 [2] is used for the evaluation. And HM4.0 S/W with the fix of bug #203 (HM 4.0-dev-bugfix (r1358)) is used as the anchor.

Table 1. Simulation result of experiment MRG_ID02_A

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.1% |
| Class B | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
|  | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 102% |  |  | 102% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.1% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% |
| Class C | 0.1% | -0.1% | 0.0% | 0.1% | 0.1% | -0.1% |
| Class D | 0.1% | 0.7% | 0.3% | 0.2% | -0.1% | -0.1% |
| Class E | 0.3% | 0.5% | 0.6% | 0.2% | 0.4% | 0.5% |
| Overall | 0.2% | 0.2% | 0.2% | 0.1% | 0.1% | 0.1% |
|  | 0.2% | 0.3% | 0.1% | 0.1% | 0.1% | 0.1% |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 101% |  |  | 98% |  |  |

Table 2. Simulation result of experiment MRG_ID02_B

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.1% |
| Class B | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 102% |  |  | 102% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.1% | 0.1% | 0.3% | 0.1% | 0.1% | 0.0% |
| Class C | 0.1% | 0.0% | -0.1% | 0.1% | 0.1% | 0.0% |
| Class D | 0.1% | 0.1% | -0.3% | 0.1% | 0.0% | -0.3% |
| Class E | 0.3% | 0.4% | 1.3% | 0.2% | 0.6% | 0.4% |
| Overall | 0.1% | 0.1% | 0.2% | 0.1% | 0.2% | 0.0% |
|  | 0.1% | 0.1% | 0.2% | 0.1% | 0.1% | 0.0% |
| Enc Time[%] | 101% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |

Table 3. Simulation result of experiment MRG_ID03_A

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.1% | -0.1% | 0.0% | 0.1% | 0.0% |
| Class B | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 103% |  |  | 102% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.1% | 0.1% | -0.2% | 0.1% | 0.0% | 0.1% |
| Class C | 0.1% | 0.3% | 0.1% | 0.1% | 0.3% | -0.2% |
| Class D | 0.2% | 0.4% | 0.2% | 0.1% | 0.0% | 0.3% |
| Class E | 0.4% | 0.4% | 1.4% | 0.2% | 0.5% | 0.4% |
| Overall | 0.2% | 0.3% | 0.3% | 0.1% | 0.2% | 0.1% |
|  | 0.2% | 0.2% | 0.3% | 0.1% | 0.1% | 0.1% |
| Enc Time[%] | 99% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 4. Simulation result of experiment MRG_ID03_B

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.1% | 0.0% | 0.0% | 0.1% | 0.1% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 103% |  |  | 102% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.1% | 0.0% | 0.0% | 0.0% | 0.1% | -0.1% |
| Class C | 0.1% | 0.0% | -0.1% | 0.1% | 0.1% | -0.2% |
| Class D | 0.1% | 0.4% | 0.1% | 0.1% | 0.2% | 0.4% |
| Class E | 0.4% | 0.7% | 0.8% | 0.3% | 0.5% | 0.2% |
| Overall | 0.2% | 0.2% | 0.1% | 0.1% | 0.2% | 0.1% |
|  | 0.2% | 0.2% | 0.1% | 0.1% | 0.2% | 0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |

Table 5. Simulation result of experiment MRG_ID04_A

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
| Class C | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.1% | -0.2% | 0.0% | 0.1% | 0.0% | -0.1% |
| Class C | 0.1% | 0.0% | 0.2% | 0.1% | 0.2% | -0.1% |
| Class D | 0.2% | -0.1% | 0.5% | 0.2% | 0.5% | 0.1% |
| Class E | 0.3% | 0.7% | 1.2% | 0.3% | 0.8% | 0.4% |
| Overall | 0.2% | 0.0% | 0.4% | 0.1% | 0.3% | 0.0% |
|  | 0.2% | 0.0% | 0.3% | 0.1% | 0.3% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 98% |  |  | 96% |  |  |

Table 6. Simulation result of experiment MRG_ID04_B

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.1% | -0.1% | 0.0% | 0.1% | 0.1% |
| Class B | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | -0.1% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.1% | -0.2% | -0.1% | 0.0% | 0.0% | -0.2% |
| Class C | 0.1% | 0.2% | 0.1% | 0.1% | 0.1% | -0.2% |
| Class D | 0.1% | 0.2% | 0.2% | 0.1% | 0.3% | 0.0% |
| Class E | 0.4% | 0.8% | 0.9% | 0.2% | 0.5% | 0.0% |
| Overall | 0.2% | 0.2% | 0.2% | 0.1% | 0.2% | -0.1% |
|  | 0.2% | 0.2% | 0.2% | 0.1% | 0.1% | -0.1% |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 98% |  |  | 97% |  |  |

# Conclusions

This contribution reports the results of CE9 experiments MRG_ID02, MRG_ID03, and MRG_ID04.

# Reference

[1]	B. Bross, “Core Experiment 9: MV Coding and Skip/Merge Operations”, JCT-VC Document, JCTVC-F909, Torino, July 2011.

[2]	F. Bossen, “Common test conditions and software reference configurations,” JCT-VC Document, JCTVC-F900, Torino, July 2011.

# Patent rights declaration(s)

ETRI and Kyung Hee University may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).