| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, 21-30 November, 2011 | Document: JCTVC-G422 |
| --- | --- |

| Title: | CE9: Cross-check for MRG_ID01 (JCTVC-G439) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Hui Yong Kim, Sung-Chang Lim, *Kyung Yong Kim, *Sang Min Kim, *Gwang Hoon Park, and Jinho Lee ETRI, 218 Gajeong-Ro, Yuseong-Gu, Daejeon, 305-700, Korea *Media Lab., Kyung Hee University 1, Seochun, Kiheung, Youngin, Gyonggi, 446-701, Korea | Tel: Email: | +82-42-860-5069 {hykim5, sclim, jinosoul} @etri.re.kr +82-31-201-3680 {*kimky, *jephiros, *ghpark} @khu.ac.kr |
| Source: | ETRI and Kyung Hee University |  |  |

_____________________________

# Abstract

This contribution reports cross-check results for CE9 experiment MRG_ID01. The R-D performance is perfectly matched with the data provided by Toshiba.

# Algorithm description

For variation A of MRG_ID01 in CE9 [1], if the two reference frames for the collocated MV candidates are identical, collocated picture for L1 motion vector is changed from RefPicList1[0] to RefPicList1[1]. And, for variation B of MRG_ID01, the same derivation process as the above is performed if both reference frames are identical and MVs of the collocated merge candidates are identical.

# Simulation results

Common test conditions specified in JCTVC-F900 [2] is used for the evaluation. HM4.0 S/W with the fix of bug #203 (HM 4.0-dev-bugfix (r1358)) is used as the anchor.

Table 1. Simulation result of MRG_ID01_A

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.2% | -0.1% | 0.0% | 0.1% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.1% | -0.1% | 0.1% | 0.1% | 0.0% | 0.1% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 110% |  |  | 107% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.3% | 0.1% | 0.3% | 0.2% | 0.2% | 0.2% |
| Class C | 0.5% | 0.4% | 0.5% | 0.5% | 0.6% | 0.4% |
| Class D | 0.2% | 0.5% | 0.2% | 0.3% | 0.1% | 0.2% |
| Class E | 0.3% | 0.2% | 0.9% | 0.3% | 0.6% | 0.1% |
| Overall | 0.3% | 0.3% | 0.4% | 0.3% | 0.3% | 0.2% |
|  | 0.3% | 0.2% | 0.4% | 0.3% | 0.3% | 0.2% |
| Enc Time[%] | 110% |  |  | 108% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |

Table 2. Simulation result of MRG_ID01_B

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | -0.1% | 0.0% | 0.1% | 0.1% |
| Class B | 0.0% | 0.0% | 0.1% | 0.0% | -0.1% | 0.1% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | -0.1% | 0.1% | 0.1% | 0.0% | 0.0% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 110% |  |  | 107% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.1% | -0.1% | 0.2% | 0.0% | 0.1% | 0.2% |
| Class C | 0.2% | 0.0% | 0.1% | 0.2% | 0.3% | 0.2% |
| Class D | 0.2% | 0.3% | 0.3% | 0.1% | 0.0% | 0.0% |
| Class E | 0.2% | 0.5% | 0.5% | 0.1% | 0.0% | -0.1% |
| Overall | 0.2% | 0.2% | 0.2% | 0.1% | 0.1% | 0.1% |
|  | 0.2% | 0.1% | 0.2% | 0.1% | 0.1% | 0.0% |
| Enc Time[%] | 110% |  |  | 108% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |

# Conclusion

The R-D performance is perfectly matched with those provided by Toshiba.

# Reference

[1]	B. Bross, et al., “Core Experiment 9: MV Coding and Skip/Merge Operations”, JCT-VC Document, JCTVC-F909, Torino, July 2011.

[2]	F. Bossen, “Common test conditions and software reference configurations”, JCT-VC Document, JCTVC-F900, Torino, July 2011.